2540 B. Total Solids Dried at 103–105°C

1. General Discussion

a. Principle: A well-mixed sample is evaporated in a weighed dish and dried to constant

weight in an oven at 103 to 105°C. The increase in weight over that of the empty dish represents

the total solids. The results may not represent the weight of actual dissolved and suspended

solids in wastewater samples (see above).

b. Interferences: Highly mineralized water with a significant concentration of calcium,

magnesium, chloride, and/or sulfate may be hygroscopic and require prolonged drying, proper

desiccation, and rapid weighing. Exclude large, floating particles or submerged agglomerates of

nonhomogeneous materials from the sample if it is determined that their inclusion is not desired

in the final result. Disperse visible floating oil and grease with a blender before withdrawing a

sample portion for analysis. Because excessive residue in the dish may form a water-trapping

crust, limit sample to no more than 200 mg residue (see Section 2540A.2).

2. Apparatus

a. Evaporating dishes: Dishes of 100-mL capacity made of one of the following materials:

1) Porcelain, 90-mm diam.

2) Platinum—Generally satisfactory for all purposes.

3) High-silica glass.#(44)*

b. Muffle furnace for operation at 550°C.

c. Steam bath.

d. Desiccator, provided with a desiccant containing a color indicator of moisture

concentration or an instrumental indicator.

e. Drying oven, for operation at 103 to 105°C.

f. Analytical balance, capable of weighing to 0.1 mg.

g. Magnetic stirrer with TFE stirring bar.

h. Wide-bore pipets.#(45)†

i. Graduated cylinder.

j. Low-form beaker.#(46)‡

3. Procedure

a. Preparation of evaporating dish: If volatile solids are to be measured ignite clean

evaporating dish at 550°C for 1 h in a muffle furnace. If only total solids are to be measured,

heat clean dish to 103 to 105°C for 1 h. Store and cool dish in desiccator until needed. Weigh

immediately before use.

b. Sample analysis: Choose a sample volume that will yield a residue between 2.5 and 200

mg. Pipet a measured volume of well-mixed sample, during mixing, to a preweighed dish. For

homogeneous samples, pipet from the approximate midpoint of the container but not in the

vortex. Choose a point both middepth and midway between wall and vortex. Evaporate to dryness on a steam bath or in a drying oven. Stir sample with a magnetic stirrer during transfer.

If necessary, add successive sample portions to the same dish after evaporation. When

evaporating in a drying oven, lower temperature to approximately 2°C below boiling to prevent

splattering. Dry evaporated sample for at least 1 h in an oven at 103 to 105°C, cool dish in

desiccator to balance temperature, and weigh. Repeat cycle of drying, cooling, desiccating, and

weighing until a constant weight is obtained, or until weight change is less than 4% of previous

weight or 0.5 mg, whichever is less. When weighing dried sample, be alert to change in weight

due to air exposure and/or sample degradation. Analyze at least 10% of all samples in duplicate.

Duplicate determinations should agree within 5% of their average weight.

4. Calculation

where:

A = weight of dried residue + dish, mg, and

B = weight of dish, mg.

5. Precision

Single-laboratory duplicate analyses of 41 samples of water and wastewater were made with

a standard deviation of differences of 6.0 mg/L.

2540 C. Total Dissolved Solids Dried at 180°C

1. General Discussion

a. Principle: A well-mixed sample is filtered through a standard glass fiber filter, and the

filtrate is evaporated to dryness in a weighed dish and dried to constant weight at 180°C. The

increase in dish weight represents the total dissolved solids. This procedure may be used for

drying at other temperatures.

The results may not agree with the theoretical value for solids calculated from chemical

analysis of sample (see above). Approximate methods for correlating chemical analysis with

dissolved solids are available.1 The filtrate from the total suspended solids determination

(Section 2540D) may be used for determination of total dissolved solids.

b. Interferences: See Section 2540A.2 and Section 2540B.1. Highly mineralized waters with a considerable calcium, magnesium, chloride, and/or sulfate content may be hygroscopic

and require prolonged drying, proper desiccation, and rapid weighing. Samples high in

bicarbonate require careful and possibly prolonged drying at 180°C to insure complete

conversion of bicarbonate to carbonate. Because excessive residue in the dish may form a

water-trapping crust, limit sample to no more than 200 mg residue.

2. Apparatus

Apparatus listed in Section 2540B.2a - h is required, and in addition:

a. Glass-fiber filter disks#(47)* without organic binder.

b. Filtration apparatus: One of the following, suitable for the filter disk selected:

1) Membrane filter funnel.

2) Gooch crucible, 25-mL to 40-mL capacity, with Gooch crucible adapter.

3) Filtration apparatus with reservoir and coarse (40- to 60-μm) fritted disk as filter

support.#(48)†

c. Suction flask, of sufficient capacity for sample size selected.

d. Drying oven, for operation at 180 ± 2°C.

3. Procedure

a. Preparation of glass-fiber filter disk: If pre-prepared glass fiber filter disks are used,

eliminate this step. Insert disk with wrinkled side up into filtration apparatus. Apply vacuum and

wash disk with three successive 20-mL volumes of reagent-grade water. Continue suction to

remove all traces of water. Discard washings.

b. Preparation of evaporating dish: If volatile solids are to be measured, ignite cleaned

evaporating dish at 550°C for 1 h in a muffle furnace. If only total dissolved solids are to be

measured, heat clean dish to 180 ± 2°C for 1 h in an oven. Store in desiccator until needed.

Weigh immediately before use.

c. Selection of filter and sample sizes: Choose sample volume to yield between 2.5 and 200

mg dried residue. If more than 10 min are required to complete filtration, increase filter size or

decrease sample volume.

d. Sample analysis: Stir sample with a magnetic stirrer and pipet a measured volume onto a

glass-fiber filter with applied vacuum. Wash with three successive 10-mL volumes of

reagent-grade water, allowing complete drainage between washings, and continue suction for

about 3 min after filtration is complete. Transfer total filtrate (with washings) to a weighed

evaporating dish and evaporate to dryness on a steam bath or in a drying oven. If necessary, add

successive portions to the same dish after evaporation. Dry evaporated sample for at least 1 h in

an oven at 180 ± 2°C, cool in a desiccator to balance temperature, and weigh. Repeat drying

cycle of drying, cooling, desiccating, and weighing until a constant weight is obtained or until

weight change is less than 4% of previous weight or 0.5 mg, whichever is less. Analyze at least

10% of all samples in duplicate. Duplicate determinations should agree within 5% of their average weight. If volatile solids are to be determined, follow procedure in Section 2540E.

4. Calculation

where:

A = weight of dried residue + dish, mg, and

B = weight of dish, mg.

5. Precision

Single-laboratory analyses of 77 samples of a known of 293 mg/L were made with a

standard deviation of differences of 21.20 mg/L.

2540 D. Total Suspended Solids Dried at 103–105°C

1. General Discussion

a. Principle: A well-mixed sample is filtered through a weighed standard glass-fiber filter

and the residue retained on the filter is dried to a constant weight at 103 to 105°C. The increase

in weight of the filter represents the total suspended solids. If the suspended material clogs the

filter and prolongs filtration, it may be necessary to increase the diameter of the filter or decrease

the sample volume. To obtain an estimate of total suspended solids, calculate the difference

between total dissolved solids and total solids.

b. Interferences: See Section 2540A.2 and Section 2540B.1. Exclude large floating particles

or submerged agglomerates of nonhomogeneous materials from the sample if it is determined

that their inclusion is not representative. Because excessive residue on the filter may form a water-entrapping crust, limit the sample size to that yielding no more than 200 mg residue. For

samples high in dissolved solids thoroughly wash the filter to ensure removal of dissolved

material. Prolonged filtration times resulting from filter clogging may produce high results

owing to increased colloidal materials captured on the clogged filter.

2. Apparatus

Apparatus listed in Section 2540B.2 and Section 2540C.2 is required, except for evaporating

dishes, steam bath, and 180°C drying oven. In addition:

Aluminum weighing dishes.

3. Procedure

a. Preparation of glass-fiber filter disk: If pre-prepared glass fiber filter disks are used,

eliminate this step. Insert disk with wrinkled side up in filtration apparatus. Apply vacuum and

wash disk with three successive 20-mL portions of reagent-grade water. Continue suction to

remove all traces of water, turn vacuum off, and discard washings. Remove filter from filtration

apparatus and transfer to an inert aluminum weighing dish. If a Gooch crucible is used, remove

crucible and filter combination. Dry in an oven at 103 to 105°C for 1 h. If volatile solids are to

be measured, ignite at 550°C for 15 min in a muffle furnace. Cool in desiccator to balance

temperature and weigh. Repeat cycle of drying or igniting, cooling, desiccating, and weighing

until a constant weight is obtained or until weight change is less than 4% of the previous

weighing or 0.5 mg, whichever is less. Store in desiccator until needed.

b. Selection of filter and sample sizes: Choose sample volume to yield between 2.5 and 200

mg dried residue. If volume filtered fails to meet minimum yield, increase sample volume up to 1

L. If complete filtration takes more than 10 min, increase filter diameter or decrease sample

volume.

c. Sample analysis: Assemble filtering apparatus and filter and begin suction. Wet filter with

a small volume of reagent-grade water to seat it. Stir sample with a magnetic stirrer at a speed to

shear larger particles, if practical, to obtain a more uniform (preferably homogeneous) particle

size. Centrifugal force may separate particles by size and density, resulting in poor precision

when point of sample withdrawal is varied. While stirring, pipet a measured volume onto the

seated glass-fiber filter. For homogeneous samples, pipet from the approximate midpoint of

container but not in vortex. Choose a point both middepth and midway between wall and vortex.

Wash filter with three successive 10-mL volumes of reagent-grade water, allowing complete

drainage between washings, and continue suction for about 3 min after filtration is complete.

Samples with high dissolved solids may require additional washings. Carefully remove filter

from filtration apparatus and transfer to an aluminum weighing dish as a support. Alternatively,

remove the crucible and filter combination from the crucible adapter if a Gooch crucible is used.

Dry for at least 1 h at 103 to 105°C in an oven, cool in a desiccator to balance temperature, and

weigh. Repeat the cycle of drying, cooling, desiccating, and weighing until a constant weight is

obtained or until the weight change is less than 4% of the previous weight or 0.5 mg, whichever

is less. Analyze at least 10% of all samples in duplicate. Duplicate determinations should agree within 5% of their average weight. If volatile solids are to be determined, treat the residue

according to 2540E.

4. Calculation

where:

A = weight of filter + dried residue, mg, and

B = weight of filter, mg.

5. Precision

The standard deviation was 5.2 mg/L (coefficient of variation 33%) at 15 mg/L, 24 mg/L

(10%) at 242 mg/L, and 13 mg/L (0.76%) at 1707 mg/L in studies by two analysts of four sets of

10 determinations each.

Single-laboratory duplicate analyses of 50 samples of water and wastewater were made with

a standard deviation of differences of 2.8 mg/L.

2540 E. Fixed and Volatile Solids Ignited at 550°C

1. General Discussion

a. Principle: The residue from Method B, C, or D is ignited to constant weight at 550°C.

The remaining solids represent the fixed total, dissolved, or suspended solids while the weight

lost on ignition is the volatile solids. The determination is useful in control of wastewater

treatment plant operation because it offers a rough approximation of the amount of organic

matter present in the solid fraction of wastewater, activated sludge, and industrial wastes.

b. Interferences: Negative errors in the volatile solids may be produced by loss of volatile

matter during drying. Determination of low concentrations of volatile solids in the presence of

high fixed solids concentrations may be subject to considerable error. In such cases, measure for

suspect volatile components by another test, for example, total organic carbon (Section 5310).

Highly alkaline residues may react with silica in sample or silica-containing crucibles.

2. Apparatus

See Section 2540B.2, Section 2540C.2, and Section 2540D.2.

3. Procedure

Ignite residue produced by Method 2540B, C, or D to constant weight in a muffle furnace at

a temperature of 550°C. Ignite a blank glass fiber filter along with samples. Have furnace up to

temperature before inserting sample. Usually, 15 to 20 min ignition are required for 200 mg

residue. However, more than one sample and/or heavier residues may overtax the furnace and

necessitate longer ignition times. Let dish or filter disk cool partially in air until most of the heat

has been dissipated. Transfer to a desiccator for final cooling in a dry atmosphere. Do not

overload desiccator. Weigh dish or disk as soon as it has cooled to balance temperature. Repeat

cycle of igniting, cooling, desiccating, and weighing until a constant weight is obtained or until

weight change is less than 4% or 0.5 mg, whichever is less. Analyze at least 10% of all samples

in duplicate. Duplicate determinations should agree within 5% of their average weight. Weight

loss of the blank filter is an indication of unsuitability of a particular brand or type of filter for

this analysis.

4. Calculation

where:

A = weight of residue + dish before ignition, mg,

B = weight of residue + dish or filter after ignition, mg, and

C = weight of dish or filter, mg.

5. Precision

The standard deviation was 11 mg/L at 170 mg/L volatile total solids in studies by three

laboratories on four samples and 10 replicates. Bias data on actual samples cannot be obtained.

2540 F. Settleable Solids

1. General Discussion

Settleable solids in surface and saline waters as well as domestic and industrial wastes may

be determined and reported on either a volume (mL/L) or a weight (mg/L) basis.

2. Apparatus

The volumetric test requires only an Imhoff cone. The gravimetric test requires all the

apparatus listed in Section 2540D.2 and a glass vessel with a minimum diameter of 9 cm.

3. Procedure

a. Volumetric: Fill an Imhoff cone to the 1-L mark with a well-mixed sample. Settle for 45

min, gently agitate sample near the sides of the cone with a rod or by spinning, settle 15 min

longer, and record volume of settleable solids in the cone as milliliters per liter. If the settled

matter contains pockets of liquid between large settled particles, estimate volume of these and

subtract from volume of settled solids. The practical lower limit of measurement depends on

sample composition and generally is in the range of 0.1 to 1.0 mL/L. Where a separation of

settleable and floating materials occurs, do not estimate the floating material as settleable matter.

Replicates usually are not required.

Where biological or chemical floc is present, the gravimetric method (3b) is preferred.

b. Gravimetric:

1) Determine total suspended solids as in Section 2540D.

2) Pour a well-mixed sample into a glass vessel of not less than 9 cm diam using not less

greater diameter and a larger volume of sample. Let stand quiescent for 1 h and, without

disturbing the settled or floating material, siphon 250 mL from center of container at a point

halfway between the surface of the settled material and the liquid surface. Determine total

suspended solids (milligrams per liter) of this supernatant liquor (Section 2540D). These are the

nonsettleable solids.

4. Calculation

mg settleable solids/L = mg total suspended solids/L − mg nonsettleable solids/L

5. Precision and Bias

Precision and bias data are not now available.

Part 3000 METALS

3010 INTRODUCTION

3010 A. General Discussion

1. Significance

The effects of metals in water and wastewater range from beneficial through troublesome to

dangerously toxic. Some metals are essential to plant and animal growth while others may

adversely affect water consumers, wastewater treatment systems, and receiving waters. The

benefits versus toxicity of some metals depend on their concentrations in waters.

2. Types of Methods

Preliminary treatment is often required to present the metals to the analytical methodology

in an appropriate form. Alternative methods for pretreatment of samples are presented in Section 3030.

Metals may be determined satisfactorily by a variety of methods, with the choice often

depending on the precision and sensitivity required. Part 3000 describes colorimetric methods as

well as instrumental methods, i.e., atomic absorption spectrometry, including flame,

electrothermal (furnace), hydride, and cold vapor techniques; flame photometry; inductively

coupled plasma emission spectrometry; inductively coupled plasma mass spectrometry, and

anodic stripping voltammetry. Flame atomic absorption methods generally are applicable at

moderate (0.1- to 10-mg/L) concentrations in clean and complex-matrix samples. Electrothermal

methods generally can increase sensitivity if matrix problems do not interfere. Inductively

coupled plasma emission techniques are applicable over a broad linear range and are especially

sensitive for refractory elements. Inductively coupled plasma mass spectrometry offers

significantly increased sensitivity for some elements (as low as 0.01 μg/L) in a variety of

environmental matrices. Flame photometry gives good results at higher concentrations for

several Group I and II elements. Anodic stripping offers high sensitivity for several elements in

relatively clean matrices. Colorimetric methods are applicable to specific metal determinations

where interferences are known not to compromise method accuracy; these methods may provide

speciation information for some metals. Table 3010:I lists the methods available in Part 3000 for

each metal.

3. Definition of Terms

a. Dissolved metals: Those metals in an unacidified sample that pass through a 0.45-μm

membrane filter.

b. Suspended metals: Those metals in an unacidified sample that are retained by a 0.45-μm

membrane filter.

c. Total metals: The concentration of metals determined in an unfiltered sample after

vigorous digestion, or the sum of the concentrations of metals in the dissolved and suspended

fractions. Note that total metals are defined operationally by the digestion procedure.

d. Acid-extractable metals: The concentration of metals in solution after treatment of an

unfiltered sample with hot dilute mineral acid. To determine either dissolved or suspended

metals, filter sample immediately after collection. Do not preserve with acid until after filtration.

3010 B. Sampling and Sample Preservation

Before collecting a sample, decide what fraction is to be analyzed (dissolved, suspended,

total, or acid-extractable). This decision will determine in part whether the sample is acidified

with or without filtration and the type of digestion required.

Serious errors may be introduced during sampling and storage because of contamination

from sampling device, failure to remove residues of previous samples from sample container,

and loss of metals by adsorption on and/or precipitation in sample container caused by failure to

acidify the sample properly.

1. Sample Containers

The best sample containers are made of quartz or TFE. Because these containers are

expensive, the preferred sample container is made of polypropylene or linear polyethylene with a

polyethylene cap. Borosilicate glass containers also may be used, but avoid soft glass containers

for samples containing metals in the microgram-per-liter range. Store samples for determination

of silver in light-absorbing containers. Use only containers and filters that have been acid rinsed.

2. Preservation

Preserve samples immediately after sampling by acidifying with concentrated nitric acid

(HNO3) to pH <2. Filter samples for dissolved metals before preserving (see Section 3030).

Usually 1.5 mL conc HNO3/L sample (or 3 mL 1 + 1 HNO3/L sample) is sufficient for

short-term preservation. For samples with high buffer capacity, increase amount of acid (5 mL

may be required for some alkaline or highly buffered samples). Use commercially available

high-purity acid#(65)* or prepare high-purity acid by sub-boiling distillation of acid.

After acidifying sample, preferably store it in a refrigerator at approximately 4°C to prevent

change in volume due to evaporation. Under these conditions, samples with metal concentrations

of several milligrams per liter are stable for up to 6 months (except mercury, for which the limit

is 5 weeks). For microgram-per-liter metal levels, analyze samples as soon as possible after

sample collection.

Alternatively, preserve samples for mercury analysis by adding 2 mL/L 20% (w/v) K2Cr2O7

solution (prepared in 1 + 1 HNO3). Store in a refrigerator not contaminated with mercury.

(CAUTION: Mercury concentrations may increase in samples stored in plastic bottles in

mercury-contaminated laboratories.)

3010 C. General Precautions

1. Sources of Contamination

Avoid introducing contaminating metals from containers, distilled water, or membrane

filters. Some plastic caps or cap liners may introduce metal contamination; for example, zinc has

been found in black bakelite-type screw caps as well as in many rubber and plastic products, and

cadmium has been found in plastic pipet tips. Lead is a ubiquitous contaminant in urban air and

dust.

2. Contaminant Removal

Thoroughly clean sample containers with a metal-free nonionic detergent solution, rinse

with tap water, soak in acid, and then rinse with metal-free water. For quartz, TFE, or glass

materials, use 1 + 1 HNO3, 1 + 1 HCl, or aqua regia (3 parts conc HCl + 1 part conc HNO3) for

soaking. For plastic material, use 1 + 1 HNO3 or 1 + 1 HCl. Reliable soaking conditions are 24 h

at 70°C. Chromic acid or chromium-free substitutes#(66)* may be used to remove organic

deposits from containers, but rinse containers thoroughly with water to remove traces of

chromium. Do not use chromic acid for plastic containers or if chromium is to be determined.

Always use metal-free water in analysis and reagent preparation (see Section 3111B.3c). In these

methods, the word ‘‘water’’ means metal-free water.

3. Airborne Contaminants

For analysis of microgram-per-liter concentrations of metals, airborne contaminants in the

form of volatile compounds, dust, soot, and aerosols present in laboratory air may become

significant. To avoid contamination use ‘‘clean laboratory’’ facilities such as commercially

available laminar-flow clean-air benches or custom-designed work stations and analyze blanks

that reflect the complete procedure.

3020 QUALITY ASSURANCE/QUALITY CONTROL

3020 A. Introduction

General information and recommendations for quality assurance (QA) and quality control

(QC) are provided in Section 1020 Quality Assurance, 1030 Data Quality, and 1040 Method

Development and Evaluation. This section discusses QA/QC requirements that are common to

the analytical methods presented in Part 3000. The requirements are recommended minimum

QA/QC activities; refer to individual methods and regulatory program requirements for

method-specific QA/QC requirements. Always consider the overall purpose of analyses. QA/QC

measures and substantiation for operational-control determinations may differ significantly from

those for determinations of trace metals at water quality criteria levels. Levels of trace metals in

environmental samples may be orders of magnitude lower than in potential sources of

contamination.

Use replicates of measurable concentration to establish precision and known-additions

recovery to determine bias. Use blanks, calibrations, control charts, known additions, standards,

and other ancillary measurement tools as appropriate. Provide adequate documentation and

record keeping to satisfy client requirements and performance criteria established by the

laboratory.

3020 B. Quality Control Practices

1. Initial Quality Control

a. Initial demonstration of capability: Verify analyst capability before analyzing any

samples and repeat periodically to demonstrate proficiency with the analytical method. Verify

that the method being used provides sufficient sensitivity for the purpose of the measurement.

Test analyst capability by analyzing at least four reagent water portions containing known

additions of the analyte of interest. Confirm proficiency by generating analytical results that

demonstrate precision and bias within acceptable limits representative of the analytical method.

b. Method detection limit (MDL): Before samples are analyzed, determine the MDL for each

analyte by the procedures of Section 1030, or other applicable procedure.1 Determine MDL at

least annually for each method and major matrix category. Verify MDL for a new analyst or

whenever instrument hardware or method operating conditions are modified. Analyze samples

for MDL determinations over a 3- to 5-d period to generate a realistic value. Preferably use

pooled data from several analysts rather than data from a single analyst.

c. Dynamic range (DR): Before using a new method, determine the dynamic range, i.e., the

concentration range over which a method has an increasing response (linear or second-order), for

each analyte by analyzing several standard solutions that bracket the range of interest. Each

standard measurement should be within 10% of the true value for acceptance into the DR

determination. Take measurements at both the low and high end of the calibration range to determine method suitability. Analytical instrumentation with curve-fitting features may allow

utilization of nonlinear instrument response.

2. Calibration

a. Initial calibration: Calibrate initially with a minimum of a blank and three calibration

standards of the analyte(s) of interest. Select calibration standards that bracket the expected

concentration of the sample and that are within the method’s dynamic range. The number of

calibration points depends on the width of the dynamic range and the shape of the calibration

curve. One calibration standard should be below the reporting limit for the method. As a general

rule, differences between calibration standard concentrations should not be greater than one

order of magnitude (i.e., 1, 10, 100, 1000). Apply linear or polynomial curve-fitting statistics, as

appropriate, for analysis of the concentration-instrument response relationship. The appropriate

linear or nonlinear correlation coefficient for standard concentration to instrument response

should be ≥ 0.995. Use initial calibration for quantitation of analyte concentration in samples.

Use calibration verification, ¶ b below, only for checks on the initial calibration and not for

sample quantitation. Repeat initial calibration daily and whenever calibration verification

acceptance criteria are not satisfied.

b. Calibration verification: Calibration verification is the periodic confirmation that

instrument response has not changed significantly from the initial calibration. Verify calibration

by analyzing a midpoint calibration standard (check standard) and calibration blank at the

beginning and end of a sample run, periodically during a run (normally after each set of ten

samples). A check standard determination outside 90 to 110% of the expected concentration

indicates a potential problem. If a check standard determination is outside 80 to 120% of the

expected concentration, immediately cease sample analyses and initiate corrective action. Repeat

initial calibration and sample determinations since the last acceptable calibration verification.

Use calculated control limits (Section 1020B) to provide better indications of system

performance and to provide tighter control limits.

c. Quality control sample: Analyze an externally generated quality control sample of known

concentration at least quarterly and whenever new calibration stock solutions are prepared.

Obtain this sample from a source external to the laboratory or prepare it from a source different

from those used to prepare working standards. Use to validate the laboratory’s working standards

both qualitatively and quantitatively.

3. Batch Quality Control

a. Method blank (MB): A method blank (also known as reagent blank) is a portion of reagent

water treated exactly as a sample, including exposure to all equipment, glassware, procedures,

and reagents. The MB is used to assess whether analytes or interference are present within the

analytical process or system. No analyte of interest should be present in the MB at a warning

level based on the end user’s requirements. Undertake immediate corrective action for MB

measurements above the MDL. Include a minimum of one MB with each set of 20 or fewer

samples. b. Laboratory-fortified blank (LFB): The laboratory-fortified blank (also known as blank spike) is a method blank that has been fortified with a known concentration of analyte. It is used to evaluate ongoing laboratory performance and analyte recovery in a clean matrix. Prepare

fortified concentrations approximating the midpoint of the calibration curve or lower with stock

solutions prepared from a source different from those used to develop working standards.

Calculate percent recovery, plot control charts, and determine control limits (Section 1020B) for

these measurements. Ensure that the LFB meets performance criteria for the method. Establish

corrective actions to be taken in the event that LFB does not satisfy acceptance criteria. Include a

minimum of one LFB with each set of 20 or fewer samples.

c. Duplicates: Use duplicate samples of measurable concentration to measure precision of

the analytical process. Randomly select routine samples to be analyzed twice. Process duplicate

sample independently through entire sample preparation and analytical process. Include a

minimum of one duplicate for each matrix type with each set of 20 or fewer samples.

d. Laboratory-fortified matrix (LFM)/laboratory-fortified matrix duplicate: Use LFM (also

known as matrix spike) and LFM duplicate to evaluate the bias and precision, respectively, of

the method as influenced by a specific matrix. Prepare by adding a known concentration of

analytes to a randomly selected routing sample. Prepare addition concentrations to

approximately double the concentration present in the original sample. If necessary, dilute

sample to bring the measurement within the established calibration curve. Limit addition volume

to 5% or less of sample volume. Calculate percent recovery and relative percent difference, plot

control charts, and determine control limits (Section 1020B). Ensure that performance criteria for

the method are satisfied. Process fortified samples independently through entire sample

preparation and analytical process. Include a minimum of one LFM/LFM duplicate with each set

of 20 or fewer samples.

e. Method of known additions: To analyze a new or unfamiliar matrix, use the method of

known additions (Section 1020B) to demonstrate freedom from interference before reporting

concentration data for the analyte. Verify absence of interferences by analyzing such samples

undiluted and in a 1:10 dilution; results should be within 10% of each other. Limit

known-addition volume to 10% or less of the sample volume.

3030 PRELIMINARY TREATMENT OF SAMPLES*#(67)

3030 A. Introduction

Samples containing particulates or organic material generally require pretreatment before

spectroscopic analysis. ‘‘Total metals’’ includes all metals, inorganically and organically bound,

both dissolved and particulate. Colorless, transparent samples (primarily drinking water) having

a turbidity of <1 NTU, no odor, and single phase may be analyzed directly by atomic absorption

spectroscopy (flame or electrothermal vaporization) or inductively coupled plasma spectroscopy

(atomic emission or mass spectrometry) for total metals without digestion. For further

verification or if changes in existing matrices are encountered, compare digested and undigested

samples to ensure comparable results. On collection, acidify such samples to pH <2 with conc

nitric acid (1.5 mL HNO3/L is usually adequate for drinking water) and analyze directly. Digest

all other samples before determining total metals. To analyze for dissolved metals, filter sample,

acidify filtrate, and store until analyses can be performed. To determine suspended metals, filter

sample, digest filter and the material on it, and analyze. To determine acid-extractable metals,

extract metals as indicated in Section 3030E through K and analyze extract.

This section describes general pretreatment for samples in which metals are to be determined

according to Section 3110 through 3500-Zn with several exceptions. The special digestion

techniques for mercury are given in Section 3112B.4b and c, and those for arsenic and selenium

in Section 3114 and Section 3500-Se.

Take care not to introduce metals into samples during preliminary treatment. During

pretreatment avoid contact with rubber, metal-based paints, cigarette smoke, paper tissues, and

all metal products including those made of stainless steel, galvanized metal, and brass.

Conventional fume hoods can contribute significantly to sample contamination, particularly

during acid digestion in open containers. Keep vessels covered with watch glasses and turn

spouts away from incoming air to reduce airborne contamination. Plastic pipet tips often are

contaminated with copper, iron, zinc, and cadmium; before use soak in 2N HCl or HNO3 for

several days and rinse with deionized water. Avoid using colored plastics, which can contain

metals. Use certified metal-free plastic containers and pipet tips when possible. Avoid using

glass if analyzing for aluminum or silica.

Use metal-free water (see Section 3111B.3c) for all operations. Check reagent-grade acids

used for preservation, extraction, and digestion for purity. If excessive metal concentrations are

found, purify the acids by distillation or use ultra-pure acids. Inductively coupled plasma mass

spectrometry (ICP-MS) may require use of ultra-pure acids and reagents to avoid measurable

contamination. Process blanks through all digestion and filtration steps and evaluate blank

results relative to corresponding sample results. Either apply corrections to sample results or take

other corrective actions as necessary or appropriate.

3030 B. Filtration for Dissolved and Suspended Metals

1. Filtration Procedures

If dissolved or suspended metals (see Section 3010A) are to be determined, filter sample at

time of collection using a preconditioned plastic filtering device with either vacuum or pressure,

containing a filter support of plastic or fluorocarbon, through a prewashed ungridded 0.4- to

0.45-μm-pore-diam membrane filter (polycarbonate or cellulose esters). Before use filter a blank

consisting of metal-free (deionized) water to insure freedom from contamination. Precondition

filter and filter device by rinsing with 50 mL deionized water. If the filter blank contains

significant metals concentrations, soak membrane filters in approximately 0.5N HCl or 1N

HNO3 (recommended for electrothermal and ICP-MS analyses) and rinse with deionized water

before use.

Before filtering, centrifuge highly turbid samples in acid-washed fluorocarbon or

high-density plastic tubes to reduce loading on filters. Stirred, pressure filter units foul less

readily than vacuum filters; filter at a pressure of 70 to 130 kPa. After filtration acidify filtrate to

pH 2 with conc HNO3 and store until analyses can be performed. If a precipitate forms on

acidification, digest acidified filtrate before analysis as directed (see Section 3030E). Retain

filter and digest it for direct determination of suspended metals.

If it is not possible to field-filter the sample without contaminating it, obtain sample in an

‘‘unpreserved’’ bottle as above and promptly cool to 4°C. Do not acid-preserve the sample.

Then, without delay, filter sample under cleaner conditions in the laboratory.

Test pH of a portion of aqueous sample upon receipt in the laboratory to ensure that the

sample has been properly filtered and acid-preserved.1

NOTE: Different filters display different sorption and filtration characteristics2; for trace

analysis, test filter and filtration sytem to verify complete recovery of metals.

If suspended metals (see Section 3010A) are to be determined, filter sample as above for

dissolved metals, but do not centrifuge before filtration. Retain filter and digest it for direct

determination of suspended metals. Record sample volume filtered and include a filter in

determination of the blank.

CAUTION: Do not use perchloric acid to digest membrane filters. (See Section 3030H for

more information on handling HClO4).

3030 C. Treatment for Acid-Extractable Metals

Extractable metals (see Section 3010A) are lightly adsorbed on particulate material. Because

some sample digestion may be unavoidable use rigidly controlled conditions to obtain

meaningful and reproducible results. Maintain constant sample volume, acid volume, and contact

time. Express results as extractable metals and specify extraction conditions.

At collection, acidify entire sample with 5 mL conc HNO3/L sample. To prepare sample, mix

well, transfer 100 mL to a beaker or flask, and add 5 mL 1 + 1 high-purity HCl. Heat 15 min on a

steam bath. Filter through a membrane filter (preconditioned as in Section 3030B) and carefully

transfer filtrate to a tared volumetric flask. Adjust volume to 100 mL with metal-free water, mix,

and analyze. If volume is greater than 100 mL, determine volume to nearest 0.1 mL by weight,

analyze, and correct final concentration measurement by multiplying by the dilution factor (final

volume ÷ 100).

3030 D. Digestion for Metals

To reduce interference by organic matter and to convert metals associated with particulates

to a form (usually the free metal) that can be determined by atomic absorption spectrometry or

inductively-coupled plasma spectroscopy, use one of the digestion techniques presented below.

Use the least rigorous digestion method required to provide acceptable and consistent recovery

compatible with the analytical method and the metal being analyzed.1-3

1. Selection of Acid

Nitric acid will digest most samples adequately (Section 3030E). Nitrate is an acceptable

matrix for both flame and electrothermal atomic absorption and the preferred matrix for

ICP-MS.4 Some samples may require addition of perchloric, hydrochloric, hydrofluoric, or

sulfuric acid for complete digestion. These acids may interfere in the analysis of some metals

and all provide a poorer matrix for both electrothermal and ICP-MS analysis. Confirm metal

recovery for each digestion and analytical procedure used. Use Table 3030:I as a guide in

determining which acids (in addition to HNO3) to use for complete digestion. As a general rule,

HNO3 alone is adequate for clean samples or easily oxidized materials; HNO3-H2SO4 or

HNO3-HCl digestion is adequate for readily oxidizable organic matter; HNO3-HClO4 or

HNO3-HClO4-HF digestion is necessary for difficult-to-oxidize organic matter or minerals

containing silicates. Although dry ashing is not generally recommended because of the loss of

many volatile elements, it may be helpful if large amounts of organic matter are present.

2. Digestion Procedures

Dilute samples with Ag concentrations greater than 1 mg/L to contain less than 1 mg Ag/L

for flame atomic absorption methods and 25 μg/L or less for electrothermal analysis.2,5,6 To address problems with silver halide solubility in HNO3, digest using method 3030F.3b.

Report digestion technique used.

Acid digestion techniques (Section 3030E through I) generally yield comparable precision

and bias for most sample types that are totally digested by the technique. Because acids used in

digestion will add metals to the samples and blanks, minimize the volume of acids used.

Because the acid digestion techniques (Section 3030E and F) normally are not total

digestions, the microwave digestion procedure (Section 3030K) may be used as an alternative.

The microwave method is a closed-vessel procedure and thus is expected to provide improved

precision when compared with hot-plate techniques. Microwave digestion is recommended for

samples being analyzed by ICP-MS. The microwave digestion method is recommended for the

analysis of Ag, Al, As, Ba, Be, Ca, Cd, Co, Cr, Cu, Fe, K, Mg, Mn, Mo, Na, Ni, Pb, Sb, Se, Tl,

V, and Zn. Microwave digestion may be acceptable for additional analytes provided its

performance for those elements is validated.

Suggested sample volumes are indicated below for flame atomic absorption spectrometry.

Lesser volumes, to a minimum of 5 mL, are appropriate for graphite furnace, ICP, and ICP-MS.

Do not subsample volumes less than 5 mL, especially when particulates are present. Instead

dilute samples with elevated analyte concentrations after digestion. If the recommended volume

exceeds digestion vessel capacity, add sample as evaporation proceeds. For samples containing

particulates, wide-bore pipets maybe useful for volume measurement and transfer.

When samples are concentrated during digestion (e.g., >100mL sample used) determine

metal recovery for each matrix digested, to verify method validity. Using larger samples will

require additional acid, which also would increase the concentration of impurities.

Estimated Metal Concentration

mg/L

Sample Volume*

mL

<0.1                                        1000

0.1–10                                      100

10–100+                                     10

*For flame atomic absorption spectrometry.

Report results as follows:

where:

A = concentration of metal in digested solution, mg/L,

B = final volume of digested solution, mL, and

C = sample size, mL.

Prepare solid samples or liquid sludges with high solids contents on a weight basis. Mix

sample and transfer a suitable amount (typically 1 g of a sludge with 15% total solids) directly

into a preweighed digestion vessel. Reweigh and calculate weight of sample. Proceed with one

of the digestion techniques presented below. However, as these digestion methods are

predominantly for dissolved and extractable metals in aqueous samples, other approaches may be

more appropriate for solid samples. For complete mineralization of solid samples, consult

methods available elsewhere.1,4,6,7 Report results on wet- or dry-weight basis as follows:

where:

A = concentration of metal in digested solution, mg/L,

B = final volume of digested solution, mL, and

D = total solids, % (see Section 2540G).

Always prepare acid blanks for each type of digestion performed. Although it is always best

to eliminate all relevant sources of contamination, a reagent blank prepared with the same acids

and subjected to the same digestion procedure as the sample can correct for impurities present in

acids and reagent water. However, blank correction is not recommended for any other sources of

contamination such as impurities adsorbed on glassware.

3030 E. Nitric Acid Digestion

Because of the wide variation in concentration levels detected by various instrumental

techniques and the need to deal adequately with sources of contamination at trace levels, this

method presents one approach for high-level analytes (>0.1 mg/L) and another for trace levels

(≤0.1 mg/L).

1. Digestion for Flame Atomic Absorption and High-Level Concentrations

a. Apparatus:

1) Hot plate.

2) Conical (erlenmeyer) flasks, 125-mL, or Griffin beakers, 150-mL, acid-washed and rinsed

with water.

3) Volumetric flasks, 100-mL.

4) Watch glasses, ribbed and unribbed.

b. Reagent:

Nitric acid, HNO3, conc, analytical or trace-metals grade.

c. Procedure: Transfer a measured volume (100 mL recommended) of well-mixed,

acid-preserved sample appropriate for the expected metals concentrations to a flask or beaker

(see Section 3030D for sample volume). In a hood, add 5 mL conc HNO3. If a beaker is used,

cover with a ribbed watch glass to minimize contamination. Boiling chips, glass beads, or

Hengar granules may be added to aid boiling and minimize spatter when high concentration

levels (>10 mg/L) are being determined. Bring to a slow boil and evaporate on a hot plate to the

lowest volume possible (about 10 to 20 mL) before precipitation occurs. Continue heating and

adding conc HNO3 as necessary until digestion is complete as shown by a light-colored, clear

solution. Do not let sample dry during digestion.

Wash down flask or beaker walls and watch glass cover (if used) with metal-free water and then filter if necessary (see Section 3030B). Transfer filtrate to a 100-mL volumetric flask with

two 5-mL portions of water, adding these rinsings to the volumetric flask. Cool, dilute to mark,

and mix thoroughly. Take portions of this solution for required metal determinations.

2. Digestion for Trace-Level (≤0.1 mg/L) Concentrations for ICP and ICP-MS1

a. Apparatus:

1) Block heater, dry, with temperature control.

2) Polypropylene tubes*#(68), graduated, round-bottom tubes with caps, 17 × 100 mm,

acid-washed and rinsed with metal-free water. Preferably use tubes that simultaneously match

the analysis instrument autosampler and the block digester. A fit with the centrifuge is secondary

but also desirable.

3) Pipetters, assorted sizes or adjustable.

4) Pipet tips.

5) Centrifuge.

b. Reagent:

Nitric acid, HNO3, conc, double distilled.†#(69)

c. Procedure: Soak new polypropylene tubes and caps overnight or for several days in 2N

HNO3. Triple rinse with metal-free water, and preferably dry in poly rackets or baskets in a

low-temperature oven overnight. Store cleaned tubes in plastic bags before use. Pipet tips also

may need to be cleaned; evaluate before use.

Pipet 10 mL well-mixed, acid-preserved sample into a precleaned, labeled tube with a

macropipet. With a minimum volume change (<0.5 mL), add appropriate amount of analyte for

matrix fortified samples. With a pipet, add 0.5 mL conc HNO3 (or 1.0 mL 1 + 1 HNO3) to all

samples, blanks, standards, and quality control samples.

Place tubes in block heater in a hood and adjust temperature to 105°C. Drape caps over each

tube to allow escape of acid vapors while preventing contamination. NOTE: Do not screw on caps

at this time. Digest samples for a minimum of 2 h. Do not let samples boil. Add more conc nitric

acid as necessary until digestion is complete by observation of a clear solution.

Remove tubes from heat and cool. Dilute back to original 10 mL volume with metal-free

water. Adjust over-volume samples to next convenient gradation for calculations and note

volume. (Apply concentration correction from Section 3030D.) If tubes contain particulates,

centrifuge and decant clear portion into another precleaned tube. Tighten screw caps and store at

4°C until ready for analysis.

3030 F. Nitric Acid-Hydrochloric Acid Digestion

1. Apparatus

See Section 3030E.1a. The following also may be needed:

Steam bath.

2. Reagents

a. Nitric acid, HNO3, conc, analytical grade or better (see Section 3030E).

b. Hydrochloric acid, HCl, 1 + 1.

c. Nitric acid, HNO3, 1 + 1.

3. Procedure

a. Total HNO3/HCl: Transfer a measured volume of well-mixed, acid-preserved sample

appropriate for the expected metals concentrations to a flask or beaker (see Section 3030D for

sample volume). In a hood add 3 mL conc HNO3 and cover with a ribbed watch glass. Place

flask or beaker on a hot plate and cautiously evaporate to less than 5 mL, making certain that

sample does not boil and that no area of the bottom of the container is allowed to go dry. Cool.

Rinse down walls of beaker and watch glass with a minimum of metal-free water and add 5 mL

conc HNO3. Cover container with a nonribbed watch glass and return to hot plate. Increase

temperature of hot plate so that a gentle reflux action occurs. Continue heating, adding additional

acid as necessary, until digestion is complete (generally indicated when the digestate is light in

color or does not change in appearance with continued refluxing). Cool. Add 10 mL 1 + 1 HCl

and 15 mL water per 100 mL anticipated final volume. Heat for an additional 15 min to dissolve

any precipitate or residue. Cool, wash down beaker walls and watch glass with water, filter to

remove insoluble material that could clog the nebulizer (see Section 3030B), and transfer filtrate

to a 100-mL volumetric flask with rinsings. Alternatively centrifuge or let settle overnight.

Adjust to volume and mix thoroughly.

b. Recoverable HNO3/HCl: For this less rigorous digestion procedure, transfer a measured

volume of well-mixed, acid-preserved sample to a flask or beaker. Add 2 mL 1 + 1 HNO3 and 10

mL 1 + 1 HCl and cover with a ribbed watch glass. Heat on a steam bath or hot plate until

volume has been reduced to near 25 mL, making certain sample does not boil. Cool and filter to

remove insoluble material or alternatively centrifuge or let settle overnight. Quantitatively

transfer sample to volumetric flask, adjust volume to 100 mL, and mix.

For trace-level digestion, use precautionary measures similar to those detailed in Section 3030E.

3030 G. Nitric Acid-Sulfuric Acid Digestion

1. Apparatus

See Section 3030E.1a.

2. Reagents

a. Nitric acid, HNO3, conc. (See Section 3030E for acid grades.)

b. Sulfuric acid, H2SO4, conc.

3. Procedure

Transfer a measured volume of well-mixed, acid-preserved sample appropriate for the

expected metals concentrations to a flask or beaker (see Section 3030D for sample volume). Add

5 mL conc HNO3 and cover with a ribbed watch glass. Bring to slow boil on hot plate and

evaporate to 15 to 20 mL. Add 5 mL conc HNO3 and 10 mL conc H2SO4, cooling flask or

beaker between additions. Evaporate on a hot plate until dense white fumes of SO3 just appear.

If solution does not clear, add 10 mL conc HNO3 and repeat evaporation to fumes of SO3. Heat

to remove all HNO3 before continuing treatment. All HNO3 will be removed when the solution

is clear and no brownish fumes are evident. Do not let sample dry during digestion.

Cool and dilute to about 50 mL with water. Heat to almost boiling to dissolve slowly soluble

salts. Filter if necessary, then complete procedure as directed in Section 3030E.1c beginning

with, ‘‘Transfer filtrate . . .’’

3030 H. Nitric Acid-Perchloric Acid Digestion

1. Apparatus

See Section 3030E.1a. The following also are needed:

a. Safety shield.

b. Safety goggles.

c. Watch glasses.

2. Reagents

a. Nitric acid, HNO3, conc.

b. Perchloric acid, HClO4.

c. Ammonium acetate solution: Dissolve 500 g NH4C2H3O2 in 600 mL water.

3. Procedure

CAUTION: Heated mixtures of HClO4 and organic matter may explode violently. Avoid this

hazard by taking the following precautions: (a) do not add HClO4 to a hot solution containing

organic matter; (b) always pretreat samples containing organic matter with HNO3 before adding

HClO4; (c) avoid repeated fuming with HClO4 in ordinary hoods (For routine operations, use a

water pump attached to a glass fume eradicator. Stainless steel fume hoods with adequate water

washdown facilities are available commercially and are acceptable for use with HClO4); and (d)

never let samples being digested with HClO4 evaporate to dryness.

Transfer a measured volume of well-mixed, acid-preserved sample appropriate for the

expected metals concentrations to a flask or beaker (see Section 3030D for sample volume). In a

hood add 5 mL conc HNO3 and cover with a ribbed watch glass. Evaporate sample to 15 to 20

mL on a hot plate. Add 10 mL each of conc HNO3 and HClO4, cooling flask or beaker between

additions. Evaporate gently on a hot plate until dense white fumes of HClO4 just appear. If

solution is not clear, keep solution just boiling until it clears. If necessary, add 10 mL conc

HNO3 to complete digestion. Cool, dilute to about 50 mL with water, and boil to expel any

chlorine or oxides of nitrogen. Filter, then complete procedure as directed in 3030E.1c beginning

with, ‘‘Transfer filtrate . . .’’

If lead is to be determined in the presence of high amounts of sulfate (e.g., determination of

Pb in power plant fly ash samples), dissolve PbSO4 precipitate as follows: Add 50 mL

ammonium acetate solution to flask or beaker in which digestion was carried out and heat to

incipient boiling. Rotate container occasionally to wet all interior surfaces and dissolve any

deposited residue. Reconnect filter and slowly draw solution through it. Transfer filtrate to a

100-mL volumetric flask, cool, dilute to mark, mix thoroughly, and set aside for determination of

lead.

3030 I. Nitric Acid-Perchloric Acid-Hydrofluoric Acid Digestion

1. Apparatus

a. Hot plate.

b. TFE beakers, 250-mL, acid-washed and rinsed with water.

c. Volumetric flasks, 100-mL, polypropylene or other suitable plastic.

2. Reagents

a. Nitric acid, HNO3, conc and 1 + 1.

b. Perchloric acid, HClO4.

c. Hydrofluoric acid, HF, 48 to 51%.

3. Procedure

CAUTION: See precautions for using HClO4 in 3030H; handle HF with extreme care and

provide adequate ventilation, especially for the heated solution. Avoid all contact with exposed

skin. Provide medical attention for HF burns.

Transfer a measured volume of well-mixed, acid-preserved sample appropriate for the

expected metals concentrations into a 250-mL TFE beaker (see Section 3030D for sample

volume). Evaporate on a hot plate to 15 to 20 mL. Add 12 mL conc HNO3 and evaporate to near

dryness. Repeat HNO3 addition and evaporation. Let solution cool, add 20 mL HClO4 and 1 mL

HF, and boil until solution is clear and white fumes of HClO4 have appeared. Cool, add about 50

mL water, filter, and proceed as directed in Section 3030E.1c beginning with, ‘‘Transfer filtrate .

. .’’

3030 J. Dry Ashing

The procedure appears in the Eighteenth Edition of Standard Methods. It has been deleted

from subsequent editions.

3030 K. Microwave-Assisted Digestion

1. Apparatus

a. Microwave unit with programmable power (minimum 545 W) to within ± 10 W of

required power, having a corrosion-resistant, well-ventilated cavity and having all electronics

protected against corrosion for safe operation. Use a unit having a rotating turntable with a

minimum speed of 3 rpm to insure homogeneous distribution of microwave radiation. Use only

laboratory-grade microwave equipment and closed digestion containers with pressure relief that

are specifically designed for hot acid.1

b. Vessels: Construction requires an inner liner of perfluoroalkoxy (PFA) TeflonTM,*#(70)

other TFE, or composite fluorinated polymers,†#(71) capable of withstanding pressures of at

least 760 ± 70 kPa (110 ± 10 psi), and capable of controlled pressure relief at the manufacturer’s

maximum pressure rating.

Acid wash all digestion vessels and rinse with water (¶ 2a). For new vessels or when

changing between high- and low-concentration samples, clean by leaching with hot‡#(72)

hydrochloric acid (1:1) for a minimum of 2 h and then with hot nitric acid (1:1) for a minimum

of 2 h; rinse with water and dry in a clean environment. Use this procedure whenever the

previous use of digestion vessels is unknown or cross-contamination from vessels is suspected.

c. Temperature feedback control system, using shielded thermocouple, fiber-optic probe, or

infrared detector.

d. Bottles, polyethylene, 125-mL, with caps.

e. Thermometer, accurate to ± 0.1°C.

f. Balance, large-capacity (1500 g), accurate to 0.1 g.

g. Filtration or centrifuge equipment (optional).

h. Plastic container with cover, 1-L, preferably made of PFA TeflonTM§.#(73)

2. Reagents

a. Metal-free water: See Section 3111B.3c.

b. Nitric acid, HNO3, conc, sub-boiling distilled. Non-sub-boiling acids can be used if they

are shown not to contribute blanks.

3. Calibration of Microwave Unit

NOTE: For microwave units equipped with temperature feedback electronic controls,

calibration of the microwave unit is not required provided performance specifications can be

duplicated.

For cavity-type microwave equipment, evaluate absolute power (watts) by measuring the

temperature rise in 1 kg water exposed to microwave radiation for a fixed time. With this

measurement, the relationship between available power (W) and the partial power setting (%) of

the unit can be estimated, and any absolute power in watts may be transferred from one unit to

another. The calibration format required depends on type of electronic system used by

manufacturer to provide partial microwave power. Few units have an accurate and precise linear

relationship between percent power settings and absorbed power. Where linear circuits have

been used, determine calibration curve by a three-point calibration method; otherwise, use the

multiple-point calibration method.

a. Three-point calibration method: Measure power at 100% and 50% power using the

procedure described in ¶ 3c and calculate power setting corresponding to required power in

watts as specified in the procedure from the two-point line. Measure absorbed power at the

calculated partial power setting. If the measured absorbed power does not correspond to the

calculated power within ± 10 W, use the multiple-point calibration method, ¶ 3b. Use this point

periodically to verify integrity of calibration.

b. Multiple-point calibration method: For each microwave unit, measure the following

power settings: 100, 99, 98, 97, 95, 90, 80, 70, 60, 50, and 40% using the procedure described in

¶ 3c. These data are clustered about the customary working power ranges. Nonlinearity

commonly is encountered at the upper end of the calibration curve. If the unit’s electronics are

known to have nonlinear deviations in any region of proportional power control, make a set of

measurements that bracket the power to be used. The final calibration point should be at the

partial power setting that will be used in the test. Check this setting periodically to evaluate the

integrity of the calibration. If a significant change (± 10 W) is detected, re-evaluate entire

calibration.

c. Equilibrate a large volume of water to room temperature (23 ± 2°C). Weigh 1 kg water

(1000 g ± 1 g) or measure (1000 mL ± 1 mL) into a plastic, not glass, container, and measure the

temperature to ± 0.1°C. Condition microwave unit by heating a glass beaker with 500 to 1000

mL tap water at full power for 5 min with the exhaust fan on. Loosely cover plastic container to

reduce heat loss and place in normal sample path (at outer edge of rotating turntable); circulate

continuously through the microwave field for 120 s at desired power setting with exhaust fan on

as it will be during normal operation. Remove plastic container and stir water vigorously. Use a

magnetic stirring bar inserted immediately after microwave irradiation; record maximum

temperature within the first 30 s to ± 0.1°C. Use a new sample for each additional measurement.

If the water is reused, return both water and beaker to 23 ± 2°C. Make three measurements at

each power setting. When any part of the high-voltage circuit, power source, or control

components in the unit have been serviced or replaced, recheck calibration power. If power

output has changed by more than ± 10 W, re-evaluate entire calibration.

Compute absorbed power by the following relationship:

where:

P = apparent power absorbed by sample, W,

K = conversion factor for thermochemical calories sec − 1 to watts (4.184),

Cp = heat capacity, thermal capacity, or specific heat (cal g−1 °C−1) of water,

m = mass of water sample, g,

ΔT = final temperature minus initial temperature, °C, and

t = time, s.

For the experimental conditions of 120 s and 1 kg water (Cp at 25°C = 0.9997), the

calibration equation simplifies to:

P = (ΔT ) (34.85)

Stable line voltage within the manufacturer’s specification is necessary for accurate and

reproducible calibration and operation. During measurement and operation it must not vary by

more than ± 2 V. A constant power supply may be necessary if line voltage is unstable.

4. Procedure

CAUTION: This method is designed for microwave digestion of waters only. It is not intended

for the digestion of solids, for which high concentrations of organic compounds may result in

high pressures and possibly unsafe conditions.

CAUTION: As a safety measure, never mix different manufacturers’ vessels in the same

procedure. Vessels constructed differently will retain heat at different rates; control of heating

conditions assumes that all vessels have the same heat-transfer characteristics. Inspect

casements for cracks and chemical corrosion. Failure to maintain the vessels’ integrity may

result in catastrophic failure.

Both prescription controls and performance controls are provided for this procedure.

Performance controls are the most general and most accurate. When equipment capability

permits, use the performance criterion.

a. Performance criterion: The following procedure is based on heating acidified samples in

two stages where the first stage is to reach 160 ± 4° in 10 min and the second stage is to permit a

slow rise to 165 to 170°C during the second 10 min. This performance criterion is based on

temperature feedback control system capability that is implemented in various ways by different

manufacturers. Because the temperature of the acid controls the reaction, this is the essential

condition that will reproduce results in this preparation method. Verification of temperature

conditions inside the vessel at these specific times is sufficient to verify the critical procedural

requirements.

b. Prescription criterion: For all PFA vessels without liners, a verified program that meets

the performance-based temperature-time profile is 545 W for 10 min followed by 344 W for 10

min using five single-wall PFA TeflonTMi#(74) digestion vessels.2 Any verified program for a

given microwave unit depends on unit power and operational power settings, heating times,

number, type, and placement of digestion vessels within the unit, and sample and acid volumes.

The change in power, time, and temperature profile is not directly proportional to the change in

the number of sample vessels. Any deviations from the verified program conditions will require

verification of the time-temperature profile to conform to the given two-stage profile. This may

be done by laboratory personnel if suitable test equipment is available, or by the manufacturer of

the microwave equipment.

c. General conditions: Weigh entire digestion vessel assembly to 0.1 g before use and record

(A). Accurately transfer 45 mL of well-shaken sample into the digestion vessel. Pipet 5 mL conc

HNO3 into each vessel. Attach all safety equipment required for appropriate and safe vessel

operation following manufacturer’s specifications. Tighten cap to manufacturer’s specifications.

Weigh each capped vessel to the nearest 0.1 g (B).

Place appropriate number of vessels evenly distributed in the carousel. Treat sample blanks,

known additions, and duplicates in the same manner as samples. For prescription control only,

when fewer samples than the appropriate number are digested, fill remaining vessels with 45 mL

water and 5 mL conc HNO3 to obtain full complement of vessels for the particular program

being used.

Place carousel in unit and seat it carefully on turntable. Program microwave unit to heat

samples to 160 ± 4°C in 10 min and then, for the second stage, to permit a slow rise to 165 to

170°C for 10 min. Start microwave generator, making sure that turntable is turning and that exhaust fan is on.

At completion of the microwave program, let vessels cool for at least 5 min in the unit before

removal. Cool samples further outside the unit by removing the carousel and letting them cool on

a bench or in a water bath. When cooled to room temperature, weigh each vessel (to 0.1 g) and

record weight (C).

If the net weight of sample plus acid decreased by more than 10%, discard sample.

Complete sample preparation by carefully uncapping and venting each vessel in a fume

hood. Follow individual manufacturer’s specifications for relieving pressure in individual vessel

types. Transfer to acid-cleaned noncontaminating plastic bottles. If the digested sample contains

particulates, filter, centrifuge, or settle overnight and decant.

5. Calculations

a. Dilution correction: Multiply results by 50/45 or 1.11 to account for the dilution caused

by the addition of 5 mL acid to 45 mL sample.

b. Discarding of sample: To determine if the net weight of sample plus acid decreased by

more than 10% during the digestion process, use the following calculation

6. Quality Control

NOTE: When nitric acid digestion is used, recoveries of silver and antimony in some

matrices may be unacceptably low. Verify recoveries using appropriate known additions.

Preferably include a quality-control sample in each loaded carousel. Prepare samples in

batches including preparation blanks, sample duplicates, and pre-digestion known additions.

Determine size of batch and frequency of quality-control samples by method of analysis and

laboratory practice. The power of the microwave unit and batch size may prevent including one

or more of the quality-control samples in each carousel. Do not group quality-control samples

together but distribute them throughout the various carousels to give the best monitoring of

digestion.

3110 METALS BY ATOMIC ABSORPTION SPECTROMETRY

Because requirements for determining metals by atomic absorption spectrometry vary with

metal and/or concentration to be determined, the method is presented as follows:

Section 3111, Metals by Flame Atomic Absorption Spectrometry, encompasses:

• Determination of antimony, bismuth, cadmium, calcium, cesium, chromium, cobalt, copper,

gold, iridium, iron, lead, lithium, magnesium, manganese, nickel, palladium, platinum,

potassium, rhodium, ruthenium, silver, sodium, strontium, thallium, tin, and zinc by direct

aspiration into an air-acetylene flame (Section 3111B),

• Determination of low concentrations of cadmium, chromium, cobalt, copper, iron, lead,

manganese, nickel, silver, and zinc by chelation with ammonium pyrrolidine dithiocarbamate

(APDC), extraction into methyl isobutyl ketone (MIBK), and aspiration into an air-acetylene

flame (Section 3111C),

• Determination of aluminum, barium, beryllium, calcium, molybdenum, osmium, rhenium,

silicon, thorium, titanium, and vanadium by direct aspiration into a nitrous oxide-acetylene

flame (Section 3111D), and

• Determination of low concentrations of aluminum and beryllium by chelation with

8-hydroxyquinoline, extraction into MIBK, and aspiration into a nitrous oxide-acetylene flame

(Section 3111E).

Section 3112 covers determination of mercury by the cold vapor technique.

Section 3113 concerns determination of micro quantities of aluminum, antimony, arsenic,

barium, beryllium, cadmium, chromium, cobalt, copper, iron, lead, manganese, molybdenum,

nickel, selenium, silver, and tin by electrothermal atomic absorption spectrometry.

Section 3114 covers determination of arsenic and selenium by conversion to their hydrides

and aspiration into an argon-hydrogen or nitrogen-hydrogen flame.

3111 METALS BY FLAME ATOMIC ABSORPTION SPECTROMETRY*#(75)

3111 A. Introduction

1. Principle

In flame atomic absorption spectrometry, a sample is aspirated into a flame and atomized. A

light beam is directed through the flame, into a monochromator, and onto a detector that measures the amount of light absorbed by the atomized element in the flame. For some metals,

atomic absorption exhibits superior sensitivity over flame emission. Because each metal has its

own characteristic absorption wavelength, a source lamp composed of that element is used; this

makes the method relatively free from spectral or radiation interferences. The amount of energy

at the characteristic wavelength absorbed in the flame is proportional to the concentration of the

element in the sample over a limited concentration range. Most atomic absorption instruments

also are equipped for operation in an emission mode, which may provide better linearity for

some elements.

2. Selection of Method

See Section 3110.

3. Interferences

a. Chemical interference: Many metals can be determined by direct aspiration of sample

into an air-acetylene flame. The most troublesome type of interference is termed ‘‘chemical’’

and results from the lack of absorption by atoms bound in molecular combination in the flame.

This can occur when the flame is not hot enough to dissociate the molecules or when the

dissociated atom is oxidized immediately to a compound that will not dissociate further at the

flame temperature. Such interferences may be reduced or eliminated by adding specific elements

or compounds to the sample solution. For example, the interference of phosphate in the

magnesium determination can be overcome by adding lanthanum. Similarly, introduction of

calcium eliminates silica interference in the determination of manganese. However, silicon and

metals such as aluminum, barium, beryllium, and vanadium require the higher-temperature,

nitrous oxide-acetylene flame to dissociate their molecules. The nitrous oxide-acetylene flame

also can be useful in minimizing certain types of chemical interferences encountered in the

air-acetylene flame. For example, the interference caused by high concentrations of phosphate in

the determination of calcium in the air-acetylene flame is reduced in the nitrous oxide-acetylene

flame.

MIBK extractions with APDC (see Section 3111C) are particularly useful where a salt

matrix interferes, for example, in seawater. This procedure also concentrates the sample so that

the detection limits are extended.

Brines and seawater can be analyzed by direct aspiration but sample dilution is

recommended. Aspiration of solutions containing high concentrations of dissolved solids often

results in solids buildup on the burner head. This requires frequent shutdown of the flame and

cleaning of the burner head. Preferably use background correction when analyzing waters that

contain in excess of 1% solids, especially when the primary resonance line of the element of

interest is below 240 nm. Make more frequent recovery checks when analyzing brines and

seawaters to insure accurate results in these concentrated and complex matrices.

Barium and other metals ionize in the flame, thereby reducing the ground state (potentially

absorbing) population. The addition of an excess of a cation (sodium, potassium, or lithium)

having a similar or lower ionization potential will overcome this problem. The wavelength of maximum absorption for arsenic is 193.7 nm and for selenium 196.0 nm—wavelengths at which

the air-acetylene flame absorbs intensely. The sensitivity for arsenic and selenium can be

improved by conversion to their gaseous hydrides and analyzing them in either a

nitrogen-hydrogen or an argon-hydrogen flame with a quartz tube (see Section 3114).

b. Background correction: Molecular absorption and light scattering caused by solid

particles in the flame can cause erroneously high absorption values resulting in positive errors.

When such phenomena occur, use background correction to obtain accurate values. Use any one

of three types of background correction: continuum-source, Zeeman, or Smith-Hieftje correction.

1) Continuum-source background correction—A continuum-source background corrector

utilizes either a hydrogen-filled hollow cathode lamp with a metal cathode or a deuterium arc

lamp. When both the line source hollow-cathode lamp and the continuum source are placed in

the same optical path and are time-shared, the broadband background from the elemental signal

is subtracted electronically, and the resultant signal will be background-compensated.

Both the hydrogen-filled hollow-cathode lamp and deuterium arc lamp have lower intensities

than either the line source hollow-cathode lamp or electrodeless discharge lamps. To obtain a

valid correction, match the intensities of the continuum source with the line source

hollow-cathode or electrodeless discharge lamp. The matching may result in lowering the

intensity of the line source or increasing the slit width; these measures have the disadvantage of

raising the detection limit and possibly causing nonlinearity of the calibration curve. Background

correction using a continuum source corrector is susceptible to interference from other absorbing

lines in the spectral bandwidth. Miscorrection occurs from significant atomic absorption of the

continuum source radiation by elements other than that being determined. When a line source

hollow-cathode lamp is used without background correction, the presence of an absorbing line

from another element in the spectral bandwidth will not cause an interference unless it overlaps

the line of interest.

Continuum-source background correction will not remove direct absorption spectral overlap,

where an element other than that being determined is capable of absorbing the line radiation of

the element under study.

2) Zeeman background correction—This correction is based on the principle that a magnetic

field splits the spectral line into two linearly polarized light beams parallel and perpendicular to

the magnetic field. One is called the pi (π) component and the other the sigma (σ) component.

These two light beams have exactly the same wavelength and differ only in the plane of polarization. The π line will be absorbed by both the atoms of the element of interest and by the

background caused by broadband absorption and light scattering of the sample matrix. The σ line will be absorbed only by the background.

Zeeman background correction provides accurate background correction at much higher

absorption levels than is possible with continuum source background correction systems. It also

virtually eliminates the possibility of error from structured background. Because no additional

light sources are required, the alignment and intensity limitations encountered using continuum

sources are eliminated. Disadvantages of the Zeeman method include reduced sensitivity for some elements, reduced linear range, and a ‘‘rollover’’ effect whereby the absorbance of some elements begins to decrease at high concentrations, resulting in a two-sided calibration curve.

3) Smith-Hieftje background correction—This correction is based on the principle that

absorbance measured for a specific element is reduced as the current to the hollow cathode lamp

is increased while absorption of nonspecific absorbing substances remains identical at all current

levels. When this method is applied, the absorbance at a high-current mode is subtracted from

the absorbance at a low-current mode. Under these conditions, any absorbance due to

nonspecific background is subtracted out and corrected for. Smith-Hieftje background correction provides a number of advantages over continuum-source correction. Accurate correction at higher absorbance levels is possible and error from structured background is virtually eliminated. In some cases, spectral interferences also can be eliminated. The usefulness of Smith-Hieftje background correction with electrodeless discharge lamps has not yet been established.

4. Sensitivity, Detection Limits, and Optimum Concentration Ranges

The sensitivity of flame atomic absorption spectrometry is defined as the metal concentration that produces an absorption of 1% (an absorbance of approximately 0.0044). The instrument detection limit is defined here as the concentration that produces absorption equivalent to twice the magnitude of the background fluctuation. Sensitivity and detection limits vary with the instrument, the element determined, the complexity of the matrix, and the technique selected. The optimum concentration range usually starts from the concentration of several times the detection limit and extends to the concentration at which the calibration curve

starts to flatten. To achieve best results, use concentrations of samples and standards within the

optimum concentration range of the spectrometer. See Table 3111:I for indication of

concentration ranges measurable with conventional atomization. In many instances the

concentration range shown in Table 3111:I may be extended downward either by scale

expansion or by integrating the absorption signal over a long time. The range may be extended

upward by dilution, using a less sensitive wavelength, rotating the burner head, or utilizing a

microprocessor to linearize the calibration curve at high concentrations.

5. Preparation of Standards

Prepare standard solutions of known metal concentrations in water with a matrix similar to

the sample. Use standards that bracket expected sample concentration and are within the

method’s working range. Very dilute standards should be prepared daily from stock solutions in

concentrations greater than 500 mg/L. Stock standard solutions can be obtained from several

commercial sources. They also can be prepared from National Institute of Standards and

Technology (NIST) reference materials or by procedures outlined in the following sections.

For samples containing high and variable concentrations of matrix materials, make the major

ions in the sample and the dilute standard similar. If the sample matrix is complex and

components cannot be matched accurately with standards, use the method of standard additions, Section 3113B.4d2), to correct for matrix effects. If digestion is used, carry standards through

the same digestion procedure used for samples.

6. Apparatus

a. Atomic absorption spectrometer, consisting of a light source emitting the line spectrum of

an element (hollow-cathode lamp or electrodeless discharge lamp), a device for vaporizing the

sample (usually a flame), a means of isolating an absorption line (monochromator or filter and

adjustable slit), and a photoelectric detector with its associated electronic amplifying and

measuring equipment.

b. Burner: The most common type of burner is a premix, which introduces the spray into a

condensing chamber for removal of large droplets. The burner may be fitted with a conventional

head containing a single slot; a three-slot Boling head, which may be preferred for direct

aspiration with an air-acetylene flame; or a special head for use with nitrous oxide and acetylene.

c. Readout: Most instruments are equipped with either a digital or null meter readout

mechanism. Most modern instruments are equipped with microprocessors or stand-alone control

computers capable of integrating absorption signals over time and linearizing the calibration

curve at high concentrations.

d. Lamps: Use either a hollow-cathode lamp or an electrodeless discharge lamp (EDL). Use

one lamp for each element being measured. Multi-element hollow-cathode lamps generally

provide lower sensitivity than single-element lamps. EDLs take a longer time to warm up and

stabilize.

e. Pressure-reducing valves: Maintain supplies of fuel and oxidant at pressures somewhat

higher than the controlled operating pressure of the instrument by using suitable reducing valves.

Use a separate reducing valve for each gas.

f. Vent: Place a vent about 15 to 30 cm above the burner to remove fumes and vapors from

the flame. This precaution protects laboratory personnel from toxic vapors, protects the

instrument from corrosive vapors, and prevents flame stability from being affected by room

drafts. A damper or variable-speed blower is desirable for modulating air flow and preventing

flame disturbance. Select blower size to provide the air flow recommended by the instrument

manufacturer. In laboratory locations with heavy particulate air pollution, use clean laboratory

facilities (Section 3010C).

7. Quality Assurance/Quality Control

Some data typical of the precision and bias obtainable with the methods discussed are

presented in Table 3111:II and Table 3111:III.

Analyze a blank between sample or standard readings to verify baseline stability. Rezero

when necessary.

To one sample out of every ten (or one sample from each group of samples if less than ten

are being analyzed) add a known amount of the metal of interest and reanalyze to confirm

recovery. The amount of metal added should be approximately equal to the amount found. If little metal is present add an amount close to the middle of the linear range of the test. Recovery

of added metal should be between 85 and 115%.

Analyze an additional standard solution after every ten samples or with each batch of

samples, whichever is less, to confirm that the test is in control. Recommended concentrations of

standards to be run, limits of acceptability, and reported single-operator precision data are listed

in Table 3111:III.

See Section 3020 for additional recommended quality control procedures.

3111 B. Direct Air-Acetylene Flame Method

1. General Discussion

This method is applicable to the determination of antimony, bismuth, cadmium, calcium, cesium, chromium, cobalt, copper, gold, iridium, iron, lead, lithium, magnesium, manganese,

nickel, palladium, platinum, potassium, rhodium, ruthenium, silver, sodium, strontium, thallium,

tin, and zinc.

2. Apparatus

Atomic absorption spectrometer and associated equipment: See Section 3111A.6. Use

burner head recommended by the manufacturer.

3. Reagents

a. Air, cleaned and dried through a suitable filter to remove oil, water, and other foreign

substances. The source may be a compressor or commercially bottled gas.

b. Acetylene, standard commercial grade. Acetone, which always is present in acetylene

cylinders, can be prevented from entering and damaging the burner head by replacing a cylinder

when its pressure has fallen to 689 kPa (100 psi) acetylene.

CAUTION: Acetylene gas represents an explosive hazard in the laboratory. Follow

instrument manufacturer’s directions in plumbing and using this gas. Do not allow gas contact

with copper, brass with >65% copper, silver, or liquid mercury; do not use copper or brass

tubing, regulators, or fittings.

c. Metal-free water: Use metal-free water for preparing all reagents and calibration

standards and as dilution water. Prepare metal-free water by deionizing tap water and/or by using

one of the following processes, depending on the metal concentration in the sample: single

distillation, redistillation, or sub-boiling. Always check deionized or distilled water to determine

whether the element of interest is present in trace amounts. (NOTE: If the source water contains

Hg or other volatile metals, single- or redistilled water may not be suitable for trace analysis

because these metals distill over with the distilled water. In such cases, use sub-boiling to

prepare metal-free water).

d. Calcium solution: Dissolve 630 mg calcium carbonate, CaCO3, in 50 mL of 1 + 5 HCl. If

necessary, boil gently to obtain complete solution. Cool and dilute to 1000 mL with water.

e. Hydrochloric acid, HCl, 1%, 10%, 20% (all v/v), 1 + 5, 1 + 1, and conc.

f. Lanthanum solution: Dissolve 58.65 g lanthanum oxide, La2O3, in 250 mL conc HCl. Add

acid slowly until the material is dissolved and dilute to 1000 mL with water.

g. Hydrogen peroxide, 30%.

h. Nitric acid, HNO3, 2% (v/v), 1 + 1, and conc.

i. Aqua regia: Add 3 volumes conc HCl to 1 volume conc HNO3.

j. Standard metal solutions: Prepare a series of standard metal solutions in the optimum

concentration range by appropriate dilution of the following stock metal solutions with water

containing 1.5 mL conc HNO3/L. Stock standard solutions are available from a number of

commercial suppliers. Alternatively, prepare as described below. Thoroughly dry reagents before use. In general, use reagents of the highest purity. For hydrates, use fresh reagents.

1) Antimony: Dissolve 0.2669 g K(SbO)C4H4O6 in water, add 10 mL 1 + 1 HCl and dilute

to 1000 mL with water; 1.00 mL = 100 μg Sb.

2) Bismuth: Dissolve 0.100 g bismuth metal in a minimum volume of 1 + 1 HNO3. Dilute to

1000 mL with 2% (v/v) HNO3; 1.00 mL = 100 μg Bi.

3) Cadmium: Dissolve 0.100 g cadmium metal in 4 mL conc HNO3. Add 8.0 mL conc

HNO3 and dilute to 1000 mL with water; 1.00 mL = 100 μg Cd.

4) Calcium: Suspend 0.2497 g CaCO3 (dried at 180° for 1 h before weighing) in water and

dissolve cautiously with a minimum amount of 1 + 1 HNO3. Add 10.0 mL conc HNO3 and

dilute to 1000 mL with water; 1.00 mL = 100 μg Ca.

5) Cesium: Dissolve 0.1267 g cesium chloride, CsCl, in 1000 mL water; 1.00 mL = 100 μg

Cs.

6) Chromium: Dissolve 0.1923 g CrO3 in water. When solution is complete, acidify with 10

mL conc HNO3 and dilute to 1000 mL with water; 1.00 mL = 100 μg Cr.

7) Cobalt: Dissolve 0.1000 g cobalt metal in a minimum amount of 1 + 1 HNO3. Add 10.0

mL 1 + 1 HCl and dilute to 1000 mL with water; 1.00 mL = 100 μg Co.

8) Copper: Dissolve 0.100 g copper metal in 2 mL conc HNO3, add 10.0 mL conc HNO3

and dilute to 1000 mL with water; 1.00 mL = 100 μg Cu.

9) Gold: Dissolve 0.100 g gold metal in a minimum volume of aqua regia. Evaporate to

dryness, dissolve residue in 5 mL conc HCl, cool, and dilute to 1000 mL with water; 1.00 mL =

100 μg Au.

10) Iridium: Dissolve 0.1147 g ammonium chloroiridate, (NH4)2IrCl6, in a minimum

volume of 1% (v/v) HCl and dilute to 100 mL with 1% (v/v) HCl; 1.00 mL = 500 μg Ir.

11) Iron: Dissolve 0.100 g iron wire in a mixture of 10 mL 1 + 1 HCl and 3 mL conc HNO3.

Add 5 mL conc HNO3 and dilute to 1000 mL with water; 1.00 mL = 100 μg Fe.

12) Lead: Dissolve 0.1598 g lead nitrate, Pb(NO3)2, in a minimum amount of 1 + 1 HNO3,

add 10 mL conc HNO3, and dilute to 1000 mL with water; 1.00 mL = 100 μg Pb.

13) Lithium: Dissolve 0.5323 g lithium carbonate, Li2CO3, in a minimum volume of 1 + 1

HNO3. Add 10.0 mL conc HNO3 and dilute to 1000 mL with water; 1.00 mL = 100 μg Li.

14) Magnesium: Dissolve 0.1658 g MgO in a minimum amount of 1 + 1 HNO3. Add 10.0

mL conc HNO3 and dilute to 1000 mL with water; 1.00 mL = 100 μg Mg.

15) Manganese: Dissolve 0.1000 g manganese metal in 10 mL conc HCl mixed with 1 mL conc HNO3. Dilute to 1000 mL with water; 1.00 mL = 100 μg Mn.

16) Nickel: Dissolve 0.1000 g nickel metal in 10 mL hot conc HNO3, cool, and dilute to

1000 mL with water; 1.00 mL = 100 μg Ni.

17) Palladium: Dissolve 0.100 g palladium wire in a minimum volume of aqua regia and

evaporate just to dryness. Add 5 mL conc HCl and 25 mL water and warm until dissolution is

complete. Dilute to 1000 mL with water; 1.00 mL = 100 μg Pd.

18) Platinum: Dissolve 0.100 g platinum metal in a minimum volume of aqua regia and

evaporate just to dryness. Add 5 mL conc HCl and 0.1 g NaCl and again evaporate just to

dryness. Dissolve residue in 20 mL of 1 + 1 HCl and dilute to 1000 mL with water; 1.00 mL =

100 μg Pt.

19) Potassium: Dissolve 0.1907 g potassium chloride, KCl, (dried at 110°C) in water and

make up to 1000 mL; 1.00 mL = 100 μg K.

20) Rhodium: Dissolve 0.386 g ammonium hexachlororhodate, (NH4)3RhCl6⋅1.5H2O, in a

minimum volume of 10% (v/v) HCl and dilute to 1000 mL with 10% (v/v) HCl; 1.00 mL = 100

μg Rh.

21) Ruthenium: Dissolve 0.205 g ruthenium chloride, RuCl3, in a minimum volume of 20%

(v/v) HCl and dilute to 1000 mL with 20% (v/v) HCl; 1.00 mL = 100 μg Ru.

22) Silver: Dissolve 0.1575 g silver nitrate, AgNO3, in 100 mL water, add 10 mL conc

HNO3, and make up to 1000 mL; 1.00 mL = 100 μg Ag.

23) Sodium: Dissolve 0.2542 g sodium chloride, NaCl, dried at 140°C, in water, add 10 mL

conc HNO3 and make up to 1000 mL; 1.00 mL = 100 μg Na.

24) Strontium: Suspend 0.1685 g SrCO3 in water and dissolve cautiously with a minimum

amount of 1 + 1 HNO3. Add 10.0 mL conc HNO3 and dilute to 1000 mL with water: 1 mL = 100

μg Sr.

25) Thallium: Dissolve 0.1303 g thallium nitrate, TlNO3, in water. Add 10 mL conc HNO3

and dilute to 1000 mL with water; 1.00 mL = 100 μg Tl.

27) Zinc: Dissolve 0.100 g zinc metal in 20 mL 1 + 1 HCl and dilute to 1000 mL with water;

1.00 mL = 100 μg Zn.

4. Procedure

a. Sample preparation: Required sample preparation depends on the metal form being

measured.

If dissolved metals are to be determined, see Section 3030B for sample preparation. If total

or acid-extractable metals are to be determined, see Section 3030C through K. For all samples,

make certain that the concentrations of acid and matrix modifiers are the same in both samples

and standards.

When determining Ca or Mg, dilute and mix 100 mL sample or standard with 10 mL

lanthanum solution (¶ 3 f) before aspirating. When determining Fe or Mn, mix 100 mL with 25

mL of Ca solution (¶ 3d) before aspirating. When determining Cr, mix 1 mL 30% H2O2 with

each 100 mL before aspirating. Alternatively use proportionally smaller volumes.

b. Instrument operation: Because of differences between makes and models of atomic

absorption spectrometers, it is not possible to formulate instructions applicable to every

instrument. See manufacturer’s operating manual. In general, proceed according to the

following: Install a hollow-cathode lamp for the desired metal in the instrument and roughly set

the wavelength dial according to Table 3111:I. Set slit width according to manufacturer’s

suggested setting for the element being measured. Turn on instrument, apply to the

hollow-cathode lamp the current suggested by the manufacturer, and let instrument warm up

until energy source stabilizes, generally about 10 to 20 min. Readjust current as necessary after

warmup. Optimize wavelength by adjusting wavelength dial until optimum energy gain is

obtained. Align lamp in accordance with manufacturer’s instructions.

Install suitable burner head and adjust burner head position. Turn on air and adjust flow rate

to that specified by manufacturer to give maximum sensitivity for the metal being measured.

Turn on acetylene, adjust flow rate to value specified, and ignite flame. Let flame stabilize for a

few minutes. Aspirate a blank consisting of deionized water containing the same concentration

of acid in standards and samples. Zero the instrument. Aspirate a standard solution and adjust

aspiration rate of the nebulizer to obtain maximum sensitivity. Adjust burner both vertically and

horizontally to obtain maximum response. Aspirate blank again and rezero the instrument.

Aspirate a standard near the middle of the linear range. Record absorbance of this standard when

freshly prepared and with a new hollow-cathode lamp. Refer to these data on subsequent

determinations of the same element to check consistency of instrument setup and aging of

hollow-cathode lamp and standard.

The instrument now is ready to operate. When analyses are finished, extinguish flame by

turning off first acetylene and then air.

c. Standardization: Select at least three concentrations of each standard metal solution

(prepared as in ¶ 3 j above) to bracket the expected metal concentration of a sample. Aspirate

blank and zero the instrument. Then aspirate each standard in turn into flame and record

absorbance.

Prepare a calibration curve by plotting on linear graph paper absorbance of standards versus

their concentrations. For instruments equipped with direct concentration readout, this step is

unnecessary. With some instruments it may be necessary to convert percent absorption to

absorbance by using a table generally provided by the manufacturer. Plot calibration curves for

Ca and Mg based on original concentration of standards before dilution with lanthanum solution.

Plot calibration curves for Fe and Mn based on original concentration of standards before

dilution with Ca solution. Plot calibration curve for Cr based on original concentration of

standard before addition of H2O2.

d. Analysis of samples: Rinse nebulizer by aspirating water containing 1.5 mL conc

HNO3/L. Aspirate blank and zero instrument. Aspirate sample and determine its absorbance.

5. Calculations

Calculate concentration of each metal ion, in micrograms per liter for trace elements, and in

milligrams per liter for more common metals, by referring to the appropriate calibration curve

prepared according to ¶ 4c. Alternatively, read concentration directly from the instrument

readout if the instrument is so equipped. If the sample has been diluted, multiply by the

appropriate dilution factor.

3111 C. Extraction/Air-Acetylene Flame Method

1. General Discussion

This method is suitable for the determination of low concentrations of cadmium, chromium,

cobalt, copper, iron, lead, manganese, nickel, silver, and zinc. The method consists of chelation

with ammonium pyrrolidine dithiocarbamate (APDC) and extraction into methyl isobutyl ketone

(MIBK), followed by aspiration into an air-acetylene flame.

2. Apparatus

a. Atomic absorption spectrometer and associated equipment: See Section 3111A.6.

b. Burner head, conventional. Consult manufacturer’s operating manual for suggested

burner head.

3. Reagents

a. Air: See Section 3111B.3a.

b. Acetylene: See Section 3111B.3b.

c. Metal-free water: See Section 3111B.3c.

d. Methyl isobutyl ketone (MIBK), reagent grade. For trace analysis, purify MIBK by

redistillation or by sub-boiling distillation.

e. Ammonium pyrrolidine dithiocarbamate (APDC) solution: Dissolve 4 g APDC in 100 mL

water. If necessary, purify APDC with an equal volume of MIBK. Shake 30 s in a separatory

funnel, let separate, and withdraw lower portion. Discard MIBK layer.

f. Nitric acid, HNO3, conc, ultrapure.

g. Standard metal solutions: See Section 3111B.3 j.

h. Potassium permanganate solution, KMnO4, 5% (w/v) aqueous.

i. Sodium sulfate, Na2SO4, anhydrous.

j. Water-saturated MIBK: Mix one part purified MIBK with one part water in a separatory

funnel. Shake 30 s and let separate. Discard aqueous layer. Save MIBK layer.

k. Hydroxylamine hydrochloride solution, 10% (w/v). This solution can be purchased

commercially.

4. Procedure

a. Instrument operation: See Section 3111B.4b. After final adjusting of burner position,

aspirate water-saturated MIBK into flame and gradually reduce fuel flow until flame is similar to

that before aspiration of solvent.

b. Standardization: Select at least three concentrations of standard metal solutions (prepared

as in Section 3111B.3 j) to bracket expected sample metal concentration and to be, after

extraction, in the optimum concentration range of the instrument. Adjust 100mL of each standard

and 100 mL of a metal-free water blank to pH 3 by adding 1N HNO3 or 1N NaOH. For

individual element extraction, use the following pH ranges to obtain optimum extraction

efficiency:

Element

pH Range for Optimum Extraction

Ag                                                      2–5 (complex unstable)

Cd                                                      1–6

Co                                                      2–10

Cr                                                       3–9

Cu                                                      0.1–8

Fe                                                       2–5

Mn                                                      2–4 (complex unstable)

Ni                                                        2–4

Pb                                                       0.1–6

Zn                                                        2–6

NOTE: For Ag and Pb extraction the optimum pH value is 2.3 ± 0.2. The Mn complex

deteriorates rapidly at room temperature, resulting in decreased instrument response. Chilling the

extract to 0°C may preserve the complex for a few hours. If this is not possible and Mn cannot be

analyzed immediately after extraction, use another analytical procedure.

Transfer each standard solution and blank to individual 200-mL volumetric flasks, add 1 mL

APDC solution, and shake to mix. Add 10 mL MIBK and shake vigorously for 30 s. (The

maximum volume ratio of sample to MIBK is 40.) Let contents of each flask separate into

aqueous and organic layers, then carefully add water (adjusted to the same pH at which the

extraction was carried out) down the side of each flask to bring the organic layer into the neck

and accessible to the aspirating tube.

Aspirate organic extracts directly into the flame (zeroing instrument on a water-saturated

MIBK blank) and record absorbance.

Prepare a calibration curve by plotting on linear graph paper absorbances of extracted

standards against their concentrations before extraction.

c. Analysis of samples: Prepare samples in the same manner as the standards. Rinse atomizer

by aspirating water-saturated MIBK. Aspirate organic extracts treated as above directly into the

flame and record absorbances.

With the above extraction procedure only hexavalent chromium is measured. To determine

total chromium, oxidize trivalent chromium to hexavalent chromium by bringing sample to a

boil and adding sufficient KMnO4 solution dropwise to give a persistent pink color while the

solution is boiled for 10 min. Destroy excess KMnO4 by adding 1 to 2 drops hydroxylamine

hydrochloride solution to the boiling solution, allowing 2 min for the reaction to proceed. If pink

color persists, add 1 to 2 more drops hydroxylamine hydrochloride solution and wait 2 min. Heat

an additional 5 min. Cool, extract with MIBK, and aspirate.

During extraction, if an emulsion forms at the water-MIBK interface, add anhydrous Na2SO4

to obtain a homogeneous organic phase. In that case, also add Na2SO4 to all standards and

blanks.

To avoid problems associated with instability of extracted metal complexes, determine

metals immediately after extraction.

5. Calculations

Calculate the concentration of each metal ion in micrograms per liter by referring to the

3111 D. Direct Nitrous Oxide-Acetylene Flame Method

1. General Discussion

This method is applicable to the determination of aluminum, barium, beryllium, calcium, molybdenum, osmium, rhenium, silicon, thorium, titanium, and vanadium.

2. Apparatus

a. Atomic absorption spectrometer and associated equipment: See Section 3111A.6.

b. Nitrous oxide burner head: Use special burner head as suggested in manufacturer’s

manual. At roughly 20-min intervals of operation it may be necessary to dislodge the carbon

crust that forms along the slit surface with a carbon rod or appropriate alternative.

c. T-junction valve or other switching valve for rapidly changing from nitrous oxide to air,

so that flame can be turned on or off with air as oxidant to prevent flashbacks.

3. Reagents

a. Air: See Section 3111B.3a.

b. Acetylene: See Section 3111B.3b.

c. Metal-free water: See Section 3111B.3c.

d. Hydrochloric acid, HCl, 1N, 1+1, and conc.

e. Nitric acid, HNO3, conc.

f. Sulfuric acid, H2SO4, 1% (v/v).

g. Hydrofluoric acid, HF, 1N.

h. Nitrous oxide, commercially available cylinders. Fit nitrous oxide cylinder with a special

nonfreezable regulator or wrap a heating coil around an ordinary regulator to prevent flashback

at the burner caused by reduction in nitrous oxide flow through a frozen regulator. (Most modern

atomic absorption instruments have automatic gas control systems that will shut down a nitrous

oxide-acetylene flame safely in the event of a reduction in nitrous oxide flow rate.)

CAUTION: Use nitrous oxide with strict adherence to manufacturer’s directions. Improper

sequencing of gas flows at startup and shutdown of instrument can produce explosions from

flashback.

i. Potassium chloride solution: Dissolve 250 g KCl in water and dilute to 1000 mL.

j. Aluminum nitrate solution: Dissolve 139 g Al(NO3)3⋅9H2O in 150 mL water. Acidify

slightly with conc HNO3 to preclude possible hydrolysis and precipitation. Warm to dissolve

completely. Cool and dilute to 200 mL.

k. Standard metal solutions: Prepare a series of standard metal solutions in the optimum

concentration ranges by appropriate dilution of stock metal solutions with water containing 1.5

mL conc HNO3/L. Stock standard solutions are available from a number of commercial

suppliers. Alternatively, prepare as described below.

1) Aluminum: Dissolve 0.100 g aluminum metal in an acid mixture of 4 mL 1 + 1 HCl and 1

mL conc HNO3 in a beaker. Warm gently to effect solution. Transfer to a 1-L flask, add 10 mL 1

+ 1 HCl, and dilute to 1000 mL with water; 1.00 mL = 100 μg Al.

2) Barium: Dissolve 0.1516 g BaCl2 (dried at 250° for 2 h), in about 10 mL water with 1 mL

1+ 1 HCl. Add 10.0 mL 1 + 1 HCl and dilute to 1000 mL with water; 1.00 mL = 100 μg Ba.

3) Beryllium: Do not dry. Dissolve 1.966 g BeSO4⋅4H2O in water, add 10.0 mL conc HNO3,

and dilute to 1000 mL with water; 1.00 mL = 100 μg Be.

4) Calcium: See Section 3111B.3 j4).

5) Molybdenum: Dissolve 0.2043 g (NH4)2 MoO4 in water and dilute to 1000 mL; 1.00 mL

= 100 μg Mo.

6) Osmium: Obtain standard 0.1M osmium tetroxide solution*#(76) and store in glass bottle;

1.00 mL = 19.02 mg Os. Make dilutions daily as needed using 1% (v/v) H2SO4. CAUTION: OsO4

is extremely toxic and highly volatile.

7) Rhenium: Dissolve 0.1554 g potassium perrhenate, KReO4, in 200 mL water. Dilute to

1000 mL with 1% (v/v) H2SO4; 1.00 mL = 100 μg Re.

8) Silica: Do not dry. Dissolve 0.4730 g Na2SiO3⋅9H2O in water. Add 10.0 mL conc HNO3

and dilute to 1000 mL with water. 1.00 mL = 100 μg SiO2. Store in polyethylene.

9) Thorium: Dissolve 0.238 g thorium nitrate, Th(NO3)4⋅4H2O in 1000 mL water; 1.00 mL

= 100 μg Th.

10) Titanium: Dissolve 0.3960 g pure (99.8 or 99.9%) titanium chloride, TiCl4,†#(77) in a

mixture of equal volumes of 1N HCl and 1N HF. Make up to 1000 mL with this acid mixture;

1.00 mL = 100 μg Ti.

11) Vanadium: Dissolve 0.2297 g ammonium metavanadate, NH4VO3, in a minimum

amount of conc HNO3. Heat to dissolve. Add 10 mL conc HNO3, and dilute to 1000 mL with

water; 1.00 mL = 100 μg V.

4. Procedure

a. Sample preparation: See Section 3111B.4a.

When determining Al, Ba, or Ti, mix 2 mL KCl solution into 100 mL sample or standard

before aspiration. When determining Mo and V, mix 2 mL Al(NO3)3⋅9H2O into 100 mL sample

or standard before aspiration.

b. Instrument operation: See Section 3111B.4b. After adjusting wavelength, install a nitrous

oxide burner head. Turn on acetylene (without igniting flame) and adjust flow rate to value

specified by manufacturer for a nitrous oxide-acetylene flame. Turn off acetylene. With both air

and nitrous oxide supplies turned on, set T-junction valve to nitrous oxide and adjust flow rate

according to manufacturer’s specifications. Turn switching valve to the air position and verify

that flow rate is the same. Turn acetylene on and ignite to a bright yellow flame. With a rapid

motion, turn switching valve to nitrous oxide. The flame should have a red cone above the ignited, let burner come to thermal equilibrium before beginning analysis.

Aspirate a blank consisting of deionized water containing 1.5 mL conc HNO3/L and check

aspiration rate. Adjust if necessary to a rate between 3 and 5 mL/ min. Zero the instrument.

Aspirate a standard of the desired metal with a concentration near the midpoint of the optimum

concentration range and adjust burner (both horizontally and vertically) in the light path to

obtain maximum response. Aspirate blank again and re-zero the instrument. The instrument now

is ready to run standards and samples.

To extinguish flame, turn switching valve from nitrous oxide to air and turn off acetylene.

This procedure eliminates the danger of flashback that may occur on direct ignition or shutdown

of nitrous oxide and acetylene. (See also discussion in Section 3111B.4b.)

c. Standardization: Select at least three concentrations of standard metal solutions (prepared

as in ¶ 3k) to bracket the expected metal concentration of a sample. Aspirate each in turn into the

flame and record absorbances.

Most modern instruments are equipped with microprocessors and digital readout which

permit calibration in direct concentration terms. If instrument is not so equipped, prepare a

calibration curve by plotting on linear graph paper absorbance of standards versus concentration.

Plot calibration curves for Al, Ba, and Ti based on original concentration of standard before

adding KCl solution. Plot calibration curves for Mo and V based on original concentration of

standard before adding Al(NO3)3 solution.

d. Analysis of samples: Rinse atomizer by aspirating water containing 1.5 mL conc HNO3/L

and zero instrument. Aspirate a sample and determine its absorbance.

5. Calculations

Calculate concentration of each metal ion in micrograms per liter by referring to the

appropriate calibration curve prepared according to ¶ 4c.

Alternatively, read the concentration directly from the instrument readout if the instrument is

so equipped. If sample has been diluted, multiply by the appropriate dilution factor.

3111 E. Extraction/Nitrous Oxide-Acetylene Flame Method

1. General Discussion

a. Application: This method is suitable for the determination of aluminum at concentrations less than 900 μg/L and beryllium at concentrations less than 30 μg/L. The method consists of

chelation with 8-hydroxyquinoline, extraction with methyl isobutyl ketone (MIBK), and

aspiration into a nitrous oxide-acetylene flame.

b. Interferences: Concentrations of Fe greater than 10 mg/L interfere by suppressing Al

absorption. Iron interference can be masked by addition of hydroxylamine

hydrochloride/1,10-phenanthroline. Mn concentrations up to 80 mg/L do not interfere if turbidity

in the extract is allowed to settle. Mg forms an insoluble chelate with 8-hydroxyquinoline at pH

8.0 and tends to remove Al complex as a coprecipitate. However, the Mg complex forms slowly

over 4 to 6 min; its interference can be avoided if the solution is extracted immediately after

adding buffer.

2. Apparatus

Atomic absorption spectrometer and associated equipment: See Section 3111A.6.

3. Reagents

a. Air: See Section 3111B.3a.

b. Acetylene: See Section 3111B.3b.

c. Ammonium hydroxide, NH4OH, conc.

d. Buffer: Dissolve 300 g ammonium acetate, NH4C2H3O2, in water, add 105 mL conc

NH4OH, and dilute to 1 L.

e. Metal-free water: See Section 3111B.3c.

f. Hydrochloric acid, HCl, conc.

g. 8-Hydroxyquinoline solution: Dissolve 20 g 8-hydroxyquinoline in about 200 mL water,

add 60 mL glacial acetic acid, and dilute to 1 L with water.

h. Methyl isobutyl ketone: See Section 3111C.3d.

i. Nitric acid, HNO3, conc.

j. Nitrous oxide: See Section 3111D.3h.

k. Standard metal solutions: Prepare a series of standard metal solutions containing 5 to

1000 μg/L by appropriate dilution of the stock metal solutions prepared according to Section

3111D.3k.

l. Iron masking solution: Dissolve 1.3 g hydroxylamine hydrochloride and 6.58 g

1,10-phenanthroline monohydrate in about 500 mL water and dilute to 1 L with water.

4. Procedure

a. Instrument operation: See Section 3111B.4b, Section 3111C.4a, and Section 3111D.4b.

After final adjusting of burner position, aspirate MIBK into flame and gradually reduce fuel flow

until flame is similar to that before aspiration of solvent. Adjust wavelength setting according to

Table 3111:I. b. Standardization: Select at least three concentrations of standard metal solutions (prepared

as in ¶ 3k) to bracket the expected metal concentration of a sample and transfer 100 mL of each

(and 100 mL water blank) to four different 200-mL volumetric flasks. Add 2 mL

8-hydroxyquinoline solution, 2 mL masking solution (if required), and 10 mL buffer to one flask,

immediately add 10 mL MIBK, and shake vigorously. The duration of shaking affects the forms

of aluminum complexed. A fast, 10-s shaking time favors monomeric Al, whereas 5 to 10 min of

shaking also will complex polymeric species. Adjustment of the 8-hydroxyquinoline to sample

ratio can improve recoveries of extremely high or low concentrations of aluminum. Treat each

blank, standard, and sample in similar fashion. Continue as in Section 3111C.4b.

c. Analysis of samples: Rinse atomizer by aspirating water-saturated MIBK. Aspirate

extracts of samples treated as above, and record absorbances.

5. Calculations

Calculate concentration of each metal in micrograms per liter by referring to the appropriate

calibration curve prepared according to ¶ 4b.

3500-Cr CHROMIUM*#(101)

3500-Cr A. Introduction

1. Occurrence and Significance

Chromium (Cr) is the first element in Group VIB in the periodic table; it has an atomic

number of 24, an atomic weight of 51.99, and valences of 1 through 6. The average abundance of

Cr in the earth’s crust is 122 ppm; in soils Cr ranges from 11 to 22 ppm; in streams it averages

about 1 μg/L, and in groundwaters it is generally 100 μg/L. Chromium is found chiefly in

chrome-iron ore (FeO⋅Cr2O3). Chromium is used in alloys, in electroplating, and in pigments.

Chromate compounds frequently are added to cooling water for corrosion control.

In natural waters trivalent chromium exists as Cr3+, Cr(OH)2+, Cr(OH)2+, and Cr(OH)4–; in

the hexavalent form chromium exists as CrO42– and as Cr2O7 2–. Cr3+ would be expected to form strong complexes with amines, and would be adsorbed by clay minerals. Chromium may exist in water supplies in both the hexavalent and the trivalent state although the trivalent form rarely occurs in potable water.

Chromium is considered nonessential for plants, but an essential trace element for animals.

Hexavalent compounds have been shown to be carcinogenic by inhalation and are corrosive to

tissue. The chromium guidelines for natural water are linked to the hardness or alkalinity of the

water (i.e., the softer the water, the lower the permitted level for chromium). The United Nations Food and Agriculture Organization recommended maximum level for irrigation waters is 100

μg/L. The U.S. EPA primary drinking water standard MCL is 0.1 mg/L for total chromium.

2. Selection of Method

The colorimetric method (B) is useful for the determination of hexavalent chromium in a

natural or treated water in the range from 100 to 1000 μg/L. This range can be extended by

appropriate sample dilution or concentration and/or use of longer cell paths. The ion

chromatographic method with photometric detection (C) is suitable for determining dissolved

hexavalent chromium in drinking water, groundwater, and industrial wastewater effluents. The

electrothermal atomic absorption spectrometric method (Section 3113B) is suitable for

determining low levels of total chromium (< 50 μg/L) in water and wastewater, and the flame

atomic absorption spectrometric methods (Section 3111B and Section 3111C) and the

inductively coupled plasma methods (Section 3120 and Section 3125) are appropriate for

measuring concentrations up to milligram-per-liter levels.

3. Sample Handling

If only the dissolved metal content is desired, filter sample through a 0.45-μm membrane

filter at time of collection, and after filtration acidify filtrate with conc nitric acid (HNO3) to pH

<2. If only dissolved hexavalent chromium is desired, adjust pH of filtrate to 8 or above with 1N

sodium hydroxide solution and refrigerate. If the total chromium content is desired, acidify

unfiltered sample at time of collection with conc HNO3 to pH <2. If total hexavalent chromium

is desired, adjust the pH of unfiltered sample to 8 or above with 1N sodium hydroxide and

refrigerate.

3500-Cr B. Colorimetric Method

1. General Discussion

a. Principle: This procedure measures only hexavalent chromium (Cr 6+). Therefore, to

determine total chromium convert all the chromium to the hexavalent state by oxidation with

potassium permanganate. NOTE: The oxidation process may not provide total conversion of all

chromium species to Cr 6+.1-3 For total chromium determination, acid-digest the sample (see

Section 3030) and follow with a suitable instrumental analysis technique. The hexavalent

chromium is determined colorimetrically by reaction with diphenylcarbazide in acid solution. A

red-violet colored complex of unknown composition is produced. The reaction is very sensitive,

the molar absorptivity based on chromium being about 40 000 L g–1 cm–1 at 540 nm. To

determine total chromium, digest the sample with a sulfuric-nitric acid mixture and then oxidize

with potassium permanganate before reacting with the diphenylcarbazide.

b. Interferences: The reaction with diphenylcarbazide is nearly specific for chromium.

Hexavalent molybdenum and mercury salts will react to form color with the reagent but the intensities are much lower than that for chromium at the specified pH. Concentrations as high as

200 mg Mo or Hg/L can be tolerated. Vanadium interferes strongly but concentrations up to 10

times that of chromium will not cause trouble. Potential interference from permanganate is

eliminated by prior reduction with azide. Iron in concentrations greater than 1 mg/L may

produce a yellow color but the ferric ion (Fe3+) color is not strong and no difficulty is

encountered normally if the absorbance is measured photometrically at the appropriate

wavelength. Interfering amounts of molybdenum, vanadium, iron, and copper can be removed by

extraction of the cupferrates of these metals into chloroform (CHCl3). A procedure for this

extraction is provided but do not use it unless necessary, because residual cupferron and CHCl3

in the aqueous solution complicate the later oxidation. Therefore, follow the extraction by

additional treatment with acid fuming to decompose these compounds.

2. Apparatus

a. Colorimetric equipment: One of the following is required:

1) Spectrophotometer, for use at 540 nm, with a light path of 1 cm or longer.

2) Filter photometer, providing a light path of 1 cm or longer and equipped with a greenish

yellow filter having maximum transmittance near 540 nm.

b. Separatory funnels, 125-mL, Squibb form, with glass or TFE stopcock and stopper.

c. Acid-washed glassware: New and unscratched glassware will minimize chromium

adsorption on glass surfaces during the oxidation procedure. Do not use glassware previously

treated with chromic acid. Thoroughly clean other used glassware and new glassware with nitric

or hydrochloric acid to remove chromium traces.

3. Reagents

Use reagent water (see Section 1080) for reagent preparation and analytical procedure.

a. Stock chromium solution: Dissolve 141.4 mg K2Cr2O7 in water and dilute to 100 mL;

1.00 mL = 500 μg Cr.

b. Standard chromium solution: Dilute 1.00 mL stock chromium solution to 100 mL; 1.00

mL = 5.00 μg Cr.

c. Nitric acid, HNO3, conc.

d. Sulfuric acid, H2SO4, conc, 18N, and 6N.

e. Sulfuric acid, H2SO4, 0.2N: Dilute 17 mL 6N H2SO4 to 500 mL with water.

f. Phosphoric acid, H3PO4, conc.

g. Methyl orange indicator solution.

h. Hydrogen peroxide, H2O2, 30%.

i. Ammonium hydroxide, NH4OH, conc.

j. Potassium permanganate solution: Dissolve 4 g KMnO4 in 100 mL water.

k. Sodium azide solution: Dissolve 0.5 g NaN3 in 100 mL water.

l. Diphenylcarbazide solution: Dissolve 250 mg 1,5-diphenylcarbazide

(1,5-diphenylcarbohydrazide) in 50 mL acetone. Store in a brown bottle. Discard when solution

becomes discolored.

m. Chloroform, CHCl3: Avoid or redistill material that comes in containers with metal or

metal-lined caps.

n. Cupferron solution: Dissolve 5 g cupferron, C6H5N(NO)ONH4, in 95 mL water.

o. Sodium hydroxide,1N: Dissolve 40 g NaOH in 1 L water. Store in plastic bottle.

4. Procedure

a. Preparation of calibration curve: To compensate for possible slight losses of chromium

during digestion or other analytical operations, treat standards by the same procedure as the

sample. Accordingly, pipet measured volumes of standard chromium solution (5 μg/mL) ranging

from 2.00 to 20.0 mL, to give standards for 10 to 100 μg Cr, into 250-mL beakers or conical

flasks. Depending on pretreatment used in ¶ b below, proceed with subsequent treatment of

standards as if they were samples, also carrying out cupferron treatment of standards if this is

required for samples.

Develop color as for samples, transfer a suitable portion of each colored solution to a 1-cm

absorption cell, and measure absorbance at 540 nm, using reagent water as reference. Correct

absorbance readings of standards by subtracting absorbance of a reagent blank carried through

the method.

Construct a calibration curve by plotting corrected absorbance values against micrograms

chromium in 102 mL final volume.

b. Treatment of sample: If sample has been filtered and/or only hexavalent chromium is

desired, start analysis within 24 h of collection and proceed to ¶ 4e. NOTE: Recent evidence4

suggests that preserved samples can be held for 30 d without substantial changes to Cr 6+

concentrations. If total dissolved chromium is desired and there are interfering amounts of

molybdenum, vanadium, copper, or iron present, proceed to ¶ 4c. If interferences are not present,

proceed to ¶ 4d.

If sample is unfiltered and total chromium is desired, digest with HNO3 and H2SO4 as in

Section 3030G. If interferences are present, proceed to ¶ 4c, ¶ 4d, and ¶ 4e. If there are no

interferences, proceed to ¶ 4d and ¶ 4e.

c. Removal of molybdenum, vanadium, iron, and copper with cupferron: Pipet a portion of

sample containing 10 to 100 μg Cr into a 125-mL separatory funnel. Dilute to about 40 mL with

water and chill in an ice bath. Add 5 mL ice-cold cupferron solution, shake well, and let stand in

ice bath for 1 min. Extract in separatory funnel with three successive 5-mL portions of CHCl3; shake each portion thoroughly with aqueous solution, let layers separate, and withdraw and

discard CHCl3 extract. Transfer extracted aqueous solution to a 125-mL conical flask. Wash

separatory funnel with a small amount of water and add wash water to flask. Boil for about 5 min

to volatilize CHCl3 and cool. Add 5 mL HNO3 and 3 mL H2SO4. Boil samples to the appearance

of SO3 fumes. Cool slightly, carefully add 5 mL HNO3, and again boil to fumes to complete

decomposition of organic matter. Cool, wash sides of flask, and boil once more to SO3 fumes,

assuming elimination of all HNO3. Cool and add 25 mL water.

d. Oxidation of trivalent chromium: Pipet a portion of digested sample with or without

interferences removed, and containing 10 to 100 μg Cr, into a 125-mL conical flask. Add several

drops of methyl orange indicator, then add conc NH4OH until solution just begins to turn yellow.

Add 1 + 1 H2SO4 dropwise until it is acidic, plus 1 mL (20 drops) in excess. Adjust volume to

about 40 mL, add two or more acid-washed glass beads, and heat to boiling. Add 2 drops

KMnO4 solution to give a dark red color. If fading occurs, add KMnO4 dropwise to maintain an

excess of about 2 drops. Boil for 2 min longer. Add 1 mL NaN3 solution and continue boiling

gently. If red color does not fade completely after boiling for approximately 30 s, add another 1

mL NaN3 solution. Continue boiling for 1 min after color has faded completely, then cool.

e. Color development and measurement: Add 0.25 mL (5 drops) H3PO4. Use 0.2N H2SO4

and a pH meter to adjust solution to pH 1.0 ± 0.3. NOTE: Recent work5 identifies the optimum

pH range for color development to be 1.6 to 2.2; the matter of optimum pH range is currently

being considered by Standard Methods. Transfer solution to a 100-mL volumetric flask, dilute to

100 mL, and mix. Add 2.0 mL diphenylcarbazide solution, mix, and let stand 5 to 10 min for full

color development. Transfer an appropriate portion to a 1-cm absorption cell and measure its

absorbance at 540 nm, using reagent water as reference. Correct absorbance reading of sample

by subtracting absorbance of a blank carried through the method (see also note below). From the

corrected absorbance, determine micrograms chromium present by reference to the calibration

curve.

NOTE: If the solution is turbid after dilution to 100 mL in ¶ e above, take an absorbance

reading before adding carbazide reagent and correct absorbance reading of final colored solution

by subtracting the absorbance measured previously.

5. Calculation

For digested samples:

where:

A = mL original sample, and

B = mL portion from 100 mL digested sample.

For undigested samples:

6. Precision and Bias

Collaborative test data from 16 laboratories were obtained on reagent water, tap water, 10%

NaCl solution, treated water from synthetic organic industrial waste, EPA extraction leachate,

process water, lake water, and effluent from a steel pickle liquor treatment plant.6 The test data

yielded the following relationships:

Reagent water:

St = 0.037x + 0.006

So = 0.022x + 0.004

Drinking or wastewater:

St = 0.067x + 0.004

So = 0.037x + 0.002

Leachate:

St = 0.032x + 0.007

So = 0.017x + 0.004

where:

St = overall precision,

So = single-operator precision, and

x = chromium concentration, mg/L.

3500-Fe IRON*#(107)

3500-Fe A. Introduction

1. Occurrence and Significance

Iron (Fe) is the first element in Group VIII of the periodic table; it has an atomic number of

26, an atomic weight of 55.85, and common valences of 2 and 3 (and occasionally valences of 1,

4, and 6). The average abundance of Fe in the earth’s crust is 6.22%; in soils Fe ranges from 0.5

to 4.3%; in streams it averages about 0.7 mg/L; and in groundwater it is 0.1 to 10 mg/L. Iron

occurs in the minerals hematite, magnetite, taconite, and pyrite. It is widely used in steel and in

other alloys.

The solubility of ferrous ion (Fe2+) is controlled by the carbonate concentration. Because

groundwater is often anoxic, any soluble iron in groundwater is usually in the ferrous state. On

exposure to air or addition of oxidants, ferrous iron is oxidized to the ferric state (Fe3+) and may

hydrolyze to form red, insoluble hydrated ferric oxide. In the absence of complex-forming ions,

ferric iron is not significantly soluble unless the pH is very low.

Elevated iron levels in water can cause stains in plumbing, laundry, and cooking utensils, and

can impart objectionable tastes and colors to foods. The United Nations Food and Agriculture

Organization recommended level for irrigation waters is 5 mg/L. The U.S. EPA secondary

drinking water standard MCL is 0.3 mg/L.

2. Selection of Method

Sensitivity and detection limits for the atomic absorption spectrometric methods (Section

3111B and Section 3111C), the inductively coupled plasma method (Section 3120), and the

phenanthroline colorimetric procedure described here (B) are similar and generally adequate for

analysis of natural or treated waters. Lower detection limits can be achieved with electrothermal atomic absorption spectrometry (Section 3113B) when an appropriate matrix modifier is used.

The complexing reagents used in the colorimetric procedures are specific for ferrous iron but the

atomic absorption procedures are not. However, because of the instability of ferrous iron, which

is changed easily to the ferric form in solutions in contact with air, determination of ferrous iron

requires special precautions and may need to be done in the field at the time of sample

collection.

The procedure for determining ferrous iron using 1,10-phenanthroline (Section

3500-Fe.B.4c) has a somewhat limited applicability; avoid long storage time or exposure of

samples to light. A rigorous quantitative distinction between ferrous and ferric iron can be

obtained with a special procedure using bathophenanthroline. Spectrophotometric methods using

bathophenanthroline1-6 and other organic complexing reagents such as ferrozine7 or TPTZ8 are

capable of determining iron concentrations as low as 1 μg/L. A chemiluminescence procedure9

is stated to have a detection limit of 5 ng/L. Additional procedures are described elsewhere.10-13

3. Sampling and Storage

Plan in advance the methods of collecting, storing, and pretreating samples. Clean sample

container with acid and rinse with reagent water. Equipment for membrane filtration of samples

in the field may be required to determine iron in solution (dissolved iron). Dissolved iron,

considered to be that passing through a 0.45-μm membrane filter, may include colloidal iron.

The value of the determination depends greatly on the care taken to obtain a representative

sample. Iron in well or tap water samples may vary in concentration and form with duration and

degree of flushing before and during sampling. When taking a sample portion for determining

iron in suspension, shake the sample bottle often and vigorously to obtain a uniform suspension

of precipitated iron. Use particular care when colloidal iron adheres to the sample bottle. This

problem can be acute with plastic bottles.

For a precise determination of total iron, use a separate container for sample collection. Treat

with acid at the time of collection to place the iron in solution and prevent adsorption or

deposition on the walls of the sample container. Take account of the added acid in measuring

portions for analysis. The addition of acid to the sample may eliminate the need for adding acid

before digestion (Section 3500-Fe.B.4a).

3500-Fe B. Phenanthroline Method

1. General Discussion

a. Principle: Iron is brought into solution, reduced to the ferrous state by boiling with acid

and hydroxylamine, and treated with 1,10-phenanthroline at pH 3.2 to 3.3. Three molecules of

phenanthroline chelate each atom of ferrous iron to form an orange-red complex. The colored

solution obeys Beer’s law; its intensity is independent of pH from 3 to 9. A pH between 2.9 and

3.5 insures rapid color development in the presence of an excess of phenanthroline. Color

standards are stable for at least 6 months.

b. Interference: Among the interfering substances are strong oxidizing agents, cyanide,

nitrite, and phosphates (polyphosphates more so than orthophosphate), chromium, zinc in

concentrations exceeding 10 times that of iron, cobalt and copper in excess of 5 mg/L, and nickel

in excess of 2 mg/L. Bismuth, cadmium, mercury, molybdate, and silver precipitate

phenanthroline. The initial boiling with acid converts polyphosphates to orthophosphate and

removes cyanide and nitrite that otherwise would interfere. Adding excess hydroxylamine

eliminates errors caused by excessive concentrations of strong oxidizing reagents. In the

presence of interfering metal ions, use a larger excess of phenanthroline to replace that

complexed by the interfering metals. Where excessive concentrations of interfering metal ions are present, the extraction method may be used.

If noticeable amounts of color or organic matter are present, it may be necessary to

evaporate the sample, gently ash the residue, and redissolve in acid. The ashing may be carried

out in silica, porcelain, or platinum crucibles that have been boiled for several hours in 6N HCl.

The presence of excessive amounts of organic matter may necessitate digestion before use of the

extraction procedure.

c. Minimum detectable concentration: Dissolved or total concentrations of iron as low as 10

μg/L can be determined with a spectrophotometer using cells with a 5 cm or longer light path.

Carry a blank through the entire procedure to allow for correction.

2. Apparatus

a. Colorimetric equipment: One of the following is required:

1) Spectrophotometer, for use at 510 nm, providing a light path of 1 cm or longer.

2) Filter photometer, providing a light path of 1 cm or longer and equipped with a green

filter having maximum transmittance near 510 nm.

3) Nessler tubes, matched, 100-mL, tall form.

b. Acid-washed glassware: Wash all glassware with conc hydrochloric acid (HCl) and rinse

with reagent water before use to remove deposits of iron oxide.

c. Separatory funnels: 125-mL, Squibb form, with ground-glass or TFE stopcocks and

stoppers.

3. Reagents

Use reagents low in iron. Use reagent water (see Section 1080 and Section 3111B.3c) in

preparing standards and reagent solutions and in procedure. Store reagents in glass-stoppered

bottles. The HCl and ammonium acetate solutions are stable indefinitely if tightly stoppered. The

hydroxylamine, phenanthroline, and stock iron solutions are stable for several months. The

standard iron solutions are not stable; prepare daily as needed by diluting the stock solution.

Visual standards in nessler tubes are stable for several months if sealed and protected from light.

a. Hydrochloric acid, HCl, conc, containing less than 0.5 ppm iron.

b. Hydroxylamine solution: Dissolve 10 g NH2OH⋅HCl in 100 mL water.

c. Ammonium acetate buffer solution: Dissolve 250 g NH4C2H3O2 in 150 mL water. Add

700 mL conc (glacial) acetic acid. Because even a good grade of NH4C2H3O2 contains a

significant amount of iron, prepare new reference standards with each buffer preparation.

d. Sodium acetate solution: Dissolve 200 g NaC2H3O2⋅3H2O in 800 mL water.

e. Phenanthroline solution: Dissolve 100 mg 1,10-phenanthroline monohydrate,

C12H8N2⋅H2O, in 100 mL water by stirring and heating to 80°C. Do not boil. Discard the

solution if it darkens. Heating is unnecessary if 2 drops conc HCl are added to the water. (NOTE: One milliliter of this reagent is sufficient for no more than 100 μg Fe.)

f. Potassium permanganate, 0.1M: Dissolve 0.316 KMnO4 in reagent water and dilute to

100 mL.

g. Stock iron solution: Use metal (1) or salt (2) for preparing the stock solution.

1) Use electrolytic iron wire, or ‘‘iron wire for standardizing,’’ to prepare the solution. If

necessary, clean wire with fine sandpaper to remove any oxide coating and to produce a bright

surface. Weigh 200.0 mg wire and place in a 1000-mL volumetric flask. Dissolve in 20 mL 6N

sulfuric acid (H2SO4) and dilute to mark with water; 1.00 mL = 200 μg Fe.

2) If ferrous ammonium sulfate is preferred, slowly add 20 mL conc H2SO4 to 50 mL water

and dissolve 1.404 g Fe(NH4)2(SO4)2⋅6H2O. Add 0.1M potassium permanganate (KMnO4)

dropwise until a faint pink color persists. Dilute to 1000 mL with water and mix; 1.00 mL = 200

μg Fe.

h. Standard iron solutions: Prepare daily for use.

1) Pipet 50.00 mL stock solution into a 1000-mL volumetric flask and dilute to mark with

water; 1.00 mL = 10.0 μg Fe.

2) Pipet 5.00 mL stock solution into a 1000-mL volumetric flask and dilute to mark with

water; 1.00 mL = 1.00 μg Fe.

i. Diisopropyl or isopropyl ether. CAUTION: Ethers may form explosive peroxides; test

before using.

4. Procedure

a. Total iron: Mix sample thoroughly and measure 50.0 mL into a 125-mL erlenmeyer flask.

If this sample volume contains more than 200 μg iron use a smaller accurately measured portion

and dilute to 50.0 mL. Add 2 mL conc HCl and 1 mL NH2OH⋅HCl solution. Add a few glass

beads and heat to boiling. To insure dissolution of all the iron, continue boiling until volume is

reduced to 15 to 20 mL. (If the sample is ashed, take up residue in 2 mL conc HCl and 5 mL

water.) Cool to room temperature and transfer to a 50- or 100-mL volumetric flask or nessler

tube. Add 10 mL NH4C2H3O2 buffer solution and 4 mL phenanthroline solution, and dilute to

mark with water. Mix thoroughly and allow a minimum of 10 min for maximum color

development.

b. Dissolved iron: Immediately after collection filter sample through a 0.45-μm membrane

filter into a vacuum flask containing 1 mL conc HCl/100 mL sample. Analyze filtrate for total

dissolved iron (¶ 4a) and/or dissolved ferrous iron (¶ 4c). (This procedure also can be used in the

laboratory if it is understood that normal sample exposure to air during shipment may result in

precipitation of iron.)

Calculate suspended iron by subtracting dissolved from total iron.

c. Ferrous iron: Determine ferrous iron at sampling site because of the possibility of change in the ferrous-ferric ratio with time in acid solutions. To determine ferrous iron only, acidify a

separate sample with 2 mL conc HCl/100 mL sample at time of collection. Fill bottle directly

from sampling source and stopper. Immediately withdraw a 50-mL portion of acidified sample

and add 20 mL phenanthroline solution and 10 mL NH4C2H3O2 solution with vigorous stirring.

Dilute to 100 mL and measure color intensity within 5 to 10 min. Do not expose to sunlight.

(Color development is rapid in the presence of excess phenanthroline. The phenanthroline

volume given is suitable for less than 50 μg total iron; if larger amounts are present, use a

correspondingly larger volume of phenanthroline or a more concentrated reagent.)

Calculate ferric iron by subtracting ferrous from total iron.

d. Color measurement: Prepare a series of standards by accurately pipetting calculated

volumes of standard iron solutions [use solution described in ¶ 3h2) to measure 1- to 10-μg

portions] into 125-mL erlenmeyer flasks, diluting to 50 mL by adding measured volumes of

water, and carrying out the steps in ¶ 4a beginning with transfer to a 100-mL volumetric flask or

nessler tube.

For visual comparison, prepare a set of at least 10 standards, ranging from 1 to 100 μg Fe in

the final 100-mL volume. Compare colors in 100-mL tall-form nessler tubes.

For photometric measurement, use Table 3500-Fe:I as a rough guide for selecting proper

light path at 510 nm. Read standards against water set at zero absorbance and plot a calibration

curve, including a blank (see ¶ 3c and General Introduction).

If samples are colored or turbid, carry a second set of samples through all steps of the

procedure without adding phenanthroline. Instead of water, use the prepared blanks to set

photometer to zero absorbance and read each sample developed with phenanthroline against the

corresponding blank without phenanthroline. Translate observed photometer readings into iron

values by means of the calibration curve. This procedure does not compensate for interfering

ions.

e. Samples containing organic interferences: Digest samples containing substantial amounts

of organic substances according to the directions given in Section 3030G or Section 3030H.

1) If a digested sample has been prepared according to the directions given in Section 3030G

or Section 3030H, pipet 10.0 mL or other suitable portion containing 20 to 500 μg Fe into a

125-mL separatory funnel. If the volume taken is less than 10 mL, add water to make up to 10

mL. To the separatory funnel add 15 mL conc HCl for a 10-mL aqueous volume; or, if the

portion taken was greater than 10.0 mL, add 1.5 mL conc HCl/mL sample. Mix, cool, and

proceed with ¶ 4e3) below.

2) To prepare a sample solely for determining iron, measure a suitable volume containing 20

to 500 μg Fe and carry it through the digestion procedure described in either Section 3030G or

Section 3030H. However, use only 5 mL H2SO4 or HClO4 and omit H2O2. When digestion is

complete, cool, dilute with 10 mL water, heat almost to boiling to dissolve slowly soluble salts,

and, if the sample is still cloudy, filter through a glass-fiber, sintered-glass, or porcelain filter,

washing with 2 to 3 mL water. Quantitatively transfer filtrate or clear solution to a 25-mL

volumetric flask and make up to 25 mL with water. Empty flask into a 125-mL separatory

funnel, rinse flask with 5 mL conc HCl and add to the funnel. Add 25 mL conc HCl measured

with the same flask. Mix and cool to room temperature.

3) Extract the iron from the HCl solution in the separatory funnel by shaking for 30 s with

25 mL isopropyl ether (CAUTION). Draw off lower acid layer into a second separatory funnel.

Extract acid solution again with 25 mL isopropyl ether, drain acid layer into a suitable clean

vessel, and add ether layer to the ether in the first funnel. Pour acid layer back into second

separatory funnel and re-extract with 25 mL isopropyl ether. Withdraw and discard acid layer

and add ether layer to first funnel. Persistence of a yellow color in the HCl solution after three

extractions does not signify incomplete separation of iron because copper, which is not

extracted, gives a similar yellow color.

Shake combined ether extracts with 25 mL water to return iron to aqueous phase and transfer

lower aqueous layer to a 100-mL volumetric flask. Repeat extraction with a second 25-mL

portion of water, adding this to the first aqueous extract. Discard ether layer.

4) Add 1 mL NH2OH⋅HCl solution, 10 mL phenanthroline solution, and 10 mL NaC2H3O2

solution. Dilute to 100 mL with water, mix thoroughly, and let stand for a minimum of 10 min.

Measure absorbance at 510 nm using a 5-cm absorption cell for amounts of iron less than 100 μg

or 1-cm cell for quantities from 100 to 500 μg. As reference, use either water or a sample blank

prepared by carrying the specified quantities of acids through the entire analytical procedure. If

water is used as reference, correct sample absorbance by subtracting absorbance of a sample

blank.

Determine micrograms of iron in the sample from the absorbance (corrected, if necessary) by

reference to the calibration curve prepared by using a suitable range of iron standards containing

the same amounts of phenanthroline, hydroxylamine, and sodium acetate as the sample.

5. Calculation

When the sample has been treated according to ¶ 4a, ¶ 4b, ¶ 4c, or ¶ 4e2):

When the sample has been treated according to ¶ 4e1):

Report details of sample collection, storage, and pretreatment if they are pertinent to

interpretation of results.

6. Precision and Bias

Precision and bias depend on the method of sample collection and storage, the method of

color measurement, the iron concentration, and the presence of interfering color, turbidity, and

foreign ions. In general, optimum reliability of visual comparison in nessler tubes is not better

than 5% and often only 10%, whereas, under optimum conditions, photometric measurement

may be reliable to 3% or 3 μg, whichever is greater. The sensitivity limit for visual observation

in nessler tubes is approximately 1 μg Fe. Sample variability and instability may affect precision

and bias of this determination more than will the errors of analysis. Serious divergences have

been found in reports of different laboratories because of variations in methods of collecting and

treating samples.

A synthetic sample containing 300 μg Fe/L, 500 μg Al/L, 50 μg Cd/L, 110 μg Cr/L, 470 μg

Cu/L, 70 μg Pb/L, 120 μg Mn/L, 150 μg Ag/L, and 650 μg Zn/L in distilled water was analyzed in 44 laboratories by the phenanthroline method, with a relative standard deviation of 25.5% and a relative error of 13.3%.

3500-Pb LEAD*#(108)

3500-Pb A. Introduction

1. Occurrence and Significance

Lead (Pb) is the fifth element in Group IVA in the periodic table; it has an atomic number of

82, an atomic weight of 207.19, and valences of 2 and 4. The average abundance of Pb in the

earth’s crust is 13 ppm; in soils it ranges from 2.6 to 25 ppm; in streams it is 3 μg/L, and in

groundwaters it is generally <0.1 mg/L. Lead is obtained chiefly from galena (PbS). It is used in

batteries, ammunition, solder, piping, pigments, insecticides, and alloys. Lead also was used in

gasoline for many years as an anti-knock agent in the form of tetraethyl lead.

The common aqueous species are Pb2+ and hydroxide and carbonate complexes. Lead in a

water supply may come from industrial, mine, and smelter discharges or from the dissolution of

plumbing and plumbing fixtures. Tap waters that are inherently noncorrosive or not suitably

treated may contain lead resulting from an attack on lead service pipes, lead interior plumbing,

brass fixtures and fittings, or solder pipe joints.

Lead is nonessential for plants and animals. It is toxic by ingestion and is a cumulative

poison. The Food and Drug Administration regulates lead content in food and in house paints.

Under the lead-copper rule, the U.S. EPA drinking water 90th percentile action level is 15 μg/L.

2. Selection of Method

The atomic absorption spectrometric method (Section 3111B) has a relatively high detection

limit in the flame mode and requires an extraction procedure (Section 3111C) for the low

concentrations common in potable water. The electrothermal atomic absorption (AA) method

(Section 3113B) is more sensitive for low concentrations and does not require extraction. The

inductively coupled plasma/mass spectrometric method (Section 3125) is even more sensitive

than the electrothermal AA method. The inductively coupled plasma method (Section 3120) has

a sensitivity similar to that of the flame atomic absorption method. Anodic stripping

voltammetry (Section 3130B) can achieve superior detection limits, but is susceptible to

interferences from copper, silver, gold, and organic compounds. The dithizone method (B) is

sensitive and specific as a colorimetric procedure.

3500-Pb B. Dithizone Method

1. General Discussion

a. Principle: An acidified sample containing microgram quantities of lead is mixed with

ammoniacal citrate-cyanide reducing solution and extracted with dithizone in chloroform

(CHCl3) to form a cherry-red lead dithizonate. The color of the mixed color solution is measured

photometrically.1,2 Sample volume taken for analysis may be 2 L when digestion is used.

b. Interference: In a weakly ammoniacal cyanide solution (pH 8.5 to 9.5) dithizone forms colored complexes with bismuth, stannous tin, and monovalent thallium. In strongly ammoniacal

citrate-cyanide solution (pH 10 to 11.5) the dithizonates of these ions are unstable and are

extracted only partially.3 This method uses a high pH, mixed color, single dithizone extraction.

Interference from stannous tin and monovalent thallium is reduced further when these ions are

oxidized during preliminary digestion. A modification of the method allows detection and

elimination of bismuth interference. Excessive quantities of bismuth, thallium, and tin may be

removed.4

Dithizone in CHCl3 absorbs at 510 nm; control its interference by using nearly equal

concentrations of excess dithizone in samples, standards, and blank.

The method is without interference for the determination of 0.0 to 30.0 μg Pb in the presence

of 20 μg Tl+, 100 μg Sn2+, 200 μg In3+, and 1000 μg each of Ba2+, Cd2+, Co2+, Cu2+, Mg2+,

Mn2+, Hg2+, Sr2+, Zn2+, Al3+, Sb3+, As3+, Cr3+, Fe3+, V3+, PO4

3–, and SO4

2–. Gram quantities

of alkali metals do not interfere. A modification is provided to avoid interference from excessive

quantities of bismuth or tin.

c. Preliminary sample treatment: At time of collection acidify with conc HNO3 to pH <2 but

avoid excess HNO3. Add 5 mL 0.1N iodine solution to avoid losses of volatile organo-lead

compounds during handling and digesting of samples. Prepare a blank of lead-free water and

carry through the procedure.

d. Digestion of samples: Unless digestion is shown to be unnecessary, digest all samples for

dissolved or total lead as described in 3030H or K.

e. Minimum detectable concentration: 1.0 μg Pb/10 mL dithizone solution.

2. Apparatus

a. Spectrophotometer for use at 510 nm, providing a light path of 1 cm or longer.

b. pH meter.

c. Separatory funnels: 250-mL Squibb type. Clean all glassware, including sample bottles,

with 1 + 1 HNO3. Rinse thoroughly with reagent water.

d. Automatic dispensing burets: Use for all reagents to minimize indeterminate

contamination errors.

3. Reagents

Prepare all reagents in lead-free water.

a. Stock lead solution: Dissolve 0.1599 g lead nitrate, Pb(NO3)2 (minimum purity 99.5%), in

approximately 200 mL water. Add 10 mL conc HNO3 and dilute to 1000 mL with water.

Alternatively, dissolve 0.1000 g pure Pb metal in 20 mL 1 + 1 HNO3 and dilute to 1000 mL with

water; 1.00 mL = 100 μg Pb.

b. Working lead solution: Dilute 2.0 mL stock solution to 100 mL with water; 1 mL = 2.00 μg Pb.

c. Nitric acid, HNO3, 1 + 4: Dilute 200 mL conc HNO3 to 1 L with water.

d. Ammonium hydroxide, NH4OH, 1 + 9: Dilute 10 mL conc NH4OH to 100 mL with water.

e. Citrate-cyanide reducing solution: Dissolve 400 g dibasic ammonium citrate,

(NH4)2HC6H5O7, 20 g anhydrous sodium sulfite, Na2SO3, 10 g hydroxylamine hydrochloride,

NH2OH⋅HCl, and 40 g potassium cyanide, KCN (CAUTION: Poison) in water and dilute to 1 L.

Mix this solution with 2 L conc NH4OH. Do not pipet by mouth. Prepare solution in a fume

hood.

f. Stock dithizone solution: The dithizone concentration in the stock dithizone solutions is

based on having a 100% pure dithizone reagent. Some commercial grades of dithizone are

contaminated with the oxidation product diphenylthiocarbodiazone or with metals. Purify

dithizone as directed below. For dithizone solutions not stronger than 0.001% (w/v), calculate

the exact concentration by dividing the absorbance of the solution in a 1.00-cm cell at 606 nm by

40.6 × 103, the molar absorptivity.

In a fume hood, dissolve 100 mg dithizone in 50 mL CHCl3 in a 150-mL beaker and filter

through a 7-cm-diam paper.*#(109) Receive filtrate in a 500-mL separatory funnel or in a

125-mL erlenmeyer flask under slight vacuum; use a filtering device designed to handle the

CHCl3 vapor. Wash beaker with two 5-mL portions CHCl3, and filter. Wash the paper with three

5-mL portions CHCl3, adding final portion dropwise to edge of paper. If filtrate is in flask,

transfer with CHCl3 to a 500-mL separatory funnel.

Add 100 mL 1 + 99 NH4OH to separatory funnel and shake moderately for 1 min; excessive

agitation produces slowly breaking emulsions. Let layers separate, swirling funnel gently to

submerge CHCl3 droplets held on surface of aqueous layer. Transfer CHCl3 layer to 250-mL

separatory funnel, retaining the orange-red aqueous layer in the 500-mL funnel. Repeat

extraction, receiving CHCl3 layer in another 250-mL separatory funnel and transferring aqueous

layer, using 1 + 99 NH4OH, to the 500-mL funnel holding the first extract. Repeat extraction,

transferring the aqueous layer to 500-mL funnel. Discard CHCl3 layer.

To combined extracts in the 500-mL separatory funnel add 1 + 1 HCl in 2-mL portions,

mixing after each addition, until dithizone precipitates and solution is no longer orange-red.

Extract precipitated dithizone with three 25-mL portions CHCl3. Dilute combined extracts to

1000 mL with CHCl3; 1.00 mL = 100 μg dithizone.

g. Dithizone working solution: Dilute 100 mL stock dithizone solution to 250 mL with

CHCl3; 1 mL = 40 μg dithizone.

h. Special dithizone solution: Dissolve 250 mg dithizone in 250 mL CHCl3. This solution

may be prepared without purification because all extracts using it are discarded.

i. Sodium sulfite solution: Dissolve 5 g anhydrous Na2SO3 in 100 mL water.

j. Iodine solution: Dissolve 40 g KI in 25 mL water, add 12.7 g resublimed iodine, and dilute

to 1000 mL.

4. Procedure

a. With sample digestion: CAUTION: Perform the following procedure (excluding use of

spectrophotometer) in a fume hood. To a digested sample containing not more than 1 mL conc

acid add 20 mL 1 + 4 HNO3 and filter through lead-free filter paper†#(110) and filter funnel

directly into a 250-mL separatory funnel. Rinse digestion beaker with 50 mL water and add to

filter. Add 50 mL ammoniacal citrate-cyanide solution, mix, and cool to room temperature. Add

10 mL dithizone working solution, shake stoppered funnel vigorously for 30 s, and let layers

separate. Insert lead-free cotton in stem of separatory funnel and draw off lower layer. Discard 1

to 2 mL CHCl3 layer, then fill absorption cell. Measure absorbance of extract at 510 nm, using

dithizone working solution, ¶ 3g, to zero spectrophotometer.

b. Without sample digestion: To 100 mL acidified sample (pH 2) in a 250-mL separatory

funnel add 20 mL 1 + 4 HNO3 and 50 mL citrate-cyanide reducing solution; mix. Add 10 mL

dithizone working solution and proceed as in ¶ 4a.

c. Calibration curve: Plot concentration of at least five standards and a blank against

absorbance. Determine concentration of lead in extract from curve. All concentrations are μg

Pb/10 mL final extract.

d. Removal of excess interferences: The dithizonates of bismuth, tin, and thallium differ

from lead dithizonate in maximum absorbance. Detect their presence by measuring sample

absorbance at 510 nm and at 465 nm. Calculate corrected absorbance of sample at each

wavelength by subtracting absorbance of blank at same wavelength. Calculate ratio of corrected

absorbance at 510 nm to corrected absorbance at 465 nm. The ratio of corrected absorbances for

lead dithizonate is 2.08 and for bismuth dithizonate is 1.07. If the ratio for the sample indicates

interference, i.e., is markedly less than 2.08, proceed as follows with a new 100-mL sample: If

the sample has not been digested, add 5 mL Na2SO3 solution to reduce iodine preservative.

Adjust sample to pH 2.5 using a pH meter and 1 + 4 HNO3 or 1 + 9 NH4OH as required.

Transfer sample to 250-mL separatory funnel, extract with a minimum of three 10-mL portions

special dithizone solution, or until the CHCl3 layer is distinctly green. Extract with 20-mL

portions CHCl3 to remove dithizone (absence of green). Add 20 mL 1 + 4 HNO3, 50 mL

citrate-cyanide reducing solution, and 10 mL dithizone working solution. Extract as in ¶ 4a and

measure absorbance.

5. Calculation

6. Precision and Bias

Single-operator precision in recovering 0.0104 mg Pb/L from Mississippi River water was

6.8% relative standard deviation and −1.4% relative error. At the level of 0.026 mg Pb/L,

recovery was made with 4.8% relative standard deviation and 15% relative error.

3500-Mg MAGNESIUM*#(112)

3500-Mg A. Introduction

1. Occurrence and Significance

Magnesium (Mg) is the second element in Group IIA of the periodic table; it has an atomic

number of 12, an atomic weight of 24.30, and a valence of 2. The average abundance of Mg in

the earth’s crust is 2.1%; in soils it is 0.03 to 0.84%; in streams it is 4 mg/L, and in groundwaters

it is >5 mg/L. Magnesium occurs commonly in the minerals magnesite and dolomite.

Magnesium is used in alloys, pyrotechnics, flash photography, drying agents, refractories,

fertilizers, pharmaceuticals, and foods.

The common aqueous species is Mg2+. The carbonate equilibrium reactions for magnesium

are more complicated than for calcium, and conditions for direct precipitation of dolomite in

natural waters are not common. Important contributors to the hardness of a water, magnesium

salts break down when heated, forming scale in boilers. Chemical softening, reverse osmosis, or

ion exchange reduces magnesium and associated hardness to acceptable levels.

Magnesium is an essential element in chlorophyll and in red blood cells. Some salts of

magnesium are toxic by ingestion or inhalation. Concentrations greater than 125 mg/L also can

have a cathartic and diuretic effect.

2. Selection of Method

The methods presented are applicable to waters and wastewaters. Direct determinations can be made with the atomic absorption spectrometric method (Section 3111B) and inductively

coupled plasma method (Section 3120). The inductively coupled plasma mass spectrometric

method (Section 3125) also may be applied successfully in most cases (with lower detection

limits), even though magnesium is not specifically listed as an analyte in the method. These

methods can be applied to most concentrations encountered, although sample dilution may be

required. Choice of method is largely a matter of personal preference and analyst experience. A

calculation method (B) also is available.

3500-Mg B. Calculation Method

Magnesium may be estimated as the difference between hardness and calcium as CaCO3 if

interfering metals are present in noninterfering concentrations in the calcium titration (Section

3500-Ca.B) and suitable inhibitors are used in the hardness titration (Section 2340C).

mg Mg/L = [total hardness (as mg CaCO3/L) − calcium hardness (as mg CaCO3/L)] × 0.243

3500-Mn MANGANESE*#(113)

3500-Mn A. Introduction

1. Occurrence and Significance

Manganese (Mn) is the first element in Group VIIB in the periodic table; it has an atomic

number of 25, an atomic weight of 54.94, and common valences of 2, 4, and 7 (and more rarely,

valences of 1, 3, 5, and 6). The average abundance of Mn in the earth’s crust is 1060 ppm; in

soils it is 61 to 1010 ppm; in streams it is 7 μg/L, and in groundwaters it is <0.1 mg/L.

Manganese is associated with iron minerals, and occurs in nodules in ocean, fresh waters, and

soils. The common ores are pyrolusite (MnO2) and psilomelane. Manganese is used in steel

alloys, batteries, and food additives.

The common aqueous species are the reduced Mn2+ and the oxidized Mn4+. The aqueous

chemistry of manganese is similar to that of iron. Since groundwater is often anoxic, any soluble

manganese in groundwater is usually in the reduced state (Mn2+). Upon exposure to air or other

oxidants, groundwater containing manganese usually will precipitate black MnO2. Elevated

manganese levels therefore can cause stains in plumbing/laundry, and cooking utensils. It is

considered an essential trace element for plants and animals. The United Nations Food and

Agriculture Organization recommended maximum level for manganese in irrigation waters is 0.2

mg/L. The U.S. EPA secondary drinking water standard MCL is 50 μg/L.

2. Selection of Method

The atomic absorption spectrometric methods (Section 3111B and Section 3111C), the

electrothermal atomic absorption method (Section 3113B), and the inductively coupled plasma

methods (Section 3120 and Section 3125) permit direct determination with acceptable

sensitivity and are the methods of choice. Of the various colorimetric methods, the persulfate

method (B) is preferred because the use of mercuric ion can control interference from a limited

chloride ion concentration.

3. Sampling and Storage

Manganese may exist in a soluble form in a neutral water when first collected, but it

oxidizes to a higher oxidation state and precipitates or becomes adsorbed on the container walls.

Determine manganese very soon after sample collection. When delay is unavoidable, total

manganese can be determined if the sample is acidified at the time of collection with HNO3 to

pH <2. See Section 3010B.

3500-Mn B. Persulfate Method

1. General Discussion

a. Principle: Persulfate oxidation of soluble manganous compounds to form permanganate is

carried out in the presence of silver nitrate. The resulting color is stable for at least 24 h if excess

persulfate is present and organic matter is absent.

b. Interference: As much as 0.1 g chloride (Cl–) in a 50-mL sample can be prevented from

interfering by adding 1 g mercuric sulfate (HgSO4) to form slightly dissociated complexes.

Bromide and iodide still will interfere and only trace amounts may be present. The persulfate

procedure can be used for potable water with trace to small amounts of organic matter if the

period of heating is increased after more persulfate has been added.

For wastewaters containing organic matter, use preliminary digestion with nitric and sulfuric

acids (HNO3 and H2SO4) (see Section 3030G). If large amounts of Cl– also are present, boiling

with HNO3 helps remove it. Interfering traces of Cl– are eliminated by HgSO4 in the special

reagent.

Colored solutions from other inorganic ions are compensated for in the final colorimetric

step.

Samples that have been exposed to air may give low results due to precipitation of

manganese dioxide (MnO2). Add 1 drop 30% hydrogen peroxide (H2O2) to the sample, after

adding the special reagent, to redissolve precipitated manganese.

c. Minimum detectable concentration: The molar absorptivity of permanganate ion is about

2300 L g–1 cm–1. This corresponds to a minimum detectable concentration (98% transmittance) of 210 μg Mn/L when a 1-cm cell is used or 42 μg Mn/L when a 5-cm cell is used.

2. Apparatus

Colorimetric equipment: One of the following is required:

a. Spectrophotometer, for use at 525 nm, providing a light path of 1 cm or longer.

b. Filter photometer, providing a light path of 1 cm or longer and equipped with a green

filter having maximum transmittance near 525 nm.

c. Nessler tubes, matched, 100-mL, tall form.

3. Reagents

a. Special reagent: Dissolve 75 g HgSO4 in 400 mL conc HNO3 and 200 mL distilled water.

Add 200 mL 85% phosphoric acid (H3PO4), and 35 mg silver nitrate (AgNO3). Dilute the cooled

solution to 1 L.

b. Ammonium persulfate, (NH4)2S2O8, solid.

c. Standard manganese solution: Prepare a 0.1N potassium permanganate (KMnO4) solution

by dissolving 3.2 g KMnO4 in distilled water and making up to 1 L. Age for several weeks in

sunlight or heat for several hours near the boiling point, then filter through a fine fritted-glass

filter crucible and standardize against sodium oxalate as follows:

Weigh several 100- to 200-mg samples of Na2C2O4 to 0.1 mg and transfer to 400-mL

beakers. To each beaker, add 100 mL distilled water and stir to dissolve. Add 10 mL 1 + 1

H2SO4 and heat rapidly to 90 to 95°C. Titrate rapidly with the KMnO4 solution to be

standardized, while stirring, to a slight pink end-point color that persists for at least 1 min. Do

not let temperature fall below 85°C. If necessary, warm beaker contents during titration; 100 mg

Na2C2O4 will consume about 15 mL permanganate solution. Run a blank on distilled water and

H2SO4.

where:

A = mL titrant for sample and

B = mL titrant for blank.

Average results of several titrations. Calculate volume of this solution necessary to prepare 1

L of solution so that 1.00 mL = 50.0 μg Mn, as follows:

To this volume add 2 to 3 mL conc H2SO4 and NaHSO3 solution dropwise, with stirring, until

the permanganate color disappears. Boil to remove excess SO2, cool, and dilute to 1000 mL with

distilled water. Dilute this solution further to measure small amounts of manganese.

d. Standard manganese solution (alternate): Dissolve 1.000 g manganese metal (99.8%

min.) in 10 mL redistilled HNO3. Dilute to 1000 mL with 1% (v/v) HCl; 1 mL = 1.000 mg Mn.

Dilute 10 mL to 200 mL with distilled water; 1 mL = 0.05 mg Mn. Prepare dilute solution daily.

e. Hydrogen peroxide, H2O2, 30%.

f. Nitric acid, HNO3, conc.

g. Sulfuric acid, H2SO4, conc.

h. Sodium nitrite solution: Dissolve 5.0 g NaNO2 in 95 mL distilled water.

i. Sodium oxalate, Na2C2O4, primary standard.

j. Sodium bisulfite: Dissolve 10 g NaHSO3 in 100 mL distilled water.

4. Procedure

a. Treatment of sample: If a digested sample has been prepared according to directions for

reducing organic matter and/or excessive chlorides in Section 3030G, pipet a portion containing

0.05 to 2.0 mg Mn into a 250-mL conical flask. Add distilled water, if necessary, to 90 mL and

proceed as in ¶ b.

b. To a suitable sample portion add 5 mL special reagent and 1 drop H2O2. Concentrate to

90 mL by boiling or dilute to 90 mL. Add 1 g (NH4)2S2O8, bring to a boil, and boil for 1 min.

Do not heat on a water bath. Remove from heat source, let stand 1 min, then cool under the tap.

(Boiling too long results in decomposition of excess persulfate and subsequent loss of

permanganate color; cooling too slowly has the same effect.) Dilute to 100 mL with distilled

water free from reducing substances and mix. Prepare standards containing 0, 5.00, . . . 1500 μg

Mn by treating various amounts of standard Mn solution in the same way.

c. Nessler tube comparison: Use standards prepared as in ¶ 4b and containing 5 to 100 μg

Mn/100 mL final volume. Compare samples and standards visually.

d. Photometric determination: Use a series of standards from 0 to 1500μg Mn/100mL final

volume. Make photometric measurements against a distilled water blank. The following table

shows light path length appropriate for various amounts of manganese in 100mL final volume:

shows light path length appropriate for various amounts of manganese in 100mL final volume:

Mn Range                                                       Light Path

μg                                                                      cm

5–200                                                     15

20–400                                                    5

50–1000                                                  2

100–1500                                                1

Prepare a calibration curve of manganese concentration vs. absorbance from the standards

and determine Mn in the samples from the curve. If turbidity or interfering color is present, make

corrections as in 4e.

e. Correction for turbidity or interfering color: Avoid filtration because of possible retention

of some permanganate on the filter paper. If visual comparison is used, the effect of turbidity

only can be estimated and no correction can be made for interfering colored ions. When

photometric measurements are made, use the following ‘‘bleaching’’ method, which also

corrects for interfering color: As soon as the photometer reading has been made, add 0.05 mL

H2O2 solution directly to the sample in the optical cell. Mix and, as soon as the permanganate

color has faded completely and no bubbles remain, read again. Deduct absorbance of bleached

solution from initial absorbance to obtain absorbance due to Mn.

5. Calculation

a. When all of the original sample is taken for analysis:

b. When a portion of the digested sample (100 mL final volume) is taken for analysis:

6. Precision and Bias

A synthetic sample containing 120 μg Mn/L, 500 μg Al/L, 50 μg Cd/L, 110 μg Cr/L, 470 μg Cu/L, 300 μg Fe/L, 70 μg Pb/L, 150 μg Ag/L, and 650 μg Zn/L in distilled water was analyzed in 33 laboratories by the persulfate method, with a relative standard deviation of 26.3% and a relative error of 0%.

A second synthetic sample, similar in all respects except for 50 μg Mn/L and 1000 μg Cu/L,

was analyzed in 17 laboratories by the persulfate method, with a relative standard deviation of

50.3% and a relative error of 7.2%.

3500-Hg MERCURY

Mercury (Hg) is the third element in Group IIB in the periodic table; it has an atomic

number of 80, an atomic weight of 200.59, and valences of 1 and 2. The average abundance of

Hg in the earth’s crust is 0.09 ppm; in soils it is 30 to 160 ppb; in streams it is 0.07 μg/L, and in

groundwaters it is 0.5 to 1 μg/L. Mercury occurs free in nature, but the chief source is cinnibar

(HgS). Mercury is used in amalgams, mirror coatings, vapor lamps, paints, measuring devices

(thermometers, barometers, manometers), pharmaceuticals, pesticides, and fungicides. It is often

used in paper mills as a mold retardant for paper.

The common aqueous species are Hg2+, Hg(OH)2

0, Hg0, and stable complexes with organic

ligands. Inorganic mercury can be methylated in sediments when sulfides are present to form

dimethyl mercury, (CH3)2Hg, which is very toxic and can concentrate in the aquatic food chain.

Mercury poisoning occurred in Japan in the 1950s as the result of consumption of shellfish that

had accumulated mercury. In times past, mercury was used in the haberdashery industry to block

hats (the cause of the ‘‘mad hatter’’ syndrome).

Mercury is considered nonessential for plants and animals. The U.S. EPA primary drinking

water standard MCL is 2 μg/L.

The cold-vapor atomic absorption method (Section 3112B) is the method of choice for all

samples. The inductively coupled plasma mass spectrometric method (Section 3125) also may be

applied successfully in some cases, even though mercury is not specifically listed as an analyte

in the method. The dithizone method detailed in the 19th edition of Standard Methods can be

used for determining high levels of mercury (>2 μg/L) in potable waters.

Because mercury can be lost readily from samples, preserve them by treating with HNO3 to

reduce the pH to <2 (see Section 1060). Glass storage containers are preferred to plastic, because

they can extend the holding time to 30 d, rather than only the 14 d allowed in plastic containers.

3500-Ni NICKEL

Nickel (Ni) is the third element in Group VIII in the periodic table; it has an atomic number

of 28, an atomic weight of 58.69, and a common valence of 2 and less commonly 1, 3, or 4. The

average abundance of Ni in the earth’s crust is 1.2 ppm; in soils it is 2.5 ppm; in streams it is 1

μg/L, and in groundwaters it is <0.1 mg/L. Nickel is obtained chiefly from pyrrhotite and

garnierite. Nickel is used in alloys, magnets, protective coatings, catalysts, and batteries.

The common aqueous species is Ni2+. In reducing conditions insoluble sulfides can form,

while in aerobic conditions nickel complexes with hydroxide, carbonates, and organic ligands

can form. It is suspected to be an essential trace element for some plants and animals. The United

Nations Food and Agriculture Organization recommended maximum level for irrigation waters

is 200 μg/L. The U.S. EPA primary drinking water standard MCL is 0.1 mg/L.

The atomic absorption spectrometric methods (Section 3111B and Section 3111C), the

inductively coupled plasma methods (Section 3120 and Section 3125), and the electrothermal

atomic absorption spectrometric method (Section 3113B) are the methods of choice for all

samples.

3500-K POTASSIUM*#(114)

3500-K A. Introduction

- Occurrence and Significance

Potassium (K) is the fourth element in Group IA of the periodic table; it has an atomic

number of 19, an atomic weight of 39.10, and a valence of 1. The average abundance of K in the

earth’s crust is 1.84%; in soils it has a range of 0.1 to 2.6%; in streams it is 2.3 mg/L, and in

groundwaters it has a range of 0.5 to 10 mg/L. Potassium is commonly associated with

aluminosilicate minerals such as feldspars. 40K is a naturally occurring radioactive isotope with

a half-life of 1.3 × 109 years. Potassium compounds are used in glass, fertilizers, baking powder,

soft drinks, explosives, electroplating, and pigments. Potassium is an essential element in both

plant and human nutrition, and occurs in groundwaters as a result of mineral dissolution, from

decomposing plant material, and from agricultural runoff.

The common aqueous species is K+. Unlike sodium, it does not remain in solution, but is

assimilated by plants and is incorporated into a number of clay-mineral structures.

2. Selection of Method

Methods for the determination of potassium include flame atomic absorption (Section

3111B), inductively coupled plasma (Section 3120), flame photometry (B), and selective ion

electrode (C). The inductively coupled plasma/mass spectrometric method (Section 3125)

usually may be applied successfully (with lower detection limits), even though potassium is not

specifically listed as an analyte in the method. The preferred methods are rapid, sensitive, and

accurate; selection depends on instrument availability and analyst choice.

3. Storage of Samples

Do not store samples in soft-glass bottles because of the possibility of contamination from

leaching of the glass. Use acid-washed polyethylene or borosilicate glass bottles. Adjust sample

to pH < 2 with nitric acid. This will dissolve potassium salts and reduce adsorption on vessel

walls.

3500-K B. Flame Photometric Method

1. General Discussion

a. Principle: Trace amounts of potassium can be determined in either a direct-reading or

internal-standard type of flame photometer at a wavelength of 766.5 nm. Because much of the

information pertaining to sodium applies equally to the potassium determination, carefully study

the entire discussion dealing with the flame photometric determination of sodium (Section

3500-Na.B) before making a potassium determination.

b. Interference: Interference in the internal-standard method may occur at

sodium-to-potassium ratios of 5:1 or greater. Calcium may interfere if the calcium-to-potassium

ratio is 10:1 or more. Magnesium begins to interfere when the magnesium-to-potassium ratio

exceeds 100:1.

c. Minimum detectable concentration: Potassium levels of approximately 0.1 mg/L can be determined.

2. Apparatus

See Section 3500-Na.B.2.

3. Reagents

To minimize potassium pickup, store all solutions in plastic bottles. Shake each container

thoroughly to dissolve accumulated salts from walls before pouring.

a. Reagent water: See Section 1080. Use this water for preparing all reagents and calibration

standards, and as dilution water.

b. Stock potassium solution: Dissolve 1.907 g KCl dried at 110°C and dilute to 1000 mL

with water; 1 mL = 1.00 mg K.

c. Intermediate potassium solution: Dilute 10.0 mL stock potassium solution with water to

100 mL; 1.00 mL = 0.100 mg K.

Use this solution to prepare calibration curve in potassium range of 1 to 10 mg/L.

d. Standard potassium solution: Dilute 10.0 mL intermediate potassium solution with water

to 100 mL; 1.00 mL = 0.010 mg K. Use this solution to prepare calibration curve in potassium

range of 0.1 to 1.0 mg/L.

4. Procedure

Make determination as described in Section 3500-Na.B.4, but measure emission intensity at

766.5 nm.

5. Calculation

See Section 3500-Na.B.5.

6. Precision and Bias

A synthetic sample containing 3.1 mg K+/L, 108 mg Ca2+/L, 82 mg Mg2+/L, 19.9 mg

Na+/L, 241 mg Cl–/L, 0.25 mg NO2

–-N/L, 1.1 mg NO3

–-N/L, 259 mg SO4

2–/L, and 42.5 mg

total alkalinity/L (contributed by NaHCO3) was analyzed in 33 laboratories by the flame

photometric method, with a relative standard deviation of 15.5% and a relative error of 2.3%.

3500-K C. Potassium-Selective Electrode Method

1. General Discussion

a. Principle: Potassium ion is measured potentiometrically by using a potassium

ion-selective electrode and a double-junction, sleeve-type reference electrode. The analysis is

performed with either a pH meter having an expanded millivolt scale capable of being read to the

nearest 0.1 mV or a specific ion meter having a direct concentration scale for potassium.

Before measurement, an ionic strength adjustor reagent is added to both standards and

samples to maintain a constant ionic strength. The electrode response is measured in standard

solutions with potassium concentrations spanning the range of interest using a calibration line

derived either by the instrument meter or manually. The electrode response in sample solutions

is measured following the same procedure and potassium concentration determined from the

calibration line or instrument direct readout.

b. Interferences: Although most sensitive to potassium, the potassium electrode will respond

to other cations at high concentrations; this can result in a positive bias. Table 3500-K:I lists the

concentration of common cations causing a 10% error at various concentrations of potassium

chloride with a background ionic strength of 0.12N sodium chloride. Of the cations listed,

ammonium ion is most often present in samples at concentrations high enough to result in a

significant bias. It can be converted to gaseous ammonia by adjusting to pH > 10.

An electrode exposed to interfering cations tends to drift and respond sluggishly. To restore

normal performance soak electrode for 1 h in distilled water and then for several hours in a

standard potassium solution.

c. Detection limits: Samples containing from 0.1 to 1000 mg K+/L may be analyzed. To

measure higher concentrations dilute the sample.

2. Apparatus

a. Expanded-scale or digital pH meter or ion-selective meter.

b. Potassium ion-selective electrode.

c. Sleeve-type double-junction reference electrode: Fill outer sleeve with reference electrode

filling solution (see ¶ 3b). Fill inner sleeve with inner filling solution provided with the

electrode.

d. pH electrode.

e. Mixer, magnetic, with a TFE-coated stirring bar.

3. Reagents

a. Ionic strength adjustor (ISA): Dissolve 29.22 g NaCl in reagent water and dilute to 100

mL.

b. Reference electrode outer sleeve filling solution: Dilute 2 mL ISA solution to 100 mL

with reagent water.

- c. Stock potassium solution: See Section 3500-K.B.3b.

d. Sodium hydroxide, NaOH, 6N.

e. Reagent water: See Section 1080.

4. Procedure

a. Preparation of standards: Prepare a series of standards containing 100.0, 10.0, 1.0, and

0.1 mg K+/L by making serial dilutions of the stock potassium solution as in Section

3500-K.B.3c and Section 3500-K.B.3d.

b. Instrument calibration: Fill reference electrode according to the manufacturer’s

instructions using reference electrode filling solution. Transfer 100 mL 0.1 mg K+/L standard

into a 150-mL beaker and add 2 mL ISA. Raise pH to about 11. Stir gently with magnetic mixer.

Immerse electrodes, wait approximately 2 min for potential stabilization and record meter

reading. Thoroughly rinse electrodes and blot dry. Repeat for each standard solution in order of

increasing concentration. Prepare calibration curve on semilogarithmic graph paper by plotting

observed potential in millivolts (linear scale) against concentration (log scale). Alternatively,

calculate calibration line by regression analysis.

c. Analysis of samples: Transfer 100 mL sample into a 150-mL beaker and follow procedure

applied to standards in ¶ 4b above. From the measured response, calculate K+ concentration

from calibration curve.

5. Precision

Reproducibility of potential measured, over the method’s range, can be expected to be ± 0.4

mV, corresponding to about ± 2.5% in concentration.

6. Quality Assurance

The slope of the calibration line should be −56 mV/10-fold concentration change. If the

slope is outside the range of −56 ±3 mV, the electrode may require maintenance (replace filling

solutions). If the proper electrode response cannot be obtained, replace electrode.

Analyze an independent check standard with a mid-range potassium concentration

throughout analysis of a series, initially, every ten samples, and after final sample. If the value

has changed by more than 5%, recalibrate electrode. Analyze a reagent blank at the same

frequency. Readings must represent a lower concentration than the lowest concentration standard

(0.1 mg/L).

3500-Ag SILVER

Silver (Ag) is the second element in Group IB of the periodic table; it has an atomic number

of 47, an atomic weight of 107.87, and valences of 1 and 2. The average abundance of Ag in the

earth’s crust is 0.08 ppm; in soils it is <0.01 to 0.5 ppm; in streams it is 0.3 μg/L; in U.S.

drinking waters it is 0.23 μg/L, and in groundwater it is <0.1 μg/L. Silver occurs in its native

state and in combination with many nonmetallic elements such as argentite (Ag2S) and horn

silver (AgCl). Lead and copper ores also may yield considerable silver. Silver is widely used in

photography, silverware, jewelry, mirrors, and batteries. Silver iodide has been used in the

seeding of clouds, and silver oxide to a limited extent is used as a disinfectant for water.

In acidic water Ag+ would predominate, and in high-chloride water a series of complexes

would be expected. Silver is nonessential for plants and animals. Silver can cause argyria, a

permanent, blue-gray discoloration of the skin and eyes that imparts a ghostly appearance.

Concentrations in the range of 0.4 to 1 mg/ L have caused pathological changes in the kidneys,

liver, and spleen of rats. Toxic effects on fish in fresh water have been observed at

concentrations as low as 0.17 μg/L. For freshwater aquatic life, total recoverable silver should not exceed 1.2 mg/L.

The atomic absorption spectrometric methods (Section 3111B and Section 3111C) and the

inductively coupled plasma methods (Section 3120 and Section 3125) are preferred. The

electrothermal atomization method (Section 3113B) is the most sensitive for determining silver

in natural waters. The dithizone method detailed in the 19th edition of Standard Methods can be

used when an atomic absorption spectrometer is unavailable. A method suitable for analysis of

silver in industrial or other wastewaters at levels above 1 mg/L is available.1

If total silver is to be determined, acidify sample with conc nitric acid (HNO3) to pH <2 at

time of collection. If sample contains particulate matter and only the ‘‘dissolved’’ metal content

is to be determined, filter through a 0.45-μm membrane filter at time of collection. After

filtration, acidify filtrate with HNO3 to pH <2. Complete analysis as soon after collection as

possible. Some samples may require special storage and digestion; see Section 3030D.

3500-Na SODIUM*#(124)

3500-Na A. Introduction

1. Occurrence and Significance

Sodium (Na) is the third element in Group IA of the periodic table; it has an atomic number

of 11, an atomic weight of 22.99, and a valence of 1. The average abundance of Na in the earth’s

crust is 2.5%; in soils it is 0.02 to 0.62%; in streams it is 6.3 mg/L, and in groundwaters it is

generally >5 mg/L. Sodium occurs with silicates and with salt deposits. Sodium compounds are

used in many applications, including caustic soda, salt, fertilizers, and water treatment

chemicals.

Sodium is very soluble, and its monovalent ion Na+ can reach concentrations as high as 15

000 mg/L in equilibrium with sodium bicarbonate. The ratio of sodium to total cations is

important in agriculture and human physiology. Soil permeability can be harmed by a high

sodium ratio. In large concentrations it may affect persons with cardiac difficulties. A limiting

concentration of 2 to 3 mg/L is recommended in feedwaters destined for high-pressure boilers.

When necessary, sodium can be removed by the hydrogen-exchange process or by distillation.

The U.S. EPA advisory limit for sodium in drinking water is 20 mg/L.

- Selection of Method

Method Section 3111B uses an atomic absorption spectrometer in the flame absorption

mode. Method Section 3120B uses inductively coupled plasma; this method is not as sensitive as

the other methods, but usually this is not important. Method Section 3500-Na.B uses either a

flame photometer or an atomic absorption spectrometer in the flame emission mode. The

inductively coupled plasma/mass spectrometric method (Section 3125) also may be applied

successfully in most cases (with lower detection limits), even though sodium is not specifically

listed as an analyte in the method. When all of these instruments are available, the choice will

depend on factors including relative quality of the instruments, precision and sensitivity

required, number of samples and analytes per sample, matrix effects, and relative ease of

instrument operation. If an atomic absorption spectrometer is used, operation in the emission

mode is preferred.

3. Storage of Sample

Store alkaline samples or samples containing low sodium concentrations in polyethylene

bottles to eliminate the possibility of sample contamination due to leaching of the glass container.

3500-Na B. Flame Emission Photometric Method

1. General Discussion

a. Principle: Trace amounts of sodium can be determined by flame emission photometry at

589 nm. Sample is nebulized into a gas flame under carefully controlled, reproducible excitation

conditions. The sodium resonant spectral line at 589 nm is isolated by interference filters or by

light-dispersing devices such as prisms or gratings. Emission light intensity is measured by a

phototube, photomultiplier, or photodiode. The light intensity at 589 nm is approximately

proportional to the sodium concentration. Alignment of the wavelength dispersing device and

wavelength readout may not be precise. The appropriate wavelength setting, which may be

slightly more or less than 589 nm, can be determined from the maximum emission intensity

when aspirating a sodium standard solution, and then used for emission measurements. The

calibration curve may be linear but has a tendency to level off or even reverse at higher

concentrations. Work in the linear to near-linear range.

b. Interferences: Minimize interference by incorporation of one or more of the following:

1) Operate at the lowest practical concentration range.

2) Add releasing agents, such as strontium or lanthanum at 1000 mg/L, to suppress

ionization and anion interference. Among common anions capable of causing interference are

Cl–, SO4

2– and HCO3

– in relatively large amounts.

3) Matrix-match standards and samples by adding identical amounts of interfering

substances present in the sample to calibration standards.

4) Apply an experimentally determined correction in those instances where the sample contains a single important interference.

5) Remove interfering ions.

6) Remove burner-clogging particulate matter from the sample by filtration through a filter

paper of medium retentiveness.

7) Use the standard addition technique as described in the flame photometric method for

strontium (Section 3500-Sr.B). The method involves preparing a calibration curve using the

sample matrix as a diluent, and determining the sample concentration either mathematically or

graphically.

8) Use the internal standard technique. Potassium and calcium interfere with sodium

determination by the internal-standard method if the potassium-to-sodium ratio is ≥5:1 and the

calcium-to-sodium ratio is ≥10:1. When these ratios are exceeded, determine calcium and

potassium concentrations and matrix-match sodium calibration standards by addition of

approximately equivalent concentrations of interfering ions. Interference from magnesium is not

significant until the magnesium-to-sodium ratio exceeds 100, a rare occurrence.

b. Minimum detectable concentration: Better flame photometers or atomic absorption

spectrometers operating in the emission mode can be used to determine sodium levels

approximating 5 μg/L.

2. Apparatus

a. Flame photometer (either direct-reading or internal-standard type) or atomic absorption

spectrometer operating in the flame emission mode.

b. Glassware: Rinse all glassware with 1 + 15 HNO3 followed by several portions of reagent

water (¶ 3a).

3. Reagents

To minimize sodium contamination, store all solutions in plastic bottles. Use small

containers to reduce the amount of dry element that may be picked up from the bottle walls when

the solution is poured. Shake each container vigorously to wash accumulated salts from walls

before pouring solution.

a. Reagent water: See Section 1080. Use reagent water to prepare all reagents and

calibration standards, and as dilution water.

b. Stock sodium solution: Dissolve 2.542 g NaCl dried at 140°C to constant weight and

dilute to 1000 mL with water; 1.00 mL = 1.00 mg Na.

c. Intermediate sodium solution: Dilute 10.00 mL stock sodium solution with water to 100.0

mL; 1.00 mL = 0.10 mg Na (1.00 mL = 100 μg Na). Use this intermediate solution to prepare

calibration curve in sodium range of 1 to 10 mg/L.

d. Standard sodium solution: Dilute 10.00 mL intermediate sodium solution with water to

100 mL; 1.00 mL = 10.0 μg Na. Use this solution to prepare calibration curve in sodium range of 0.1 to 1.0 mg/L.

4. Procedure

a. Pretreatment of polluted water and wastewater samples: Follow the procedures described

in Section 3030.

b. Instrument operation: Because of differences between makes and models of instruments,

it is impossible to formulate detailed operating instructions. Follow manufacturer’s

recommendation for selecting proper photocell and wavelength, adjusting slit width and

sensitivity, appropriate fuel and oxidant gas pressures, and the steps for warm-up, correcting for

interferences and flame background, rinsing of burner, igniting flame, and measuring emission

intensity.

c. Direct-intensity measurement: Prepare a blank and sodium calibration standards in

stepped amounts in any of the following applicable ranges: 0 to 1.0, 0 to 10, or 0 to 100 mg/L.

Determine emission intensity at 589 nm. Aspirate calibration standards and samples enough

times to secure a reliable average reading for each. Construct a calibration curve from the

sodium standards. Determine sodium concentration of sample from the calibration curve. Where

a large number of samples must be run routinely, the calibration curve provides sufficient

accuracy. If greater precision and less bias are desired and time is available, use the bracketing

approach described in ¶ 4d below.

d. Bracketing approach: From the calibration curve, select and prepare sodium standards

that immediately bracket the emission intensity of the sample. Determine emission intensities of

the bracketing standards (one sodium standard slightly less and the other slightly greater than the

sample) and the sample as nearly simultaneously as possible. Repeat the determination on

bracketing standards and sample. Calculate the sodium concentration by the equation in ¶ 5b and

average the findings.

5. Calculation

a. For direct reference to the calibration curve:

mg Na/L = (mg Na/L in portion) × D

b. For the bracketing approach:

where:

B = mg Na/L in upper bracketing standard,

A = mg Na/L in lower bracketing standard,

b = emission intensity of upper bracketing standard,

a = emission intensity of lower bracketing standard,

s = emission intensity of sample, and

D = dilution ratio,

6. Precision and Bias

A synthetic sample containing 19.9 mg Na+/L, 108 mg Ca2+/L, 82 mg Mg2+/L, 3.1 mg

K+/L, 241 mg Cl–/L, 0.25 mg NO2

–-N/L, 1.1 mg NO3

–-N/L, 259 mg SO4

2–/L, and 42.5 mg total

alkalinity/L (as CaCO3) was analyzed in 35 laboratories by the flame photometric method, with

a relative standard deviation of 17.3% and a relative error of 4.0%.

3500-Zn ZINC*#(128)

3500-Zn A. Introduction

1. Occurrence and Significance

Zinc (Zn) is the first element in Group IIB in the periodic table; it has an atomic number of

30, an atomic weight of 65.38, and a valence of 2. The average abundance of Zn in the earth’s

crust is 76 ppm; in soils it is 25 to 68 ppm; in streams it is 20 μg/L, and in groundwaters it is

<0.1 mg/L. The solubility of zinc is controlled in natural waters by adsorption on mineral

surfaces, carbonate equilibrium, and organic complexes. Zinc is used in a number of alloys such

as brass and bronze, and in batteries, fungicides, and pigments. Zinc is an essential growth

element for plants and animals but at elevated levels it is toxic to some species of aquatic life.

The United Nations Food and Agriculture Organization recommended level for zinc in irrigation

waters is 2 mg/L. The U.S. EPA secondary drinking water standard MCL is 5 mg/L.

Concentrations above 5 mg/L can cause a bitter astringent taste and an opalescence in alkaline

waters. Zinc most commonly enters the domestic water supply from deterioration of galvanized

iron and dezincification of brass. In such cases lead and cadmium also may be present because

they are impurities of the zinc used in galvanizing. Zinc in water also may result from industrial

waste pollution.

2. Selection of Method

The atomic absorption spectrometric methods (Section 3111B and Section 3111C) and

inductively coupled plasma methods (Section 3120 and Section 3125) are preferred. The zincon

method (B), suitable for analysis of both potable and polluted waters, may be used if

instrumentation for the preferred methods is not available..

3. Sampling and Storage

See Section 3010B.2 for sample handling and storage.

4110 DETERMINATION OF ANIONS BY ION CHROMATOGRAPHY*#(1)

4110 A. Introduction

Because of rapid changes in technology, this section is currently undergoing substantial

revision.

Determination of the common anions such as bromide, chloride, fluoride, nitrate, nitrite,

phosphate, and sulfate often is desirable to characterize a water and/or to assess the need for

specific treatment. Although conventional colorimetric, electrometric, or titrimetric methods are

available for determining individual anions, only ion chromatography provides a single

instrumental technique that may be used for their rapid, sequential measurement. Ion

chromatography eliminates the need to use hazardous reagents and it effectively distinguishes

among the halides (Br–, Cl–, and F–) and the oxy-ions (SO3

2–, SO4

2– or NO2

–, NO3

–).

This method is applicable, after filtration to remove particles larger than 0.2 μm, to surface, ground, and wastewaters as well as drinking water. Some industrial process waters, such as

boiler water and cooling water, also may be analyzed by this method.

4110 B. Ion Chromatography with Chemical Suppression of Eluent

Conductivity

1. General Discussion

a. Principle: A water sample is injected into a stream of carbonate-bicarbonate eluent and

passed through a series of ion exchangers. The anions of interest are separated on the basis of

their relative affinities for a low capacity, strongly basic anion exchanger (guard and separator

columns). The separated anions are directed through a hollow fiber cation exchanger membrane

(fiber suppressor) or micromembrane suppressor bathed in continuously flowing strongly acid

solution (regenerant solution). In the suppressor the separated anions are converted to their

highly conductive acid forms and the carbonate-bicarbonate eluent is converted to weakly

conductive carbonic acid. The separated anions in their acid forms are measured by conductivity.

They are identified on the basis of retention time as compared to standards. Quantitation is by

measurement of peak area or peak height.

b. Interferences: Any substance that has a retention time coinciding with that of any anion to

be determined and produces a detector response will interfere. For example, relatively high

concentrations of low-molecular-weight organic acids interfere with the determination of

chloride and fluoride by isocratic analyses. A high concentration of any one ion also interferes

with the resolution, and sometimes retention, of others. Sample dilution or gradient elution

overcomes many interferences. To resolve uncertainties of identification or quantitation use the

method of known additions. Spurious peaks may result from contaminants in reagent water,

glassware, or sample processing apparatus. Because small sample volumes are used,

scrupulously avoid contamination. Modifications such as preconcentration of samples, gradient

elution, or reinjection of portions of the eluted sample may alleviate some interferences but

require individual validation for precision and bias.

c. Minimum detectable concentration: The minimum detectable concentration of an anion is

a function of sample size and conductivity scale used. Generally, minimum detectable

concentrations are near 0.1 mg/L for Br–, Cl–, NO3–, NO2–, PO43–, and SO42– with a 100-μL

sample loop and a 10-μS/cm full-scale setting on the conductivity detector. Lower values may be

achieved by using a higher scale setting, an electronic integrator, or a larger sample size.

d. Limitations: This method is not recommended for the determination of F– in unknown

matrices. Equivalency studies have indicated positive or negative bias and poor precision in

some samples. Recent interlaboratory studies show acceptable results. Two effects are common:

first, F– is difficult to quantitate at low concentrations because of the major negative contribution

of the ‘‘water dip’’ (corresponding to the elution of water); second, the simple organic acids

(formic, carbonic, etc.) elute close to fluoride and will interfere. Determine precision and bias before analyzing samples. F– can be determined accurately by ion chromatography using special

techniques such as dilute eluent or gradient elution using an NaOH eluent or alternative columns.

2. Apparatus

a. Ion chromatograph, including an injection valve, a sample loop, guard column, separator

column, and fiber or membrane suppressors, a temperature-compensated small-volume

conductivity cell and detector (6 μL or less), and a strip-chart recorder capable of full-scale

response of 2 s or less. An electronic peak integrator is optional. Use an ion chromatograph

capable of delivering 2 to 5 mL eluent/min at a pressure of 1400 to 6900 kPa.

b. Anion separator column, with styrene divinylbenzene-based low-capacity pellicular

anion-exchange resin capable of resolving Br–, Cl–, NO3–, NO2–, PO43–, and SO42–.*#(2)

c. Guard column, identical to separator column†#(3) to protect separator column from

fouling by particulates or organics.

d. Fiber suppressor or membrane suppressor:‡#(4) Cation-exchange membrane capable of

continuously converting eluent and separated anions to their acid forms. Alternatively, use

continuously regenerated suppression systems.

3. Reagents

a. Deionized or distilled water free from interferences at the minimum detection limit of each

constituent, filtered through a 0.2-μm membrane filter to avoid plugging columns, and having a

conductance of < 0.1 μS/cm.

b. Eluent solution, sodium bicarbonate-sodium carbonate, 0.0017M NaHCO3-0.0018M

Na2CO3: Dissolve 0.5712 g NaHCO3 and 0.7632 g Na2CO3 in water and dilute to 4 L.

c. Regenerant solution, H2SO4, 0.025N: Dilute 2.8 mL conc H2SO4 to 4 L.

d. Standard anion solutions, 1000 mg/L: Prepare a series of standard anion solutions by

weighing the indicated amount of salt, dried to a constant weight at 105°C, to 1000 mL. Store in

plastic bottles in a refrigerator; these solutions are stable for at least 1 month. Verify stability.

Anion§                                     Salt                             Amount g/L

Cl–                                         NaCl                        1.6485

Br–                                         NaBr                        1.2876

NO3–                                    NaNO3                          1.3707 (226 mg NO3 –-N/L)

NO2–                                    NaNO2                           1.4998i (304 mg NO2–-N/L)

PO43–                                  KH2PO4                         1.4330 (326 mg PO4 3–-P/L

SO42–                                  K2SO4                             1.8141

§ Expressed as compound.

i Do not oven-dry, but dry to constant weight in a desiccator.

e. Combined working standard solution, high range: Combine 12 mL of standard anion

solutions, 1000 mg/L (¶ 3d) of NO2–, NO3–, HPO42–, and Br–, 20 mL of Cl–, and 80 mL of SO42–. Dilute to 1000 mL and store in a plastic bottle protected from light. Solution contains 12

mg/L each of NO2–, NO3–, HPO42–, and Br–, 20 mg/L of Cl–, and 80 mg/L of SO42–. Prepare

fresh daily.

f. Combined working standard solution, low range: Dilute 25 mL of the high-range mixture

(¶ 3e) to 100 mL and store in a plastic bottle protected from light. Solution contains 3 mg/L each

of NO2–, NO3–, HPO42–, and Br–, 5 mg/L Cl–, and 20 mg/L of SO42–. Prepare fresh daily.

g. Alternative combined working standard solutions: Prepare appropriate combinations

according to anion concentration to be determined. If NO2– and PO43– are not included, the

combined working standard is stable for 1 month. Dilute solutions containing NO2– and PO43–

must be made daily.

4. Procedure

a. System equilibration: Turn on ion chromatograph and adjust eluent flow rate to

approximate the separation achieved in Figure 4110:1 (about 2 mL/min). Adjust detector to

desired setting (usually 10 to 30 μS) and let system come to equilibrium (15 to 20 min). A stable

base line indicates equilibrium conditions. Adjust detector offset to zero out eluent conductivity;

with the fiber or membrane suppressor adjust the regeneration flow rate to maintain stability,

usually 2.5 to 3 mL/min.

b. Calibration: Inject standards containing a single anion or a mixture and determine

approximate retention times. Observed times vary with conditions but if standard eluent and

anion separator column are used, retention always is in the order F–, Cl–, NO2–, Br–, NO3–,

HPO42–, and SO42–. Inject at least three different concentrations (one near the minimum

reporting limit) for each anion to be measured and construct a calibration curve by plotting peak

height or area against concentration on linear graph paper. Recalibrate whenever the detector

setting, eluent, or regenerant is changed. To minimize the effect of the ‘‘water dip’’##(5) on F–

analysis, analyze standards that bracket the expected result or eliminate the water dip by diluting

the sample with eluent or by adding concentrated eluent to the sample to give the same HCO3/CO3

2– concentration as in the eluent. If sample adjustments are made, adjust standards

and blanks identically.

If linearity is established for a given detector setting, single standard calibration is

acceptable. Record peak height or area and retention time for calculation of the calibration

factor, F. However, a calibration curve will result in better precision and bias. HPO42– is nonlinear below 1.0 mg/L.

c. Sample analysis: Remove sample particulates, if necessary, by filtering through a

prewashed 0.2-μm-pore-diam membrane filter. Using a prewashed syringe of 1 to 10 mL

capacity equipped with a male luer fitting inject sample or standard. Inject enough sample to

flush sample loop several times: for 0.1 mL sample loop inject at least 1 mL. Switch ion

chromatograph from load to inject mode and record peak heights and retention times on strip

chart recorder. After the last peak (SO4

2–) has appeared and the conductivity signal has returned

to base line, another sample can be injected.

5. Calculations

Calculate concentration of each anion, in milligrams per liter, by referring to the appropriate

calibration curve. Alternatively, when the response is shown to be linear, use the following

equation:

C = H × F × D

where:

C = mg anion/L,

H = peak height or area,

F = response factor = concentration of standard/height (or area) of standard, and

D = dilution factor for those samples requiring dilution.

6. Quality Control

See Section 4020 for minimum QC guidelines.

7. Precision and Bias

The data in Table 4110:I, Table 4110:II, Table 4110:III, Table 4110:IV, Table 4110:V,

Table 4110:VI, and Table 4110:VII were produced in a joint validation study with EPA and

ASTM participation. Nineteen laboratories participated and used known additions of six

prepared concentrates in three waters (reagent, waste, and drinking) of their choice.

4500-Cl– CHLORIDE*#(34)

4500-Cl– A. Introduction

1. Occurrence

Chloride, in the form of chloride (Cl–) ion, is one of the major inorganic anions in water and

wastewater. The salty taste produced by chloride concentrations is variable and dependent on the

chemical composition of water. Some waters containing 250 mg Cl–/L may have a detectable

salty taste if the cation is sodium. On the other hand, the typical salty taste may be absent in

waters containing as much as 1000 mg/L when the predominant cations are calcium and

magnesium.

The chloride concentration is higher in wastewater than in raw water because sodium

chloride (NaCl) is a common article of diet and passes unchanged through the digestive system.

Along the sea coast, chloride may be present in high concentrations because of leakage of salt

water into the sewerage system. It also may be increased by industrial processes.

A high chloride content may harm metallic pipes and structures, as well as growing plants.

2. Selection of Method

Six methods are presented for the determination of chloride. Because the first two are

similar in most respects, selection is largely a matter of personal preference. The argentometric

method (B) is suitable for use in relatively clear waters when 0.15 to 10 mg Cl– are present in the

portion titrated. The end point of the mercuric nitrate method (C) is easier to detect. The

potentiometric method (D) is suitable for colored or turbid samples in which color-indicated end

points might be difficult to observe. The potentiometric method can be used without a

pretreatment step for samples containing ferric ions (if not present in an amount greater than the

chloride concentration), chromic, phosphate, and ferrous and other heavy-metal ions. The

ferricyanide method (E) is an automated technique. Flow injection analysis (G), an automated

colorimetric technique, is useful for analyzing large numbers of samples. Preferably determine

chloride by ion chromatography (Section 4110). Chloride also can be determined by the capillary

ion electrophoresis method (Section 4140). Methods (C and G) in which mercury, a highly toxic

reagent, is used require special disposal practices to avoid improper sewage discharges. Follow

appropriate regulatory procedures (see Section 1090).

3. Sampling and Storage

Collect representative samples in clean, chemically resistant glass or plastic bottles. The

maximum sample portion required is 100 mL. No special preservative is necessary if the sample

is to be stored.

4500-Cl– B. Argentometric Method

1. General Discussion

a. Principle: In a neutral or slightly alkaline solution, potassium chromate can indicate the

end point of the silver nitrate titration of chloride. Silver chloride is precipitated quantitatively

before red silver chromate is formed.

b. Interference: Substances in amounts normally found in potable waters will not interfere.

Bromide, iodide, and cyanide register as equivalent chloride concentrations. Sulfide, thiosulfate,

and sulfite ions interfere but can be removed by treatment with hydrogen peroxide.

Orthophosphate in excess of 25 mg/L interferes by precipitating as silver phosphate. Iron in

excess of 10 mg/L interferes by masking the end point.

2. Apparatus

a. Erlenmeyer flask, 250-mL.

b. Buret, 50-mL.

3. Reagents

a. Potassium chromate indicator solution: Dissolve 50 g K2CrO4 in a little distilled water.

Add AgNO3 solution until a definite red precipitate is formed. Let stand 12 h, filter, and dilute to

1 L with distilled water.

b. Standard silver nitrate titrant, 0.0141M (0.0141N): Dissolve 2.395 g AgNO3 in distilled

water and dilute to 1000 mL. Standardize against NaCl by the procedure described in ¶ 4b

below; 1.00 mL = 500 μg Cl–. Store in a brown bottle.

c. Standard sodium chloride, 0.0141M (0.0141N): Dissolve 824.0 mg NaCl (dried at 140°C)

in distilled water and dilute to 1000 mL; 1.00 mL = 500 μg Cl–.

d. Special reagents for removal of interference:

1) Aluminum hydroxide suspension: Dissolve 125 g aluminum potassium sulfate or

aluminum ammonium sulfate, AlK(SO4)2⋅12H2O or AlNH4(SO4)2⋅12H2O, in 1 L distilled

water. Warm to 60°C and add 55 mL conc ammonium hydroxide (NH4OH) slowly with stirring.

Let stand about 1 h, transfer to a large bottle, and wash precipitate by successive additions, with

thorough mixing and decanting with distilled water, until free from chloride. When freshly

prepared, the suspension occupies a volume of approximately 1 L.

2) Phenolphthalein indicator solution.

3) Sodium hydroxide, NaOH, 1N.

4) Sulfuric acid, H2SO4, 1N.

5) Hydrogen peroxide, H2O2, 30%.

4. Procedure

a. Sample preparation: Use a 100-mL sample or a suitable portion diluted to 100 mL. If the

sample is highly colored, add 3 mL Al(OH)3 suspension, mix, let settle, and filter.

If sulfide, sulfite, or thiosulfate is present, add 1 mL H2O2 and stir for 1 min.

b. Titration: Directly titrate samples in the pH range 7 to 10. Adjust sample pH to 7 to 10

with H2SO4 or NaOH if it is not in this range. For adjustment, preferably use a pH meter with a

non-chloride-type reference electrode. (If only a chloride-type electrode is available, determine

amount of acid or alkali needed for adjustment and discard this sample portion. Treat a separate

portion with required acid or alkali and continue analysis.) Add 1.0 mL K2CrO4 indicator

solution. Titrate with standard AgNO3 titrant to a pinkish yellow end point. Be consistent in

end-point recognition.

Standardize AgNO3 titrant and establish reagent blank value by the titration method outlined

above. A blank of 0.2 to 0.3 mL is usual.

5. Calculation

where:

A = mL titration for sample,

B = mL titration for blank, and

N = normality of AgNO3.

mg NaCl/L = (mg Cl–/L) × 1.65

6. Precision and Bias

A synthetic sample containing 241 mg Cl–/L, 108 mg Ca/L, 82 mg Mg/L; 3.1 mg K/L, 19.9

mg Na/L, 1.1 mg NO3–-N/L, 0.25 mg NO2–- N/L, 259 mg SO42–/L, and 42.5 mg total

alkalinity/L (contributed by NaHCO3) in distilled water was analyzed in 41 laboratories by the

argentometric method, with a relative standard deviation of 4.2% and a relative error of 1.7%.

4500-Cl– C. Mercuric Nitrate Method

1. General Discussion

a. Principle: Chloride can be titrated with mercuric nitrate, Hg(NO3)2, because of the

formation of soluble, slightly dissociated mercuric chloride. In the pH range 2.3 to 2.8,

diphenylcarbazone indicates the titration end point by formation of a purple complex with the

excess mercuric ions. Xylene cyanol FF serves as a pH indicator and end-point enhancer.

Increasing the strength of the titrant and modifying the indicator mixtures extend the range of

measurable chloride concentrations.

b. Interference: Bromide and iodide are titrated with Hg(NO3)2 in the same manner as

chloride. Chromate, ferric, and sulfite ions interfere when present in excess of 10 mg/L.

2. Apparatus

a. Erlenmeyer flask, 250-mL.

b. Microburet, 5-mL with 0.01-mL graduation intervals.

3. Reagents

a. Standard sodium chloride, 0.0141M (0.0141N): See Method B, ¶ B.3c above.

b. Nitric acid, HNO3, 0.1N.

c. Sodium hydroxide, NaOH, 0.1N.

d. Reagents for chloride concentrations below 100 mg/L:

1) Indicator-acidifier reagent: The HNO3 concentration of this reagent is an important factor

in the success of the determination and can be varied as indicated in a) or b) to suit the alkalinity

range of the sample. Reagent a) contains sufficient HNO3 to neutralize a total alkalinity of 150

mg as CaCO3/L to the proper pH in a 100-mL sample. Adjust amount of HNO3 to accommodate

samples of alkalinity different from 150 mg/L.

a) Dissolve, in the order named, 250 mg s-diphenylcarbazone, 4.0 mL conc HNO3, and 30

mg xylene cyanol FF in 100 mL 95% ethyl alcohol or isopropyl alcohol. Store in a dark bottle in

a refrigerator. This reagent is not stable indefinitely. Deterioration causes a slow end point and

high results.

b) Because pH control is critical, adjust pH of highly alkaline or acid samples to 2.5 ± 0.1

with 0.1N HNO3 or NaOH, not with sodium carbonate (Na2CO3). Use a pH meter with a

nonchloride type of reference electrode for pH adjustment. If only the usual chloride-type

reference electrode is available for pH adjustment, determine amount of acid or alkali required to obtain a pH of 2.5 ± 0.1 and discard this sample portion. Treat a separate sample portion with the

determined amount of acid or alkali and continue analysis. Under these circumstances, omit

HNO3 from indicator reagent.

2) Standard mercuric nitrate titrant, 0.007 05M (0.0141N): Dissolve 2.3 g Hg(NO3)2 or 2.5

g Hg(NO3)2⋅H2O in 100 mL distilled water containing 0.25 mL conc HNO3. Dilute to just under

1 L. Make a preliminary standardization by following the procedure described in ¶ 4a. Use

replicates containing 5.00 mL standard NaCl solution and 10 mg sodium bicarbonate (NaHCO3)

diluted to 100 mL with distilled water. Adjust titrant to 0.0141N and make a final

standardization; 1.00 mL = 500 μg Cl–. Store away from light in a dark bottle.

e. Reagent for chloride concentrations greater than 100 mg/L:

1) Mixed indicator reagent: Dissolve 0.50 g diphenylcarbazone powder and 0.05 g

bromphenol blue powder in 75 mL 95% ethyl or isopropyl alcohol and dilute to 100 mL with the

same alcohol.

2) Strong standard mercuric nitrate titrant, 0.0705M (0.141N) Dissolve 25 g Hg(NO3)2⋅H2O

in 900 mL distilled water containing 5.0 mL conc HNO3. Dilute to just under 1 L and

standardize by following the procedure described in ¶ 4b. Use replicates containing 25.00 mL

standard NaCl solution and 25 mL distilled water. Adjust titrant to 0.141N and make a final

standardization; 1.00 mL = 5.00 mg Cl–.

4. Procedure

a. Titration of chloride concentrations less than 100 mg/L: Use a 100-mL sample or smaller

portion so that the chloride content is less than 10 mg.

Add 1.0 mL indicator-acidifier reagent. (The color of the solution should be green-blue at

this point. A light green indicates pH less than 2.0; a pure blue indicates pH more than 3.8.) For

most potable waters, the pH after this addition will be 2.5 ± 0.1. For highly alkaline or acid

waters, adjust pH to about 8 before adding indicator-acidifier reagent.

Titrate with 0.0141N Hg(NO3)2 titrant to a definite purple end point. The solution turns from

green-blue to blue a few drops before the end point.

Determine blank by titrating 100 mL distilled water containing 10 mg NaHCO3.

b. Titration of chloride concentrations greater than 100 mg/L: Use a sample portion (5 to 50

mL) requiring less than 5 mL titrant to reach the end point. Measure into a 150-mL beaker. Add

approximately 0.5 mL mixed indicator reagent and mix well. The color should be purple. Add

0.1N HNO3 dropwise until the color just turns yellow. Titrate with strong Hg(NO3)2 titrant to

first permanent dark purple. Titrate a distilled water blank using the same procedure.

5. Calculation

where:

A = mL titration for sample,

B = mL titration for blank, and

N = normality of Hg(NO3)2.

mg NaCl/L = (mg Cl–/L) × 1.65

6. Precision and Bias

A synthetic sample containing 241 mg Cl–/L, 108 mg Ca/L, 82 mg Mg/L, 3.1 mg K/L, 19.9

mg Na/L, 1.1 mg NO3

–-N/L, 0.25 mg NO2

–-N/L, 259 mg SO4

2–/L, and 42.5 mg total

alkalinity/L (contributed by NaHCO3) in distilled water was analyzed in 10 laboratories by the

mercurimetric method, with a relative standard deviation of 3.3% and a relative error of 2.9%.

7. Bibliography

KOLTHOFF, I.M. & V.A. STENGER. 1947. Volumetric Analysis, 2nd ed. Vol. 2. Interscience

Publishers, New York, N.Y., pp. 334-335.

DOMASK, W.C. & K.A. KOBE. 1952. Mercurimetric determination of chlorides and water-soluble

chlorohydrins. Anal. Chem. 24:989.

GOLDMAN, E. 1959. New indicator for the mercurimetric chloride determination in potable

water. Anal. Chem. 31:1127.

4500-Cl– D. Potentiometric Method

1. General Discussion

a. Principle: Chloride is determined by potentiometric titration with silver nitrate solution

with a glass and silver-silver chloride electrode system. During titration an electronic voltmeter

is used to detect the change in potential between the two electrodes. The end point of the titration

is that instrument reading at which the greatest change in voltage has occurred for a small and

constant increment of silver nitrate added.

b. Interference: Iodide and bromide also are titrated as chloride. Ferricyanide causes high

results and must be removed. Chromate and dichromate interfere and should be reduced to the

chromic state or removed. Ferric iron interferes if present in an amount substantially higher than

the amount of chloride. Chromic ion, ferrous ion, and phosphate do not interfere.

Grossly contaminated samples usually require pretreatment. Where contamination is minor, some contaminants can be destroyed simply by adding nitric acid.

2. Apparatus

a. Glass and silver-silver chloride electrodes: Prepare in the laboratory or purchase a silver

electrode coated with AgCl for use with specified instruments. Instructions on use and care of

electrodes are supplied by the manufacturer.

b. Electronic voltmeter, to measure potential difference between electrodes: A pH meter may

be converted to this use by substituting the appropriate electrode.

c. Mechanical stirrer, with plastic-coated or glass impeller.

3. Reagents

a. Standard sodium chloride solution, 0.0141M (0.0141N): See Section 4500-Cl–.B.3c.

b. Nitric acid, HNO3, conc.

c. Standard silver nitrate titrant, 0.0141M (0.0141N): See Section 4500-Cl–.B.3b.

d. Pretreatment reagents:

1) Sulfuric acid, H2SO4, 1 + 1.

2) Hydrogen peroxide, H2O2, 30%.

3) Sodium hydroxide, NaOH, 1N.

4. Procedure

a. Standardization: The various instruments that can be used in this determination differ in

operating details; follow the manufacturer’s instructions. Make necessary mechanical

adjustments. Then, after allowing sufficient time for warmup (10 min), balance internal electrical

components to give an instrument setting of 0 mV or, if a pH meter is used, a pH reading of 7.0.

1) Place 10.0 mL standard NaCl solution in a 250-mL beaker, dilute to about 100 mL, and

add 2.0 mL conc HNO3. Immerse stirrer and electrodes.

2) Set instrument to desired range of millivolts or pH units. Start stirrer.

3) Add standard AgNO3 titrant, recording scale reading after each addition. At the start, large

increments of AgNO3 may be added; then, as the end point is approached, add smaller and equal

increments (0.1 or 0.2 mL) at longer intervals, so that the exact end point can be determined.

Determine volume of AgNO3 used at the point at which there is the greatest change in

instrument reading per unit addition of AgNO3.

4) Plot a differential titration curve if the exact end point cannot be determined by inspecting

the data. Plot change in instrument reading for equal increments of AgNO3 against volume of

AgNO3 added, using average of buret readings before and after each addition. The procedure is

illustrated in Figure 4500-Cl–:1. b. Sample analysis:

1) Pipet 100.0 mL sample, or a portion containing not more than 10 mg Cl–, into a 250-mL

beaker. In the absence of interfering substances, proceed with ¶ 3) below.

2) In the presence of organic compounds, sulfite, or other interferences (such as large

amounts of ferric iron, cyanide, or sulfide) acidify sample with H2SO4, using litmus paper. Boil

for 5 min to remove volatile compounds. Add more H2SO4, if necessary, to keep solution acidic.

Add 3 mL H2O2 and boil for 15 min, adding chloride-free distilled water to keep the volume

above 50 mL. Dilute to 100 mL, add NaOH solution dropwise until alkaline to litmus, then 10

drops in excess. Boil for 5 min, filter into a 250-mL beaker, and wash precipitate and paper

several times with hot distilled water.

3) Add conc HNO3 dropwise until acidic to litmus paper, then 2.0 mL in excess. Cool and

dilute to 100 mL if necessary. Immerse stirrer and electrodes and start stirrer. Make any

necessary adjustments according to the manufacturer’s instructions and set selector switch to

appropriate setting for measuring the difference of potential between electrodes.

4) Complete determination by titrating according to ¶ 4a4). If an end-point reading has been

established from previous determinations for similar samples and conditions, use this

predetermined end point. For the most accurate work, make a blank titration by carrying

chloride-free distilled water through the procedure.

5. Calculation

where:

A = mL AgNO3,

B = mL blank, and

N = normality of titrant.

6. Precision and Bias

In the absence of interfering substances, the precision and bias are estimated to be about

0.12 mg for 5 mg Cl–, or 2.5% of the amount present. When pretreatment is required to remove

interfering substances, the precision and bias are reduced to about 0.25 mg for 5 mg Cl–, or 5%

of amount present.

4500-Cl– E. Automated Ferricyanide Method

1. General Discussion

a. Principle: Thiocyanate ion is liberated from mercuric thiocyanate by the formation of

soluble mercuric chloride. In the presence of ferric ion, free thiocyanate ion forms a highly

colored ferric thiocyanate, of which the intensity is proportional to the chloride concentration.

b. Interferences: Remove particulate matter by filtration or centrifugation before analysis.

Guard against contamination from reagents, water, glassware, and sample preservation process.

No chemical interferences are significant.

c. Application: The method is applicable to potable, surface, and saline waters, and domestic

and industrial wastewaters. The concentration range is 1 to 200 mg Cl–/L; it can be extended by

dilution.

2. Apparatus

a. Automated analytical equipment: An example of the continuous-flow analytical instrument

consists of the interchangeable components shown in Figure 4500-Cl–:2.

b. Filters, 480-nm.

3. Reagents

a. Stock mercuric thiocyanate solution: Dissolve 4.17 g Hg(SCN)2 in about 500 mL

methanol, dilute to 1000 mL with methanol, mix, and filter through filter paper.

b. Stock ferric nitrate solution: Dissolve 202 g Fe(NO3)3⋅9H2O in about 500 mL distilled

water, then carefully add 21 mL conc HNO3. Dilute to 1000 mL with distilled water and mix.

Filter through paper and store in an amber bottle.

c. Color reagent: Add 150 mL stock Hg(SCN)2 solution to 150 mL stock Fe(NO3)3 solution.

Mix and dilute to 1000 mL with distilled water. Add 0.5 mL polyoxyethylene 23 lauryl

ether.*#(35)

d. Stock chloride solution: Dissolve 1.6482 g NaCl, dried at 140°C, in distilled water and

dilute to 1000 mL; 1.00 mL = 1.00 mg Cl–.

e. Standard chloride solutions: Prepare chloride standards in the desired concentration range,

such as 1 to 200 mg/L, using stock chloride solution.

4. Procedure

Set up manifold as shown in Figure 4500-Cl–:2 and follow general procedure described by

the manufacturer.

5. Calculation

Prepare standard curves by plotting response of standards processed through the manifold

against chloride concentrations in standards. Compute sample chloride concentration by

comparing sample response with standard curve.

6. Precision and Bias

With an automated system in a single laboratory six samples were analyzed in septuplicate.

At a concentration ranging from about 1 to 50 mg Cl–/L the average standard deviation was 0.39

mg/L. The coefficient of variation was 2.2%. In two samples with added chloride, recoveries

were 104% and 97%.

4500-N NITROGEN*#(51)

4500-N A. Introduction

In waters and wastewaters the forms of nitrogen of greatest interest are, in order of decreasing oxidation state, nitrate, nitrite, ammonia, and organic nitrogen. All these forms of nitrogen, as well as nitrogen gas (N2), are biochemically interconvertible and are components of the nitrogen cycle. They are of interest for many reasons.

Organic nitrogen is defined functionally as organically bound nitrogen in the trinegative

oxidation state. It does not include all organic nitrogen compounds. Analytically, organic

nitrogen and ammonia can be determined together and have been referred to as ‘‘kjeldahl

nitrogen,’’ a term that reflects the technique used in their determination. Organic nitrogen

includes such natural materials as proteins and peptides, nucleic acids and urea, and numerous

synthetic organic materials. Typical organic nitrogen concentrations vary from a few hundred

micrograms per liter in some lakes to more than 20 mg/L in raw sewage.

Total oxidized nitrogen is the sum of nitrate and nitrite nitrogen. Nitrate generally occurs in

trace quantities in surface water but may attain high levels in some groundwater. In excessive

amounts, it contributes to the illness known as methemoglobinemia in infants. A limit of 10 mg

nitrate as nitrogen/L has been imposed on drinking water to prevent this disorder. Nitrate is

found only in small amounts in fresh domestic wastewater but in the effluent of nitrifying

biological treatment plants nitrate may be found in concentrations of up to 30 mg nitrate as nitrogen/ L. It is an essential nutrient for many photosynthetic autotrophs and in some cases has

been identified as the growth-limiting nutrient.

Nitrite is an intermediate oxidation state of nitrogen, both in the oxidation of ammonia to

nitrate and in the reduction of nitrate. Such oxidation and reduction may occur in wastewater

treatment plants, water distribution systems, and natural waters. Nitrite can enter a water supply

system through its use as a corrosion inhibitor in industrial process water. Nitrite is the actual

etiologic agent of methemoglobinemia. Nitrous acid, which is formed from nitrite in acidic

solution, can react with secondary amines (RR′NH) to form nitrosamines (RR′N-NO), many of which are known to be carcinogens. The toxicologic significance of nitrosation reactions in vivo and in the natural environment is the subject of much current concern and research.

Ammonia is present naturally in surface and wastewaters. Its concentration generally is low

in groundwaters because it adsorbs to soil particles and clays and is not leached readily from

soils. It is produced largely by deamination of organic nitrogen-containing compounds and by

hydrolysis of urea. At some water treatment plants ammonia is added to react with chlorine to

form a combined chlorine residual. Ammonia concentrations encountered in water vary from less

than 10 μg ammonia nitrogen/L in some natural surface and groundwaters to more than 30 mg/L in some wastewaters.

In this manual, organic nitrogen is referred to and reported as organic N, nitrate nitrogen as NO3-N, nitrite nitrogen as NO2–-N, and ammonia nitrogen as NH3-N.

Total nitrogen can be determined through oxidative digestion of all digestible nitrogen forms

to nitrate, followed by quantitation of the nitrate. Two procedures, one using a persulfate/UV

digestion (Section 4500-N.B), and the other using persulfate digestion (Section 4500-N.C) are

presented. The procedures give good results for total nitrogen, composed of organic nitrogen

(including some aromatic nitrogen-containing compounds), ammonia, nitrite, and nitrate.

Molecular nitrogen is not determined and recovery of some industrial nitrogen-containing

compounds is low.

Chloride ions do not interfere with persulfate oxidation, but the rate of reduction of nitrate to

nitrite (during subsequent nitrate analysis by cadmium reduction) is significantly decreased by

chlorides. Ammonium and nitrate ions adsorbed on suspended pure clay or silt particles should

give a quantitative yield from persulfate digestion. If suspended matter remains after digestion,

remove it before the reduction step.

If suspended organic matter is dissolved by the persulfate digestion reagent, yields

comparable to those from true solutions are obtained; if it is not dissolved, the results are

unreliable and probably reflect a negative interference. The persulfate method is not effective in

wastes with high organic loadings. Dilute such samples and re-analyze until results from two

dilutions agree.

4500-N B. In-Line UV/Persulfate Digestion and Oxidation with Flow Injection

Analysis (PROPOSED)

1. General Discussion

a. Principle: Nitrogen compounds are digested and oxidized in-line to nitrate by use of

heated alkaline persulfate and ultraviolet radiation. The digested sample is injected onto the

manifold where its nitrate is reduced to nitrite by a cadmium granule column. The nitrite then is

determined by diazotization with sulfanilamide under acidic conditions to form a diazonium ion.

The diazonium ion is coupled with N-(1-naphthyl)ethylenediamine dihydrochloride. The

resulting pink dye absorbs at 540 nm and is proportional to total nitrogen.

This method recovers nearly all forms of organic and inorganic nitrogen, reduced and

oxidized, including ammonia, nitrate, and nitrite. It differs from the total kjeldahl nitrogen

method described in Section 4500-Norg.D, which does not recover the oxidized forms of

nitrogen. This method recovers nitrogen components of biological origin such as amino acids,

proteins, and peptides as ammonia, but may not recover the nitrogenous compounds of some

industrial wastes such as amines, nitro-compounds, hydrazones, oximes, semicarbazones, and

some refractory tertiary amines.

See Section 4500-N.A for a discussion of the various forms of nitrogen found in waters and

wastewaters, Section 4500-Norg.A and Section 4500-Norg.B for a discussion of total nitrogen

methods, and Section 4130, Flow Injection Analysis (FIA). Also see Section 4500-N.C for a

similar, batch total nitrogen method that uses only persulfate.

b. Interferences: Remove large or fibrous particulates by filtering sample though glass wool.

Guard against contamination from reagents, water, glassware, and the sample preservation

process.

2. Apparatus

Flow injection analysis equipment consisting of:

a. FIA injection valve with sample loop or equivalent.

b. Multichannel proportioning pump.

c. FIA manifold (Figure 4500-N:1) with tubing heater, in-line ultraviolet digestion fluidics

including a debubbler consisting of a gas-permeable TFE membrane and its holder, and flow

cell. In Figure 4500-N:1, relative flow rates only are shown. Tubing volumes are given as an

example only; they may be scaled down proportionally. Use manifold tubing of an inert material

such as TFE. The block marked ‘‘UV’’ should consist of TFE tubing irradiated by a mercury

discharge ultraviolet lamp emitting radiation at 254 nm.

d. Absorbance detector, 540 nm, 10-nm bandpass.

e. Injection valve control and data acquisition system.

3. Reagents

Use reagent water (>10 megohm) to prepare carrier and for all solutions. To prevent bubble

formation, degas carrier and all reagents with helium. Pass He at 140 kPa (20 psi) through a

helium degassing tube. Bubble He through 1 L solution for 1 min. As an alternative to preparing reagents by weight/weight, use weight/volume.

a. Borate solution, Na2B4O7⋅10H2O: In a 1-L volumetric flask dissolve 38.0 g

Na2B4O7⋅10H2O and 3.0 g sodium hydroxide, NaOH, in approximately 900 mL water, using a

magnetic stirring bar. Gentle heating will speed dissolution. Adjust to pH 9.0 with NaOH or conc

hydrochloric acid (HCl). Dilute to mark and invert to mix.

b. Persulfate solution, K2S2O8: Potassium persulfate solid reagent usually contains nitrogen

contamination. Higher contamination levels result in larger blank peaks.

To a tared 1-L container, add 975 g water and 49 g K2S2O8. Add a magnetic stirring bar,

dissolve persulfate, and dilute to mark. Invert to mix.

c. Ammonium chloride buffer: CAUTION: Fumes. Use a hood. To a 1-L volumetric flask add

500 mL water, 105 mL conc HCl, and 95 mL conc ammonium hydroxide, NH4OH. Dissolve,

dilute to mark, and invert to mix. Adjust to pH 8.5 with 1N HCl or 1N NaOH solution.

d. Sulfanilamide color reagent: To a tared, dark, 1-L container add 876 g water, 170 g 85%

phosphoric acid, H3PO4, 40.0 g sulfanilamide, and 1.0 g N-(1-naphthyl)ethylenediamine

dihydrochloride (NED). Shake to wet solids and stir for 30 min to dissolve. Store in a dark bottle

and discard when solution turns dark pink.

e. Cadmium column: See Section 4500-NO3–.I.3c, d, and e.

f. Stock nitrate standard, 1000 mg N/L: In a 1-L volumetric flask dissolve 7.221 g potassium

nitrate, KNO3 (dried at 60°C for 1 h), or 4.93 g sodium nitrite, NaNO2, in about 800 mL water.

Dilute to mark and invert to mix. When refrigerated the standard may be stored for up to 3

months.

g. Standard solutions: Prepare nitrate standards in the desired concentration range, using

stock nitrate standards (¶ 3 f ), and diluting with water.

4. Procedure

Set up a manifold equivalent to that in Figure 4500-N:1 and follow method supplied by

manufacturer, or laboratory standard operating procedure for this method.

Carry both standards and samples through this procedure. If samples have been preserved

with sulfuric acid, preserve standards similarly. Samples may be homogenized. Turbid samples

may be filtered, since digestion effectiveness on nitrogen-containing particles is unknown;

however, organic nitrogen may be lost in the filtration.

5. Calculation

Prepare standard curves by plotting absorbance of standards processed through the manifold

versus nitrogen concentration. The calibration curve is linear.

Verify digestion efficiency by determining urea, glutamic acid, or nicotinic acid standards

(Section 4500-N.C.3d) at regular intervals. In the concentration range of the method, the recovery of these compounds should be >95%.

6. Quality Control

See Section 4130B.

7. Precision and Bias

a. MDL: Using a 70-μL sample loop and a published MDL method,1 analysts ran 21

replicates of a 0.20-mg N/L standard. These gave a mean of 0.18 mg N/L, a standard deviation of

0.008 mg N/L, and MDL of 0.020 mg N/L.

b. Precision study: Ten injections each of a 4.00-mg N/L standard and of a 10.0-mg N/L

standard both gave a relative standard deviation of 0.6%.

c. Recovery of total nitrogen: Table 4500-N:I shows recoveries for various nitrogen

compounds determined at 10 mg N/L and 4.0 mg N/L. All compounds were determined in

triplicate.

d. Ammonia recoveries from wastewater treatment plant effluent with known additions: To a

sample of wastewater treatment plant effluent, ammonium chloride was added at two

concentrations, 2.50 and 5.00 mg N/L, and analyses were made in triplicate to give mean

recoveries of 96% and 95%, respectively. A sample with no additions also was diluted twofold

in triplicate to give a mean recovery of 99%.

4500-N C. Persulfate Method

1. General Discussion

The persulfate method determines total nitrogen by oxidation of all nitrogenous compounds

to nitrate. Should ammonia, nitrate, and nitrite be determined individually, ‘‘organic nitrogen’’

can be obtained by difference.

a. Principle: Alkaline oxidation at 100 to 110°C converts organic and inorganic nitrogen to

nitrate. Total nitrogen is determined by analyzing the nitrate in the digestate.

b. Selection of nitrate measurement method: Automated or manual cadmium reduction may

be used to determine total nitrogen levels below 2.9 mg N/L. Results summarized in Table

4500-N:II were obtained using automated cadmium reduction.

2. Apparatus

a. Autoclave, or hotplate and pressure cooker capable of developing 100 to 110°C for 30 min.

b. Glass culture tubes:*#(52) 30-mL screw-capped (polypropylene linerless caps), 20 mm

OD × 150 mm long. Clean before initial use by autoclaving with digestion reagent.

c. Apparatus for nitrate determination: See Section 4500-NO3–.E or Section 4500-NO3–.F.

d. Automated analytical equipment: An example of the continuous-flow analytical

instrument consists of components shown in Figure 4500-NO3–:2.

3. Reagents

a. Ammonia-free and nitrate-free water: Prepare by ion exchange or distillation methods as

directed in Section 4500-NH3.B.3a and Section 4500-NO3–.B.3a.

b. Stock nitrate solution: Prepare as directed in Section 4500-NO3–.B.3b.

c. Intermediate nitrate solution: Prepare as directed in Section 4500-NO3–.B.3c.

d. Stock glutamic acid solution: Dry glutamic acid, C3H5NH2(COOH)2, in an oven at 105°C

for 24 h. Dissolve 1.051 g in water and dilute to 1000 mL; 1.00 mL = 100 μg N. Preserve with 2

mL CHCl3/L.

e. Intermediate glutamic acid solution: Dilute 100 mL stock glutamic acid solution to 1000

mL with water; 1.00 mL = 10.0 μg N. Preserve with 2 mL CHCl3/L.gc

f. Digestion reagent: Dissolve 20.1 g low nitrogen (<0.001% N) potassium persulfate,

K2S2O8, and 3.0 g NaOH in water and dilute to 1000 mL just before use.

g. Borate buffer solution: Dissolve 61.8 g boric acid, H3BO3, and 8.0 g NaOH in water and

dilute to 1000 mL.

h. Copper sulfate solution: Dissolve 2.0 g CuSO4⋅5H2O in 90 mL water and dilute to 100

mL.

i. Ammonium chloride solution: Dissolve 10.0 g NH4Cl in 1 L water. Adjust to pH 8.5 by

adding three or four NaOH pellets as necessary or NaOH solution before bringing to volume.

This reagent is stable for 2 weeks when refrigerated.

j. Color reagent: Combine 1500 mL water, 200.0 mL conc phosphoric acid, H3PO4, 20.0 g

sulfanilamide, and 1.0 g N-(1-naphthyl)-ethylenediamine dihydrochloride. Dilute to 2000 mL.

Add 2.0 mL polyoxyethylene 23 lauryl ether.†#(53) Store at 4°C in the dark. Prepare fresh

reagent every 6 weeks. Alternatively, prepare proportionally smaller volumes to minimize waste.

4. Procedure

a. Calibration curve: Prepare NO3– calibration standards in the range 0 to 2.9 mg NO3–-N/L

by diluting to 100 mL the following volumes of intermediate nitrate solution: 0, 1.00, 2.00, 4.00 . . . 29.0 mL. Treat standards in the same manner as samples.

b. Digestion check standard: Prepare glutamic acid digestion check standard of 2.9 mg N/L

by diluting, to 100 mL, a 29.0-mL volume of intermediate glutamic acid solution. Treat digestion

check standard in the same manner as samples.

c. Digestion: Samples preserved with acid cannot be analyzed by this method. To a culture

tube, add 10.0 mL sample or standard or a portion diluted to 10.0 mL. Add 5.0 mL digestion

reagent. Cap tightly. Mix by inverting twice. Heat for 30 min in an autoclave or pressure cooker

at 100 to 110°C. Slowly cool to room temperature. Add 1.0 mL borate buffer solution. Mix by

inverting at least twice.

d. Blank: Carry a reagent blank through all steps of the procedure and apply necessary

corrections to the results.

e. Nitrate measurement: Determine nitrate by cadmium reduction. Set up manifold as shown

in Figure 4500-NO3–:2, but use reagents specified in Section 4500-Norg.C.3.

5. Calculation

Prepare the standard curve by plotting the absorbances or peak heights of the nitrate

calibration standards carried through the digestion procedure against their nitrogen

concentrations. Compute organic N sample concentration by comparing sample absorbance or

peak height with the standard curve.

6. Precision and Bias

See Table 4500-N:II.

4500-NH3 NITROGEN (AMMONIA)*#(54)

4500-NH3 A. Introduction

1. Selection of Method

The two major factors that influence selection of the method to determine ammonia are

concentration and presence of interferences. In general, direct manual determination of low

concentrations of ammonia is confined to drinking waters, clean surface or groundwater, and

good-quality nitrified wastewater effluent. In other instances, and where interferences are present

or greater precision is necessary, a preliminary distillation step (B) is required.

A titrimetric method (C), an ammonia-selective electrode method (D), an ammonia-selective

electrode method using known addition (E), a phenate method (F), and two automated versions

of the phenate method (G and H) are presented. Methods D, E, F, G, and H may be used either

with or without sample distillation. The data presented in Table 4500-NH3:I and Table

4500-NH3:III should be helpful in selecting the appropriate method of analysis.

Nesslerization has been dropped as a standard method, although it has been considered a

classic water quality measurement for more than a century. The use of mercury in this test

warrants its deletion because of the disposal problems.

The distillation and titration procedure is used especially for NH3-N concentrations greater

than 5 mg/L. Use boric acid as the absorbent following distillation if the distillate is to be

titrated.

The ammonia-selective electrode method is applicable over the range from 0.03 to 1400 mg

NH3-N/L.

The manual phenate method is applicable to both fresh water and seawater and is linear to

0.6 mg NH3-N/L. Distill into sulfuric acid (H2SO4) absorbent for the phentate method when

interferences are present.

The automated phenate method is applicable over the range of 0.02 to 2.0 mg NH3-N/L.

2. Interferences

Glycine, urea, glutamic acid, cyanates, and acetamide hydrolyze very slowly in solution on

standing but, of these, only urea and cyanates will hydrolyze on distillation at pH of 9.5.

Hydrolysis amounts to about 7% at this pH for urea and about 5% for cyanates. Volatile alkaline

compounds such as hydrazine and amines will influence titrimetric results. Residual chlorine

reacts with ammonia; remove by sample pretreatment. If a sample is likely to contain residual

chlorine, immediately upon collection, treat with dechlorinating agent as in Section

4500-NH3.B.3d.

3. Storage of Samples

Most reliable results are obtained on fresh samples. If samples are to be analyzed within 24 h unacidified, or preserve samples by acidifying to pH <2 and storing at 4°C. If acid preservation

is used, neutralize samples with NaOH or KOH immediately before making the determination.

CAUTION: Although acidification is suitable for certain types of samples, it produces

interferences when exchangeable ammonium is present in unfiltered solids.

4500-NH3 B. Preliminary Distillation Step

1. General Discussion

The sample is buffered at pH 9.5 with a borate buffer to decrease hydrolysis of cyanates and

organic nitrogen compounds. It is distilled into a solution of boric acid when titration is to be

used or into H2SO4 when the phenate method is used. The ammonia in the distillate can be

determined either colorimetrically by the phenate method or titrimetrically with standard H2SO4

and a mixed indicator or a pH meter. The choice between the colorimetric and the acidimetric

methods depends on the concentration of ammonia. Ammonia in the distillate also can be

determined by the ammonia-selective electrode method, using 0.04N H2SO4 to trap the

ammonia.

2. Apparatus

a. Distillation apparatus: Arrange a borosilicate glass flask of 800- to 2000-mL capacity

attached to a vertical condenser so that the outlet tip may be submerged below the surface of the

receiving acid solution. Use an all-borosilicate-glass apparatus or one with condensing units

constructed of block tin or aluminum tubes.

b. pH meter.

3. Reagents

a. Ammonia-free water: Prepare by ion-exchange or distillation methods:

1) Ion exchange—Prepare ammonia-free water by passing distilled water through an

ion-exchange column containing a strongly acidic cation-exchange resin mixed with a strongly

basic anion-exchange resin. Select resins that will remove organic compounds that interfere with

the ammonia determination. Some anion-exchange resins tend to release ammonia. If this occurs,

prepare ammonia-free water with a strongly acidic cation-exchange resin. Regenerate the column high blank value.

2) Distillation—Eliminate traces of ammonia in distilled water by adding 0.1 mL conc

H2SO4 to 1 L distilled water and redistilling. Alternatively, treat distilled water with sufficient

bromine or chlorine water to produce a free halogen residual of 2 to 5 mg/ L and redistill after

standing at least 1 h. Discard the first 100 mL distillate. Check redistilled water for the

possibility of a high blank.

It is very difficult to store ammonia-free water in the laboratory without contamination from

gaseous ammonia. However, if storage is necessary, store in a tightly stoppered glass container

to which is added about 10 g ion-exchange resin (preferably a strongly acidic cation-exchange

resin)/L ammonia-free water. For use, let resin settle and decant ammonia-free water. If a high

blank value is produced, replace the resin or prepare fresh ammonia-free water.

Use ammonia-free distilled water for preparing all reagents, rinsing, and sample dilution.

b. Borate buffer solution: Add 88 mL 0.1N NaOH solution to 500 mL approximately 0.025M

sodium tetraborate (Na2B4O7) solution (9.5 g Na2B4O7⋅10 H2O/L) and dilute to 1 L.

c. Sodium hydroxide, 6N.

d. Dechlorinating reagent: Dissolve 3.5 g sodium thiosulfate (Na2S2O3⋅5H2O) in water and

dilute to 1 L. Prepare fresh weekly. Use 1 mL reagent to remove 1 mg/L residual chlorine in

500-mL sample.

e. Neutralization agent.

1) Sodium hydroxide, NaOH, 1N.

2) Sulfuric acid, H2SO4, 1N.

f. Absorbent solution, plain boric acid: Dissolve 20 g H3BO3 in water and dilute to 1 L.

g. Indicating boric acid solution: See Section 4500-NH3.C.3a and b.

h. Sulfuric acid, 0.04N: Dilute 1.0 mL conc H2SO4 to 1 L.

4. Procedure

a. Preparation of equipment: Add 500 mL water and 20 mL borate buffer, adjust pH to 9.5

with 6N NaOH solution, and add to a distillation flask. Add a few glass beads or boiling chips

and use this mixture to steam out the distillation apparatus until distillate shows no traces of

ammonia.

b. Sample preparation: Use 500 mL dechlorinated sample or a known portion diluted to 500

mL with water. When NH3-N concentration is less than 100 μg/L, use a sample volume of 1000

mL. Remove residual chlorine by adding, at the time of collection, dechlorinating agent

equivalent to the chlorine residual. If necessary, neutralize to approximately pH 7 with dilute

acid or base, using a pH meter.

Add 25 mL borate buffer solution and adjust to pH 9.5 with 6N NaOH using a pH meter.

c. Distillation: To minimize contamination, leave distillation apparatus assembled after

steaming out and until just before starting sample distillation. Disconnect steaming-out flask and

immediately transfer sample flask to distillation apparatus. Distill at a rate of 6 to 10 mL/min

with the tip of the delivery tube below the surface of acid receiving solution. Collect distillate in

a 500-mL erlenmeyer flask containing 50 mL indicating boric acid solution for titrimetric

method. Distill ammonia into 50 mL 0.04N H2SO4 for the ammonia-selective electrode method

and for the phenate method. Collect at least 200 mL distillate. Lower distillation receiver so that

the end of the delivery tube is free of contact with the liquid and continue distillation during the

last minute or two to cleanse condenser and delivery tube. Dilute to 500 mL with water.

When the phenate method is used for determining NH3-N, neutralize distillate with 1N

NaOH solution.

d. Ammonia determination: Determine ammonia by the titrimetric method (C), the

ammonia-selective electrode methods (D and E), or the phenate methods (F and G).

4500-NH3 C. Titrimetric Method

1. General Discussion

The titrimetric method is used only on samples that have been carried through preliminary

distillation (see Section 4500-NH3.B). The following table is useful in selecting sample volume

for the distillation and titration method.

Ammonia Nitrogen in Sample              Sample Volume

mg/L                                                           mL

5–10                                                     250

10–20                                                   100

20–50                                                     50.0

50–100                                                   25.0

2. Apparatus

Distillation apparatus: See Section 4500-NH3.B.2a and Section 4500-NH3.B.2b.

3. Reagents

Use ammonia-free water in making all reagents and dilutions.

a. Mixed indicator solution: Dissolve 200 mg methyl red indicator in 100 mL 95% ethyl or

isopropyl alcohol. Dissolve 100 mg methylene blue in 50 mL 95% ethyl or isopropyl alcohol.

Combine solutions. Prepare monthly.

b. Indicating boric acid solution: Dissolve 20 g H3BO3 in water, add 10 mL mixed indicator

solution, and dilute to 1 L. Prepare monthly.

c. Standard sulfuric acid titrant, 0.02N: Prepare and standardize as directed in Alkalinity,

Section 2320B.3c. For greatest accuracy, standardize titrant against an amount of Na2CO3 that

has been incorporated in the indicating boric acid solution to reproduce the actual conditions of

sample titration; 1.00 mL = 14 × normality × 1000 μg N. (For 0.02N, 1.00 mL = 280 μg N.)

4. Procedure

a. Proceed as described in Section 4500-NH3.B using indicating boric acid solution as

absorbent for the distillate.

b. Sludge or sediment samples: Rapidly weigh to within ±1% an amount of wet sample,

equivalent to approximately 1 g dry weight, in a weighing bottle or crucible. Wash sample into a

500-mL kjeldahl flask with water and dilute to 250 mL. Proceed as in ¶ 4a but add a piece of

paraffin wax to distillation flask and collect only 100 mL distillate.

c. Titrate ammonia in distillate with standard 0.02N H2SO4 titrant until indicator turns a pale

lavender.

d. Blank: Carry a blank through all steps of the procedure and apply the necessary correction

to the results.

5. Calculation

a. Liquid samples:

b. Sludge or sediment samples:

where:

A = volume of H2SO4 titrated for sample, mL, and

B = volume of H2SO4 titrated for blank, mL.

6. Precision and Bias

Three synthetic samples containing ammonia and other constituents dissolved in distilled

water were distilled and analyzed by titration.

Sample 1 contained 200 μg NH3-N/L, 10 mg Cl−/L, 1.0 mg NO3−-N/L, 1.5 mg organic N/L, 10.0 mg PO43−/L, and 5.0 mg silica/L. The relative standard deviation and relative error for the 21 participating laboratories were 69.8% and 20%, respectively. Sample 2 contained 800 μg NH3-N/L, 200 mg Cl−/L, 1.0 mg NO3−-N/L, 0.8 mg organic N/L, 5.0 mg PO4 3−/L, and 15.0 mg silica/L. The relative standard deviation and relative error for the 20 participating laboratories were 28.6% and 5%, respectively. Sample 3 contained 1500 μg NH3-N/L, 400 mg Cl−/L, 1.0 mg NO3−-N/L, 0.2 mg organic N/L, 0.5 mg PO4

3−/L, and 30.0 mg silica/L. The relative standard deviation and relative error for the 21 participating laboratories were 21.6%, and 2.6%, respectively.

4500-NH3 D. Ammonia-Selective Electrode Method

1. General Discussion

a. Principle: The ammonia-selective electrode uses a hydrophobic gas-permeable membrane

to separate the sample solution from an electrode internal solution of ammonium chloride.

Dissolved ammonia (NH3(aq) and NH4+) is converted to NH3(aq) by raising pH to above 11 with

a strong base. NH3(aq) diffuses through the membrane and changes the internal solution pH that

is sensed by a pH electrode. The fixed level of chloride in the internal solution is sensed by a

chloride ion-selective electrode that serves as the reference electrode. Potentiometric

measurements are made with a pH meter having an expanded millivolt scale or with a specific

ion meter.

b. Scope and application: This method is applicable to the measurement of 0.03 to 1400 mg

NH3-N/L in potable and surface waters and domestic and industrial wastes. High concentrations

of dissolved ions affect the measurement, but color and turbidity do not. Sample distillation is

unnecessary. Use standard solutions and samples that have the same temperature and contain

about the same total level of dissolved species. The ammonia-selective electrode responds

slowly below 1 mg NH3-N/L; hence, use longer times of electrode immersion (2 to 3 min) to

obtain stable readings.

c. Interference: Amines are a positive interference. This may be enhanced by acidification.

Mercury and silver interfere by complexing with ammonia, unless the NaOH/EDTA solution

(3c) is used.

d. Sample preservation: Refrigerate at 4°C for samples to be analyzed within 24 h. Preserve

samples high in organic and nitrogenous matter, and any other samples for longer storage, by

lowering pH to 2 or less with conc H2SO4.

2. Apparatus

a. Electrometer: A pH meter with expanded millivolt scale capable of 0.1 mV resolution

between −700 mV and +700 mV or a specific ion meter.

b. Ammonia-selective electrode.*#(55)

c. Magnetic stirrer, thermally insulated, with TFE-coated stirring bar.

3. Reagents

a. Ammonia-free water: See Section 4500-NH3.B.3a. Use for making all reagents.

b. Sodium hydroxide, 10N.

c. NaOH/EDTA solution, 10N: Dissolve 400 g NaOH in 800 mL water. Add 45.2 g

ethylenediaminetetraacetic acid, tetrasodium salt, tetrahydrate (Na4EDTA⋅4 H2O) and stir to

dissolve. Cool and dilute to 1000 mL.

d. Stock ammonium chloride solution: Dissolve 3.819 g anhydrous NH4Cl (dried at 100°C)

in water, and dilute to 1000 mL; 1.00 mL = 1.00 mg N = 1.22 mg NH3.

e. Standard ammonium chloride solutions: See ¶ 4a below.

4. Procedure

a. Preparation of standards: Prepare a series of standard solutions covering the

concentrations of 1000, 100, 10, 1, and 0.1 mg NH3-N/L by making decimal dilutions of stock

NH4Cl solution with water.

b. Electrometer calibration: Place 100 mL of each standard solution in a 150-mL beaker.

Immerse electrode in standard of lowest concentration and mix with a magnetic stirrer. Limit

stirring speed to minimize possible loss of ammonia from the solution. Maintain the same

stirring rate and a temperature of about 25°C throughout calibration and testing procedures. Add

a sufficient volume of 10N NaOH solution (1 mL usually is sufficient) to raise pH above 11. If

the presence of silver or mercury is possible, use NaOH/EDTA solution in place of NaOH

solution. If it is necessary to add more than 1 mL of either NaOH or NaOH/ EDTA solution, note

volume used, because it is required for subsequent calculations. Keep electrode in solution until

a stable millivolt reading is obtained. Do not add NaOH solution before immersing electrode,

because ammonia may be lost from a basic solution. Repeat procedure with remaining standards,

proceeding from lowest to highest concentration. Wait until the reading has stablized (at least 2

to 3 min) before recording millivolts for standards and samples containing ≤ 1 mg NH3-N/L.

c. Preparation of standard curve: Using semilogarithmic graph paper, plot ammonia

concentration in milligrams NH3-N per liter on the log axis vs. potential in millivolts on the

linear axis starting with the lowest concentration at the bottom of the scale. If the electrode is

functioning properly a tenfold change of NH3-N concentration produces a potential change of

about 59 mV.

d. Calibration of specific ion meter: Refer to manufacturer’s instructions and proceed as in ¶s

4a and b.

e. Measurement of samples: Dilute if necessary to bring NH3-N concentration to within

calibration curve range. Place 100 mL sample in 150-mL beaker and follow procedure in ¶ 4b

above. Record volume of 10N NaOH added. Read NH3-N concentration from standard curve.

5. Calculation

where:

A = dilution factor,

B = concentration of NH3-N/L, mg/L, from calibration curve,

C = volume of 10N NaOH added to calibration standards, mL, and D = volume of 10N NaOH added to sample, mL.

6. Precision and Bias

For the ammonia-selective electrode in a single laboratory using surface water samples at

concentrations of 1.00, 0.77, 0.19, and 0.13 mg NH3-N/L, standard deviations were ±0.038,

±0.017, ±0.007, and ±0.003, respectively. In a single laboratory using surface water samples at

concentrations of 0.10 and 0.13 mg NH3-N/L, recoveries were 96% and 91%, respectively. The

results of an interlaboratory study involving 12 laboratories using the ammonia-selective

electrode on distilled water and effluents are summarized in Table 4500-NH3:I.

4500-NH3 E. Ammonia-Selective Electrode Method Using Known Addition

1. General Discussion

a. Principle: When a linear relationship exists between concentration and response, known

addition is convenient for measuring occasional samples because no calibration is needed.

Because an accurate measurement requires that the concentration at least double as a result of the

addition, sample concentration must be known within a factor of three. Total concentration of

ammonia can be measured in the absence of complexing agents down to 0.8 mg NH3-N/L or in

the presence of a large excess (50 to 100 times) of complexing agent. Known addition is a

convenient check on the results of direct measurement.

b. See Section 4500-NH3.D.1 for further discussion.

2. Apparatus

Use apparatus specified in Section 4500-NH3.D.2.

3. Reagents

Use reagents specified in Section 4500-NH3.D.3.

Add standard ammonium chloride solution approximately 10 times as concentrated as

samples being measured.

4. Procedure

a. Dilute 1000 mg/L stock solution to make a standard solution about 10 times as

concentrated as the sample concentrate.

b. Add 1 mL 10N NaOH to each 100 mL sample and immediately immerse electrode. When

checking a direct measurement, leave electrode in 100 mL of sample solution. Use magnetic

stirring throughout. Measure mV reading and record as E1.

c. Pipet 10 mL of standard solution into sample. Thoroughly stir and immediately record new

mV reading as E2.

5. Calculation

a. Δ E = E1 − E2.

b. From Table 4500-NH3:II find the concentration ratio, Q, corresponding to change in

potential, Δ E. To determine original total sample concentration, multiply Q by the concentration

of the added standard:

Co = Q Cs

where:

Co = total sample concentration, mg/L,

Q = reading from known-addition table, and

Cs = concentration of added standard, mg/L.

c. To check a direct measurement, compare results of the two methods. If they agree within

±4%, the measurements probably are good. If the known-addition result is much larger than the

direct measurement, the sample may contain complexing agents.

6. Precision and Bias

In 38 water samples analyzed by both the phenate and the known-addition

ammonia-selective electrode method, the electrode method yielded a mean recovery of 102% of

the values obtained by the phenate method when the NH3-N concentrations varied between 0.30

and 0.78 mg/L. In 57 wastewater samples similarly compared, the electrode method yielded a

mean recovery of 108% of the values obtained by the phenate method using distillation when the

NH3-N concentrations varied between 10.2 and 34.7 mg N/L. In 20 instances in which two to

four replicates of these samples were analyzed, the mean standard deviation was 1.32 mg N/L. In

three measurements at a sewer outfall, distillation did not change statistically the value obtained

by the electrode method. In 12 studies using standards in the 2.5- to 30-mg N/L range, average recovery by the phenate method was 97% and by the electrode method 101%.

4500-NH3 F. Phenate Method

1. General Discussion

a. Principle: An intensely blue compound, indophenol, is formed by the reaction of

ammonia, hypochlorite, and phenol catalyzed by sodium nitroprusside.

b. Interferences: Complexing magnesium and calcium with citrate eliminates interference

produced by precipitation of these ions at high pH. There is no interference from other trivalent

forms of nitrogen. Remove interfering turbidity by distillation orfiltration. If hydrogen sulfide is

present, remove by acidifying samples to pH 3 with dilute HCl and aerating vigorously until

sulfide odor no longer can be detected.

2. Apparatus

Spectrophotometer for use at 640 nm with a light path of 1 cm or greater.

3. Reagents

a. Phenol solution: Mix 11.1 mL liquified phenol (≥89%) with 95% v/v ethyl alcohol to a

final volume of 100 mL. Prepare weekly. CAUTION: Wear gloves and eye protection when

handling phenol; use good ventilation to minimize all personnel exposure to this toxic volatile

substance.

b. Sodium nitroprusside, 0.5% w/v: Dissolve 0.5 g sodium nitroprusside in 100 mL deionized

water. Store in amber bottle for up to 1 month.

c. Alkaline citrate: Dissolve 200 g trisodium citrate and 10 g sodium hydroxide in deionized

water. Dilute to 1000 mL.

d. Sodium hypochlorite, commercial solution, about 5%. This solution slowly decomposes

once the seal on the bottle cap is broken. Replace about every 2 months.

e. Oxidizing solution: Mix 100 mL alkaline citrate solution with 25 mL sodium hypochlorite.

Prepare fresh daily.

f. Stock ammonium solution: See Section 4500-NH3.D.3d.

g. Standard ammonium solution: Use stock ammonium solution and water to prepare a

calibration curve in a range appropriate for the concentrations of the samples.

4. Procedure

To a 25-mL sample in a 50-mL erlenmeyer flask, add, with thorough mixing after each

addition, 1 mL phenol solution, 1 mL sodium nitroprusside solution, and 2.5 mL oxidizing

solution. Cover samples with plastic wrap or paraffin wrapper film. Let color develop at room

temperature (22 to 27°C) in subdued light for at least 1 h. Color is stable for 24 h. Measure

absorbance at 640 nm. Prepare a blank and at least two other standards by diluting stock ammonia solution into the sample concentration range. Treat standards the same as samples.

5. Calculations

Prepare a standard curve by plotting absorbance readings of standards against ammonia

concentrations of standards. Compute sample concentration by comparing sample absorbance

with the standard curve.

6. Precision and Bias

For the manual phenate method, reagent water solutions of ammonium sulfate were prepared

and analyzed by two analysts in each of three laboratories. Results are summarized in Table

4500-NH3:III.

4500-NH3 G. Automated Phenate Method

1. General Discussion

a. Principle: Alkaline phenol and hypochlorite react with ammonia to form indophenol blue

that is proportional to the ammonia concentration. The blue color formed is intensified with

sodium nitroprusside.

b. Interferences: Seawater contains calcium and magnesium ions in sufficient concentrations

to cause precipitation during analysis. Adding EDTA and sodium potassium tartrate reduces the

problem. Eliminate any marked variation in acidity or alkalinity among samples because

intensity of measured color is pH-dependent. Likewise, insure that pH of wash water and

standard ammonia solutions approximates that of sample. For example, if sample has been

preserved with 0.8 mL conc H2SO4/L, include 0.8 mL conc H2SO4/L in wash water and

standards. Remove interfering turbidity by filtration. Color in the samples that absorbs in the

photometric range used for analysis interferes.

c. Application: Ammonia nitrogen can be determined in potable, surface, and saline waters as

well as domestic and industrial wastewaters over a range of 0.02 to 2.0 mg/L when photometric

measurement is made at 630 to 660 nm in a 10- to 50-mm tubular flow cell at rates of up to 60

samples/h. Determine higher concentrations by diluting the sample.

2. Apparatus

Automated analytical equipment. An example of the required continuous-flow analytical instrument consists of the interchangeable components shown in Figure 4500-NH3:1.

3. Reagents

a. Ammonia-free distilled water: See Section 4500-NH3.B.3a. Use for preparing all reagents

and dilutions.

b. Sulfuric acid, H2SO4, 5N, air scrubber solution: Carefully add 139 mL conc H2SO4 to

approximately 500 mL water, cool to room temperature, and dilute to 1 L.

c. Sodium phenate solution: In a 1-L erlenmeyer flask, dissolve 93 mL liquid (≥89%) phenol

in 500 mL water. In small increments and with agitation, cautiously add 32 g NaOH. Cool flask

under running water and dilute to 1 L. CAUTION: Minimize exposure of personnel to this

compound by wearing gloves and eye protection, and using proper ventilation.

d. Sodium hypochlorite solution: Dilute 250 mL bleach solution containing 5.25% NaOCl to

500 mL with water.

e. EDTA reagent: Dissolve 50 g disodium ethylenediamine tetraacetate and approximately

six pellets NaOH in 1 L water. For salt-water samples where EDTA reagent does not prevent

precipitation of cations, use sodium potassium tartrate solution prepared as follows:

Sodium potassium tartrate solution: To 900 mL water add 100 g NaKC4H4O6⋅4H2O, two

pellets NaOH, and a few boiling chips, and boil gently for 45 min. Cover, cool, and dilute to 1 L.

Adjust pH to 5.2 ± 0.05 with H2SO4. Let settle overnight in a cool place and filter to remove

precipitate. Add 0.5 mL polyoxyethylene 23 lauryl ether*#(56) solution and store in stoppered

bottle.

f. Sodium nitroprusside solution: Dissolve 0.5 g Na2(NO)Fe(CN)5⋅2H2O in 1 L water.

g. Ammonia standard solutions: See Section 4500-NH3.D.3c and d. Use standard ammonia

solution and water to prepare the calibration curve in the appropriate ammonia concentration

range. To analyze saline waters use substitute ocean water of the following composition to

prepare calibration standards:

Constituent                                     Concentration

g/L

NaCl                                           24.53

MgCl2                                                   5.20

CaCl2                                                   1.16

KCl                                              0.70

SrCl2                                                    0.03

Na2SO4                                              4.09

NaHCO3                                             0.20

KBr                                              0.10

H3BO3                                                 0.03

NaF                                             0.003

Subtract blank background response of substitute seawater from standards before preparing

standard curve.

4. Procedure

a. Eliminate marked variation in acidity or alkalinity among samples. Adjust pH of wash

water and standard ammonia solutions to approximately that of sample.

b. Set up manifold and complete system as shown in Figure 4500-NH3:1.

c. Obtain a stable base line with all reagents, feeding wash water through sample line.

d. Typically, use a 60/h, 6:1 cam with a common wash.

5. Calculation

Prepare standard curves by plotting response of standards processed through the manifold

against NH3-N concentrations in standards. Compute sample NH3-N concentration by

comparing sample response with standard curve.

6. Precision and Bias

For an automated phenate system in a single laboratory using surface water samples at

concentrations of 1.41, 0.77, 0.59, and 0.43 mg NH3-N/L, the standard deviation was ±0.005

mg/L, and at concentrations of 0.16 and 1.44 mg NH3-N/L, recoveries were 107 and 99%,

respectively.

4500-NH3 H. Flow Injection Analysis (PROPOSED)

1. General Discussion

a. Principle: A water sample containing ammonia or ammonium cation is injected into an

FIA carrier stream to which a complexing buffer, alkaline phenol, and hypochlorite are added.

This reaction, the Berthelot reaction, produces the blue indophenol dye. The blue color is

intensified by the addition of nitroferricyanide. The resulting peak’s absorbance is measured at

630 nm. The peak area is proportional to the concentration of ammonia in the original sample.

Also see Section 4500-NH3.F and Section 4130, Flow Injection Analysis (FIA).

b. Interferences: Remove large or fibrous particulates by filtering sample through glass

wool. Guard against contamination from reagents, water, glassware, and the sample preservation

process.

Also see Section 4500-NH3.A. Some interferents are removed by distillation; see Section

4500-NH3.B.

2. Apparatus

Flow injection analysis equipment consisting of:

a. FIA injection valve with sample loop or equivalent.

b. Multichannel proportioning pump.

c. FIA manifold (Figure 4500-NH3:2) with tubing heater and flow cell. In Figure

4500-NH3:2, relative flow rates only are shown. Tubing volumes are given as an example only;

they may be scaled down proportionally. Use manifold tubing of an inert material such as

TFE.*#(57)

d. Absorbance detector, 630 nm, 10-nm bandpass.

e. Injection valve control and data acquisition system.

3. Reagents

Use reagent water (>10 megohm) to prepare carrier and all solutions. To prevent bubble

formation, degas carrier and buffer with helium. Use He at 140 kPa (20 psi) through a helium

degassing tube. Bubble He through 1 L solution for 1 min. As an alternative to preparing

reagents by weight/weight, use weight/volume.

a. Buffer: To a 1-L tared container add 50.0 g disodium ethylenediamine tetraacetate and 5.5

g sodium hydroxide, NaOH. Add 968 mL water. Mix with a magnetic stirrer until dissolved.

b. Phenolate: CAUTION: Wear gloves. Phenol causes severe burns and is rapidly absorbed

into the body through the skin. To a tared 1-L container, add 888 g water. Add 94.2 g 88%

liquefied phenol or 83 g crystalline phenol, C6H5OH. While stirring, slowly add 32 g NaOH.

Cool and invert to mix thoroughly. Do not degas.

c. Hypochlorite: To a tared 500-mL container add 250 g 5.25% sodium hypochlorite, NaOCl

bleach solution†#(58) and 250 g water. Stir or shake to mix.

d. Nitroprusside: To a tared 1-L container add 3.50 g sodium nitroprusside (sodium

nitroferricyanide), Na2Fe(CN)5NO⋅2H2O, and 1000 g water. Invert to mix.

e. Stock ammonia standard, 1000 mg N/L: In a 1-L volumetric flask dissolve 3.819 g

ammonium chloride, NH4Cl, that has been dried for 2 h at 110°C, in about 800 mL water. Dilute

to mark and invert to mix.

f. Standard ammonia solutions: Prepare ammonia standards in desired concentration range,

using the stock standard (¶ 3e), and diluting with water.

4. Procedure

Set up a manifold equivalent to that in Figure 4500-NH3:2 and follow method supplied by

manufacturer or laboratory standard operating procedure for this method. Follow quality control

procedures described in Section 4020.

5. Calculations

Prepare standard curves by plotting the absorbance of standards processed through the

manifold versus ammonia concentration. The calibration curve is linear.

6. Precision and Bias

a. Recovery and relative standard deviation: The results of single-laboratory studies with

various matrices are given in Table 4500-NH3:IV.

b. MDL: A 650-μL sample loop was used in the method described above. Using a published

MDL method,1 analysts ran 21 replicates of a 0.020-mg N/L standard. These gave a mean of

0.0204 mg N/L, a standard deviation of 0.0007 mg N/L, and an MDL of 0.002 mg N/L.

4500-NO2– NITROGEN (NITRITE)*#(59)

4500-NO2– A. Introduction

1. Occurrence and Significance

For a discussion of the chemical characteristics, sources, and effects of nitrite nitrogen, see

Section 4500-N.

2. Selection of Method

The colorimetric method (B) is suitable for concentrations of 5 to 1000 μg NO2–-N/L (See ¶

B.1a). Nitrite values can be obtained by the automated method given in Section 4500-NO3–.E

with the Cu-Cd reduction step omitted. Additionally, nitrite nitrogen can be determined by ion

chromatography (Section 4110), and by flow injection analysis (see Section 4130 and Section

4500-NO3–.I).

4500-NO2– B. Colorimetric Method

1. General Discussion

a. Principle: Nitrite (NO2–) is determined through formation of a reddish purple azo dye

produced at pH 2.0 to 2.5 by coupling diazotized sulfanilamide with N-(1-naphthyl)-ethylenediamine dihydrochloride (NED dihydrochloride). The applicable range

of the method for spectrophotometric measurements is 10 to 1000 μg NO2–-N/L. Photometric

measurements can be made in the range 5 to 50 μg N/L if a 5-cm light path and a green color

filter are used. The color system obeys Beer’s law up to 180 μg N/ L with a 1-cm light path at 543 nm. Higher NO2– concentrations can be determined by diluting a sample.

b. Interferences: Chemical incompatibility makes it unlikely that NO2–, free chlorine, and

nitrogen trichloride (NCl3) will coexist. NCl3 imparts a false red color when color reagent is

added. The following ions interfere because of precipitation under test conditions and should be

absent: Sb3+, Au3+, Bi3+, Fe3+, Pb2+, Hg2+, Ag+, chloroplatinate (PtCl62–), and metavanadate

(VO32–). Cupric ion may cause low results by catalyzing decomposition of the diazonium salt.

Colored ions that alter the color system also should be absent. Remove suspended solids by

filtration.

c. Storage of sample: Never use acid preservation for samples to be analyzed for NO2–.

Make the determination promptly on fresh samples to prevent bacterial conversion of NO2– to

NO3– or NH3. For short-term preservation for 1 to 2 d, freeze at −20°C or store at 4°C.

2. Apparatus

Colorimetric equipment: One of the following is required:

a. Spectrophotometer, for use at 543 nm, providing a light path of 1 cm or longer.

b. Filter photometer, providing a light path of 1 cm or longer and equipped with a green

filter having maximum transmittance near 540 nm.

3. Reagents

a. Nitrite-free water: If it is not known that the distilled or demineralized water is free from

NO2–, use either of the following procedures to prepare nitrite-free water:

1) Add to 1 L distilled water one small crystal each of KMnO4 and either Ba(OH)2 or

Ca(OH)2. Redistill in an all-borosilicate-glass apparatus and discard the initial 50 mL of

distillate. Collect the distillate fraction that is free of permanganate; a red color with DPD

reagent (Section 4500-Cl.F.2b) indicates the presence of permanganate.

2) Add 1 mL conc H2SO4 and 0.2 mL MnSO4 solution (36.4 g MnSO4⋅H2O/100 mL

distilled water) to each 1 L distilled water, and make pink with 1 to 3 mL KMnO4 solution (400

mg KMnO4/ L distilled water). Redistill as described in the preceding paragraph.

Use nitrite-free water in making all reagents and dilutions.

b. Color reagent: To 800 mL water add 100 mL 85% phosphoric acid and 10 g

sulfanilamide. After dissolving sulfanilamide completely, add 1 g

N-(1-naphthyl)-ethylenediamine dihydrochloride. Mix to dissolve, then dilute to 1 L with water.

Solution is stable for about a month when stored in a dark bottle in refrigerator.

c. Sodium oxalate, 0.025M (0.05N): Dissolve 3.350 g Na2C2O4, primary standard grade, in

water and dilute to 1000 mL.

d. Ferrous ammonium sulfate, 0.05M (0.05N): Dissolve 19.607 g Fe(NH4)2 (SO4)2⋅6H2O

plus 20 mL conc H2SO4 in water and dilute to 1000 mL. Standardize as in Section 5220B.3d.

e. Stock nitrite solution: Commercial reagent-grade NaNO2 assays at less than 99%.

Because NO2– is oxidized readily in the presence of moisture, use a fresh bottle of reagent for

preparing the stock solution and keep bottles tightly stoppered against the free access of air when

not in use. To determine NaNO2 content, add a known excess of standard 0.01M (0.05N)

KMnO4 solution (see ¶ h below), discharge permanganate color with a known quantity of

standard reductant such as 0.025M Na2C2O4 or 0.05M Fe(NH4)2(SO4)2, and back-titrate with

standard permanganate solution.

1) Preparation of stock solution—Dissolve 1.232 g NaNO2 in water and dilute to 1000 mL;

1.00 mL = 250 μg N. Preserve with 1 mL CHCl3.

2) Standardization of stock nitrite solution—Pipet, in order, 50.00 mL standard 0.01M

(0.05N) KMnO4, 5 mL conc H2SO4, and 50.00 mL stock NO2– solution into a glass-stoppered

flask or bottle. Submerge pipet tip well below surface of permanganate-acid solution while adding stock NO2– solution. Shake gently and warm to 70 to 80°C on a hot plate. Discharge

permanganate color by adding sufficient 10-mL portions of standard 0.025M Na2C2O4. Titrate

excess Na2C2O4 with 0.01M (0.05N) KMnO4 to the faint pink end point. Carry a water blank

through the entire procedure and make the necessary corrections in the final calculation as shown

in the equation below.

If standard 0.05M ferrous ammonium sulfate solution is substituted for Na2C2O4, omit

heating and extend reaction period between KMnO4 and Fe2+ to 5 min before making final

KMnO4 titration.

Calculate NO2–-N content of stock solution by the following equation:

where:

A = mg NO2–-N/mL in stock NaNO2 solution,

B = total mL standard KMnO4 used,

C = normality of standard KMnO4,

D = total mL standard reductant added,

E = normality of standard reductant, and

F = mL stock NaNO2 solution taken for titration.

Each 1.00 mL 0.01M (0.05N) KMnO4 consumed by the NaNO2 solution corresponds to 1750 μg NO2–-N.

f. Intermediate nitrite solution: Calculate the volume, G, of stock NO2– solution required for

the intermediate NO2– solution from G = 12.5/A. Dilute the volume G (approximately 50 mL) to

250 mL with water; 1.00 mL = 50.0 μg N. Prepare daily.

g. Standard nitrite solution: Dilute 10.00 mL intermediate NO2– solution to 1000 mL with

water; 1.00 mL = 0.500 μg N. Prepare daily.

h. Standard potassium permanganate titrant, 0.01M (0.05N): Dissolve 1.6 g KMnO4 in 1 L

distilled water. Keep in a brown glass-stoppered bottle and age for at least 1 week. Carefully

decant or pipet supernate without stirring up any sediment. Standardize this solution frequently

by the following procedure:

Weigh to the nearest 0.1 mg several 100- to 200-mg samples of anhydrous Na2C2O4 into

400-mL beakers. To each beaker, in turn, add 100 mL distilled water and stir to dissolve. Add 10 mL 1 + 1 H2SO4 and heat rapidly to 90 to 95°C. Titrate rapidly with permanganate solution to be

standardized, while stirring, to a slight pink end-point color that persists for at least 1 min. Do

not let temperature fall below 85°C. If necessary, warm beaker contents during titration; 100 mg

will consume about 6 mL solution. Run a blank on distilled water and H2SO4.

where:

A = mL titrant for sample and

B = mL titrant for blank.

Average the results of several titrations.

4. Procedure

a. Removal of suspended solids: If sample contains suspended solids, filter through a

0.45-μm-pore-diam membrane filter.

b. Color development: If sample pH is not between 5 and 9, adjust to that range with 1N HCl

or NH4OH as required. To 50.0 mL sample, or to a portion diluted to 50.0 mL, add 2 mL color

reagent and mix.

c. Photometric measurement: Between 10 min and 2 h after adding color reagent to samples

and standards, measure absorbance at 543 nm. As a guide use the following light paths for the

indicated NO2–-N concentrations:

Light Path Length                       NO2–-N

Cm                                                μg/L

1                                           2–25

5                                           2–6

10                                         <2

5. Calculation

Prepare a standard curve by plotting absorbance of standards against NO2–-N concentration.

Compute sample concentration directly from curve.

6. Precision and Bias

In a single laboratory using wastewater samples at concentrations of 0.04, 0.24, 0.55, and 1.04 mg NO–3 + NO2–-N/L, the standard deviations were ±0.005, ± 0.004, ± 0.005, and ± 0.01,

respectively. In a single laboratory using wastewater samples at concentrations of 0.24, 0.55, and

1.05 mg NO3– + NO2–-N/L, the recoveries were 100%, 102%, and 100%, respectively.1

4500-NO2– C. (Reserved)

4500-NO3– NITROGEN (NITRATE)*#(60)

4500-NO3– A. Introduction

1. Selection of Method

Determination of nitrate (NO3–) is difficult because of the relatively complex procedures

required, the high probability that interfering constituents will be present, and the limited

concentration ranges of the various techniques.

An ultraviolet (UV) technique (Method B) that measures the absorbance of NO3– at 220 nm

is suitable for screening uncontaminated water (low in organic matter).

Screen a sample; if necessary, then select a method suitable for its concentration range and probable interferences. Nitrate may be determined by ion chromatography (Section 4110) or

capillary ion electrophoresis (Section 4140). Applicable ranges for other methods are: nitrate

electrode method (D), 0.14 to 1400 mg NO3–-N/L; cadmium reduction method (E), 0.01 to 1.0

mg NO3–-N/L; automated cadmium reduction methods (F and I), 0.001 to 10 mg NO3–-N/L. For

higher NO3–-N concentrations, dilute into the range of the selected method.

Colorimetric methods (B, E) require an optically clear sample. Filter turbid sample through

0.45-μm-pore-diam membrane filter. Test filters for nitrate contamination.

2. Storage of Samples

Start NO3– determinations promptly after sampling. If storage is necessary, store for up to 2

d at 4°C; disinfected samples are stable much longer without acid preservation. For longer

storage of unchlorinated samples, preserve with 2 mL conc H2SO4/L and store at 4°C. NOTE:

When sample is preserved with acid, NO3– and NO2– cannot be determined as individual

species.

4500-NO3– B. Ultraviolet Spectrophotometric Screening Method

1. General Discussion

a. Principle: Use this technique only for screening samples that have low organic matter

contents, i.e., uncontaminated natural waters and potable water supplies. The NO3– calibration

curve follows Beer’s law up to 11 mg N/L.

Measurement of UV absorption at 220 nm enables rapid determination of NO3–. Because

dissolved organic matter also may absorb at 220 nm and NO3– does not absorb at 275 nm, a

second measurement made at 275 nm may be used to correct the NO3– value. The extent of this

empirical correction is related to the nature and concentration of organic matter and may vary

from one water to another. Consequently, this method is not recommended if a significant

correction for organic matter absorbance is required, although it may be useful in monitoring

NO3– levels within a water body with a constant type of organic matter. Correction factors for

organic matter absorbance can be established by the method of additions in combination with

analysis of the original NO3– content by another method. Sample filtration is intended to remove

possible interference from suspended particles. Acidification with 1N HCl is designed to prevent

interference from hydroxide or carbonate concentrations up to 1000 mg CaCO3/L. Chloride has

no effect on the determination.

b. Interference: Dissolved organic matter, surfactants, NO2–, and Cr6+ interfere. Various

inorganic ions not normally found in natural water, such as chlorite and chlorate, may interfere.

Inorganic substances can be compensated for by independent analysis of their concentrations and

preparation of individual correction curves. For turbid samples, see ¶ A.1.

2. Apparatus

Spectrophotometer, for use at 220 nm and 275 nm with matched silica cells of 1-cm or

longer light path.

3. Reagents

a. Nitrate-free water: Use redistilled or distilled, deionized water of highest purity to

prepare all solutions and dilutions.

b. Stock nitrate solution: Dry potassium nitrate (KNO3) in an oven at 105°C for 24 h.

Dissolve 0.7218 g in water and dilute to 1000 mL; 1.00 mL = 100 μg NO3–-N. Preserve with 2

mL CHCl3/L. This solution is stable for at least 6 months.

c. Intermediate nitrate solution: Dilute 100 mL stock nitrate solution to 1000 mL with

water; 1.00 mL = 10.0 μg NO3–-N. Preserve with 2 mL CHCl3/L. This solution is stable for 6

months.

d. Hydrochloric acid solution, HCl, 1N.

4. Procedure

a. Treatment of sample: To 50 mL clear sample, filtered if necessary, add 1 mL HCl solution

and mix thoroughly.

b. Preparation of standard curve: Prepare NO3– calibration standards in the range 0 to 7 mg

NO3–-N/L by diluting to 50 mL the following volumes of intermediate nitrate solution: 0, 1.00,

2.00, 4.00, 7.00 . . . 35.0 mL. Treat NO3– standards in same manner as samples.

c. Spectrophotometric measurement: Read absorbance or transmittance against redistilled

water set at zero absorbance or 100% transmittance. Use a wavelength of 220 nm to obtain NO3–

reading and a wavelength of 275 nm to determine interference due to dissolved organic matter.

5. Calculation

For samples and standards, subtract two times the absorbance reading at 275 nm from the

reading at 220 nm to obtain absorbance due to NO3–. Construct a standard curve by plotting

absorbance due to NO3– against NO3–-N concentration of standard. Using corrected sample

absorbances, obtain sample concentrations directly from standard curve. NOTE: If correction

value is more than 10% of the reading at 220 nm, do not use this method.

4500-NO3– C. (Reserved)

4500-NO3– D. Nitrate Electrode Method

1. General Discussion

a. Principle: The NO3– ion electrode is a selective sensor that develops a potential across a

thin, porous, inert membrane that holds in place a water-immiscible liquid ion exchanger. The

electrode responds to NO3– ion activity between about 10–5 and 10–1 M (0.14 to 1400 mg

NO3–-N/L). The lower limit of detection is determined by the small but finite solubility of the

liquid ion exchanger.

b. Interferences: Chloride and bicarbonate ions interfere when their weight ratios to NO3–-N

are >10 or >5, respectively. Ions that are potential interferences but do not normally occur at

significant levels in potable waters are NO2–, CN–, S2–, Br–, I–, ClO3–, and ClO4–. Although the

electrodes function satisfactorily in buffers over the range pH 3 to 9, erratic responses have been

noted where pH is not held constant. Because the electrode responds to NO3– activity rather than

concentration, ionic strength must be constant in all samples and standards. Minimize these

problems by using a buffer solution containing Ag2SO4 to remove Cl–, Br–, I–, S2–, and CN–,

sulfamic acid to remove NO2–, a buffer at pH 3 to eliminate HCO3– and to maintain a constant

pH and ionic strength, and Al2(SO4)3 to complex organic acids.

2. Apparatus

a. pH meter, expanded-scale or digital, capable of 0.1 mV resolution.

b. Double-junction reference electrode.*#(61) Fill outer chamber with (NH4)2SO4 solution.

c. Nitrate ion electrode:†#(62) Carefully follow manufacturer’s instructions regarding care

and storage.

d. Magnetic stirrer: TFE-coated stirring bar.

3. Reagents

a. Nitrate-free water: Prepare as described in ¶ B.3a. Use for all solutions and dilutions.

b. Stock nitrate solution: Prepare as described in ¶ B.3b.

c. Standard nitrate solutions: Dilute 1.0, 10, and 50 mL stock nitrate solution to 100 mL

with water to obtain standard solutions of 1.0, 10, and 50 mg NO3–-N/L, respectively.

d. Buffer solution: Dissolve 17.32 g Al2(SO4)318H2O, 3.43 g Ag2SO4, 1.28 g H3BO3, and

2.52 g sulfamic acid (H2NSO3H), in about 800 mL water. Adjust to pH 3.0 by slowly adding

0.10N NaOH. Dilute to 1000 mL and store in a dark glass bottle.

e. Sodium hydroxide, NaOH, 0.1N.

f. Reference electrode filling solution: Dissolve 0.53 g (NH4)2SO4 in water and dilute to 100

mL.

4. Procedure

a. Preparation of calibration curve: Transfer 10 mL of 1 mg NO3–-N/L standard to a 50-mL

beaker, add 10 mL buffer, and stir with a magnetic stirrer. Immerse tips of electrodes and record

millivolt reading when stable (after about 1 min). Remove electrodes, rinse, and blot dry. Repeat

for 10-mg NO3–-N/L and 50-mg NO3–-N/L standards. Plot potential measurements against

NO3–-N concentration on semilogarithmic graph paper, with NO3–-N concentration on the

logarithmic axis (abscissa) and potential (in millivolts) on the linear axis (ordinate). A straight

line with a slope of +57 ±3 mV/decade at 25°C should result. Recalibrate electrodes several

times daily by checking potential reading of the 10 mg NO3–-N standard and adjusting the

calibration control until the reading plotted on the calibration curve is displayed again.

b. Measurement of sample: Transfer 10 mL sample to a 50-mL beaker, add 10 mL buffer

solution, and stir (for about 1 min) with a magnetic stirrer. Measure standards and samples at

about the same temperature. Immerse electrode tips in sample and record potential reading when

stable (after about 1 min). Read concentration from calibration curve.

5. Precision

Over the range of the method, precision of ±0.4 mV, corresponding to 2.5% in

concentration, is expected.

4500-NO3– E. Cadmium Reduction Method

1. General Discussion

a. Principle: NO3– is reduced almost quantitatively to nitrite (NO2–) in the presence of

cadmium (Cd). This method uses commercially available Cd granules treated with copper sulfate

(CuSO4) and packed in a glass column.

The NO2– produced thus is determined by diazotizing with sulfanilamide and coupling with

N-(1-naphthyl)-ethylenediamine dihydrochloride to form a highly colored azo dye that is

measured colorimetrically. A correction may be made for any NO2– present in the sample by

analyzing without the reduction step. The applicable range of this method is 0.01 to 1.0 mg

NO3–-N/L. The method is recommended especially for NO3– levels below 0.1 mg N/L where

other methods lack adequate sensitivity.

b. Interferences: Suspended matter in the column will restrict sample flow. For turbid

samples, see ¶ A.1. Concentrations of iron, copper, or other metals above several milligrams per

liter lower reduction efficiency. Add EDTA to samples to eliminate this interference. Oil and

grease will coat the Cd surface. Remove by pre-extraction with an organic solvent (see Section

5520). Residual chlorine can interfere by oxidizing the Cd column, reducing its efficiency.

Check samples for residual chlorine (see DPD methods in Section 4500-Cl). Remove residual

chlorine by adding sodium thiosulfate (Na2S2O3) solution (Section 4500-NH3.B.3d). Sample

color that absorbs at about 540 nm interferes.

2. Apparatus

a. Reduction column: Purchase or construct the column*#(63) (Figure 4500-NO3–:1) from a

100-mL volumetric pipet by removing the top portion. The column also can be constructed from

two pieces of tubing joined end to end: join a 10-cm length of 3-cm-ID tubing to a 25-cm length

of 3.5-mm-ID tubing. Add a TFE stopcock with metering valve1 to control flow rate.

b. Colorimetric equipment: One of the following is required:

1) Spectrophotometer, for use at 543 nm, providing a light path of 1 cm or longer.

2) Filter photometer, with light path of 1 cm or longer and equipped with a filter having

maximum transmittance near 540 nm.

3. Reagents

a. Nitrate-free water: See ¶ B.3a. The absorbance of a reagent blank prepared with this

water should not exceed 0.01. Use for all solutions and dilutions.

b. Copper-cadmium granules: Wash 25 g new or used 20- to 100-mesh Cd granules†#(64)

with 6N HCl and rinse with water. Swirl Cd with 100 mL 2% CuSO4 solution for 5 min or until

blue color partially fades. Decant and repeat with fresh CuSO4 until a brown colloidal precipitate

begins to develop. Gently flush with water to remove all precipitated Cu.

c. Color reagent: Prepare as directed in Section 4500-NO2–B.3b.

d. Ammonium chloride-EDTA solution: Dissolve 13 g NH4Cl and 1.7 g disodium

ethylenediamine tetraacetate in 900 mL water. Adjust to pH 8.5 with conc NH4OH and dilute to

1 L.

e. Dilute ammonium chloride-EDTA solution: Dilute 300 mL NH4Cl-EDTA solution to 500

mL with water.

f. Hydrochloric acid, HCl, 6N.

g. Copper sulfate solution, 2%: Dissolve 20 g CuSO4.5H2O in 500 mL water and dilute to 1

L.

h. Stock nitrate solution: Prepare as directed in ¶ B.3b.

i. Intermediate nitrate solution: Prepare as directed in ¶ B.3c.

j. Stock nitrite solution: See Section 4500-NO2–.B.3e.

k. Intermediate nitrite solution: See Section 4500-NO2–.B.3 f.

l. Working nitrite solution: Dilute 50.0 mL intermediate nitrite solution to 500 mL with

nitrite-free water; 1.00 mL = 5 μg NO2–-N.

4. Procedure

a. Preparation of reduction column: Insert a glass wool plug into bottom of reduction

column and fill with water. Add sufficient Cu-Cd granules to produce a column 18.5 cm long.

Maintain water level above Cu-Cd granules to prevent entrapment of air. Wash column with 200

mL dilute NH4Cl-EDTA solution. Activate column by passing through it, at 7 to 10 mL/min, at

least 100 mL of a solution composed of 25% 1.0 mg NO3–-N/L standard and 75% NH4Cl-EDTA

solution.

b. Treatment of sample:

1) Turbidity removal—For turbid samples, see ¶ A.1.

2) pH adjustment—Adjust pH to between 7 and 9, as necessary, using a pH meter and dilute

HCl or NaOH. This insures a pH of 8.5 after adding NH4Cl- EDTA solution.

3) Sample reduction—To 25.0 mL sample or a portion diluted to 25.0 mL, add 75 mL

NH4Cl-EDTA solution and mix. Pour mixed sample into column and collect at a rate of 7 to 10

mL/min. Discard first 25 mL. Collect the rest in original sample flask. There is no need to wash

columns between samples, but if columns are not to be reused for several hours or longer, pour 50 mL dilute NH4Cl-EDTA solution on to the top and let it pass through the system. Store

Cu-Cd column in this solution and never let it dry.

4) Color development and measurement—As soon as possible, and not more than 15 min

after reduction, add 2.0 mL color reagent to 50 mL sample and mix. Between 10 min and 2 h

afterward, measure absorbance at 543 nm against a distilled water-reagent blank. NOTE: If NO3–

concentration exceeds the standard curve range (about 1 mg N/L), use remainder of reduced

sample to make an appropriate dilution and analyze again.

c. Standards: Using the intermediate NO3–- N solution, prepare standards in the range 0.05

to 1.0 mg NO3–-N/L by diluting the following volumes to 100 mL in volumetric flasks: 0.5, 1.0,

2.0, 5.0, and 10.0 mL. Carry out reduction of standards exactly as described for samples.

Compare at least one NO2– standard to a reduced NO3– standard at the same concentration to

verify reduction column efficiency. Reactivate Cu-Cd granules as described in ¶ 3b above when

efficiency of reduction falls below about 75%.

5. Calculation

Obtain a standard curve by plotting absorbance of standards against NO3–-N concentration.

Compute sample concentrations directly from standard curve. Report as milligrams oxidized N

per liter (the sum of NO3–-N plus NO2–-N) unless the concentration of NO2–-N is separately

determined and subtracted.

6. Precision and Bias

In a single laboratory using wastewater samples at concentrations of 0.04, 0.24, 0.55, and

1.04 mg NO3– + NO2–-N/L, the standard deviations were ±0.005, ±0.004, ±0.005, and ±0.01,

respectively. In a single laboratory using wastewater with additions of 0.24, 0.55, and 1.05 mg

NO3– + NO2–-N/L, the recoveries were 100%, 102%, and 100%, respectively.2

4500-NO3– F. Automated Cadmium Reduction Method

1. General Discussion

a. Principle: See ¶ E.1a.

b. Interferences: Sample turbidity may interfere. Remove by filtration before analysis.

Sample color that absorbs in the photometric range used for analysis also will interfere.

c. Application: Nitrate and nitrite, singly or together in potable, surface, and saline waters

and domestic and industrial wastewaters, can be determined over a range of 0.5 to 10 mg N/L.

2. Apparatus

Automated analytical equipment: An example of the continuous-flow analytical instrument

consists of the components shown in Figure 4500-NO3–:2.

3. Reagents

a. Deionized distilled water: See ¶ B.3a.

b. Copper sulfate solution: Dissolve 20 g CuSO4⋅5H2O in 500 mL water and dilute to 1 L.

c. Wash solution: Use water for unpreserved samples. For samples preserved with H2SO4,

add 2 mL conc H2SO4/L wash water.

d. Copper-cadmium granules: See ¶ E.3b.

e. Hydrochloric acid, HCl, conc.

f. Ammonium hydroxide, NH4OH, conc.

g. Color reagent: To approximately 800 mL water, add, while stirring, 100 mL conc H3PO4,

40 g sulfanilamide, and 2 g N-(1-naphthyl)-ethylenediamine dihydrochloride. Stir until dissolved

and dilute to 1 L. Store in brown bottle and keep in the dark when not in use. This solution is

stable for several months.

h. Ammonium chloride solution: Dissolve 85 g NH4Cl in water and dilute to 1 L. Add 0.5

mL polyoxyethylene 23 lauryl ether.*#(65)

i. Stock nitrate solution: See ¶ B.3b.

j. Intermediate nitrate solution: See ¶ B.3c.

k. Standard nitrate solutions: Using intermediate NO3–-N solution and water, prepare

standards for calibration curve in appropriate nitrate range. Compare at least one NO2– standard to a NO3– standard at the same concentration to verify column reduction efficiency. To examine

saline waters prepare standard solutions with the substitute ocean water described in Section

4500-NH3.G.3g.

l. Standard nitrite solution: See Section 4500-NO2–.B.3g.

4. Procedure

Set up manifold as shown in Figure 4500-NO3–:2 and follow general procedure described

by the manufacturer.

If sample pH is below 5 or above 9, adjust to between 5 and 9 with either conc HCl or conc

NH4OH.

5. Calculation

Prepare standard curves by plotting response of standards processed through the manifold

against NO3–-N concentration in standards. Compute sample NO3–-N concentration by

comparing sample response with standard curve.

6. Precision and Bias

Data obtained in three laboratories with an automated system based on identical chemical

principles but having slightly different configurations are given in the table below. Analyses

were conducted on four natural water samples containing exact increments of inorganic nitrate:

Increment as

NO3–-N

μg/L                   Standard Deviation

μg N/L                  Bias %              Bias μg N/L

290                        12                      +5.75                  +17

350                        92                      +18.10                +63

2310                      318                    +4.47                  +103

2480                      176                    −2.69                   −67

In a single laboratory using surface water samples at concentrations of 100, 200, 800, and

2100 μg N/L, the standard deviations were 0, ±40, ±50, and ±50 μg/L, respectively, and at

concentrations of 200 and 2200 μg N/L, recoveries were 100 and 96%, respectively.

Precision and bias for the system described herein are believed to be comparable.

4500-NO3– G. (Reserved)

4500-NO3– H. Automated Hydrazine Reduction Method

1. General Discussion

a. Principle: NO3– is reduced to NO2– with hydrazine sulfate. The NO2–(originally present)

plus reduced NO3– is determined by diazotization with sulfanilamide and coupling with

N-(1-naphthyl)-ethylenediamine dihydrochloride to form a highly colored azo dye that is

measured colorimetrically.

b. Interferences: Sample color that absorbs in the photometric range used will interfere.

Concentrations of sulfide ion of less than 10 mg/L cause variations of NO3– and NO2–

concentrations of ±10%.

c. Application: NO3– + NO2– in potable and surface water and in domestic and industrial

wastes can be determined over a range of 0.01 to 10 mg N/L.

2. Apparatus

Automated analytical equipment: An example of the continuous-flow analytical instrument

consists of the components shown in Figure 4500-NO3–:3.

3. Reagents

a. Color developing reagent: To approximately 500 mL water add 200 mL conc phosphoric

acid and 10 g sulfanilamide. After sulfanilamide is dissolved completely, add 0.8 g

N-(1-naphthyl)-ethylenediamine dihydrochloride. Dilute to 1 L with water, store in a dark bottle,

and refrigerate. Solution is stable for approximately 1 month.

b. Copper sulfate stock solution: Dissolve 2.5 g CuSO4⋅5H2O in water and dilute to 1 L.

c. Copper sulfate dilute solution: Dilute 20 mL stock solution to 2 L.

d. Sodium hydroxide stock solution, 10N: Dissolve 400 g NaOH in 750 mL water, cool, and dilute to 1 L.

e. Sodium hydroxide, 1.0N: Dilute 100 mL stock NaOH solution to 1 L.

f. Hydrazine sulfate stock solution: Dissolve 27.5 g N2H4⋅H2SO4 in 900 mL water and dilute

to 1 L. This solution is stable for approximately 6 months. CAUTION: Toxic if ingested. Mark

container with appropriate warning.

g. Hydrazine sulfate dilute solution: Dilute 22 mL stock solution to 1 L.

h. Stock nitrate solution: See ¶ B.3b.

i. Intermediate nitrate solution: See ¶ B.3c.

j. Standard nitrate solutions: Prepare NO3– calibration standards in the range 0 to 10 mg/L

by diluting to 100 mL the following volumes of stock nitrate solution: 0, 0.5, 1.0, 2.0 . . . 10.0

mL. For standards in the range of 0.01 mg/L use intermediate nitrate solution. Compare at least

one nitrite standard to a nitrate standard at the same concentration to verify the efficiency of the

reduction.

k. Standard nitrite solution: See Section 4500-NO2–.B.3e, Section 4500-NO2–.B.3 f, and

Section 4500-NO2–.B.3g.

4. Procedure

Set up manifold as shown in Figure 4500-NO3–:3 and follow general procedure described

by manufacturer. Run a 2.0-mg NO3–-N/L and a 2.0-mg NO2–-N/L standard through the system

to check for 100% reduction of nitrate to nitrite. The two peaks should be of equal height; if not,

adjust concentration of the hydrazine sulfate solution: If the NO3– peak is lower than the NO2–

peak, increase concentration of hydrazine sulfate until they are equal; if the NO3– peak is higher

than the NO2– reduce concentration of hydrazine sulfate. When correct concentration has been

determined, no further adjustment should be necessary.

5. Calculation

Prepare a standard curve by plotting response of processed standards against known

concentrations. Compute concentrations of samples by comparing response with standard curve.

6. Precision and Bias

In a single laboratory using drinking water, surface water, and industrial waste at

concentrations of 0.39, 1.15, 1.76, and 4.75 mg NO3–-N/L, the standard deviations were ±0.02,±0.01, ±0.02, and ±0.03, respectively. In a single laboratory using drinking water at

concentrations of 0.75 and 2.97 mg NO3–-N/ L, the recoveries were 99% and 101%.1

4500-NO3– I. Cadmium Reduction Flow Injection Method (PROPOSED)

1. General Discussion

a. Principle: The nitrate in the sample is reduced quantitatively to nitrite by passage of the

sample through a copperized cadmium column. The resulting nitrite plus any nitrite originally in

the sample is determined as a sum by diazotizing the nitrite with sulfanilamide followed by

coupling with N-(1-naphthyl)ethylenediamine dihydrochloride. The resulting water-soluble dye

has a magenta color; absorbance of the color at 540 nm is proportional to the nitrate + nitrite in

the sample. This sum also is known as total oxidized nitrogen (TON).

Nitrite alone can be determined by removing the cadmium column, recalibrating the method,

and repeating the sample analyses. A TON and a nitrite FIA method also can be run in parallel

for a set of samples. In this arrangement, the concentrations determined in the nitrite method can

be subtracted from the corresponding concentrations determined in the TON method to give the

resulting nitrate concentrations of the samples.

Also see Section 4500-NO2– and Section 4130, Flow Injection Analysis (FIA).

b. Interferences: Remove large or fibrous particulates by filtering sample through glass

wool. Guard against nitrate and nitrite contamination from reagents, water, glassware, and the

sample preservation process.

Residual chlorine can interfere by oxidizing the cadmium reduction column. Samples that

contain large concentrations of oil and grease will coat the surface of the cadmium. Eliminate

this interference by pre-extracting sample with an organic solvent.

Low results would be obtained for samples that contain high concentrations of iron, copper,

or other metals. In this method, EDTA is added to the buffer to reduce this interference.

Also see Section 4500-NO2–.B.1b and c and Section 4500-NO3–.A.2 and Section

4500-NO3–.B.1b.

2. Apparatus

Flow injection analysis equipment consisting of:

a. FIA injection valve with sample loop or equivalent.

b. Multichannel proportioning pump.

c. FIA manifold (Figure 4500-NO3–:4) with flow cell. Relative flow rates only are shown in

Figure 4500-NO3–:4. Tubing volumes are given as an example only; they may be scaled down

proportionally. Use manifold tubing of an inert material such as TFE.

d. Absorbance detector, 540 nm, 10-nm bandpass.

e. Injection valve control and data acquisition system.

3. Reagents

Use reagent water (>10 megohm) to prepare carrier and all solutions. To prevent bubble

formation, degas carrier and buffer with helium. Pass He at 140 kPa (20 psi) through a helium

degassing tube. Bubble He through 1 L solution for 1 min. As an alternative to preparing

reagents by weight/weight, use weight/volume.

a. Ammonium chloride buffer: CAUTION: Fumes. Use a hood. To a tared 1-L container add

800.0 g water, 126 g conc hydrochloric acid, HCl, 55.6 g ammonium hydroxide, NH4OH, and

1.0 g disodium EDTA. Shake until dissolved. The pH of this buffer should be 8.5.

b. Sulfanilamide color reagent: To a tared, dark 1-L container add 876 g water, 170 g 85%

phosphoric acid, H3PO4, 40.0 g sulfanilamide, and 1.0 g N-(1-naphthyl)ethylenediamine

dihydrochloride (NED). Shake until wetted and stir with stir bar for 30 min until dissolved. This

solution is stable for 1 month.

c. Hydrochloric acid, HCl, 1M: To a 100-mL container, add 92 g water, then add 9.6 g conc

HCl. Stir or shake to mix.

d. Copper sulfate solution, 2%: To a 1-L container, add 20 g copper sulfate pentahydrate,

CuSO4⋅5H2O, to 991 g water. Stir or shake to dissolve.

e. Copperized cadmium granules: Place 10 to 20 g coarse cadmium granules (0.3- to

1.5-mm-diam) in a 250-mL beaker. Wash with 50 mL acetone, then water, then two 50-mL

portions 1M HCl (¶ 3c). Rinse several times with water. CAUTION: Cadmium is toxic and

carcinogenic. Collect and store all waste cadmium. When handling cadmium, wear gloves and

follow the precautions described on the cadmium’s Material Safety Data Sheet.

Add 100 mL 2% copper sulfate solution (¶ 3d) to cadmium prepared above. Swirl for about

5 min, then decant the liquid and repeat with a fresh 100-mL portion of the 2% copper sulfate

solution. Continue this process until the blue aqueous copper color persists. Decant and wash

with at least five portions of ammonium chloride buffer (¶ 3a) to remove colloidal copper. The

cadmium should be black or dark gray. The copperized cadmium granules may be stored in a

bottle under ammonium chloride buffer.

f. Stock nitrate standard, 200 mg N/L: In a 1-L volumetric flask dissolve 1.444 g potassium

nitrate, KNO3, in about 600 mL water. Add 2 mL chloroform. Dilute to mark and invert to mix.

This solution is stable for 6 months.

g. Stock nitrite standard, 200.0 mg N/L: In a 1-L volumetric flask dissolve 0.986 g sodium nitrite, NaNO2, or 1.214 g potassium nitrite, KNO2, in approximately 800 mL water. Add 2 mL

chloroform. Dilute to mark and invert to mix. Refrigerate.

h. Standard solution: Prepare nitrate or nitrite standards in the desired concentration range,

using the stock standards (¶ 3 f or g), and diluting with water.

4. Procedure

Set up a manifold equivalent to that in Figure 4500-NO3–:4 and pack column with

copperized cadmium granules. Follow methods supplied by column and instrument manufacturer

or laboratory’s standard operating procedure for this method. Follow quality control procedures

outlined in Section 4020.

5. Calculations

Prepare standard curves by plotting absorbance of standards processed through the manifold

versus TON or nitrite concentration. The calibration curve is linear.

If TON includes measurable nitrite concentrations, it is important that the cadmium column

be 100% efficient. If the efficiency is less, the nitrite in the sample will give a positive percent

error equal to the difference from 100%, causing an error in TON and nitrate determinations. To

measure efficiency of the cadmium column, prepare two calibration curves, one using nitrate

standards and one using equimolar nitrite standards. The column efficiency is:

Column efficiency = 100% × (slope of nitrate curve/slope of nitrite curve)

Determine column efficiency at least weekly.

6. Precision and Bias

In the studies described below, nitrate was measured. There was no significant concentration

of nitrite in the samples.

a. Recovery and relative standard deviation: Table 4500-NO3–:I gives results of

single-laboratory studies.

b. MDL: A 800-μL sample loop was used in the method described above. Using a published

MDL method,1 analysts ran 21 replicates of a 2.00-μg N/L standard. These gave a mean of 1.82

μg N/L, a standard deviation of 0.098 μg N/L, and MDL of 0.25 μg N/L. A lower MDL may be obtained by increasing the sample loop volume and increasing the ratio of carrier flow rate to

reagent flow rate.

4500-Norg A. Introduction

1. Selection of Method

The kjeldahl methods (B and C) determine nitrogen in the trinegative state. They fail to

account for nitrogen in the form of azide, azine, azo, hydrazone, nitrate, nitrite, nitrile, nitro,

nitroso, oxime, and semi-carbazone. ‘‘Kjeldahl nitrogen’’ is the sum of organic nitrogen and

ammonia nitrogen.

The major factor that influences the selection of a macro- or semi-micro-kjeldahl method to

determine organic nitrogen is its concentration. The macro-kjeldahl method is applicable for

samples containing either low or high concentrations of organic nitrogen but requires a relatively

large sample volume for low concentrations. In the semi-micro-kjeldahl method, which is

applicable to samples containing high concentrations of organic nitrogen, the sample volume

should be chosen to contain organic plus ammonia nitrogen in the range of 0.2 to 2 mg.

The block digestion method (D) is a micro method with an automated analysis step capable

of measuring organic nitrogen as low as 0.1 mg/L when blanks are carefully controlled.

2. Storage of Samples

The most reliable results are obtained on fresh samples. If an immediate analysis is not

possible, preserve samples for kjeldahl digestion by acidifying to pH 1.5 to 2.0 with concentrated

H2SO4 and storing at 4°C. Do not use HgCl2 because it will interfere with ammonia removal.

3. Interferences

a. Nitrate: During kjeldahl digestion, nitrate in excess of 10 mg/L can oxidize a portion of

the ammonia released from the digested organic nitrogen, producing N2O and resulting in a

negative interference. When sufficient organic matter in a low state of oxidation is present,

nitrate can be reduced to ammonia, resulting in a positive interference. The conditions under

which significant interferences occur are not well defined and there is no proven way to

eliminate the interference in conjunction with the kjeldahl methods described herein.

b. Inorganic salts and solids: The acid and salt content of the kjeldahl digestion reagent is

intended to produce a digestion temperature of about 380°C. If the sample contains a very large

quantity of salt or inorganic solids that dissolve during digestion, the temperature may rise above

400°C, at which point pyrolytic loss of nitrogen begins to occur. To prevent an excessive

digestion temperature, add more H2SO4 to maintain the acid-salt balance. Not all salts cause

precisely the same temperature rise, but adding 1 mL H2SO4/g salt in the sample gives

reasonable results. Add the extra acid and the digestion reagent to both sample and reagent blank. Too much acid will lower the digestion temperature below 380°C and result in incomplete

digestion and recovery. If necessary, add sodium hydroxide-sodium thiosulfate before the final

distillation step to neutralize the excess acid.

Large amounts of salt or solids also may cause bumping during distillation. If this occurs,

add more dilution water after digestion.

c. Organic matter: During kjeldahl digestion, H2SO4 oxidizes organic matter to CO2 and

H2O. If a large amount of organic matter is present, a large amount of acid will be consumed, the

ratio of salt to acid will increase, and the digestion temperature will increase. If enough organic

matter is present, the temperature will rise above 400°C, resulting in pyrolytic loss of nitrogen.

To prevent this, add to the digestion flask 10 mL conc H2SO4/3 g COD. Alternately, add 50 mL

more digestion reagent/g COD. Additional sodium hydroxide-sodium thiosulfate reagent may be

necessary to keep the distillation pH high. Because reagents may contain traces of ammonia,

treat the reagent blank identically with the samples.

4. Use of a Catalyst

Mercury has been the catalyst of choice for kjeldahl digestion. Because of its toxicity and

problems associated with legal disposal of mercury residues, a less toxic catalyst is

recommended. Digestion of some samples may be complete or nearly complete without the use

of a catalyst. Effective digestion results from the use of a reagent having a salt/acid ratio of 1

g/mL with copper as catalyst (¶ B.3a), and specified temperature (¶ B.2a) and time (¶ B.4c)). If

a change is made in the reagent formula, report the change and indicate percentage recovery

relative to the results for similar samples analyzed using the previous formula.

Before results are considered acceptable, determine nitrogen recovery from samples with

known additions of nicotinic acid, to test completeness of digestion; and with ammonium

chloride to test for loss of nitrogen.

4500-Norg B. Macro-Kjeldahl Method

1. General Discussion

a. Principle: In the presence of H2SO4, potassium sulfate (K2SO4), and cupric sulfate

(CuSO4) catalyst, amino nitrogen of many organic materials is converted to ammonium. Free

ammonia also is converted to ammonium. After addition of base, the ammonia is distilled from

an alkaline medium and absorbed in boric or sulfuric acid. The ammonia may be determined

colorimetrically, by ammonia-selective electrode, or by titration with a standard mineral acid.

b. Selection of ammonia measurement method: The sensitivity of colorimetric methods

makes them particularly useful for determining organic nitrogen levels below 5 mg/L. The

titrimetric and selective electrode methods of measuring ammonia in the distillate are suitable for

determining a wide range of organic nitrogen concentrations. Selective electrode methods and automated colorimetric methods may be used for measurement of ammonia in digestate without

distillation. Follow equipment manufacturer’s instructions.

2. Apparatus

a. Digestion apparatus: Kjeldahl flasks with a total capacity of 800 mL yield the best results.

Digest over a heating device adjusted so that 250 mL water at an initial temperature of 25°C can

be heated to a rolling boil in approximately 5 min. For testing, preheat heaters for 10 min if

gas-operated or 30 min if electric. A heating device meeting this specification should provide the

temperature range of 375 to 385°C for effective digestion.

b. Distillation apparatus: See Section 4500-NH3.B.2a.

c. Apparatus for ammonia determination: See Section 4500-NH3.C.2, 4500-NH3.D.2,

Section 4500-NH3.F.2, or Section 4500-NH3.G.2.

3. Reagents

Prepare all reagents and dilutions in ammonia-free water.

All of the reagents listed for the determination of Nitrogen (Ammonia), Section

4500-NH3.C.3, Section 4500-NH3.D.3, Section 4500-NH3.F.3, or Section 4500-NH3.G.3, are

required, plus the following:

a. Digestion reagent: Dissolve 134 g K2SO4 and 7.3 g CuSO4 in about 800 mL water.

Carefully add 134 mL conc H2SO4. When it has cooled to room temperature, dilute the solution

to 1 L with water. Mix well. Keep at a temperature close to 20°C to prevent crystallization.

b. Sodium hydroxide-sodium thiosulfate reagent: Dissolve 500 g NaOH and 25 g

Na2S2O3⋅5H2O in water and dilute to 1 L.

c. Borate buffer solution: See Section 4500-NH3.B.3b.

d. Sodium hydroxide, NaOH, 6N.

4. Procedure

a. Selection of sample volume and sample preparation: Place a measured volume of sample

in an 800-mL kjeldahl flask. Select sample size from the following tabulation:

Organic Nitrogen in Sample         Sample Size

mg/L                                                       mL

0–1                                               500

1–10                                             250

10–20                                           100

20–50                                            50.0

50–100                                          25.0

If necessary, dilute sample to 300 mL, neutralize to pH 7, and dechlorinate as described in

Section 4500-NH3.B.4b.

b. Ammonia removal: Add 25 mL borate buffer and then 6N NaOH until pH 9.5 is reached.

Add a few glass beads or boiling chips such as Hengar Granules #12 and boil off 300 mL. If

desired, distill this fraction and determine ammonia nitrogen. Alternately, if ammonia has been

determined by the distillation method, use residue in distilling flask for organic nitrogen

determination.

For sludge and sediment samples, weigh wet sample in a crucible or weighing bottle,

transfer contents to a kjeldahl flask, and determine kjeldahl nitrogen. Follow a similar procedure

for ammonia nitrogen and organic nitrogen determined by difference. Determinations of organic

and kjeldahl nitrogen on dried sludge and sediment samples are not accurate because drying

results in loss of ammonium salts. Measure dry weight of sample on a separate portion.

c. Digestion: Cool and add carefully 50 mL digestion reagent (or substitute 6.7 mL conc

H2SO4, 6.7 g K2SO4, and 0.365 g CuSO4) to distillation flask. Add a few glass beads and, after

mixing, heat under a hood or with suitable ejection equipment to remove acid fumes. Boil briskly

until the volume is greatly reduced (to about 25 to 50 mL) and copious white fumes are observed

(fumes may be dark for samples high in organic matter). Then continue to digest for an

additional 30 min. As digestion continues, colored or turbid samples will become transparent and

pale green. After digestion, let cool, dilute to 300 mL with water, and mix. Tilt flask away from

personnel and carefully add 50 mL sodium hydroxide-thiosulfate reagent to form an alkaline

layer at flask bottom. Connect flask to a steamed-out distillation apparatus and swirl flask to

insure complete mixing. The pH of the solution should exceed 11.0.

d. Distillation: Distill and collect 200 mL distillate. Use 50 mL indicating boric acid as

absorbent solution when ammonia is to be determined by titration. Use 50 mL 0.04N H2SO4

solution as absorbent for manual phenate or electrode methods. Extend tip of condenser well

below level of absorbent solution and do not let temperature in condenser rise above 29°C.

Lower collected distillate free of contact with condenser tip and continue distillation during last

1 or 2 min to cleanse condenser.

e. Final ammonia measurement: Use the titration, ammonia-selective electrode, manual

phenate, or automated phenate method, Section 4500-NH3.C, Section 4500-NH3.D, Section

4500-NH3.F, and Section 4500-NH3.G, respectively.

f. Standards: Carry a reagent blank and standards through all steps of the procedure.

5. Calculation

See Section 4500-NH3.C.5, Section 4500-NH3.D.5, Section 4500-NH3.F.5, or Section

4500-NH3.G.5.

6. Precision and Bias

Two analysts in one laboratory prepared reagent water solutions of nicotinic acid and

digested them by the macro-kjeldahl method. Ammonia in the distillate was determined by

titration. Results are summarized in Table 4500-Norg:I.

4500-Norg C. Semi-Micro-Kjeldahl Method

1. General Discussion

See Section 4500-Norg.B.1.

2. Apparatus

a. Digestion apparatus: Use kjeldahl flasks with a capacity of 100 mL in a

semi-micro-kjeldahl digestion apparatus*#(67) equipped with heating elements to accommodate

kjeldahl flasks and a suction outlet to vent fumes. The heating elements should provide the

temperature range of 375 to 385°C for effective digestion.

b. Distillation apparatus: Use an all-glass unit equipped with a steam-generating vessel

containing an immersion heater†#(68) (Figure 4500-Norg:1).

c. pH meter.

d. Apparatus for ammonia determination: See Section 4500-NH3.C.2, Section

4500-NH3.D.2, Section 4500-NH3.F.2, or Section 4500-NH3.G.2.

3. Reagents

All of the reagents listed for the determination of Nitrogen (Ammonia) (Section

4500-NH3.B.3) and Nitrogen (Organic) macro-kjeldahl (Section 4500-Norg.B.3) are required. Prepare all reagents and dilutions with ammonia-free water.

4. Procedure

a. Selection of sample volume: Determine the sample size from the following tabulation:

Organic Nitrogen in Sample                   Sample Size

mg/L                                                                  mL

4–40                                                       50

8–80                                                        25

20–200                                                    10

40–400                                                     5

For sludge and sediment samples weigh a portion of wet sample containing between 0.2 and

2 mg organic nitrogen in a crucible or weighing bottle. Transfer sample quantitatively to a 100-

mL beaker by diluting it and rinsing the weighing dish several times with small quantities of

water. Make the transfer using as small a quantity of water as possible and do not exceed a total

volume of 50 mL. Measure dry weight of sample on a separate portion.

b. Ammonia removal: Pipet 50 mL sample or an appropriate volume diluted to 50 mL with

water into a 100-mL beaker. Add 3 mL borate buffer and adjust to pH 9.5 with 6N NaOH, using

a pH meter. Quantitatively transfer sample to a 100-mL kjeldahl flask and boil off 30 mL.

Alternatively, if ammonia removal is not required, digest samples directly as described in ¶ c

below. Distillation following this direct digestion yields kjeldahl nitrogen concentration rather

than organic nitrogen.

c. Digestion: Carefully add 10 mL digestion reagent to kjeldahl flask containing sample. Add

five or six glass beads (3- to 4-mm size) to prevent bumping during digestion. Set each heating

unit on the micro-kjeldahl digestion apparatus to its medium setting and heat flasks under a hood

or with suitable ejection equipment to remove fumes of SO3. Continue to boil briskly until

solution becomes transparent and pale green and copious fumes are observed. Then turn each

heating unit up to its maximum setting and digest for an additional 30 min. Cool. Quantitatively

transfer digested sample by diluting and rinsing several times into micro-kjeldahl distillation

apparatus so that total volume in distillation apparatus does not exceed 30 mL. Add 10 mL

sodium hydroxide-thiosulfate reagent and turn on steam.

d. Distillation: Control rate of steam generation to boil contents in distillation unit so that

neither escape of steam from tip of condenser nor bubbling of contents in receiving flask occurs.

Distill and collect 30 to 40 mL distillate below surface of 10 mL absorbent solution contained in

a 125-mL erlenmeyer flask. Use indicating boric acid for a titrimetric finish. Use 10 mL 0.04N

H2SO4 solution for collecting distillate for the phenate or electrode methods. Extend tip of

condenser well below level of absorbent solution and do not let temperature in condenser rise above 29°C. Lower collected distillate free of contact with condenser tip and continue

distillation during last 1 or 2 min to cleanse condenser.

e. Standards: Carry a reagent blank and standards through all steps of procedure and apply

necessary correction to results.

f. Final ammonia measurement: Use the titration, ammonia-selective electrode, manual

phenate, or automated phenate method, Section 4500-NH3.C, Section 4500-NH3.D, Section

4500-NH3.F, and Section 4500-NH3.G, respectively.

5. Calculation

See Section 4500-NH3.C.5, Section 4500-NH3.D.5, Section 4500-NH3.F.5, or Section

4500-NH3.G.5.

6. Precision and Bias

No data on the precision and bias of the semi-micro-kjeldahl method are available.

4500-Norg D. Block Digestion and Flow Injection Analysis (PROPOSED)

1. General Discussion

a. Principle: Samples of drinking, ground, and surface waters and of domestic and industrial

wastes are digested in a block digestor with sulfuric acid and copper sulfate as a catalyst. The

digestion recovers nitrogen components of biological origin, such as amino acids, proteins, and

peptides, as ammonia, but may not recover the nitrogenous compounds of some industrial wastes

such as amines, nitro compounds, hydrazones, oximes, semicarbazones, and some refractory

tertiary amines. Nitrate is not recovered. See Section 4500-N for a discussion of the various

forms of nitrogen found in waters and wastewaters, Section 4500-Norg.A and Section

4500-Norg.B for a discussion of kjeldahl nitrogen methods, and Section 4130, Flow Injection

Analysis (FIA).

The digested sample is injected onto the FIA manifold where its pH is controlled by raising

it to a known, basic pH by neutralization with a concentrated buffer. This in-line neutralization

converts the ammonium cation to ammonia, and also prevents undue influence of the sulfuric

acid matrix on the pH-sensitive color reaction that follows. The ammonia thus produced is

heated with salicylate and hypochlorite to produce a blue color that is proportional to the

ammonia concentration. The color is intensified by adding sodium nitroprusside. The presence of

EDTA in the buffer prevents precipitation of calcium and magnesium. The resulting peak’s

absorbance is measured at 660 nm. The peak area is proportional to the concentration of total kjeldahl nitrogen in the original sample.

b. Interferences: Remove large or fibrous particulates by filtering the sample through glass

wool.

The main source of interference is ammonia. Ammonia is an airborne contaminant that is

removed rapidly from ambient air by the digestion solution. Guard against ammonia

contamination in reagents, water, glassware, and the sample preservation process. It is

particularly important to prevent ammonia contamination in the sulfuric acid used for the

digestion. Open sulfuric acid bottles away from laboratories in which ammonia or ammonium

chloride have been used as reagents and store sulfuric acid away from such reagents. Ensure that

the open ends of the block digestor’s tubes can be covered to prevent ammonia from being

scrubbed from the fume hood make-up air during the digestion.

If a sample consumes more than 10% of the sulfuric acid during digestion, the pH-dependent

color reaction will show a matrix effect. The color reaction buffer will accommodate a range of

5.4%±0.4% H2SO4 (v/v) in the diluted digested sample. Sample matrices with a high

concentration of carbohydrates or other organic material may consume more than 10% of the

acid during digestion. If this effect is suspected, titrate digested sample with standardized sodium

hydroxide to determine whether more than 10% of the sulfuric acid has been consumed during

digestion. The block digestor also should have a means to prevent loss of sulfuric acid from the

digestion tubes during the digestion period.

Also see Section 4500-Norg.A and Section 4500-Norg.B.

2. Apparatus

Digestion and flow injection analysis equipment consisting of:

a. Block digestor capable of maintaining a temperature of 380°C for 2 h.

b. Digestion tubes capable of being heated to 380°C for 2 h and having a cover to prevent

ammonia contamination and loss of sulfuric acid.

c. FIA injection valve with sample loop or equivalent.

d. Multichannel proportioning pump.

e. FIA manifold (Figure 4500-Norg:2) with tubing heater and flow cell. Relative flow rates

only are shown in Figure 4500-Norg:2. Tubing volumes are given as an example only; they may

be scaled down proportionally. Use manifold tubing of an inert material such as TFE.

f. Absorbance detector, 660 nm, 10-nm bandpass.

g. Injection valve control and data acquisition system.

3. Reagents

Use reagent water (>10 megohm) for all solutions. To prevent bubble formation, degas

carrier and buffer with helium. Pass He at 140 kPa (20 psi) through a helium degassing tube.

Bubble He through 1 L solution for 1 min. As an alternative to preparing reagents by weight/weight, use weight/volume.

a. Digestion solution: In a 1-L volumetric flask, dissolve 134.0 g potassium sulfate, K2SO4,

and 7.3 g copper sulfate, CuSO4, in 800 mL water. Then add slowly while swirling 134 mL conc

sulfuric acid, H2SO4. Let cool, dilute to mark, and invert to mix.

b. Carrier and diluent: To a tared 1-L container, add 496 g digestion solution (¶ 3a) and 600

g water. Shake until dissolved.

c. Sodium hydroxide, NaOH, 0.8M: To a tared 1-L plastic container, add 32.0 g NaOH and

985.0 g water. Stir or shake until dissolved.

d. Buffer: To a tared 1-L container add 941 g water. Add and completely dissolve 35.0 g

sodium phosphate dibasic heptahydrate, Na2HPO4⋅7H2O. Add 20.0 g disodium EDTA

(ethylenediaminetetracetic acid disodium salt). The EDTA will not dissolve but will form a

turbid solution. Finally, add 50 g NaOH. Stir or shake until dissolved.

e. Salicylate/nitroprusside: To a tared 1-L dark container, add 150.0 g sodium salicylate

(salicylic acid sodium salt), C6H4(OH)(COO)Na, 1.00 g sodium nitroprusside (sodium

nitroferricyanide dihydrate), Na2Fe(CN)5NO⋅2H2O, and 908 g water. Stir or shake until

dissolved. Prepare fresh monthly.

f. Hypochlorite: To a tared 250-mL container, add 16 g commercial 5.25% sodium

hypochlorite bleach solution*#(69) and 234 g deionized water. Shake to mix.

g. Stock standard, 250 mg N/L: In a 1-L volumetric flask dissolve 0.9540 g ammonium

chloride, NH4Cl (dried for 2 h at 110°C), in about 800 mL water. Dilute to mark and invert to

mix.

h. Standard ammonia solutions: Prepare ammonia standards in desired concentration range,

using the stock standard (¶ 3g) and diluting with water.

i. Simulated digested standards: To prepare calibration standards without having to digest

the standards prepared in ¶ 3h, proceed as follows:

Stock standard, 5.00 mg N/L: To a tared 250-mL container add about 5.0 g stock standard

(250 mg N/L). Divide actual weight of solution added by 0.02 and make up to this resulting total

weight with diluent (¶ 3b). Shake to mix. Prepare working standards from this stock standard,

diluting with the diluent (¶ 3b), not water.

4. Procedure

a. Digestion procedure: Carry both standards and samples through this procedure.

To a 75-mL block digestor tube add 25.0 mL sample or standard and then add 10 mL

digestion solution (¶ 3a) and mix. Add four alundum granules to each tube for smooth boiling.

Place tubes in preheated block digestor for 1 h at 200°C. After 1 h, increase block temperature to

380°C and continue to digest for 1 h at 380°C. Remove tubes from block and let cool for about

10 min. Dilute each to 25.0 mL with water and mix with vortex mixer. Cover tubes to prevent ammonia contamination.

b. FIA analysis: Set up a manifold equivalent to that in Figure 4500-Norg:2 and analyze

digested standards and samples by method supplied by manufacturer or laboratory standard

operating procedure. Follow quality control protocols described in Section 4020.

5. Calculations

Prepare standard curves by plotting absorbance of standards processed through the manifold

versus ammonia concentration. The calibration curve is linear.

6. Precision and Bias

a. Recovery and relative standard deviation: Table 4500-Norg:II gives results of

single-laboratory studies.

b. MDL: A 130-μL sample loop was used in the method described above. Using a published

MDL method,1 analysts ran 21 replicates of a 0.1-mg N/L standard. These gave a mean of 0.103

mg N/L, a standard deviation of 0.014 mg N/L, and MDL of 0.034 mg N/L. A lower MDL may

be obtained by increasing the sample loop volume and increasing the ratio of carrier flow rate to

reagents flow rate.

4500-O OXYGEN (DISSOLVED)*#(70)

4500-O A. Introduction

1. Significance

Dissolved oxygen (DO) levels in natural and wastewaters depend on the physical, chemical,

and biochemical activities in the water body. The analysis for DO is a key test in water pollution

and waste treatment process control.

2. Selection of Method

Two methods for DO analysis are described: the Winkler or iodometric method and its

modifications and the electrometric method using membrane electrodes. The iodometric method1

is a titrimetric procedure based on the oxidizing property of DO while the membrane electrode

procedure is based on the rate of diffusion of molecular oxygen across a membrane.2 The choice of procedure depends on the interferences present, the accuracy desired, and, in some cases,

convenience or expedience.

4500-O B. Iodometric Methods

1. Principle

The iodometric test is the most precise and reliable titrimetric procedure for DO analysis. It

is based on the addition of divalent manganese solution, followed by strong alkali, to the sample

in a glass-stoppered bottle. DO rapidly oxidizes an equivalent amount of the dispersed divalent

manganous hydroxide precipitate to hydroxides of higher valency states. In the presence of

iodide ions in an acidic solution, the oxidized manganese reverts to the divalent state, with the

liberation of iodine equivalent to the original DO content. The iodine is then titrated with a

standard solution of thiosulfate.

The titration end point can be detected visually, with a starch indicator, or electrometrically,

with potentiometric or dead-stop techniques.1 Experienced analysts can maintain a precision of

±50 μg/L with visual end-point detection and a precision of ±5 μg/L with electrometric

end-point detection.1,2

The liberated iodine also can be determined directly by simple absorption

spectrophotometers.3 This method can be used on a routine basis to provide very accurate

estimates for DO in the microgram-per-liter range provided that interfering particulate matter,

color, and chemical interferences are absent.

2. Selection of Method

Before selecting a method consider the effect of interferences, particularly oxidizing or

reducing materials that may be present in the sample. Certain oxidizing agents liberate iodine

from iodides (positive interference) and some reducing agents reduce iodine to iodide (negative

interference). Most organic matter is oxidized partially when the oxidized manganese precipitate

is acidified, thus causing negative errors.

Several modifications of the iodometric method are given to minimize the effect of

interfering materials.2 Among the more commonly used procedures are the azide modification,4

the permanganate modification,5 the alum flocculation modification,6 and the copper

sulfate-sulfamic acid flocculation modification.7,8 The azide modification (C) effectively removes interference caused by nitrite, which is the most common interference in biologically

treated effluents and incubated BOD samples. Use the permanganate modification (D) in the

presence of ferrous iron. When the sample contains 5 or more mg ferric iron salts/L, add

potassium fluoride (KF) as the first reagent in the azide modification or after the permanganate

treatment for ferrous iron. Alternately, eliminate Fe(III) interference by using 85 to 87%

phosphoric acid (H3PO4) instead of sulfuric acid (H2SO4) for acidification. This procedure has

not been tested for Fe(III) concentrations above 20 mg/L.

Use the alum flocculation modification (E) in the presence of suspended solids that cause

interference and the copper sulfate-sulfamic acid flocculation modification (F) on

activated-sludge mixed liquor.

3. Collection of Samples

Collect samples very carefully. Methods of sampling are highly dependent on source to be

sampled and, to a certain extent, on method of analysis. Do not let sample remain in contact with

air or be agitated, because either condition causes a change in its gaseous content. Samples from

any depth in streams, lakes, or reservoirs, and samples of boiler water, need special precautions

to eliminate changes in pressure and temperature. Procedures and equipment have been

developed for sampling waters under pressure and unconfined waters (e.g., streams, rivers, and

reservoirs). Sampling procedures and equipment needed are described in American Society for

Testing and Materials Special Technical Publication No. 148-1 and in U.S. Geological Survey

Water Supply Paper No. 1454.

Collect surface water samples in narrow-mouth glass-stoppered BOD bottles of 300-mL

capacity with tapered and pointed ground-glass stoppers and flared mouths. Avoid entraining or

dissolving atmospheric oxygen. In sampling from a line under pressure, attach a glass or rubber

tube to the tap and extend to bottom of bottle. Let bottle overflow two or three times its volume

and replace stopper so that no air bubbles are entrained.

Suitable samplers for streams, ponds, or tanks of moderate depth are of the APHA type

shown in Figure 4500-O:1. Use a Kemmerer-type sampler for samples collected from depths

greater than 2 m. Bleed sample from bottom of sampler through a tube extending to bottom of a

250- to 300-mL BOD bottle. Fill bottle to overflowing (overflow for approximately 10 s), and

prevent turbulence and formation of bubbles while filling. Record sample temperature to nearest

degree Celsius or more precisely.

4. Preservation of Samples

Determine DO immediately on all samples containing an appreciable oxygen or iodine

demand. Samples with no iodine demand may be stored for a few hours without change after

adding manganous sulfate (MnSO4) solution, alkali-iodide solution, and H2SO4, followed by

shaking in the usual way. Protect stored samples from strong sunlight and titrate as soon as

possible.

For samples with an iodine demand, preserve for 4 to 8 h by adding 0.7 mL conc H2SO4 and 1 mL sodium azide solution (2 g NaN3/100 mL distilled water) to the BOD bottle. This will

arrest biological activity and maintain DO if the bottle is stored at the temperature of collection

or water-sealed and kept at 10 to 20°C. As soon as possible, complete the procedure, using 2 mL

MnSO4 solution, 3 mL alkali-iodide solution, and 2 mL conc H2SO4.

4500-O C. Azide Modification

1. General Discussion

Use the azide modification for most wastewater, effluent, and stream samples, especially if

samples contain more than 50 μg NO2–-N/L and not more than 1 mg ferrous iron/L. Other

reducing or oxidizing materials should be absent. If 1 mL KF solution is added before the sample

is acidified and there is no delay in titration, the method is applicable in the presence of 100 to

200 mg ferric iron/L.

2. Reagents

a. Manganous sulfate solution: Dissolve 480 g MnSO4⋅4H2O, 400 g MnSO4⋅2H2O, or 364 g

MnSO4⋅H2O in distilled water, filter, and dilute to 1 L. The MnSO4 solution should not give a color with starch when added to an acidified potassium iodide (KI) solution.

b. Alkali-iodide-azide reagent:

1) For saturated or less-than-saturated samples—Dissolve 500 g NaOH (or 700 g KOH) and

135 g NaI (or 150 g KI) in distilled water and dilute to 1 L. Add 10 g NaN3 dissolved in 40 mL

distilled water. Potassium and sodium salts may be used interchangeably. This reagent should

not give a color with starch solution when diluted and acidified.

2) For supersaturated samples—Dissolve 10 g NaN3 in 500 mL distilled water. Add 480 g

sodium hydroxide (NaOH) and 750 g sodium iodide (NaI), and stir until dissolved. There will be

a white turbidity due to sodium carbonate (Na2CO3), but this will do no harm. CAUTION—Do

not acidify this solution because toxic hydrazoic acid fumes may be produced.

c. Sulfuric acid, H2SO4, conc: One milliliter is equivalent to about 3 mL alkali-iodide-azide

reagent.

d. Starch: Use either an aqueous solution or soluble starch powder mixtures.

To prepare an aqueous solution, dissolve 2 g laboratory-grade soluble starch and 0.2 g

salicylic acid, as a preservative, in 100 mL hot distilled water.

e. Standard sodium thiosulfate titrant: Dissolve 6.205 g Na2S2O3⋅5H2O in distilled water.

Add 1.5 mL 6N NaOH or 0.4 g solid NaOH and dilute to 1000 mL. Standardize with bi-iodate

solution.

f. Standard potassium bi-iodate solution, 0.0021M: Dissolve 812.4 mg KH(IO3)2 in distilled

water and dilute to 1000 mL.

Standardization—Dissolve approximately 2 g KI, free from iodate, in an erlenmeyer flask

with 100 to 150 mL distilled water. Add 1 mL 6N H2SO4 or a few drops of conc H2SO4 and

20.00 mL standard bi-iodate solution. Dilute to 200 mL and titrate liberated iodine with

thiosulfate titrant, adding starch toward end of titration, when a pale straw color is reached.

When the solutions are of equal strength, 20.00 mL 0.025M Na2S2O3 should be required. If not,

adjust the Na2S2O3 solution to 0.025M.

3. Procedure

a. To the sample collected in a 250- to 300-mL bottle, add 1 mL MnSO4 solution, followed

by 1 mL alkali-iodide-azide reagent. If pipets are dipped into sample, rinse them before returning

them to reagent bottles. Alternatively, hold pipet tips just above liquid surface when adding

reagents. Stopper carefully to exclude air bubbles and mix by inverting bottle a few times. When

precipitate has settled sufficiently (to approximately half the bottle volume) to leave clear

supernate above the manganese hydroxide floc, add 1.0 mL conc H2SO4. Restopper and mix by

inverting several times until dissolution is complete. Titrate a volume corresponding to 200 mL

original sample after correction for sample loss by displacement with reagents. Thus, for a total

of 2 mL (1 mL each) of MnSO4 and alkali-iodide-azide reagents in a 300-mL bottle, titrate 200 × 300/(300 − 2) = 201 mL.

b. Titrate with 0.025M Na2S2O3 solution to a pale straw color. Add a few drops of starch

solution and continue titration to first disappearance of blue color. If end point is overrun,

back-titrate with 0.0021M bi-iodate solution added dropwise, or by adding a measured volume of

treated sample. Correct for amount of bi-iodate solution or sample. Disregard subsequent

recolorations due to the catalytic effect of nitrite or to traces of ferric salts that have not been

complexed with fluoride.

4. Calculation

a. For titration of 200 mL sample, 1 mL 0.025M Na2S2O3 = 1 mg DO/L.

b. To express results as percent saturation at 101.3 kPa, use the solubility data in Table

4500-O:I. Equations for correcting solubilities to barometric pressures other than mean sea level

and for various chlorinities are given below the table.

5. Precision and Bias

DO can be determined with a precision, expressed as a standard deviation, of about 20 μg/L

in distilled water and about 60 μg/L in wastewater and secondary effluents. In the presence of

appreciable interference, even with proper modifications, the standard deviation may be as high

as 100 μg/L. Still greater errors may occur in testing waters having organic suspended solids or

heavy pollution. Avoid errors due to carelessness in collecting samples, prolonging the

completion of test, or selecting an unsuitable modification.

4500-O D. Permanganate Modification

1. General Discussion

Use the permanganate modification only on samples containing ferrous iron. Interference

from high concentrations of ferric iron (up to several hundred milligrams per liter), as in acid

mine water, may be overcome by the addition of 1 mL potassium fluoride (KF) and azide,

provided that the final titration is made immediately after acidification.

This procedure is ineffective for oxidation of sulfite, thiosulfate, polythionate, or the organic

matter in wastewater. The error with samples containing 0.25% by volume of digester waste

from the manufacture of sulfite pulp may amount to 7 to 8 mg DO/L. With such samples, use the

alkali-hypochlorite modification.1 At best, however, the latter procedure gives low results, the

deviation amounting to 1 mg/L for samples containing 0.25% digester wastes.

2. Reagents

All the reagents required for Method C, and in addition:

a. Potassium permanganate solution: Dissolve 6.3 g KMnO4 in distilled water and dilute to 1 L.

b. Potassium oxalate solution: Dissolve 2 g K2C2O4⋅H2O in 100 mL distilled water; 1 mL

will reduce about 1.1 mL permanganate solution.

c. Potassium fluoride solution: Dissolve 40 g KF⋅2H2O in distilled water and dilute to 100 mL.

3. Procedure

a. To a sample collected in a 250- to 300-mL bottle add, below the surface, 0.70 mL conc

H2SO4, 1 mL KMnO4 solution, and 1 mL KF solution. Stopper and mix by inverting. Never add

more than 0.7 mL conc H2SO4 as the first step of pretreatment. Add acid with a 1-mL pipet

graduated to 0.1 mL. Add sufficient KMnO4 solution to obtain a violet tinge that persists for 5

min. If the permanganate color is destroyed in a shorter time, add additional KMnO4 solution,

but avoid large excesses.

b. Remove permanganate color completely by adding 0.5 to 1.0 mL K2C2O4 solution. Mix

well and let stand in the dark to facilitate the reaction. Excess oxalate causes low results; add

only enough K2C2O4 to decolorize the KMnO4 completely without an excess of more than 0.5

mL. Complete decolorization in 2 to 10 min. If it is impossible to decolorize the sample without

adding a large excess of oxalate, the DO result will be inaccurate.

c. From this point the procedure closely parallels that in Section 4500-O.C.3. Add 1 mL

MnSO4 solution and 3 mL alkali-iodide-azide reagent. Stopper, mix, and let precipitate settle a short time; acidify with 2 mL conc H2SO4. When 0.7 mL acid, 1 mL KF solution, 1 mL KMnO4

solution, 1 mL K2C2O4 solution, 1 mL MnSO4 solution, and 3 mL alkali-iodide-azide (or a total

of 7.7 mL reagents) are used in a 300-mL bottle, take 200 × 300/(300 − 7.7) = 205 mL for

titration.

This correction is slightly in error because the KMnO4 solution is nearly saturated with DO

and 1 mL would add about 0.008 mg oxygen to the DO bottle. However, because precision of

the method (standard deviation, 0.06 mL thiosulfate titration, or 0.012 mg DO) is 50% greater

than this error, a correction is unnecessary. When substantially more KMnO4 solution is used

routinely, use a solution several times more concentrated so that 1 mL will satisfy the

permanganate demand.

4500-O E. Alum Flocculation Modification

1. General Discussion

Samples high in suspended solids may consume appreciable quantities of iodine in acid

solution. The interference due to solids may be removed by alum flocculation.

2. Reagents

All the reagents required for the azide modification (Section 4500-O.C.2) and in addition:

a. Alum solution: Dissolve 10 g aluminum potassium sulfate, AlK(SO4)2⋅12H2O, in distilled

water and dilute to 100 mL.

b. Ammonium hydroxide, NH4OH, conc.

3. Procedure

Collect sample in a glass-stoppered bottle of 500 to 1000 mL capacity, using the same

precautions as for regular DO samples. Add 10 mL alum solution and 1 to 2 mL conc NH4OH.

Stopper and invert gently for about 1 min. Let sample settle for about 10 min and siphon clear

supernate into a 250- to 300-mL DO bottle until it overflows. Avoid sample aeration and keep

siphon submerged at all times. Continue sample treatment as in Section 4500-O.C.3 or an

appropriate modification.

4500-O F. Copper Sulfate-Sulfamic Acid Flocculation Modification

1. General Discussion

This modification is used for biological flocs such as activated sludge mixtures, which have

high oxygen utilization rates.

2. Reagents

All the reagents required for the azide modification (Section 4500-O.C.2) and,, in addition:

Copper sulfate-sulfamic acid inhibitor solution: Dissolve 32 g technical-grade NH2SO2OH

without heat in 475 mL distilled water. Dissolve 50 g CuSO4⋅5H2O in 500 mL distilled water.

Mix the two solutions and add 25 mL conc acetic acid.

3. Procedure

Add 10 mL CuSO4-NH2SO2OH inhibitor to a 1-L glass-stoppered bottle. Insert bottle in a

special sampler designed so that bottle fills from a tube near bottom and overflows only 25 to

50% of bottle capacity. Collect sample, stopper, and mix by inverting. Let suspended solids

settle and siphon relatively clear supernatant liquor into a 250- to 300-mL DO bottle. Continue

sample treatment as rapidly as possible by the azide (Section 4500-O.C.3) or other appropriate

modification.

4500-O G. Membrane Electrode Method

1. General Discussion

Various modifications of the iodometric method have been developed to eliminate or

minimize effects of interferences; nevertheless, the method still is inapplicable to a variety of

industrial and domestic wastewaters.1 Moreover, the iodometric method is not suited for field

testing and cannot be adapted easily for continuous monitoring or for DO determinations in situ.

Polarographic methods using the dropping mercury electrode or the rotating platinum

electrode have not been reliable always for the DO analysis in domestic and industrial

wastewaters because impurities in the test solution can cause electrode poisoning or other

interferences.2,3 With membrane-covered electrode systems these problems are minimized,

because the sensing element is protected by an oxygen-permeable plastic membrane that serves

as a diffusion barrier against impurities.4–6 Under steady-state conditions the current is directly

proportional to the DO concentration.*#(71)

Membrane electrodes of the polarographic4 as well as the galvanic5 type have been used for

DO measurements in lakes and reservoirs,8 for stream survey and control of industrial

effluents,9,10 for continuous monitoring of DO in activated sludge units,11 and for estuarine and

oceanographic studies.12 Being completely submersible, membrane electrodes are suited for

analysis in situ. Their portability and ease of operation and maintenance make them particularly

convenient for field applications. In laboratory investigations, membrane electrodes have been used for continuous DO analysis in bacterial cultures, including the BOD test.5,13

Membrane electrodes provide an excellent method for DO analysis in polluted waters, highly

colored waters, and strong waste effluents. They are recommended for use especially under

conditions that are unfavorable for use of the iodometric method, or when that test and its

modifications are subject to serious errors caused by interferences.

a. Principle: Oxygen-sensitive membrane electrodes of the polarographic or galvanic type

are composed of two solid metal electrodes in contact with supporting electrolyte separated from

the test solution by a selective membrane. The basic difference between the galvanic and the

polarographic systems is that in the former the electrode reaction is spontaneous (similar to that

in a fuel cell), while in the latter an external source of applied voltage is needed to polarize the

indicator electrode. Polyethylene and fluorocarbon membranes are used commonly because they

are permeable to molecular oxygen and are relatively rugged.

Membrane electrodes are commercially available in some variety. In all these instruments

the ‘‘diffusion current’’ is linearly proportional to the concentration of molecular oxygen. The

current can be converted easily to concentration units (e.g., milligrams per liter) by a number of

calibration procedures.

Membrane electrodes exhibit a relatively high temperature coefficient largely due to changes

in the membrane permeability.6 The effect of temperature on the electrode sensitivity, φ

(microamperes per milligram per liter), can be expressed by the following simplified

relationship:6

log φ = 0.43 mt + b

where:

t = temperature, °C,

m = constant that depends on the membrane material, and

b = constant that largely depends on membrane thickness.

If values of φ and m are determined for one temperature (φ0 and t0), it is possible to calculate the sensitivity at any desired temperature (φ and t) as follows:

log φ = log φ0 + 0.43 m (t − t0)

Nomographic charts for temperature correction can be constructed easily7 and are available from

some manufacturers. An example is shown in Figure 4500-O:2, in which, for simplicity,

sensitivity is plotted versus temperature on semilogarithmic coordinates. Check one or two

points frequently to confirm original calibration. If calibration changes, the new calibration

should be parallel to the original, provided that the same membrane material is used.

Temperature compensation also can be made automatically by using thermistors in the

electrode circuit.4 However, thermistors may not compensate fully over a wide temperature range. For certain applications where high accuracy is required, use calibrated nomographic

charts to correct for temperature effect.

To use the DO membrane electrode in estuarine waters or in wastewaters with varying ionic

strength, correct for effect of salting-out on electrode sensitivity.6,7 This effect is particularly

significant for large changes in salt content. Electrode sensitivity varies with salt concentration

according to the following relationship:

log φS = 0.43 mSCS + log φ0

where:

φS, φ0 = sensitivities in salt solution and distilled water, respectively,

CS = salt concentration (preferably ionic strength), and

mS = constant (salting-out coefficient).

If φ0 and mS are determined, it is possible to calculate sensitivity for any value of CS.

Conductivity measurements can be used to approximate salt concentration (CS). This is

particularly applicable to estuarine waters. Figure 4500-O:3 shows calibration curves for

sensitivity of varying salt solutions at different temperatures.

b. Interference: Plastic films used with membrane electrode systems are permeable to a

variety of gases besides oxygen, although none is depolarized easily at the indicator electrode.

Prolonged use of membrane electrodes in waters containing such gases as hydrogen sulfide

(H2S) tends to lower cell sensitivity. Eliminate this interference by frequently changing and

calibrating the membrane electrode.

c. Sampling: Because membrane electrodes offer the advantage of analysis in situ they

eliminate errors caused by sample handling and storage. If sampling is required, use the same

precautions suggested for the iodometric method.

2. Apparatus

Oxygen-sensitive membrane electrode, polarographic or galvanic, with appropriate meter.

3. Procedure

a. Calibration: Follow manufacturer’s calibration procedure exactly to obtain guaranteed

precision and accuracy. Generally, calibrate membrane electrodes by reading against air or a

sample of known DO concentration (determined by iodometric method) as well as in a sample

with zero DO. (Add excess sodium sulfite, Na2SO3, and a trace of cobalt chloride, CoCl2, to

bring DO to zero.) Preferably calibrate with samples of water under test. Avoid an iodometric

calibration where interfering substances are suspected. The following illustrate the recommended

procedures:

1) Fresh water—For unpolluted samples where interfering substances are absent, calibrate in the test solution or distilled water, whichever is more convenient.

2) Salt water—Calibrate directly with samples of seawater or waters having a constant salt

concentration in excess of 1000 mg/L.

3) Fresh water containing pollutants or interfering substances— Calibrate with distilled water

because erroneous results occur with the sample.

4) Salt water containing pollutants or interfering substances— Calibrate with a sample of

clean water containing the same salt content as the sample. Add a concentrated potassium

chloride (KCl) solution ( see Conductivity, Section 2510 and Table 2510:I ) to distilled water to

produce the same specific conductance as that in the sample. For polluted ocean waters, calibrate

with a sample of unpolluted seawater.

5) Estuary water containing varying quantities of salt—Calibrate with a sample of

uncontaminated seawater or distilled or tap water. Determine sample chloride or salt

concentration and revise calibration to account for change of oxygen solubility in the estuary

water.7

b. Sample measurement: Follow all precautions recommended by manufacturer to insure

acceptable results. Take care in changing membrane to avoid contamination of sensing element

and also trapping of minute air bubbles under the membrane, which can lead to lowered response

and high residual current. Provide sufficient sample flow across membrane surface to overcome

erratic response (see Figure 4500-O:4 for a typical example of the effect of stirring).

c. Validation of temperature effect: Check frequently one or two points to verify temperature

correction data.

4. Precision and Bias

With most commercially available membrane electrode systems an accuracy of ±0.1 mg

DO/L and a precision of ±0.05 mg DO/ L can be obtained.

4500-P PHOSPHORUS*#(73)

4500-P A. Introduction

1. Occurrence

Phosphorus occurs in natural waters and in wastewaters almost solely as phosphates. These

are classified as orthophosphates, condensed phosphates (pyro-, meta-, and other

polyphosphates), and organically bound phosphates. They occur in solution, in particles or

detritus, or in the bodies of aquatic organisms.

These forms of phosphate arise from a variety of sources. Small amounts of orthophosphate

or certain condensed phosphates are added to some water supplies during treatment. Larger

quantities of the same compounds may be added when the water is used for laundering or other

cleaning, because these materials are major constituents of many commercial cleaning

preparations. Phosphates are used extensively in the treatment of boiler waters. Orthophosphates

applied to agricultural or residential cultivated land as fertilizers are carried into surface waters

with storm runoff and to a lesser extent with melting snow. Organic phosphates are formed

primarily by biological processes. They are contributed to sewage by body wastes and food

residues, and also may be formed from orthophosphates in biological treatment processes or by

receiving water biota.

Phosphorus is essential to the growth of organisms and can be the nutrient that limits the

primary productivity of a body of water. In instances where phosphate is a growth-limiting

nutrient, the discharge of raw or treated wastewater, agricultural drainage, or certain industrial

wastes to that water may stimulate the growth of photosynthetic aquatic micro- and

macroorganisms in nuisance quantities.

inorganic forms and incorporated into organic compounds.

2. Definition of Terms

Phosphorus analyses embody two general procedural steps: (a+) conversion of the

phosphorus form of interest to dissolved orthophosphate, and (b) colorimetric determination of

dissolved orthophosphate. The separation of phosphorus into its various forms is defined

analytically but the analytical differentiations have been selected so that they may be used for

interpretive purposes.

Filtration through a 0.45-μm-pore-diam membrane filter separates dissolved from suspended

forms of phosphorus. No claim is made that filtration through 0.45-μm filters is a true separation of suspended and dissolved forms of phosphorus; it is merely a convenient and replicable analytical technique designed to make a gross separation.

Membrane filtration is selected over depth filtration because of the greater likelihood of

obtaining a consistent separation of particle sizes. Prefiltration through a glass fiber filter may be

used to increase the filtration rate.

Phosphates that respond to colorimetric tests without preliminary hydrolysis or oxidative

digestion of the sample are termed ‘‘reactive phosphorus.’’ While reactive phosphorus is largely

a measure of orthophosphate, a small fraction of any condensed phosphate present usually is

hydrolyzed unavoidably in the procedure. Reactive phosphorus occurs in both dissolved and

suspended forms.

Acid hydrolysis at boiling-water temperature converts dissolved and particulate condensed

phosphates to dissolved orthophosphate. The hydrolysis unavoidably releases some phosphate

from organic compounds, but this may be reduced to a minimum by judicious selection of acid

strength and hydrolysis time and temperature. The term ‘‘acid-hydrolyzable phosphorus’’ is

preferred over ‘‘condensed phosphate’’ for this fraction.

The phosphate fractions that are converted to orthophosphate only by oxidation destruction

of the organic matter present are considered ‘‘organic’’ or ‘‘organically bound’’ phosphorus.

The severity of the oxidation required for this conversion depends on the form—and to some

extent on the amount—of the organic phosphorus present. Like reactive phosphorus and

acid-hydrolyzable phosphorus, organic phosphorus occurs both in the dissolved and suspended

fractions.

The total phosphorus as well as the dissolved and suspended phosphorus fractions each may

be divided analytically into the three chemical types that have been described: reactive,

acid-hydrolyzable, and organic phosphorus. Figure 4500-P:1 shows the steps for analysis of

individual phosphorus fractions. As indicated, determinations usually are conducted only on the

unfiltered and filtered samples. Suspended fractions generally are determined by difference;

however, they may be determined directly by digestion of the material retained on a glass-fiber

filter.

3. Selection of Method

a. Digestion methods: Because phosphorus may occur in combination with organic matter, a

digestion method to determine total phosphorus must be able to oxidize organic matter

effectively to release phosphorus as orthophosphate. Three digestion methods are given in

Section 4500-P.B.3, Section 4500-P.B.4, and Section 4500-P.B.5. The perchloric acid method,

the most drastic and time-consuming method, is recommended only for particularly difficult

samples such as sediments. The nitric acid-sulfuric acid method is recommended for most

samples. By far the simplest method is the persulfate oxidation technique. Persulfate oxidation is

coupled with ultraviolet light for a more efficient digestion in an automated in-line

digestion/determination by flow injection analysis (4500-P.I). It is recommended that persulfate

oxidation methods be checked against one or more of the more drastic digestion techniques and

be adopted if identical recoveries are obtained.

After digestion, determine liberated orthophosphate by Method C, D, E, F, G, or H. The

colorimetric method used, rather than the digestion procedure, governs in matters of interference

and minimum detectable concentration.

b. Colorimetric method: Three methods of orthophosphate determination are described.

Selection depends largely on the concentration range of orthophosphate. The

vanadomolybdophosphoric acid method (C) is most useful for routine analysis in the range of 1

to 20 mg P/L. The stannous chloride method (D) or the ascorbic acid method (E) is more suited

for the range of 0.01 to 6 mg P/L. An extraction step is recommended for the lower levels of this

range and when interferences must be overcome. Automated versions of the ascorbic acid

method (F, G, and H) also are presented. Careful attention to procedure may allow application of

these methods to very low levels of phosphorus, such as those found in unimpaired fresh water.

Ion chromatography (Section 4110) and capillary ion electrophoresis (Section 4140) are

useful for determination of orthophosphate in undigested samples.

4. Precision and Bias

To aid in method selection, Table 4500-P:I presents the results of various combinations of

digestions, hydrolysis, and colorimetric techniques for three synthetic samples of the following

compositions:

Sample 1: 100 μg orthosphosphate phosphorus (PO43–-P/L), 80 μg condensed phosphate

phosphorus/L (sodium hexametaphosphate), 30 μg organic phosphorus/L (adenylic acid), 1.5mg

NH3-N/L, 0.5 mg NO3-N/L, and 400 mg Cl–/L.

Sample 2: 600 μg PO43–-P/L, 300 μg condensed phosphate phosphorus/L (sodium

hexametaphosphate), 90 μg organic phosphorus/L (adenylic acid), 0.8 mg NH3-N/L, 5.0 mg

NO3–-N/L, and 400 mg Cl–/L.

Sample 3: 7.00 mg PO43–-P/L, 3.00 mg condensed phosphate phosphorus/L (sodium hexametaphosphate), 0.230 mg organic phosphorus/L (adenylic acid), 0.20 mg NH3-N/L, 0.05

mg NO3–- N/L, and 400 mg Cl–/L.

5. Sampling and Storage

If dissolved phosphorus forms are to be differentiated, filter sample immediately after

collection. Preserve by freezing at or below −10°C. In some cases 40 mg HgCl2/L may be added

to the samples, especially when they are to be stored for long periods before analysis. CAUTION:

HgCl2 is a hazardous substance; take appropriate precautions in disposal; use of HgCl2 is not

encouraged. Do not add either acid or CHCl3 as a preservative when phosphorus forms are to be

determined. If total phosphorus alone is to be determined, add H2SO4 or HCl to pH<2 and cool

to 4°C, or freeze without any additions.

Do not store samples containing low concentrations of phosphorus in plastic bottles unless

kept in a frozen state because phosphates may be adsorbed onto the walls of plastic bottles.

Rinse all glass containers with hot dilute HCl, then rinse several times in reagent water.

Never use commercial detergents containing phosphate for cleaning glassware used in phosphate

analysis.

4500-P B. Sample Preparation

For information on selection of digestion method (¶s 3 through 5 below), see 4500-P.A.3a.

1. Preliminary Filtration

Filter samples for determination of dissolved reactive phosphorus, dissolved

acid-hydrolyzable phosphorus, and total dissolved phosphorus through 0.45-μm membrane

filters. A glass fiber filter may be used to prefilter hard-to-filter samples.

Wash membrane filters by soaking in distilled water before use because they may contribute

significant amounts of phosphorus to samples containing low concentrations of phosphate. Use one of two washing techniques: (a) soak 50 filters in 2 L distilled water for 24 h; (b) soak 50

filters in 2 L distilled water for 1 h, change distilled water, and soak filters an additional 3 h.

Membrane filters also may be washed by running several 100-mL portions of distilled water

through them. This procedure requires more frequent determination of blank values to ensure

consistency in washing and to evaluate different lots of filters.

2. Preliminary Acid Hydrolysis

The acid-hydrolyzable phosphorus content of the sample is defined operationally as the

difference between reactive phosphorus as measured in the untreated sample and phosphate

found after mild acid hydrolysis. Generally, it includes condensed phosphates such as pyro-,

tripoly-, and higher-molecular-weight species such as hexametaphosphate. In addition, some

natural waters contain organic phosphate compounds that are hydrolyzed to orthophosphate

under the test conditions. Polyphosphates generally do not respond to reactive phosphorus tests

but can be hydrolyzed to orthophosphate by boiling with acid.

After hydrolysis, determine reactive phosphorus by a colorimetric method (C, D, or E).

Interferences, precision, bias, and sensitivity will depend on the colorimetric method used.

a. Apparatus:

Autoclave or pressure cooker, capable of operating at 98 to 137 kPa.

b. Reagents:

1) Phenolphthalein indicator aqueous solution.

2) Strong acid solution: Slowly add 300 mL conc H2SO4 to about 600 mL distilled water.

When cool, add 4.0 mL conc HNO3 and dilute to 1 L.

3) Sodium hydroxide, NaOH, 6N.

c. Procedure: To 100-mL sample or a portion diluted to 100 mL, add 0.05 mL (1 drop)

phenolphthalein indicator solution. If a red color develops, add strong acid solution dropwise, to

just discharge the color. Then add 1 mL more.

Boil gently for at least 90 min, adding distilled water to keep the volume between 25 and 50

mL. Alternatively, heat for 30 min in an autoclave or pressure cooker at 98 to 137 kPa. Cool,

neutralize to a faint pink color with NaOH solution, and restore to the original 100-mL volume

with distilled water.

Prepare a calibration curve by carrying a series of standards containing orthophosphate (see

colorimetric method C, D, or E) through the hydrolysis step. Do not use orthophosphate

standards without hydrolysis, because the salts added in hydrolysis cause an increase in the color

intensity in some methods.

Determine reactive phosphorus content of treated portions, using Method C, D, or E. This

gives the sum of polyphosphate and orthophosphate in the sample. To calculate its content of

acid-hydrolyzable phosphorus, determine reactive phosphorus in a sample portion that has not

been hydrolyzed, using the same colorimetric method as for treated sample, and subtract.

3. Perchloric Acid Digestion

a. Apparatus:

1) Hot plate: A 30- × 50-cm heating surface is adequate.

2) Safety shield.

3) Safety goggles.

4) Erlenmeyer flasks, 125-mL, acid-washed and rinsed with distilled water.

b. Reagents:

1) Nitric acid, HNO3, conc.

2) Perchloric acid, HClO4⋅2H2O, purchased as 70 to 72% HClO4, reagent-grade.

3) Sodium hydroxide, NaOH, 6N.

4) Methyl orange indicator solution.

5) Phenolphthalein indicator aqueous solution.

c. Procedure: CAUTION—Heated mixtures of HClO4 and organic matter may explode

violently. Avoid this hazard by taking the following precautions: (a) Do not add HClO4 to a hot

solution that may contain organic matter. (b) Always initiate digestion of samples containing

organic matter with HNO3. Complete digestion using the mixture of HNO3 and HClO4. (c) Do

not fume with HClO4 in ordinary hoods. Use hoods especially constructed for HClO4 fuming or

a glass fume eradicator*#(74) connected to a water pump. (d) Never let samples being digested

with HClO4 evaporate to dryness.

Measure sample containing the desired amount of phosphorus (this will be determined by

whether Method C, D, or E is to be used) into a 125-mL erlenmeyer flask. Acidify to methyl

orange with conc HNO3, add another 5 mL conc HNO3, and evaporate on a steam bath or hot

plate to 15 to 20 mL.

Add 10 mL each of conc HNO3 and HClO4 to the 125-mL conical flask, cooling the flask

between additions. Add a few boiling chips, heat on a hot plate, and evaporate gently until dense

white fumes of HClO4 just appear. If solution is not clear, cover neck of flask with a watch glass

and keep solution barely boiling until it clears. If necessary, add 10 mL more HNO3 to aid

oxidation.

Cool digested solution and add 1 drop aqueous phenolphthalein solution. Add 6N NaOH

solution until the solution just turns pink. If necessary, filter neutralized solution and wash filter

liberally with distilled water. Make up to 100 mL with distilled water.

Determine the PO43–-P content of the treated sample by Method C, D, or E.

Prepare a calibration curve by carrying a series of standards containing orthophosphate (see

Method C, D, or E) through digestion step. Do not use orthophosph treatment.

4. Sulfuric Acid-Nitric Acid Digestion

a. Apparatus:

1) Digestion rack: An electrically or gas-heated digestion rack with provision for

withdrawal of fumes is recommended. Digestion racks typical of those used for micro-kjeldahl

digestions are suitable.

2) Micro-kjeldahl flasks.

b. Reagents:

1) Sulfuric acid, H2SO4, conc.

2) Nitric acid, HNO3, conc.

3) Phenolphthalein indicator aqueous solution.

4) Sodium hydroxide, NaOH, 1N.

c. Procedure: Into a micro-kjeldahl flask, measure a sample containing the desired amount

of phosphorus (this is determined by the colorimetric method used). Add 1 mL conc H2SO4 and

5 mL conc HNO3.

Digest to a volume of 1 mL and then continue until solution becomes colorless to remove

HNO3.

Cool and add approximately 20 mL distilled water, 0.05 mL (1 drop) phenolphthalein

indicator, and as much 1N NaOH solution as required to produce a faint pink tinge. Transfer

neutralized solution, filtering if necessary to remove particulate material or turbidity, into a

100-mL volumetric flask. Add filter washings to flask and adjust sample volume to 100 mL with

distilled water.

Determine phosphorus by Method C, D, or E, for which a separate calibration curve has been

constructed by carrying standards through the acid digestion procedure.

5. Persulfate Digestion Method

a. Apparatus:

1) Hot plate: A 30- × 50-cm heating surface is adequate.

2) Autoclave: An autoclave or pressure cooker capable of developing 98 to 137 kPa may be

used in place of a hot plate.

3) Glass scoop, to hold required amounts of persulfate crystals.

b. Reagents:

1) Phenolphthalein indicator aqueous solution.

2) Sulfuric acid solution: Carefully add 300 mL conc H2SO4 to approximately 600 mL

distilled water and dilute to 1 L with distilled water.

3) Ammonium persulfate, (NH4)2S2O8, solid, or potassium persulfate, K2S2O8, solid.

4) Sodium hydroxide, NaOH, 1N.

c. Procedure: Use 50 mL or a suitable portion of thoroughly mixed sample. Add 0.05 mL (1

drop) phenolphthalein indicator solution. If a red color develops, add H2SO4 solution dropwise

to just discharge the color. Then add 1 mL H2SO4 solution and either 0.4 g solid (NH4)2S2O8 or

0.5 g solid K2S2O8.

Boil gently on a preheated hot plate for 30 to 40 min or until a final volume of 10 mL is

reached. Organophosphorus compounds such as AMP may require as much as 1.5 to 2 h for

complete digestion. Cool, dilute to 30 mL with distilled water, add 0.05 mL (1 drop)

phenolphthalein indicator solution, and neutralize to a faint pink color with NaOH. Alternatively,

heat for 30 min in an autoclave or pressure cooker at 98 to 137 kPa. Cool, add 0.05 mL (1 drop)

phenolphthalein indicator solution, and neutralize to a faint pink color with NaOH. Make up to

100 mL with distilled water. In some samples a precipitate may form at this stage, but do not

filter. For any subsequent subdividing of the sample, shake well. The precipitate (which is

possibly a calcium phosphate) redissolves under the acid conditions of the colorimetric reactive

phosphorus test. Determine phosphorus by Method C, D, or E, for which a separate calibration

curve has been constructed by carrying standards through the persulfate digestion procedure.

4500-P C. Vanadomolybdophosphoric Acid Colorimetric Method

1. General Discussion

a. Principle: In a dilute orthophosphate solution, ammonium molybdate reacts under acid

conditions to form a heteropoly acid, molybdophosphoric acid. In the presence of vanadium,

yellow vanadomolybdophosphoric acid is formed. The intensity of the yellow color is

proportional to phosphate concentration.

b. Interference: Positive interference is caused by silica and arsenate only if the sample is

heated. Negative interferences are caused by arsenate, fluoride, thorium, bismuth, sulfide,

thiosulfate, thiocyanate, or excess molybdate. Blue color is caused by ferrous iron but this does

not affect results if ferrous iron concentration is less than 100 mg/L. Sulfide interference may be

removed by oxidation with bromine water. Ions that do not interfere in concentrations up to 1000 mg/L are Al3+, Fe3+, Mg2+, Ca2+, Ba2+, Sr2+, Li+, Na+, K+, NH4+, Cd2+, Mn2+, Pb2+, Hg+,

Hg2+, Sn2+, Cu2+, Ni2+, Ag+, U4+, Zr4+, AsO3–, Br–, CO32–, ClO4–, CN–, IO3–, SiO44–, NO3–,

NO2–, SO42–, SO32–, pyrophosphate, molybdate, tetraborate, selenate, benzoate, citrate, oxalate,

lactate, tartrate, formate, and salicylate. If HNO3 is used in the test, Cl– interferes at 75 mg/L.

c. Minimum detectable concentration: The minimum detectable concentration is 200 μg P/L

in 1-cm spectrophotometer cells.

2. Apparatus

a. Colorimetric equipment: One of the following is required:

1) Spectrophotometer, for use at 400 to 490 nm.

2) Filter photometer, provided with a blue or violet filter exhibiting maximum transmittance

between 400 and 470 nm.

The wavelength at which color intensity is measured depends on sensitivity desired, because

sensitivity varies tenfold with wavelengths 400 to 490 nm. Ferric iron causes interference at low

wavelengths, particularly at 400 nm. A wavelength of 470 nm usually is used. Concentration

ranges for different wavelengths are:

P Range mg/L                                Wavelength nm

1.0– 5.0                                       400

2.0–10                                         420

4.0–18                                         470

b. Acid-washed glassware: Use acid-washed glassware for determining low concentrations of

phosphorus. Phosphate contamination is common because of its absorption on glass surfaces.

Avoid using commercial detergents containing phosphate. Clean all glassware with hot dilute

HCl and rinse well with distilled water. Preferably, reserve the glassware only for phosphate

determination, and after use, wash and keep filled with water until needed. If this is done, acid

treatment is required only occasionally.

c. Filtration apparatus and filter paper.*#(75)

3. Reagents

a. Phenolphthalein indicator aqueous solution.

b. Hydrochloric acid, HCl, 1 + 1. H2SO4, HClO4, or HNO3 may be substituted for HCl. The

acid concentration in the determination is not critical but a final sample concentration of 0.5N is

recommended.

c. Activated carbon.†#(76) Remove fine particles by rinsing with distilled water.

d. Vanadate-molybdate reagent:

1) Solution A: Dissolve 25 g ammonium molybdate, (NH4)6Mo7O24⋅4H2O, in 300 mL

distilled water.

2) Solution B: Dissolve 1.25 g ammonium metavanadate, NH4VO3, by heating to boiling in

300 mL distilled water. Cool and add 330 mL conc HCl. Cool Solution B to room temperature,

pour Solution A into Solution B, mix, and dilute to 1 L.

e. Standard phosphate solution: Dissolve in distilled water 219.5 mg anhydrous KH2PO4 and

dilute to 1000 mL; 1.00 mL = 50.0 μg PO43–-P.

4. Procedure

a. Sample pH adjustment: If sample pH is greater than 10, add 0.05 mL (1 drop)

phenolphthalein indicator to 50.0 mL sample and discharge the red color with 1 + 1 HCl before

diluting to 100 mL.

b. Color removal from sample: Remove excessive color in sample by shaking about 50 mL

with 200 mg activated carbon in an erlenmeyer flask for 5 min and filter to remove carbon.

Check each batch of carbon for phosphate because some batches produce high reagent blanks.

c. Color development in sample: Place 35 mL or less of sample, containing 0.05 to 1.0 mg P,

in a 50-mL volumetric flask. Add 10 mL vanadate-molybdate reagent and dilute to the mark with

distilled water. Prepare a blank in which 35 mL distilled water is substituted for the sample.

After 10 min or more, measure absorbance of sample versus a blank at a wavelength of 400 to

490 nm, depending on sensitivity desired (see ¶ 2a above). The color is stable for days and its

intensity is unaffected by variation in room temperature.

d. Preparation of calibration curve: Prepare a calibration curve by using suitable volumes of

standard phosphate solution and proceeding as in ¶ 4c. When ferric ion is low enough not to

interfere, plot a family of calibration curves of one series of standard solutions for various

wavelengths. This permits a wide latitude of concentrations in one series of determinations.

Analyze at least one standard with each set of samples.

5. Calculation

6. Precision and Bias

See Table 4500-P:I.

4500-P D. Stannous Chloride Method

1. General Discussion

a. Principle: Molybdophosphoric acid is formed and reduced by stannous chloride to

intensely colored molybdenum blue. This method is more sensitive than Method C and makes

feasible measurements down to 7 μg P/L by use of increased light path length. Below 100 μg P/L an extraction step may increase reliability and lessen interference.

b. Interference: See Section 4500-P.C.1b.

c. Minimum detectable concentration: The minimum detectable concentration is about 3 μg

P/L. The sensitivity at 0.3010 absorbance is about 10 μg P/L for an absorbance change of 0.009.

2. Apparatus

The same apparatus is required as for Method C, except that a pipetting bulb is required for

the extraction step. Set spectrophotometer at 625 nm in the measurement of benzene-isobutanol

extracts and at 690 nm for aqueous solutions. If the instrument is not equipped to read at 690 nm,

use a wavelength of 650 nm for aqueous solutions, with somewhat reduced sensitivity and

precision.

3. Reagents

a. Phenolphthalein indicator aqueous solution.

b. Strong-acid solution: Prepare as directed in Section 4500-P.B.2b2).

c. Ammonium molybdate reagent I: Dissolve 25 g (NH4)6Mo7O24⋅ 4H2O in 175 mL distilled

water. Cautiously add 280 mL conc H2SO4 to 400 mL distilled water. Cool, add molybdate

solution, and dilute to 1 L.

d. Stannous chloride reagent I: Dissolve 2.5 g fresh SnCl2⋅2H2O in 100 mL glycerol. Heat in

a water bath and stir with a glass rod to hasten dissolution. This reagent is stable and requires

neither preservatives nor special storage.

e. Standard phosphate solution: Prepare as directed in Section 4500-P.C.3e.

f. Reagents for extraction:

1) Benzene-isobutanol solvent: Mix equal volumes of benzene and isobutyl alcohol.

(CAUTION—This solvent is highly flammable.)

2) Ammonium molybdate reagent II: Dissolve 40.1 g (NH4)6Mo7O24⋅4H2O in approximately

500 mL distilled water. Slowly add 396 mL ammonium molybdate reagent I. Cool and dilute to 1L.

3) Alcoholic sulfuric acid solution: Cautiously add 20 mL conc H2SO4 to 980 mL methyl

alcohol with continuous mixing.

4) Dilute stannous chloride reagent II: Mix 8 mL stannous chloride reagent I with 50 mL

glycerol. This reagent is stable for at least 6 months.

4. Procedure

a. Preliminary sample treatment: To 100 mL sample containing not more than 200 μg P and

free from color and turbidity, add 0.05 mL (1 drop) phenolphthalein indicator. If sample turns

pink, add strong acid solution dropwise to discharge the color. If more than 0.25 mL (5 drops) is

required, take a smaller sample and dilute to 100 mL with distilled water after first discharging

the pink color with acid.

b. Color development: Add, with thorough mixing after each addition, 4.0 mL molybdate

reagent I and 0.5 mL (10 drops) stannous chloride reagent I. Rate of color development and

intensity of color depend on temperature of the final solution, each 1°C increase producing about

1% increase in color. Hence, hold samples, standards, and reagents within 2°C of one another

and in the temperature range between 20 and 30°C.

c. Color measurement: After 10 min, but before 12 min, using the same specific interval for

all determinations, measure color photometrically at 690 nm and compare with a calibration

curve, using a distilled water blank. Light path lengths suitable for various concentration ranges

are as follows:

Approximate P Range                     Light Path

mg/L                                                      cm

0.3–2                                            0.5

0.1–1                                            2

0.007–0.2                                    10

progressively and later fades, maintain equal timing conditions for samples and standards.

Prepare at least one standard with each set of samples or once each day that tests are made. The

calibration curve may deviate from a straight line at the upper concentrations of the 0.3 to

2.0-mg/L range.

d. Extraction: When increased sensitivity is desired or interferences must be overcome,

extract phosphate as follows: Pipet a 40-mL sample, or one diluted to that volume, into a

125-mL separatory funnel. Add 50.0 mL benzene-isobutanol solvent and 15.0 mL molybdate

reagent II. Close funnel at once and shake vigorously for exactly 15 s. If condensed phosphate is

present, any delay will increase its conversion to orthophosphate. Remove stopper and withdraw

25.0 mL of separated organic layer, using a pipet with safety bulb. Transfer to a 50-mL

volumetric flask, add 15 to 16 mL alcoholic H2SO4 solution, swirl, add 0.50 mL (10 drops)

dilute stannous chloride reagent II, swirl, and dilute to the mark with alcoholic H2SO4. Mix

thoroughly. After 10 min, but before 30 min, read against the blank at 625 nm. Prepare blank by

carrying 40 mL distilled water through the same procedure used for the sample. Read phosphate

concentration from a calibration curve prepared by taking known phosphate standards through

the same procedure used for samples.

5. Calculation

Calculate as follows:

- Direct procedure:

- Extraction procedure:

6. Precision and Bias

See Table 4500-P:I.

4500-P E. Ascorbic Acid Method

1. General Discussion

a. Principle: Ammonium molybdate and potassium antimonyl tartrate react in acid medium

with orthophosphate to form a heteropoly acid—phosphomolybdic acid—that is reduced to

intensely colored molybdenum blue by ascorbic acid.

b. Interference: Arsenates react with the molybdate reagent to produce a blue color similar to

that formed with phosphate. Concentrations as low as 0.1 mg As/L interfere with the phosphate

determination. Hexavalent chromium and NO2

– interfere to give results about 3% low at

concentrations of 1 mg/L and 10 to 15% low at 10 mg/L. Sulfide (Na2S) and silicate do not

interfere at concentrations of 1.0 and 10 mg/L.

c. Minimum detectable concentration: Approximately 10 μg P/L. P ranges are as follows:

Approximate P Range                      Light Path

mg/L                                                        cm

0.30–2.0                                        0.5

0.15–1.30                                      1.0

0.01–0.25                                      5.0

2. Apparatus

a. Colorimetric equipment: One of the following is required:

1) Spectrophotometer, with infrared phototube for use at 880 nm, providing a light path of

2.5 cm or longer.

2) Filter photometer, equipped with a red color filter and a light path of 0.5 cm or longer.

b. Acid-washed glassware: See Section 4500-P.C.2b.

3. Reagents

a. Sulfuric acid, H2SO4, 5N: Dilute 70 mL conc H2SO4 to 500 mL with distilled water.

b. Potassium antimonyl tartrate solution: Dissolve 1.3715 g K(SbO)C4H4O6⋅1/2H2O in 400

mL distilled water in a 500-mL volumetric flask and dilute to volume. Store in a glass-stoppered

bottle.

c. Ammonium molybdate solution: Dissolve 20 g (NH4)6Mo7O24⋅ 4H2O in 500 mL distilled

water. Store in a glass-stoppered bottle.

d. Ascorbic acid, 0.1M: Dissolve 1.76 g ascorbic acid in 100 mL distilled water. The solution

is stable for about 1 week at 4°C.

e. Combined reagent: Mix the above reagents in the following proportions for 100 mL of the

combined reagent: 50 mL 5N H2SO4, 5 mL potassium antimonyl tartrate solution, 15 mL

ammonium molybdate solution, and 30 mL ascorbic acid solution. Mix after addition of each

reagent. Let all reagents reach room temperature before they are mixed and mix in the order

given. If turbidity forms in the combined reagent, shake and let stand for a few minutes until

turbidity disappears before proceeding. The reagent is stable for 4 h.

f. Stock phosphate solution: See Section 4500-P.C.3e.

g. Standard phosphate solution: Dilute 50.0 mL stock phosphate solution to 1000 mL with

distilled water; 1.00 mL = 2.50 μg P.

4. Procedure

a. Treatment of sample: Pipet 50.0 mL sample into a clean, dry test tube or 125-mL

erlenmeyer flask. Add 0.05 mL (1 drop) phenolphthalein indicator. If a red color develops add

5N H2SO4 solution dropwise to just discharge the color. Add 8.0 mL combined reagent and mix

thoroughly. After at least 10 min but no more than 30 min, measure absorbance of each sample

at 880 nm, using reagent blank as the reference solution.

b. Correction for turbidity or interfering color: Natural color of water generally does not

interfere at the high wavelength used. For highly colored or turbid waters, prepare a blank by

adding all reagents except ascorbic acid and potassium antimonyl tartrate to the sample. Subtract

blank absorbance from absorbance of each sample.

c. Preparation of calibration curve: Prepare individual calibration curves from a series of six

standards within the phosphate ranges indicated in ¶ 1c above. Use a distilled water blank with

the combined reagent to make photometric readings for the calibration curve. Plot absorbance vs.

phosphate concentration to give a straight line passing through the origin. Test at least one

phosphate standard with each set of samples.

5. Calculation

6. Precision and Bias

The precision and bias values given in Table 4500-P:I are for a single-solution procedure

given in the 13th edition. The present procedure differs in reagent-to-sample ratios, no addition

of solvent, and acidity conditions. It is superior in precision and bias to the previous technique in

the analysis of both distilled water and river water at the 228-μg P/L level (Table 4500-P:II).

4500-P F. Automated Ascorbic Acid Reduction Method

1. General Discussion

a. Principle: Ammonium molybdate and potassium antimonyl tartrate react with

orthophosphate in an acid medium to form an antimony-phosphomolybdate complex, which, on

reduction with ascorbic acid, yields an intense blue color suitable for photometric measurement.

b. Interferences: As much as 50 mg Fe3+/L, 10 mg Cu/L, and 10 mg SiO2/L can be tolerated.

High silica concentrations cause positive interference.

In terms of phosphorus, the results are high by 0.005, 0.015, and 0.025 mg/L for silica

concentrations of 20, 50, and 100 mg/ L, respectively. Salt concentrations up to 20% (w/v) cause

an error of less than 1%. Arsenate (AsO43–) is a positive interference.

Eliminate interference from NO2– and S2– by adding an excess of bromine water or a saturated potassium permanganate (KMnO4) solution. Remove interfering turbidity by filtration

before analysis. Filter samples for total or total hydrolyzable phosphorus only after digestion.

Sample color that absorbs in the photometric range used for analysis also will interfere. See also

Section 4500-P.E.1b.

c. Application: Orthophosphate can be determined in potable, surface, and saline waters as

well as domestic and industrial wastewaters over a range of 0.001 to 10.0 mg P/L when

photometric measurements are made at 650 to 660 or 880 nm in a 15-mm or 50-mm tubular flow

cell. Determine higher concentrations by diluting sample. Although the automated test is

designed for orthophosphate only, other phosphorus compounds can be converted to this reactive

form by various sample pretreatments described in Section 4500-P.B.1, Section 4500-P.B.2, and

Section 4500-P.B.5.

2. Apparatus

a. Automated analytical equipment: An example of the continuous-flow analytical instrument

consists of the interchangeable components shown in Figure 4500-P:2. A flow cell of 15 or 50

mm and a filter of 650 to 660 or 880 nm may be used.

b. Hot plate or autoclave.

c. Acid-washed glassware: See Section 4500-P.C.2b.

3. Reagents

a. Potassium antimonyl tartrate solution: Dissolve 0.3 g K(SbO)C4H4O6⋅1/2H2O in

approximately 50 mL distilled water and dilute to 100 mL. Store at 4°C in a dark,

glass-stoppered bottle.

b. Ammonium molybdate solution: Dissolve 4 g (NH4)6Mo7O24⋅ 4H2O in 100 mL distilled

water. Store in a plastic bottle at 4°C.

c. Ascorbic acid solution: See Section 4500-P.E.3d.

d. Combined reagent: See Section 4500-P.E.3e.

e. Dilute sulfuric acid solution: Slowly add 140 mL conc H2SO4 to 600 mL distilled water.

When cool, dilute to 1 L.

f. Ammonium persulfate, (NH4)2S2O8, crystalline.

g. Phenolphthalein indicator aqueous solution.

h. Stock phosphate solution: Dissolve 439.3 mg anhydrous KH2PO4, dried for 1 h at 105°C,

in distilled water and dilute to 1000 mL; 1.00 mL = 100 μg P.

i. Intermediate phosphate solution: Dilute 100.0 mL stock phosphate solution to 1000 mL

with distilled water; 1.00 mL = 10.0 μg P.

j. Standard phosphate solutions: Prepare a suitable series of standards by diluting appropriate

volumes of intermediate phosphate solution.

4. Procedure

Set up manifold as shown in Figure 4500-P:2 and follow the general procedure described by

the manufacturer.

Add 0.05 mL (1 drop) phenolphthalein indicator solution to approximately 50 mL sample. If

a red color develops, add H2SO4 (¶ 3e) dropwise to just discharge the color.

5. Calculation

Prepare standard curves by plotting response of standards processed through the manifold

against P concentration in standards. Compute sample P concentration by comparing sample

response with standard curve.

6. Precision and Bias

Six samples were analyzed in a single laboratory in septuplicate. At an average PO4 3–

concentration of 0.340 mg/L, the average deviation was 0.015 mg/L. The coefficient of variation

was 6.2%. In two samples with added PO43–, recoveries were 89 and 96%.

4500-P G. Flow Injection Analysis for Orthophosphate (PROPOSED)

1. General Discussion

a. Principle: The orthophosphate ion (PO4

3–) reacts with ammonium molybdate and

antimony potassium tartrate under acidic conditions to form a complex. This complex is reduced

with ascorbic acid to form a blue complex that absorbs light at 880 nm. The absorbance is

proportional to the concentration of orthophosphate in the sample.

Also see Section 4500-P.A, Section 4500-P.B, and Section 4500-P.F, and Section 4130,

Flow Injection Analysis (FIA).

b. Interferences: Remove large or fibrous particulates by filtering sample through glass wool. Guard against contamination from reagents, water, glassware, and the sample preservation

process. Silica forms a pale blue complex that also absorbs at 880 nm. This interference is generally insignificant because a silica concentration of approximately 30 mg/L would be required to produce a 0.005 mg P/L positive error in orthophosphate.

Concentrations of ferric iron greater than 50 mg/L cause a negative error due to competition

with the complex for the reducing agent ascorbic acid. Treat samples high in iron with sodium

bisulfite to eliminate this interference, as well as the interference due to arsenates.

Glassware contamination is a problem in low-level phosphorus determinations. Wash

glassware with hot dilute HCl and rinse with reagent water. Commercial detergents are rarely

needed but, if they are used, use special phosphate-free preparations.

Also see Section 4500-P.F.

2. Apparatus

Flow injection analysis equipment consisting of:

a. FIA injection valve with sample loop or equivalent.

b. Multichannel proportioning pump.

c. FIA manifold (Figure 4500-P:3) with tubing heater and flow cell. Relative flow rates only

are shown in Figure 4500-P:3. Tubing volumes are given as an example only; they may be scaled

down proportionally. Use manifold tubing of an inert material such as TFE.

d. Absorbance detector, 880 nm, 10-nm bandpass.

e. Injection valve control and data acquisition system.

3. Reagents

Use reagent water (>10 megohm) to prepare carrier and all solutions. To prevent bubble

formation, degas carrier and buffer with helium. Pass He at 140 kPa (20 psi) through a helium

degassing tube. Bubble He through 1 L solution for 1 min. As an alternative to preparing

reagents by weight/weight, use weight/volume.

a. Stock ammonium molybdate solution: To a tared 1-L container add 40.0 g ammonium

molybdate tetrahydrate [(NH4)6Mo7O24⋅4H2O] and 983 g water. Mix with a magnetic stirrer for

at least 4 h. Store in plastic and refrigerate.

b. Stock antimony potassium tartrate solution: To a 1-L dark, tared container add 3.0 g

antimony potassium tartrate (potassium antimonyl tartrate hemihydrate),

K(SbO)C4H4O6⋅1/2H2O, and 995 g water. Mix with a magnetic stirrer until dissolved. Store in a

dark bottle and refrigerate.

c. Working molybdate color reagent: To a tared 1-L container add 680 g water, then add

64.4 g conc sulfuric acid. (CAUTION: This solution becomes very hot!) Swirl to mix. When

mixture can be handled comfortably, add 213 g stock ammonium molybdate solution (¶ 3a) and

72.0 g stock antimony potassium tartrate solution (¶ 3b). Shake and degas with helium.

d. Ascorbic acid solution: To a tared 1-L container, add 60.0 g granular ascorbic acid and

975 g water. Stir or shake until dissolved. Degas this reagent with helium, then add 1.0 g dodecyl

sulfate, CH3(CH2)11OSO3Na, stirring gently to mix. Prepare fresh weekly.

e. Stock orthophosphate standard, 25.00 mg P/L: In a 1-L volumetric flask dissolve 0.1099 g

primary standard grade anhydrous potassium phosphate monobasic (KH2PO4) that has been

dried for 1 h at 105°C in about 800 mL water. Dilute to mark with water and invert to mix.

f. Standard orthophosphate solutions: Prepare orthophosphate standards in desired

concentration range, using stock standard (¶ 3e) and diluting with water.

4. Procedure

Set up a manifold equivalent to that in Figure 4500-P:3 and follow method supplied by

manufacturer or laboratory standard operating procedure. Use quality control protocols outlined

in Section 4020.

5. Calculations

Prepare standard curves by plotting absorbance of standards processed through the manifold

versus orthophosphate concentration. The calibration curve is linear.

6. Precision and Bias

a. Recovery and relative standard deviation: Table 4500-P:III gives results of

single-laboratory studies.

b. MDL: A 700-μL sample loop was used in the method described above. Using a published

MDL method,1 analysts ran 21 replicates of a 5.0-μg P/L standard. These gave a mean of 5.26

μg P/L, a standard deviation of 0.264 μg P/L, and MDL of 0.67 μg P/L.

4500-P H. Manual Digestion and Flow Injection Analysis for Total

Phosphorus (PROPOSED)

1. General Discussion

a. Principle: Polyphosphates are converted to the orthophosphate form by a sulfuric acid

digestion and organic phosphorus is converted to orthophosphate by a persulfate digestion.

When the resulting solution is injected onto the manifold, the orthophosphate ion (PO4 3–) reacts

with ammonium molybdate and antimony potassium tartrate under acidic conditions to form a complex. This complex is reduced with ascorbic acid to form a blue complex that absorbs light at

880 nm. The absorbance is proportional to the concentration of total phosphorus in the sample.

See Section 4500-P.A for a discussion of the various forms of phosphorus found in waters

and wastewaters, Section 4500-P.B for a discussion of sample preparation and digestion, and

Section 4130, Flow Injection Analysis (FIA).

b. Interferences: See Section 4500-P.G.1b.

2. Apparatus

Digestion and flow injection analysis equipment consisting of:

a. Hotplate or autoclave.

b. FIA injection valve with sample loop or equivalent.

c. Multichannel proportioning pump.

d. FIA manifold (Figure 4500-P:4) with tubing heater and flow cell. Relative flow rates

only are shown in Figure 4500-P:4. Tubing volumes are given as an example only; they may be

scaled down proportionally. Use manifold tubing of an inert material such as TFE.

3. Reagents

Use reagent water (>10 megohm) for all solutions. To prevent bubble formation, degas

carrier and buffer with helium. Pass He at 140 kPa (20 psi) through a helium degassing tube.

Bubble He through 1 L solution for 1 min. As an alternative to preparing reagents by

weight/weight, use weight/volume.

Prepare reagents listed in Section 4500-P.G.3a, b, d, e, and f, and in addition:

a. Sulfuric acid carrier, H2SO4, 0.13M: To a tared 1-L container add 993 g water, then add

13.3 g conc H2SO4. Shake carefully to mix. Degas daily. Prepare fresh weekly.

b. Molybdate color reagent: To a tared 1-L container add 694 g water, then add 38.4 g conc

H2SO4. (CAUTION: The solution becomes very hot!) Swirl to mix. When mixture can be handled

comfortably, add 72.0 g stock antimony potassium tartrate (¶ G.3b) and 213 g stock ammonium

molybdate (¶ G.3a). Shake to mix, and degas.

4. Procedure

See Section 4500-P.B.4 or 5 for digestion procedures. Carry both standards and samples

through the digestion. The resulting solutions should be about 0.13M in sulfuric acid to match

the concentration of the carrier. If the solutions differ more than 10% from this concentration,

adjust concentration of carrier’s sulfuric acid to match that of digested samples.

Set up a manifold equivalent to that in Figure 4500-P:4 and analyze digested samples and

standards by following method supplied by manufacturer or laboratory’s standard operating

procedure. Use quality control protocols outlined in Section 4020.

5. Calculations

Prepare standard curves by plotting absorbance of standards processed through the manifold

versus phosphorus concentration. The calibration curve is linear.

6. Precision and Bias

a. MDL: A 780-μL sample loop was used in the method described above. Using a published

MDL method,1 analysts ran 21 replicates of a 3.5-μg P/L standard. These gave a mean of 3.53

μg P/L, a standard deviation of 0.82 μg P/L, and MDL of 2.0 μg P/L. The MDL is limited

mainly by the precision of the digestion.

b. Precision study: Ten injections of a 100.0-μg P/L standard gave a percent relative

standard deviation of 0.3%.

4500-P I. In-line UV/Persulfate Digestion and Flow Injection Analysis for Total

Phosphorus (PROPOSED)

1. General Discussion

a. Principle: Organic phosphorus is converted in-line to orthophosphate by heat, ultraviolet

radiation, and persulfate digestion. At the same time, inorganic polyphosphates are converted to

orthophosphate by in-line sulfuric acid digestion. The digestion processes occur before sample

injection. A portion of the digested sample is then injected and its orthophosphate concentration

determined by the flow injection method described in Section 4500-P.H.1.

See Section 4500-P.A for a discussion of the various forms of phosphorus found in waters

and wastewaters, Section 4500-P.B for a discussion of sample preparation and digestion, and

Section 4130, Flow Injection Analysis (FIA).

b. Interferences: See Section 4500-P.G.1b.

2. Apparatus

Flow injection analysis equipment consisting of:

a. FIA injection valve with sample loop or equivalent.

b. Multichannel proportioning pump.

c. FIA manifold (Figure 4500-P:5) with tubing heater, in-line ultraviolet digestion fluidics

including a debubbler consisting of a gas-permeable TFE membrane and its holder, and flow

cell. Relative flow rates only are shown in Figure 4500-P:5. Tubing volumes are given as an

example only; they may be scaled down proportionally. Use manifold tubing of an inert material

as TFE. The block marked ‘‘UV’’ should consist of TFE tubing irradiated by a mercury

discharge ultraviolet lamp emitting radiation at 254 nm.

d. Absorbance detector, 880 nm, 10-nm bandpass.

e. Injection valve control and data acquisition system.

3. Reagents

Use reagent water (>10 megohm) for all solutions. To prevent bubble formation, degas

carrier and all reagents with helium. Pass He at 140 kPa (20 psi) through a helium degassing

tube. Bubble He through 1 L solution for 1 min. As an alternative to preparing reagents by

weight/weight, use weight/volume.

a. Digestion reagent 1: To a tared 1-L container, add 893.5 g water, then slowly add 196.0 g

sulfuric acid, H2SO4. CAUTION: This solution becomes very hot! Prepare weekly. Degas before

using.

b. Digestion reagent 2: To a tared 1-L container, add 1000 g water, then add 26 g potassium

persulfate, K2S4O8. Mix with a magnetic stirrer until dissolved. Prepare weekly. Degas before

using.

c. Sulfuric acid carrier, 0.71M: To a tared 1-L container, slowly add 70 g H2SO4 to 962 g

water. Add 5 g sodium chloride, NaCl. Let cool, then degas with helium. Add 1.0 g sodium

dodecyl sulfate. Invert to mix. Prepare weekly.

d. Stock ammonium molybdate: To a tared 1-L container add 40.0 g ammonium molybdate

tetrahydrate, (NH4)6Mo7O24⋅4H2O, and 983 g water. Mix with a magnetic stirrer for at least 4 h.

The solution can be stored in plastic for up to 2 months if refrigerated.

e. Stock antimony potassium tartrate: To a 1-L dark, plastic, tared container add 3.0 g

antimony potassium tartrate (potassium antimonyl tartrate trihydrate), C8H4K2O12Sb2⋅3H2O,

and 995 g water. Mix with a magnetic stirrer until dissolved. The solution can be stored in a dark

plastic container for up to 2 months if refrigerated.

f. Molybdate color reagent: To a tared 1-L container add 715 g water, then 213 g stock

ammonium molybdate (¶ 3e) and 72.0 g stock antimony potassium tartrate (¶ 3 f). Add and

dissolve 22.8 g sodium hydroxide, NaOH. Shake and degas with helium. Prepare weekly.

g. Ascorbic acid: To a tared 1-L container add 70.0 g ascorbic acid and 975 g water. Mix

with a magnetic stirrer until dissolved. Degas with helium. Add 1.0 g sodium dodecyl sulfate.

Mix with a magnetic stirrer. Prepare fresh every 2 d.

h. Stock orthophosphate standard, 1000 mg P/L: In a 1-L volumetric flask dissolve 4.396 g

primary standard grade anhydrous potassium phosphate monobasic, KH2PO4 (dried for 1 h at

105°C), in about 800 mL water. Dilute to mark with water and invert to mix. Prepare monthly.

i. Standard solutions: Prepare orthophosphate standards in desired concentration range,

using stock orthophosphate standards (¶ 3i), and diluting with water. If the samples are preserved

concentration.

4. Procedure

Set up a manifold equivalent to that in Figure 4500-P:5 and follow method supplied by

manufacturer or laboratory’s standard operating procedure. Use quality control procedures

described in Section 4020.

5. Calculations

Prepare standard curves by plotting absorbance of standards processed through manifold

versus phosphorus concentration. The calibration curve is linear.

Verify digestion efficiency by determining tripolyphosphate and trimethylphosphate

standards at regular intervals. In the concentration range of the method, the recovery of either of

these compounds should be >95%.

6. Precision and Bias

a. MDL: A 390-μL sample loop was used in the method described above. Using a published

MDL method,1 analysts ran 21 replicates of a 0.10-mg P/L orthophosphate standard. These gave

a mean of 0.10 mg P/L, a standard deviation of 0.003 mg P/L, and MDL of 0.007 mg P/L.

b. Precision of recovery study: Ten injections of a 10.0-mg P/L trimethylphosphate standard

gave a mean percent recovery of 98% and a percent relative standard deviation of 0.8%.

c. Recovery of total phosphorus: Two organic and two inorganic complex phosphorus

compounds were determined in triplicate at three concentrations. The results are shown in Table

4500-P:IV.

d. Comparison of in-line digestion with manual digestion method: Samples from a

wastewater treatment plant influent and effluent and total phosphorus samples at 2.0 mg P/L

were determined in duplicate with both manual persulfate digestion followed by the method in

Section 4500-PH and in-line digestion method. Table 4500-P:V gives the results of this

comparison, and Figure 4500-P:6 shows the correlation between manual and in-line total phosphorus methods.

4500-S2– SULFIDE*#(82)

4500-S2– A. Introduction

1. Occurrence and Significance

Sulfide often is present in groundwater, especially in hot springs. Its common presence in

wastewaters comes partly from the decomposition of organic matter, sometimes from industrial

wastes, but mostly from the bacterial reduction of sulfate. Hydrogen sulfide escaping into the air

from sulfide-containing wastewater causes odor nuisances. The threshold odor concentration of

H2S in clean water is between 0.025 and 0.25 μg/L. Gaseous H2S is very toxic and has claimed

the lives of numerous workers in sewers. At levels toxic to humans it interferes with the

olfactory system, giving a false sense of the safe absence of H2S. It attacks metals directly and

indirectly has caused serious corrosion of concrete sewers because it is oxidized biologically to

H2SO4 on the pipe wall. Dissolved H2S is toxic to fish and other aquatic organisms.

2. Categories of Sulfides

From an analytical standpoint, three categories of sulfide in water and wastewater are

distinguished.

a. Total sulfide includes dissolved H2S and HS–, as well as acid-soluble metallic sulfides

present in suspended matter. The S2– is negligible, amounting to less than 0.5% of the dissolved

sulfide at pH 12, less than 0.05% at pH 11, etc. Copper and silver sulfides are so insoluble that

they do not respond in ordinary sulfide determinations; they can be ignored for practical

purposes.

b. Dissolved sulfide is that remaining after suspended solids have been removed by

flocculation and settling.

c. Un-ionized hydrogen sulfide may be calculated from the concentration of dissolved sulfide, the sample pH, and the practical ionization constant of H2S.

Figure 4500-S2–:1 shows analytical flow paths for sulfide determinations under various

conditions and options.

3. Sampling and Storage

Take samples with minimum aeration. Either analyze samples immediately after collection

or preserve for later analysis with zinc acetate solution. To preserve a sample for a total sulfide

determination put zinc acetate and sodium hydroxide solutions into bottle before filling it with

sample. Use 4 drops of 2N zinc acetate soution per 100 mL sample. Increase volume of zinc

acetate solution if the sulfide concentration is expected to be greater than 64 mg/L. The final pH

should be at least 9. Add more NaOH if necessary. Fill bottle completely and stopper.

4. Qualitative Tests

A qualitative test for sulfide often is useful. It is advisable in the examination of industrial

wastes containing interfering substances that may give a false negative result in the methylene

blue method (D).

a. Antimony test: To about 200 mL sample, add 0.5 mL saturated solution of potassium

antimony tartrate and 0.5 mL 6N HCl in excess of phenolphthalein alkalinity.

Yellow antimony sulfide (Sb2S3) is discernible at a sulfide concentration of 0.5 mg/L.

Comparisons with samples of known sulfide concentration make the technique roughly

quantitative. The only known interferences are metallic ions such as lead, which hold the sulfide

so firmly that it does not produce Sb2S3, and dithionite, which decomposes in acid solution to

produce sulfide.

b. Silver-silver sulfide electrode test: Dilute sample 1:1 with alkaline antioxidant reagent

(see ¶ G.3a below). Measure electrode potential relative to a double-junction reference electrode

and estimate the sulfide concentration from an old calibration curve or the example calibration

curve in the electrode manual. This gives a reasonable estimate of sulfide concentration if the

electrode is in good condition.

c. Lead acetate paper and silver foil tests: Confirm odors attributed to H2S with lead acetate

paper. On exposure to the vapor of a slightly acidified sample, the paper becomes blackened by

formation of PbS. A strip of silver foil is more sensitive than lead acetate paper. Clean the silver

by dipping in NaCN solution and rinse. CAUTION: NaCN is toxic, handle with care. Silver is

suitable particularly for long-time exposure in the vicinity of possible H2S sources because black

Ag2S is permanent whereas PbS slowly oxidizes.

5. Selection of Quantitative Methods

Iodine oxidizes sulfide in acid solution. A titration based on this reaction is an accurate

method for determining sulfide at concentrations above 1 mg/L if interferences are absent and if

loss of H2S is avoided. The iodometric method (F) is useful for standardizing the methylene blue

colorimetric methods (D, E, and I) and is suitable for analyzing samples freshly taken from wells

or springs. The method can be used for wastewater and partly oxidized water from sulfur springs

if interfering substances are removed first. The automated methylene blue method with

distillation (I) is useful for a variety of samples containing more than 1 mg S–/L.

The methylene blue method (D) is based on the reaction of sulfide, ferric chloride, and

dimethyl-p-phenylenediamine to produce methylene blue. Ammonium phosphate is added after

color development to remove ferric chloride color. The procedure is applicable at sulfide

concentrations between 0.1 and 20.0 mg/L. The automated methlylene blue method (E) is similar

to Method D. A gas dialysis technique separates the sulfide from the sample matrix. Gas dialysis

eliminates most interferences, including turbidity and color. The addition of the antioxidant

ascorbic acid improves sulfide recoveries. The method is applicable at sulfide concentrations

between 0.002 and 0.100 mg/L.

Potentiometric methods utilizing a silver electrode (G) may be suitable. From the potential of

the electrode relative to a reference electrode an estimate can be made of the sulfide

concentration, but careful attention to details of procedures and frequent standardizations are

needed to secure good results. The electrode is useful particularly as an end-point indicator for

titration of dissolved sulfide with silver nitrate. The ion-selective electrode method is unaffected

by sample color or turbidity and is applicable for concentrations greater than 0.03 mg/L.

6. Preparation of Sulfide Standards

Take care in preparing reliable stock solutions of sulfide for calibration and quality control.

Prepare sulfide standards from sodium sulfide nonahydrate (Na2S⋅9H2O) crystals. These crystals

usually have excess water present on the surface, in addition to a layer of contamination from

oxidation products (polysulfides, polythionates, and sulfate) of sulfide reacting with atmospheric

oxygen. Further, solutions of sulfide are prone to ready oxidation by dissolved and atmospheric

oxygen. Use reagent water to prepare sulfide standards and sample dilutions. Boil and degas with

either argon or nitrogen while cooling. Purchase the smallest amount of solid standards possible

and keep no longer than 1 year. Preferably handle and store solid sulfide standards and stock

solutions in an inert atmosphere glove bag or glove box to reduce contamination due to

oxidation.

Preferably remove single crystals of Na2S⋅9H2O from reagent bottle with nonmetallic

tweezers; quickly rinse in degassed reagent water to remove surface contamination. Blot crystal

dry with a tissue, then rapidly transfer to a tared, stoppered weighing bottle containing 5 to 10

mL degassed reagent water. Repeat procedure until desired amount of sodium sulfide is in

weighing bottle. Determine amount of Na2S⋅9H2O in weighing bottle by difference, then

multiply the weight by 0.133 to determine the amount of S2–. Avoid excess agitation and mixing

of the solution with atmospheric oxygen. Quantitatively transfer and dilute entire contents of

weighing bottle to an appropriate size volumetric flask with degassed reagent water to prepare a

known concentration sulfide stock solution (3.750 g Na2S⋅9H2O diluted to a final volume of 500

mL will give a stock solution of which 1.00 mL = 1.00 mg S2–). Alternatively, purchase precertified stock solutions of sulfide. Verify concentration of stock solution daily using the

iodometric method (F). Store stock solution with minimum headspace for no more than 1 week.

4500-S2– B. Separation of Soluble and Insoluble Sulfides

Unless the sample is entirely free from suspended solids (dissolved sulfide equals total

sulfide), to measure dissolved sulfide first remove insoluble matter. This can be done by

producing an aluminum hydroxide floc that is settled, leaving a clear supernatant for analysis.

1. Apparatus

Glass bottles with stoppers: Use 100 mL if sulfide will be determined by the methylene blue

method and 500 to 1000 mL if by the iodometric method.

2. Reagents

a. Sodium hydroxide solution, NaOH, 6N.

b. Aluminum chloride solution: Because of the hygroscopic and caking tendencies of this

chemical, purchase 100-g bottles of AlCl3⋅6H2O. Dissolve contents of a previously unopened

100-g bottle in 144 mL distilled water.

3. Procedure

a. To a 100-mL glass bottle add 0.2 mL (nominally 4 drops) 6N NaOH. Fill bottle with

sample and immediately add 0.2 mL (4 drops) AlCl3 solution. Stopper bottle with no air under

stopper. Rotate back and forth about a transverse axis vigorously for 1 min or longer to

flocculate contents. Vary volumes of these added chemicals to get good clarification without

using excessively large amounts and to produce a pH of 6 to 9. If a 500- or 1000-mL bottle is

used, add proportionally larger amounts of reagents.

b. Let settle until reasonably clear supernatant can be drawn off. With proper flocculation, this may take 5 to 15 min. Do not wait longer than necessary.

c. Either analyze the supernatant immediately or preserve with 2N zinc acetate (see Section

4500-S2–.C).

4500-S2– C. Sample Pretreatment to Remove Interfering Substances or to

Concentrate the Sulfide

The iodometric method suffers interference from reducing substances that react with iodine,

including thiosulfate, sulfite, and various organic compounds, both solid and dissolved.

Strong reducing agents also interfere in the methylene blue method (D) by preventing

formation of the blue color. Thiosulfate at concentrations about 10 mg/L may retard color

formation or completely prevent it. Ferrocyanide produces a blue color. Sulfide itself prevents

the reaction if its concentration is very high, in the range of several hundred milligrams per liter.

To avoid the possibility of false negative results, use the antimony method to obtain a qualitative

result in industrial wastes likely to contain sulfide but showing no color by the methylene blue

method. Iodide, which is likely to be present in oil-field wastewaters, may diminish color

formation if its concentration exceeds 2 mg/L. Many metals (e.g., Hg, Cd, Cu) form insoluble

sulfides and give low recoveries.

Eliminate interferences due to sulfite, thiosulfate, iodide, and many other soluble substances,

but not ferrocyanide, by first precipitating ZnS, removing the supernatant, and replacing it with

distilled water. Use the same procedure, even when not needed for removal of interferences, to

concentrate sulfide. The automated methylene blue method (E) is relatively free from

interferences because gas dialysis separates the sulfide from the sample matrix.

1. Apparatus

Glass bottles with stoppers: See Section 4500-S2–.B.1.

2. Reagents

a. Zinc acetate solution: Dissolve 220 g Zn(C2H3O2)2⋅2H2O in 870 mL water; this makes 1

L solution.

b. Sodium hydroxide solution, NaOH, 6N.

3. Procedure

a. Put 0.20 mL (4 drops) zinc acetate solution and 0.10 mL (2 drops) 6N NaOH into a

100-mL glass bottle, fill with sample, and add 0.10 mL (2 drops) 6N NaOH solution. Stopper

with no air bubbles under stopper and mix by rotating back and forth vigorously about a

transverse axis. For the iodometric procedure, use a 500-mL bottle or other convenient size, with

proportionally larger volumes of reagents. Vary volume of reagents added according to sample

so that the resulting precipitate is not excessively bulky and settles readily. Add enough NaOH

to raise the pH above 9. Let precipitate settle for 30 min. The treated sample is relatively stable

and can be held for several hours. However, if much iron is present, oxidation may be fairly rapid.

b. If the iodometric method is to be used, collect precipitate on a glass fiber filter and

continue at once with titration according to the procedure of Method F. If the methylene blue

method (D) is used, let precipitate settle for 30 min and decant as much supernatant as possible

without loss of precipitate. Refill bottle with distilled water, shake to resuspend precipitate, and

quickly withdraw a sample. If interfering substances are present in high concentration, settle,

decant, and refill a second time. If sulfide concentration is known to be low, add only enough

water to bring volume to one-half or one-fifth of original volume. Use this technique for

analyzing samples of very low sulfide concentrations. After determining the sulfide

concentration colorimetrically, multiply the result by the ratio of final to initial volume. No

concentration or pretreatment steps to remove interferences are necessary for Method E.

4500-S2– D. Methylene Blue Method

1. Apparatus

a. Matched test tubes, approximately 125 mm long and 15 mm OD.

b. Droppers, delivering 20 drops/mL methylene blue solution. To obtain uniform drops hold

dropper in a vertical position and let drops form slowly.

c. If photometric rather than visual color determination will be used, either:

1) Spectrophotometer, for use at a wavelength of 664 nm with cells providing light paths of 1

cm and 1 mm, or other path lengths, or

2) Filter photometer, with a filter providing maximum transmittance near 660 nm.

2. Reagents

a. Amine-sulfuric acid stock solution: Dissolve 27 g N,N-dimethyl-p-phenylenediamine

oxalate*#(83) in an iced mixture of 50 mL conc H2SO4 and 20 mL distilled water. Cool and

dilute to 100 mL with distilled water. Use fresh oxalate because an old supply may be oxidized

and discolored to a degree that results in interfering colors in the test. Store in a dark glass bottle.

When this stock solution is diluted and used in the procedure with a sulfide-free sample, it first

will be pink but then should become colorless within 3 min.

b. Amine-sulfuric acid reagent: Dilute 25 mL amine-sulfuric acid stock solution with 975 mL

1 + 1 H2SO4. Store in a dark glass bottle.

c. Ferric chloride solution: Dissolve 100 g FeCl3⋅6H2O in 40 mL water.

d. Sulfuric acid solution, H2SO4, 1 + 1.

e. Diammonium hydrogen phosphate solution: Dissolve 400 g (NH4)2HPO4 in 800 mL

distilled water.

f. Methylene blue solution I: Use USP grade dye or one certified by the Biological Stain Commission. The dye content should be reported on the label and should be 84% or more.

Dissolve 1.0 g in distilled water and make up to 1 L. This solution will be approximately the

correct strength, but because of variation between different lots of dye, standardize against

sulfide solutions of known strength and adjust its concentration so that 0.05 mL (1 drop) = 1.0

mg sulfide/L.

Standardization—Prepare five known-concentration sulfide standards ranging from 1 to 8

mg/L as described in 4500-S2–.A.6, or proceed as follows: Put several grams of clean, washed

crystals of Na2S⋅9H2O into a small beaker. Add somewhat less than enough water to cover

crystals. Stir occasionally for a few minutes, then pour solution into another vessel. This solution

reacts slowly with oxygen but the change is insignificant if analysis is performed within a few

hours. Prepare solution daily. To 1 L distilled water add 1 drop of Na2S solution and mix.

Immediately determine sulfide concentration by the methylene blue procedure and by the

iodometric procedure. Repeat, using more than 1 drop Na2S solution or smaller volumes of

water, until at least five tests have been made, with a range of sulfide concentrations between 1

and 8 mg/L. Calculate average percent error of the methylene blue result as compared to the

iodometric result. If the average error is negative, that is, methylene blue results are lower than

iodometric results, dilute methylene blue solution by the same percentage, so that a greater

volume will be used in matching colors. If methylene blue results are high, increase solution

strength by adding more dye.

g. Methylene blue solution II: Dilute 10.00 mL of adjusted methylene blue solution I to 100

mL with reagent water.

3. Procedure

a. Color development: Transfer 7.5 mL sample to each of two matched test tubes, using a

special wide-tip pipet or filling to marks on test tubes. If sample has been preserved with zinc

acetate, shake vigorously before taking subsample. Add to Tube A 0.5 mL amine-sulfuric acid

reagent and 0.15 mL (3 drops) FeCl3 solution. Mix immediately by inverting slowly, only once.

(Excessive mixing causes low results by loss of H2S as a gas before it has had time to react). To

Tube B add 0.5 mL 1 + 1 H2SO4 and 0.15 mL (3 drops) FeCl3 solution and mix. The presence

of S2– will be indicated by the appearance of blue color in Tube A. Color development usually is

complete in about 1 min, but a longer time often is required for fading out of the initial pink

color. Wait 3 to 5 min and add 1.6 mL (NH4)2HPO4 solution to each tube. Wait 3 to 15 min and

make color comparisons. If zinc acetate was used, wait at least 10 min before making a visual

color comparison.

b. Color determination:

1) Visual color estimation—Add methylene blue solution I or II, depending on sulfide

concentration and desired accuracy, dropwise, to the second tube, until color matches that

developed in first tube. If the concentration exceeds 20 mg/L, repeat test with a portion of sample diluted tenfold.

With methylene blue solution I, adjusted so that 0.05 mL (1 drop) = 1.0 mg S2–/L when 7.5

mL of sample are used:

mg S2–/L = no. drops solution I + 0.1 (no. drops solution II)

2) Photometric color measurement—A cell with a light path of 1 cm is suitable for measuring

sulfide concentrations from 0.1 to 2.0 mg/L. Use shorter or longer light paths for higher or lower

concentrations. This method is suitable for sample concentrations up to 20 mg/L. Zero

instrument with a portion of treated sample from Tube B. Prepare calibration curves on basis of

colorimetric tests made on Na2S solutions simultaneously analyzed by the iodometric method,

plotting concentration vs. absorbance. A linear relationship between concentration and

absorbance can be assumed from 0 to 1.0 mg/L.

Read sulfide concentration from calibration curve.

4. Precision and Bias

In a study by two chemists working in the same laboratory, the standard deviation estimated

from 34 sets of duplicate sulfide measurements was 0.04 mg/L for concentrations between 0.2

and 1.5 mg/L. The average recoveries of known additions were 92% for 40 samples containing

0.5 to 1.5 mg/L and 89% for samples containing less than 0.1 mg/L.

4500-S2– E. Gas Dialysis, Automated Methylene Blue Method

1. Apparatus

a. Automated analytical equipment: An example of the continuous-flow analytical instrument

consists of the interchangeable components shown in Figure 4500-S2–:2.

The sampler is equipped with a mixer to stir samples before analysis and the gas dialysis

membrane, which is maintained at room temperature, separates H2S from the sample matrix.

2. Reagents

a. N,N-dimethyl-p-phenylenediamine stock solution: Dissolve 1 g

N,N-dimethyl-p-phenylenediamine dihydrochloride in 500 mL 6N HCl. Prepare fresh monthly.

Store in an amber bottle.

b. N,N-dimethyl-p-phenylenediamine working solution: Dilute 190 mL N,N-dimethyl-p-phenylenediamine stock solution to 1 L. Store in an amber bottle. Prepare

weekly.

c. Ferric chloride stock solution: Dissolve 13.5 g FeCl3⋅6H2O in 500 mL 5N HCl. Store in

an amber bottle. Prepare fresh monthly.

d. Working ferric chloride solution: Dilute 190 mL ferric chloride stock solution to 1 L.

Store in an amber bottle. Prepare fresh weekly.

e. Hydrochloric acid, HCl, 6N:

f. Sodium hydroxide stock solution, NaOH, 1N.

g. Sodium hydroxide, NaOH, 0.01N: Dilute 10 mL NaOH stock solution to 1 L.

h. Sulfide stock solution, 1.00 mg S2–/1.00 mL: See Section 4500-S2–.A.6.

i. Sulfide intermediate standard solution: Dilute 10 mL sulfide stock solution to 1 L with

water. Prepare fresh daily. Standardize by iodometric titration method, Section 4500-S2–.F. 1 mL

8 0.01 mg S2–.

j. Sulfide tertiary standard solution: Dilute 50 mL sulfide intermediate solution to 500 mL

with 0.01N NaOH. Prepare fresh daily. Use standardization value from ¶ 2i to determine exact

concentration. 1.00 mL 8 0.001 mg S2–.

k. Working sulfide standard solutions: Prepare a suitable series of standards by diluting

appropriate volumes of sulfide tertiary standing solutions with 0.01N NaOH. Prepare fresh daily.

l. Zinc acetate preservative solution: Dissolve 220 g Zn(C2H3O2)2⋅2H2O in 870 mL water

(this makes 1 L solution).

3. Procedure

For unpreserved, freshly collected samples and sulfide working standards, add, in order, 4

drops 2N zinc acetate, 0.5 mL 6N NaOH, and 400 mg ascorbic acid/100 mL. For preserved

samples, add 0.5 mL 6N NaOH and 400 mg ascorbic acid/100 mL. Shake well.

Let precipitate settle for at least 30 min. Pour a portion of well-mixed sample or working

standard into a sample cup. Set up manifold as shown in Figure 4500-S2–:2 and follow the

general procedure described by the manufacturer. Determine absorbance at 660 nm.

4. Calculation

Prepare standard curves by plotting peak heights of standards processed through the

manifold against S2– concentration in the standards. Compute S2– sample concentration by

comparing sample response with standard curve.

5. Precision and Bias

In a single laboratory, samples with S2– concentrations of 0.012, 0.015, 0.034, and 0.085

mg/L had standard deviations of 0.001, 0.001, 0.001, and 0.001 mg/L, respectively, with

coefficients of variation of 8.3%, 6.3%, 2.9%, and 1.2%, respectively. In two environmental samples with added S2–, recoveries were 104.2% and 97.6%.

6. Bibliography

FRANCOM, D., L.R. GOODWIN & F.P. DIEKEN. 1989. Determination of low level sulfides in

environmental waters by automated gas dialysis/methylene blue colorimetry. Anal. Lett.

22:2587.

4500-S2– F. Iodometric Method

1. Reagents

a. Hydrochloric acid, HCl, 6N.

b. Standard iodine solution, 0.0250N: Dissolve 20 to 25 g KI in a little water and add 3.2 g

iodine. After iodine has dissolved, dilute to 1000 mL and standardize against 0.0250N Na2S2O3,

using starch solution as indicator.

c. Standard sodium thiosulfate solution, 0.0250N: See Section 4500-O.C.2e.

d. Starch solution: See Section 4500-O.C.2d.

2. Procedure

a. Measure from a buret into a 500-mL flask an amount of iodine solution estimated to be an

excess over the amount of sulfide present. Add distilled water, if necessary, to bring volume to

about 20 mL. Add 2 mL 6N HCl. Pipet 200 mL sample into flask, discharging sample under

solution surface. If iodine color disappears, add more iodine until color remains. Back-titrate

with Na2S2O3 solution, adding a few drops of starch solution as end point is approached, and

continuing until blue color disappears.

b. If sulfide was precipitated with zinc and ZnS filtered out, return filter with precipitate to

original bottle and add about 100 mL water. Add iodine solution and HCl and titrate as in ¶ 2a

above.

3. Calculation

One milliliter 0.0250N iodine solution reacts with 0.4 mg S2–:

where:

A = mL iodine solution,

B = normality of iodine solution, D = normality of Na2S2O3 solution.

4. Precision

The precision of the end point varies with the sample. In clean waters it should be

determinable within 1 drop, which is equivalent to 0.1 mg/L in a 200-mL sample.

4500-S2– G. Ion-Selective Electrode Method

1. General Discussion

a. Principle: The potential of a silver/sulfide ion-selective electrode (ISE) is related to the

sulfide ion activity. An alkaline antioxidant reagent (AAR) is added to samples and standards to

inhibit oxidation of sulfide by oxygen and to provide a constant ionic strength and pH. Use of the

AAR allows calibration in terms of total dissolved sulfide concentration. All samples and

standards must be at the same temperature. Sulfide concentrations between 0.032 mg/L (1 ×

10–6M) and 100 mg/L can be measured without preconcentration. For lower concentrations,

preconcentration is necessary.

b. Interferences: Humic substances may interfere with Ag/S-ISE measurements. For highly

colored water (high concentration of humic substances), use the method of standard additions to

check results. Sulfide is oxidized by dissolved oxygen. Sulfide oxidation may cause potential

readings to drift in the direction of decreasing concentration, i.e., to more positive values. Flush

surface of samples and standards with nitrogen to minimize contact with atmospheric oxygen for

low-level measurements. Temperature changes may cause potentials to drift either upward or

downward. Therefore, let standards and samples come to the same temperature. If samples

cannot be analyzed immediately, preserve dissolved sulfide by precipitating with zinc acetate

(Section 4500-S2–.C ).

2. Apparatus

a. Silver/sulfide electrode:*#(84)

b. Double-junction reference electrode.

c. Electrode polishing strips.†#(85)

d. pH meter with millivolt scale, capable of 0.1-mV resolution. Meters that can be calibrated

in concentration and that perform standard-additions calculations are available.

e. Electrochemical cell: Make suitable cell from a 150-mL beaker and a sheet of rigid plastic

(PVC or acrylic) with holes drilled to allow insertion of the electrodes and a tube for flushing the

headspace with nitrogen. Alternatively, purchase a polarographic cell with gas transfer

tube.‡#(86)

f. Gas dispersion tube: Use to deaerate water for preparing reagents and standards.

g. Magnetic stirrer and stirring bar: Use a piece of styrofoam or cardboard to insulate the

cell from the magnetic stirrer.

3. Reagents

a. Alkaline antioxidant reagent (AAR): To approximately 600 mL deaerated reagent water

(DRW) in a 1-L volumetric flask, add 80 g NaOH, 35 g ascorbic acid, and 67 g Na2H2EDTA.

Swirl to dissolve and dilute to 1 L. The color of freshly prepared AAR will range from colorless

to yellow. Store in a tightly capped brown glass bottle. Discard when solution becomes brown.

b. Lead perchlorate, 0.1M: Dissolve 4.60 g Pb(ClO4)2⋅3H2O in 100 mL reagent water.

Standardize by titrating with Na2H2EDTA. Alternatively, use commercially available 0.1M

Pb(ClO4)2 solutions.

c. Sulfide stock solution, 130 mg/L: See 4500-S2–.A.6, and dilute 13.0 mL of 1.00 mg

S2–/mL stock to 100.0 mL with AAR. Alternatively, add 500 mL AAR and 10 g Na2S⋅9H2O to a 1-L volumetric flask; dissolve. Dilute to 1 L with DRW. Use deaerated artificial seawater

(DASW), Table 8010:III, or 0.7M NaCl if sulfide concentrations are to be determined in

seawater. Standardize stock solution by titrating with 0.1M Pb(ClO4)2. Pipet 50 mL sulfide stock

solution into the electrochemical cell. (Use 10 mL with a small-volume polarographic cell.)

Insert Ag/S electrode and reference electrode and read initial potential. Titrate with 0.1M

Pb(ClO4)2. Let electrode potential stabilize and record potential after each addition. Locate

equivalence point as in Section 4500-Cl–.D.4a. Alternatively, linearize the titration curve.1

Calculate the function F1 for points before the equivalence point.

where:

Vo = volume of stock solution, mL,

V = titrant volume, mL,

E = potential, mV, and

m = slope of calibration curve, mV/log unit.

Plot F1 as a function of titrant volume. Extrapolate to find the intersection with the x-axis;

that is, the equivalence point. Calculate sulfide concentration in the stock solution from:

where:

C = sulfide concentration, mg/L,

Veq = equivalence volume, mL,

[Pb] = concentration of Pb in titrant, mg/L, and

Vo = volume of stock solution, mL.

Store stock solution in a tightly capped bottle for 1 week or less. The stock solution also can

be standardized iodometrically (see Section 4500-S2–.E). CAUTION: Store in a fume hood.

d. Sulfide standards: Prepare sulfide standards daily by serial dilution of stock. Add AAR

and Zn(C2H3O2)2 solutions to 100-mL volumetric flasks. Add sulfide solutions and dilute to

volume with DRW (or DASW). Refer to Table 4500-S2–:I for volumes. Prepare at least one

standard with a concentration less than the lowest sample concentration.

4. Procedure

Check electrode performance and calibrate daily. Check electrode potential in a sulfide

standard every 2 h. The procedure depends on the sulfide concentration and the time between

sample collection and sulfide determination. If the total sulfide concentration is greater than 0.03

mg/L (1 × 10−6M) and the time delay is only a few minutes, sulfide can be determined directly.

Otherwise, precipitate ZnS and filter as described in Section 4500-S2–.C.

a. Check electrode performance: Pipet 50 mL AAR, 50 mL DWR, and 1 mL sulfide stock

solution into the measurement cell. Place Ag/S and reference electrodes in the solution and read

potential. Add 10 mL stock solution and read potential. The change in potential should be −28 ±

2 mV. If it is not, follow the troubleshooting procedure in the electrode manual.

b. Calibration: Place electrodes in the most dilute standard but use calibration standards that

bracket the sulfide concentrations in the samples. Record potential when the rate of change is

less than 0.3 mV/min. (This may take up to 30 min for very low sulfide concentrations, i.e., less

than 0.03 mg/L.) Rinse electrodes, blot dry with a tissue, and read potential of the next highest

standard. For a meter that can be calibrated directly in concentration, follow manufacturer’s

directions. For other meters, plot potential as a function of the logarithm (base 10) of the sulfide

concentration. For potentials in the linear range, calculate the slope and intercept of the linear

portion of the calibration plot.

c. Sulfide determination by comparison with calibration curve, no ZnS precipitation: Add 40

mL AAR, 0.15 mL (3 drops) zinc acetate, and 50 mL sample to a 100-mL volumetric flask.

Dilute to 100 mL with AAR. Pour into the electrochemical cell and insert the electrodes. Record

potential when the rate of change is less than 0.3 mV/min. Read sulfide concentration from the

calibration curve. Alternatively, for potentials in the linear range, calculate the sulfide concentration from:

where:

E = electrode potential and

b and m are the intercept and slope of the calibration curve. For a meter that can be calibrated

directly in concentration, follow the manufacturer’s directions.

d. Sulfide determination by comparison with calibration curve, with ZnS precipitation: Place

filter with ZnS precipitate in a 150-mL beaker containing a stir bar. Wash sample bottle with 50

mL AAR and 20 mL DRW and pour the washings into the beaker. Stir to dissolve precipitate.

Remove filter with forceps while rinsing it into the beaker with a minimum amount of DRW.

Quantitatively transfer to a 100-mL volumetric flask and dilute to mark with DRW. Pour into

the electrochemical cell and place the electrodes in the solution. Measure potential as in ¶ 4c

above. Calculate sulfide concentration (¶ 4c).

e. Sulfide determination by standard addition with or without ZnS precipitation: Measure the

Ag/S-ISE electrode potential as in ¶ c or d above. Add sulfide stock solution and measure

potential again. Calculate sulfide concentration as follows:

where:

Co and Cs = sulfide concentrations in sample and known addition,

Eo and Es = potentials measured for sample and known addition,

m = slope of calibration curve (approximately 28 mV/log S2−, and

f = ratio of known-addition volume to sample volume.

f. Sulfide determination by titration: Use the same procedure as for standardizing the sulfide

stock solution (¶ 3c). The minumum sulfide concentration for determination by titration is 0.3

mg/L (10−5M).

5. Precision

For sulfide determination by comparison with the calibration curve, the relative standard

deviation varies with the sulfide concentration. RSD values of 23% for 0.0091 mg/L and 5% for

0.182 mg/L have been reported.2 (0.0091 μg/L was below the range for which the potential

varied linearly with the logarithm of the sulfide concentration, i.e., the Nernstian range.) For sulfide determination by standard addition, the precision is greatest if the amount of sulfide

added is as large as possible while staying within the linear range.3

4500-S2– H. Calculation of Un-ionized Hydrogen Sulfide

Hydrogen sulfide (H2S) and bisulfide ion (HS−), which together constitute dissolved sulfide,

are in equilibrium with hydrogen ions:

H2S H+ + HS−

The conditional ionization constant, which is valid for the temperature and ionic strength of

the water of interest, relates the concentrations of H2S and HS−:

The conditional constant is used to calculate the distribution of dissolved sulfide between the

two species. The conditional ionization constant of H2S is approximately 7.0. It differs from 7.0

by less than 0.2 log units for the ionic strengths and temperatures likely to be encountered in

water-quality monitoring. The fraction of sulfide present as H2S can be estimated with an error

of less than 40% from Figure 4500-S2−:3. If more accuracy is needed, use the methods given

below.

1. Calculation for Fresh Water and Brackish Water (I < 0.1M)

Calculate the dissociation constant for zero ionic strength (pK1) and the temperature of

interest.1 If the temperature is 25°C, then pK1 is 6.98. Otherwise:

pK1 (T ) = 32.55 + 1519.44/T − 15.672 log10T + 0.02722T

where T is temperature (°K, i.e., T °C + 273.15). Next, calculate the ionic strength I as in Table

2330:I, the Debye-Huckel A parameter, and the negative logarithm of the monovalent ion

activity coefficient (pf m):

A = 0.7083 − 2.277 × 10−3T + 5.399 × 10−6T 2

Calculate the conditional ionization constant, K′1, and the hydrogen ion concentration, [H+]:

K′1 = 10−pK1+2 pf m

[H+] = 10−pH+ pf m

Finally, calculate the un-ionized hydrogen sulfide concentration, [H2S], from the total sulfide

concentration, ST :

Sample calculation: Total sulfide concentration 0.32 mg/L (1.0 × 10−5M), pH 6.75, ionic

strength 0.02M, temperature 15.5°C.

2. Calculation for Seawater and Estuarine Water

This procedure differs only in calculating the conditional ionization constant, which can be

calculated accurately.1 The (potentially) largest source of error in calculating un-ionized

hydrogen sulfide in seawater is the hydrogen ion concentration. Calibrate the pH electrode in

artificial seawater at the temperature of the water of interest.2 Alternatively, if the pH electrode

is calibrated using NIST buffers (as in Section 4500-H), measure pH of dilute acid (10−4–10−3N

HNO3, HCl, or HClO4) in artificial seawater diluted to the salinity of the water of interest and at

the temperature of interest and calculate a correction factor.3 (Prepare artificial seawater as in

Table 8010:III, substituting NaCl for NaF, NaHCO3, and Na2SiO3⋅9H2O on an equimolar basis.)

Calculate pK′1 as outlined in Section 4500-S2−.H.1. Calculate the coefficients A and B1 (A

and B are not Debye-Huckel parameters):

Calculate pK′1:

where:

S = salinity, g/kg.

Calculate K′1:

K′1= 10−pK′1

Sample calculation: Total sulfide concentration 0.32 mg/L (1 × 10−5M), pH 6.75, salinity 35

g/kg (I = 0.7M), temperature 15.5°C.

From 4500-S2−.H.1, pK1 = 7.11.

4500-S2– I. Distillation, Methylene Blue Flow Injection Analysis (PROPOSED)

- General Discussion

a. Principle: Water and wastewater samples are distilled into a sodium hydroxide trapping

solution and the distillate is analyzed. Hydrogen sulfide (H2S) reacts in acid media and in the

presence of ferric chloride with two molecules of N, N-dimethyl-p-phenylenediamine to form

methylene blue. The resulting color is read at 660 nm.

b. Sample preservation: Because H2S oxidizes rapidly, analyze samples and standards

without delay. To preserve samples, add 4 drops 2M zinc acetate to 100 mL sample and adjust

pH to >9 with 6M NaOH, then cool to 4°C. Samples are distilled into a trapping solution

resulting in 0.25M NaOH matrix.

Also see Section 4500-S2–.A, Section 4500-S2–.B, and Section 4500-S2–.E, and Section

4130, Flow Injection Analysis (FIA).

c. Interferences: This method measures total sulfide, which is defined as the acid-soluble

sulfide fraction of a sample. Total sulfide includes both acid-soluble sulfides such as H2S, and

acid-soluble metal sulfides present in suspended matter. This method does not measure

acid-insoluble sulfides such as CuS.

Most nonvolatile interferences are eliminated by distillation. Strong reducing agents inhibit

color formation at concentrations of several hundred milligrams per liter. Iodide interferes at

concentrations greater than 2 mg I/L.

Also see Section 4500-S2–.A and Section 4500-S2–.B.

2. Apparatus

a. Distillation apparatus consisting of a glass or polypropylene micro-distillation

device*#(87) capable of distilling 6 mL or more of sample into a 0.25M NaOH final

concentration trapping solution.

b. Flow injection analysis equipment consisting of:

1) FIA injection valve with sample loop or equivalent.

2) Multichannel proportioning pump.

3) FIA manifold (Figure 4500-S2–:4) with cation exchange column and flow cell. Relative

flow rates only are shown in Figure 4500-S2–:4. Tubing volumes are given as an example only;

they may be scaled down proportionally. Use manifold tubing of an inert material such as TFE.

4) Absorbance detector, 660 nm, 10-nm bandpass.

5) Injection valve control and data acquisition system.

3. Reagents

Use reagent water (>10 megohm) for all solutions. To prevent bubble formation, degas

carrier and buffer with helium. Pass He at 140 kPa (20 psi) through a helium degassing tube.

Bubble He through 1 L solution for 1 min.

- Sodium hydroxide carrier and diluent, NaOH, 0.25M: In a 2-L volumetric flask, dissolve 20 g NaOH in approximately 1800 mL water. Dilute to mark and mix with a magnetic stirrer

until dissolved. Store in a plastic container.

b. Hydrochloric acid, HCl, 3M: To a tared 1-L container, add 752 g water and then slowly

add 295 g conc HCl. Invert to mix.

c. Hydrochloric acid, HCl, 0.20M: To a tared 1-L container, add 983.5 g water. Then add

19.7 g conc HCl. Invert to mix.

d. N,N-dimethyl-p-phenylenediamine: In a 1-L volumetric flask dissolve 1.0 g

N,N-dimethyl-p-phenylenediamine dihydrochloride, (CH3)2NC6H4NH2⋅2HCl, in about 800 mL

3M HCl (¶ 3b). Dilute to mark and invert to mix. If solution appears dark, it is likely that the

N,N-dimethyl-p-phenylenediamine dihydrochloride is decomposed; discard, and use fresh

reagent.

e. Ferric chloride: In a 500-mL volumetric flask dissolve 6.65 g ferric chloride hexahydrate,

FeCl3⋅6H2O, in about 450 mL 0.20M HCl (¶ 3c). Dilute to mark with water and invert to mix.

f. Stock sulfide standard, 100 mg S2–/L: In a 1-L volumetric flask dissolve 0.7491 g sodium

sulfide nonahydrate, Na2S⋅9H2O, in approximately 900 mL NaOH diluent (¶ 3a). Dilute to mark

and invert to mix.

g. Standard solutions: Prepare sulfide standards in desired concentration range, using stock

standard (¶ 3 f), and diluting with NaOH diluent (¶ 3a).

h. Sulfuric acid distillation releasing solution, H2SO4, 9M: To a tared 500-mL container,

add 150.0 g water, then add slowly while swirling, in increments of 40 g, 276 g conc H2SO4.

CAUTION: Solution will become very hot. Allow to cool before using.

4. Procedure

a. Distillation: This procedure is designed for the determination of sulfides in aqueous

solutions, solid waste materials, or effluents. To preserve and remove sulfide from interfering

substances, distill samples immediately after collection.

Follow manufacturer’s instructions for use of distillation apparatus. Add sufficient 9M

H2SO4 (¶ 3h) to sample to dissolve ZnS (s), digest total sulfides, and release the sulfide as

hydrogen sulfide gas. Immediately place sample on-line with the receiving vessel or collector

tube and distill hydrogen sulfide and water in the sample into a 0.25M trapping solution.

b. Flow injection analysis: Set up a manifold equivalent to that in Figure 4500-S2–:4 and

follow method supplied by the manufacturer or laboratory standard operating procedure. The

carrier concentration should be identical to the final concentration of NaOH in the trapping

solution from the distillation procedure (¶ 4a). Follow quality control protocols outlined in

Section 4020.

5. Calculations

Prepare standard curves by plotting absorbance of standards processed through the manifold versus sulfide concentration.

6. Precision and Bias

a. MDL: A 200-μL sample loop was used in the method described above. Using a published

method,1 analysts ran 21 replicates of a 10.0-mg S2–/L standard. These gave a mean of 9.0 mg

S2–/L, a standard deviation of 0.23 mg S2–/L, and MDL of 0.58 mg S2–/L. A higher MDL may

be obtained by decreasing sample loop volume.

b. Precision: Ten injections of a distilled 50-mg S2–/L standard gave a mean of 49.4 mg

S2–/L, a standard deviation of 0.27 mg S2–/L, and percent relative standard deviation of 0.54.

4500-SO3

2– SULFITE*#(88)

4500-SO3 2– A. Introduction

1. Occurrence

Sulfite ions (SO3

2–) may occur in boilers and boiler feedwaters treated with sulfite for

dissolved oxygen control, in natural waters or wastewaters as a result of industrial pollution, and

in treatment plant effluents dechlorinated with sulfur dioxide (SO2). Excess sulfite ion in boiler

waters is deleterious because it lowers the pH and promotes corrosion. Control of sulfite ion in

wastewater treatment and discharge may be important environmentally, principally because of its

toxicity to fish and other aquatic life and its rapid oxygen demand.

2. Selection of Method

The iodometric titration method is suitable for relatively clean waters with concentrations

above 2 mg SO32–/L. The phenanthroline colorimetric determination, following evolution of

sulfite from the sample matrix as SO2, is preferred for low levels of sulfite.

4500-SO3 2– B. Iodometric Method

1. General Discussion

a. Principle: An acidified sample containing sulfite (SO3

2–) is titrated with a standardized

potassium iodide-iodate titrant. Free iodine, liberated by the iodide-iodate reagent, reacts with

SO3 2–. The titration endpoint is signalled by the blue color resulting from the first excess of

iodine reacting with a starch indicator.

b. Interferences: The presence of other oxidizable materials, such as sulfide, thiosulfate, and

Fe2+ ions, can cause apparently high results for sulfite. Some metal ions, such as Cu2+, may

catalyze the oxidation of SO3 2– to SO4

2– when the sample is exposed to air, thus leading to low

results. NO2– will react with SO32– in the acidic reaction medium and lead to low sulfite results

unless sulfamic acid is added to destroy nitrite. Addition of EDTA as a complexing agent at the

time of sample collection inhibits Cu2+ catalysis and promotes oxidation of ferrous to ferric iron

before analysis. Sulfide and thiosulfate ions normally would be expected only in samples

containing certain industrial discharges, but must be accounted for if present. Sulfide may be

removed by adding about 0.5 g zinc acetate and analyzing the supernatant of the settled sample.

However, thiosulfate may have to be determined by an independent method (e.g., the

formaldehyde/iodometric method1), and then the sulfite determined by difference.

c. Minimum detectable concentration: 2 mg SO3 2–/L.

2. Reagents

a. Sulfuric acid: H2SO4, 1 + 1.

b. Standard potassium iodide-iodate titrant, 0.002083M: Dissolve 0.4458 g primary-grade

anhydrous KIO3 (dried for 4 h at 120°C), 4.35 g KI, and 310 mg sodium bicarbonate (NaHCO3)

in distilled water and dilute to 1000 mL; 1.00 mL = 500 μg SO3 2–.

c. Sulfamic acid, NH2SO3H, crystalline.

d. EDTA reagent: Dissolve 2.5 g disodium EDTA in 100 mL distilled water.

e. Starch indicator: To 5 g starch (potato, arrowroot, or soluble) in a mortar, add a little cold

distilled water and grind to a paste. Add mixture to 1 L boiling distilled water, stir, and let settle

overnight. Use clear supernatant. Preserve by adding either 1.3 g salicylic acid, 4 g ZnCl2, or a

combination of 4 g sodium propionate and 2 g sodium azide to 1 L starch solution.

3. Procedure

a. Sample collection: Collect a fresh sample, taking care to minimize contact with air. Fix

cooled samples (<50°C) immediately by adding 1 mL EDTA solution/100 mL sample. Cool hot

samples to 50°C or below. Do not filter.

b. Titration: Add 1 mL H2SO4 and 0.1 g NH2SO3H crystals to a 250-mL erlenmeyer flask or

other suitable titration vessel. Accurately measure 50 to 100 mL EDTA-stabilized sample into

flask, keeping pipet tip below liquid surface. Add 1 mL starch indicator solution. Titrate immediately with standard KI-KIO3 titrant, while swirling flask, until a faint permanent blue

color develops. Analyze a reagent blank using distilled water instead of sample.

- Calculation

where:

A = mL titrant for sample,

B = mL titrant for blank, and

M = molarity of KI-KIO3 titrant.

5. Precision and Bias

Three laboratories analyzed five replicate portions of a standard sulfite solution and of

secondary treated wastewater effluent to which sulfite was added. The data are summarized

below. Individual analyst’s precision ranged from 0.7 to 3.6% standard deviation (N = 45).

Sample                         ẋ                      Standard Deviation, σ    Relative Standard Deviation

mg/L                               mg/L                                         %

Standard,

6.3 mg SO3 2−/L    4.5                           0. 25                                   5.5

Secondary

effluent with

2.0 mg SO3 2−/L        2.1                     0.28                                       13.4

Secondary

effluent with

4.0 mg SO3 2−/L      3.6                     0.17                                          4.8

6. Reference

1. KURTENACKER, A. 1924. The aldehyde-bisulfite reaction in mass analysis. Z. Anal.

Chem. 64:56.

4500-SO3 2– C. Phenanthroline Method

1. General Discussion

a. Principle: An acidified sample is purged with nitrogen gas and the liberated SO2 is

trapped in an absorbing solution containing ferric ion and 1,10-phenanthroline. Ferric iron is

reduced to the ferrous state by SO2, producing the orange tris(1,10-phenanthroline) iron(II)

complex. After excess ferric iron is removed with ammonium bifluoride, the phenanthroline

complex is measured colorimetrically at 510 nm.1

b. Interferences: See Section 4500-SO3 2–.B..

c. Minimum detectable concentration: 0.01 mg SO3 2–/L.

2. Apparatus

a. Apparatus for evolution of SO2: Figure 4500-SO3 2–:1 shows the following components:

1) Gas flow meter, with a capacity to measure 2 L/min of pure nitrogen gas.

2) Gas washing bottle, 250-mL, with coarse-porosity, 12- mm-diam fritted cylinder gas

dispersion tube.

3) Tubing connectors, quick-disconnect, polypropylene.

4) Tubing, flexible PVC, for use in all connections.

5) Nessler tube, 100-mL.

b. Colorimetric equipment: One of the following is required:

1) Spectrophotometer, for use at 510 nm, providing a light path of 1 cm or longer.

2) Filter photometer, providing a light path of 1 cm or longer and equipped with a green

filter having maximum transmittance near 510 nm.

3. Reagents

a. 1,10-phenanthroline solution, 0.03M: Dissolve 5.95 g 1,10-phenanthroline in 100 mL 95%

ethanol. Dilute to 1 L with distilled water. Discard if solution becomes colored.

b. Ferric ammonium sulfate solution, 0.01M: Dissolve 4.82 g NH4Fe(SO4)2⋅12H2O in 1 L

distilled water to which has been added 1 mL conc H2SO4 to suppress ferric hydrolysis. Filter

through a glass fiber filter if insoluble matter is visible. If necessary, adjust volume of acid so

that a mixture of 10 parts of phenanthroline solution and one part of ferric ammonium sulfate

solution will have a pH between 5 and 6.

c. Ammonium bifluoride, 5%: Dissolve 25 g NH4HF2 in 500 mL distilled water. Store in a polyethylene bottle and dispense with a plastic pipet.

d. Potassium tetrachloromercurate, (TCM), K2HgCl4, 0.04M: Dissolve 10.86 g HgCl2, 5.96

g KCl, and 0.066 g disodium EDTA in distilled water and dilute to 1 L. Adjust pH to 5.2. This

reagent normally is stable for 6 months, but discard if a precipitate forms.2

e. Dilute TCM-stabilized sulfite standard: Dissolve 0.5 g Na2SO3 in 500 mL distilled water.

Standardize on the day of preparation, but wait at least 30 min to allow the rate of oxidation to

slow. Determine molarity by titrating with standard 0.0125M potassium iodide-iodate titrant

using starch indicator (see Section 4500-SO3 2–.B). Calculate molarity of working standard as follows:

where:

A = titrant for sample, mL,

B = titrant for blank, mL,

M = molarity of potassium iodide-iodate titrant.

Because stock Na2SO3 solution is unstable, immediately after standardization, pipet 10 mL into

a 500-mL volumetric flask partially filled with TCM and dilute to mark with TCM. Calculate the

concentration of this dilute sulfite solution by multiplying the stock solution concentration by

0.02. This TCM-stabilized standard is stable for 30 d if stored at 5°C. Discard as soon as any

precipitate is noticed at the bottom.

f. Standard potassium iodide-iodate titrant, 0.0125M: See Section 4500-SO3 2–.B..

g. Hydrochloric acid, 1 + 1.

h. Octyl alcohol, reagent-grade.

i. Sulfamic acid, 10%: Dissolve 10 g NH2SO3H in 100 mL distilled water. This reagent can

be kept for a few days if protected from air.

j. EDTA reagent: See Section 4500-SO3 2–.B..

4. Procedure

a. Sample collection: Collect a fresh sample taking care to minimize contact with air. Fix

cooled samples (<50°C) immediately by adding 1 mL EDTA solution for each 100 mL sample.

b. SO2 evolution: Prepare the absorbing solution by adding 5 mL 1,10-phenanthroline

solution, 0.5 mL ferric ammonium sulfate solution, 25 mL distilled water, and 5 drops octyl

alcohol (to act as defoamer) to a 100-mL nessler tube; insert a gas dispersion tube. Add 1 mL

sulfamic acid solution to the gas washing bottle and 100 mL of sample or a portion containing

less than 100 μg SO3 2– diluted to 100 mL. Add 10 mL 1 + 1 HCl and immediately connect the

gas washing bottle to the gas train as shown in Figure 4500-SO3 2–:1. Place a spring or rubber

band on the gas washing bottle to keep the top securely closed during gas flow. Adjust nitrogen

flow to 2.0 L/min and purge for 60 min.

c. Colorimetric measurement: After exactly 60 min, turn off nitrogen flow, disconnect

nessler tube, and immediately add 1 mL ammonium bifluoride solution. Remove gas dispersion

tube after rinsing it with distilled water into the tube and forcing the rinse water into the nessler

tube with a rubber bulb. Dilute to 50 mL in the nessler tube and mix by rapidly moving the tube

in a circular motion. Do not let rubber stoppers or PVC tubing come in contact with the

absorbing solution. After at least 5 min from the time of adding ammonium bifluoride, read the

absorbance versus distilled water at 510 nm using either a 5-cm cell for a range of 0 to 30 μg

SO3 2– per portion or a 1-cm cell for a range of 0 to 100 μg SO3 2–. Avoid transferring octyl

alcohol into the cell by letting it rise to the surface of the absorbing solution and transferring the

clear lower solution to the cell with a pipet. Make a calibration curve by analyzing a procedure

blank and at least three standards. Run at least one standard with each set of samples. For

maximum accuracy hold samples and standards at the same temperature and keep the time

interval from start of purging to the addition of ammonium bifluoride constant. This is easier to

do if several gas trains are used simultaneously in parallel. If ambient temperatures are subject to

frequent fluctuation, a water bath may be used to control color development at a fixed

temperature.

- Calculation

6. Precision and Bias

Three laboratories analyzed five replicate portions of a standard sulfite solution and of

secondary treated wastewater effluent to which sulfite was added. The data are summarized

below. Individual analyst’s precision ranged from 4.1 to 10.5% standard deviation (N = 45).

Sample                          ẋ           Standard Deviation, σ                       Relative Standard Deviation

mg/L                     mg/L                                                              %

Standard,

4.7 mg SO3 2–/L      3.7                 0.78                                                      21

Sample

Secondary

effluent with

0.12 mg SO3 2–/L   0.12                0.03                                                       25

Secondary

effluent with

4.0 mg SO3 2–/L     3.7                  0.30                                                       8.0

4500-SO4 2– SULFATE*#(89)

4500-SO4 2– A. Introduction

1. Occurrence

Sulfate (SO4 2–) is widely distributed in nature and may be present in natural waters in concentrations ranging from a few to several thousand milligrams per liter. Mine drainage wastes

may contribute large amounts of SO4 2– through pyrite oxidation. Sodium and magnesium sulfate

exert a cathartic action.

2. Selection of Method

The ion chromatographic method (4110) and capillary ion electrophoresis (CIE—see

Section 4140) are suitable for sulfate concentrations above 0.1 mg/L. The gravimetric methods

(C and D) are suitable for SO4 2– concentrations above 10 mg/L. The turbidimetric method (E) is

applicable in the range of 1 to 40 mg SO4 2– /L. The automated methylthymol blue methods (F

and G) are the procedures for analyzing large numbers of samples for sulfate alone when the

equipment is available; over 30 samples can be analyzed per hour. Methods C, D, F, G, 4110, or CIE (4140) are preferred for accurate results.

3. Sampling and Storage

In the presence of organic matter certain bacteria may reduce SO4 2– to S2–. To avoid this,

store samples at 4°C.

4500-SO4 2– B. (Reserved)

4500-SO4 2– C. Gravimetric Method with Ignition of Residue

1. General Discussion

a. Principle: Sulfate is precipitated in a hydrochloric acid (HCl) solution as barium sulfate

(BaSO4) by the addition of barium chloride (BaCl2).

The precipitation is carried out near the boiling temperature, and after a period of digestion

the precipitate is filtered, washed with water until free of Cl–, ignited or dried, and weighed as

BaSO4.

b. Interference: The gravimetric determination of SO4 2– is subject to many errors, both

positive and negative. In potable waters where the mineral concentration is low, these may be of

minor importance.

1) Interferences leading to high results—Suspended matter, silica, BaCl2 precipitant, NO3–,

SO3 2– and occluded mother liquor in the precipitate are the principal factors in positive errors.

Suspended matter may be present in both the sample and the precipitating solution; soluble

silicate may be rendered insoluble and SO3 2– may be oxidized to SO4 2– during analysis. Barium

nitrate [Ba(NO3)2], BaCl2, and water are occluded to some extent with the BaSO4 although

water is driven off if the temperature of ignition is sufficiently high.

2) Interferences leading to low results—Alkali metal sulfates frequently yield low results.

This is true especially of alkali hydrogen sulfates. Occlusion of alkali sulfate with BaSO4 causes

substitution of an element of lower atomic weight than barium in the precipitate. Hydrogen

sulfates of alkali metals act similarly and, in addition, decompose on being heated. Heavy

metals, such as chromium and iron, cause low results by interfering with the complete

precipitation of SO4 2– and by formation of heavy metal sulfates. BaSO4 has small but significant

solubility, which is increased in the presence of acid. Although an acid medium is necessary to

prevent precipitation of barium carbonate and phosphate, it is important to limit its concentration

to minimize the solution effect.

2. Apparatus

a. Steam bath.

b. Drying oven, equipped with thermostatic control.

c. Muffle furnace, with temperature indicator.

d. Desiccator.

e. Analytical balance, capable of weighing to 0.1 mg.

f. Filter: Use one of the following:

1) Filter paper, acid-washed, ashless hard-finish, sufficiently retentive for fine precipitates.

2) Membrane filter, with a pore size of about 0.45 μm.

g. Filtering apparatus, appropriate to the type of filter selected. (Coat membrane filter holder

with silicone fluid to prevent precipitate from adhering.)

3. Reagents

a. Methyl red indicator solution: Dissolve 100 mg methyl red sodium salt in distilled water

and dilute to 100 mL.

b. Hydrochloric acid, HCl, 1 + 1.

c. Barium chloride solution: Dissolve 100 g BaCl2⋅2H2O in 1 L distilled water. Filter

through a membrane filter or hard- finish filter paper before use; 1 mL is capable of precipitating

approximately 40 mg SO4 2–.

d. Silver nitrate-nitric acid reagent: Dissolve 8.5 g AgNO3 and 0.5 mL conc HNO3 in 500

mL distilled water.

e. Silicone fluid.*#(90)

4. Procedure

a. Removal of silica: If the silica concentration exceeds 25 mg/L, evaporate sample nearly to

dryness in a platinum dish on a steam bath. Add 1 mL HCl, tilt, and rotate dish until the acid

comes in complete contact with the residue. Continue evaporation to dryness. Complete drying

in an oven at 180°C and if organic matter is present, char over flame of a burner. Moisten residue

with 2 mL distilled water and 1 mL HCl, and evaporate to dryness on a steam bath. Add 2 mL

HCl, take up soluble residue in hot water, and filter. Wash insoluble silica with several small

portions of hot distilled water. Combine filtrate and washings. Discard residue.

b. Precipitation of barium sulfate: Adjust volume of clarified sample to contain

approximately 50 mg SO4 2– in a 250-mL volume. Lower concentrations of SO4 2– may be

tolerated if it is impracticable to concentrate sample to the optimum level, but in such cases limit

total volume to 150 mL. Adjust pH with HCl to pH 4.5 to 5.0, using a pH meter or the orange

color of methyl red indicator. Add 1 to 2 mL HCl. Heat to boiling and, while stirring gently, slowly add warm BaCl2 solution until precipitation appears to be complete; then add about 2 mL

in excess. If amount of precipitate is small, add a total of 5 mL BaCl2 solution. Digest precipitate

at 80 to 90°C, preferably overnight but for not less than 2 h.

c. Filtration and weighing: Mix a small amount of ashless filter paper pulp with the BaSO4,

quantitatively transfer to a filter, and filter at room temperature. The pulp aids filtration and

reduces the tendency of the precipitate to creep. Wash precipitate with small portions of warm

distilled water until washings are free of Cl– as indicated by testing with AgNO3-HNO3 reagent.

Place filter and precipitate in a weighed platinum crucible and ignite at 800°C for 1 h. Do not let

filter paper flame. Cool in desiccator and weigh.

- Calculation

6. Precision and Bias

A synthetic sample containing 259 mg SO4 2–/L, 108 mg Ca2+/L, 82 mg Mg2+/L, 3.1 mg

K+/L, 19.9 mg Na+/L, 241 mg Cl–/L, 0.250 mg NO2 –-N/L, 1.1 mg NO3–-N/L, and 42.5 mg total

alkalinity/L (contributed by NaHCO3) was analyzed in 32 laboratories by the gravimetric

method, with a relative standard deviation of 4.7% and a relative error of 1.9%.

4500-SO4 2– D. Gravimetric Method with Drying of Residue

1. General Discussion

See Method C, preceding.

2. Apparatus

With the exception of the filter paper, all of the apparatus cited in Section 4500-SO4 2–.C. is

required, plus the following:

- Filters: Use one of the following:

1) Fritted-glass filter, fine (‘‘F’’) porosity, with a maximum pore size of 5 μm.

2) Membrane filter, with a pore size of about 0.45 μm.

b. Vacuum oven.

3. Reagents

All the reagents listed in Section 4500-SO4 2–.C. are required.

4. Procedure

a. Removal of interference: See Section 4500-SO4 2–.C..

b. Precipitation of barium sulfate: See Section 4500-SO4 2–.C..

c. Preparation of filters:

1) Fritted glass filter—Dry to constant weight in an oven maintained at 105°C or higher, cool

in desiccator, and weigh.

2) Membrane filter—Place filter on a piece of filter paper or a watch glass and dry to

constant weight*#(91) in a vacuum oven at 80°C, while maintaining a vacuum of at least 85 kPa

or in a conventional oven at a temperature of 103 to 105°C. Cool in desiccator and weigh

membrane only.

d. Filtration and weighing: Filter BaSO4 at room temperature. Wash precipitate with several

small portions of warm distilled water until washings are free of Cl–, as indicated by testing with

AgNO3-HNO3 reagent. If a membrane filter is used add a few drops of silicone fluid to the

suspension before filtering, to prevent adherence of precipitate to holder. Dry filter and

precipitate by the same procedure used in preparing filter. Cool in a desiccator and weigh.

- Calculation

4500-SO4 2– E. Turbidimetric Method

1. General Discussion

a. Principle: Sulfate ion (SO4 2–) is precipitated in an acetic acid medium with barium

chloride (BaCl2) so as to form barium sulfate (BaSO4) crystals of uniform size. Light absorbance

of the BaSO4 suspension is measured by a photometer and the SO4 2– concentration is

determined by comparison of the reading with a standard curve.

b. Interference: Color or suspended matter in large amounts will interfere. Some suspended

matter may be removed by filtration. If both are small in comparison with the SO4 2–

concentration, correct for interference as indicated in ¶ 4d below. Silica in excess of 500 mg/L

will interfere, and in waters containing large quantities of organic material it may not be possible

to precipitate BaSO4 satisfactorily.

In potable waters there are no ions other than SO4 2– that will form insoluble compounds

with barium under strongly acid conditions. Make determination at room temperature; variation

over a range of 10°C will not cause appreciable error.

c. Minimum detectable concentration: Approximately 1 mg SO4 2–/L.

2. Apparatus

a. Magnetic stirrer: Use a constant stirring speed. It is convenient to incorporate a fixed

resistance in series with the motor operating the magnetic stirrer to regulate stirring speed. Use

magnets of identical shape and size. The exact speed of stirring is not critical, but keep it

constant for each run of samples and standards and adjust it to prevent splashing.

b. Photometer: One of the following is required, with preference in the order given:

1) Nephelometer.

2) Spectrophotometer, for use at 420 nm, providing a light path of 2.5 to 10 cm.

3) Filter photometer, equipped with a violet filter having maximum transmittance near 420

nm and providing a light path of 2.5 to 10 cm.

c. Stopwatch or electric timer.

d. Measuring spoon, capacity 0.2 to 0.3 mL.

3. Reagents

a. Buffer solution A: Dissolve 30 g magnesium chloride, MgCl2⋅6H2O, 5 g sodium acetate,

CH3COONa⋅3H2O, 1.0 g potassium nitrate, KNO3, and 20 mL acetic acid, CH3COOH (99%), in 500 mL distilled water and make up to 1000 mL.

b. Buffer solution B (required when the sample SO4 2– concentration is less than 10 mg/L):

Dissolve 30 g MgCl2⋅6H2O, 5 g CH3COONa⋅3H2O, 1.0 g KNO3, 0.111 g sodium sulfate,

Na2SO4, and 20 mL acetic acid (99%) in 500 mL distilled water and make up to 1000 mL.

c. Barium chloride, BaCl2, crystals, 20 to 30 mesh. In standardization, uniform turbidity is produced with this mesh range and the appropriate buffer.

d. Standard sulfate solution: Prepare a standard sulfate solution as described in 1) or 2)

below; 1.00 mL = 100 μg SO4 2–.

1) Dilute 10.4 mL standard 0.0200N H2SO4 titrant specified in Alkalinity, Section 2320B.3c,

to 100 mL with distilled water.

2) Dissolve 0.1479 g anhydrous Na2SO4 in distilled water and dilute to 1000 mL.

4. Procedure

a. Formation of barium sulfate turbidity: Measure 100 mL sample, or a suitable portion

made up to 100 mL, into a 250-mL erlenmeyer flask. Add 20 mL buffer solution and mix in

stirring apparatus. While stirring, add a spoonful of BaCl2 crystals and begin timing

immediately. Stir for 60 ± 2 s at constant speed.

b. Measurement of barium sulfate turbidity: After stirring period has ended, pour solution

into absorption cell of photometer and measure turbidity at 5 ± 0.5 min.

c. Preparation of calibration curve: Estimate SO4 2– concentration in sample by comparing

turbidity reading with a calibration curve prepared by carrying SO4 2– standards through the

entire procedure. Space standards at 5-mg/L increments in the 0- to 40-mg/L SO4 2– range.

Above 40 mg/L accuracy decreases and BaSO4 suspensions lose stability. Check reliability of

calibration curve by running a standard with every three or four samples.

d. Correction for sample color and turbidity: Correct for sample color and turbidity by

running blanks to which BaCl2 is not added.

- Calculation

If buffer solution A was used, determine SO4 2– concentration directly from the calibration

curve after subtracting sample absorbance before adding BaCl2. If buffer solution B was used

subtract SO4 2– concentration of blank from apparent SO4 2– concentration as determined above;

because the calibration curve is not a straight line, this is not equivalent to subtracting blank

absorbance from sample absorbance.

6. Precision and Bias

With a turbidimeter,*#(92) in a single laboratory with a sample having a mean of 7.45 mg

SO4 2–/L, a standard deviation of 0.13 mg/L and a coefficient of variation of 1.7% were obtained.

Two samples dosed with sulfate gave recoveries of 85 and 91%.

4500-SO4 2– F. Automated Methylthymol Blue Method

1. General Discussion

a. Principle: Barium sulfate is formed by the reaction of the SO4 2– with barium chloride

(BaCl2) at a low pH. At high pH excess barium reacts with methylthymol blue to produce a blue

chelate. The uncomplexed methylthymol blue is gray. The amount of gray uncomplexed

methylthymol blue indicates the concentration of SO4 2–.

b. Interferences: Because many cations interfere, use an ion-exchange column to remove

interferences.

Molybdenum, often used to treat cooling waters, has been shown to cause a strong positive

bias with this method, even with as little as 1 mg Mo/L.

c. Application: This method is applicable to potable, ground, surface, and saline waters as

well as domestic and industrial wastewaters over a range from about 10 to 300 mg SO4 2–/L.

2. Apparatus

a. Automated analytical equipment: An example of the required continuous-flow analytical

instrument consists of the interchangeable components shown in Figure 4500-SO4 2–:1.

b. Ion-exchange column: Fill a piece of 2-mm-ID glass tubing about 20 cm long with the

ion-exchange resin.*#(93) To simplify filling column put resin in distilled water and aspirate it

into the tubing, which contains a glass-wool plug. After filling, plug other end of tube with glass

wool. Avoid trapped air in the column.

3. Reagents

a. Barium chloride solution: Dissolve 1.526 g BaCl2⋅2H2O in 500 mL distilled water and

dilute to 1 L. Store in a polyethylene bottle.

b. Methylthymol blue reagent: Dissolve 118.2 mg methylthymol blue†#(94) in 25 mL BaCl2

solution. Add 4 mL 1N HCl and 71 mL distilled water and dilute to 500 mL with 95% ethanol.

Store in a brown glass bottle. Prepare fresh daily.

c. Buffer solution, pH 10.1: Dissolve 6.75 g NH4Cl in 500 mL distilled water. Add 57 mL

conc NH4OH and dilute to 1 L with distilled water. Adjust pH to 10.1 and store in a

polyethylene bottle. Prepare fresh monthly.

d. EDTA reagent: Dissolve 40 g tetrasodium ethylenediaminetetraacetate in 500 mL pH 10.1

buffer solution. Dilute to 1 L with pH 10.1 buffer solution and store in a polyethylene bottle.

e. Sodium hydroxide solution, 0.36N: Dissolve 7.2 g NaOH in 250 mL distilled water. Cool

and make up to 500 mL with distilled water.

f. Stock sulfate solution: Dissolve 1.479 g anhydrous Na2SO4 in 500 mL distilled water and

dilute to 1000 mL; 1.00 mL = 1.00 mg SO4 2–.

g. Standard sulfate solutions: Prepare in appropriate concentrations from 10 to 300 mg

SO4 2–/L, using the stock sulfate solution.

4. Procedure

Set up the manifold as shown in Figure 4500-SO4 2–:1 and follow the general procedure

described by the manufacturer.

After use, rinse methylthymol blue and NaOH reagent lines in water for a few minutes, rinse

them in the EDTA solution for 10 min, and then rinse in distilled water.

5. Calculation

Prepare standard curves by plotting peak heights of standards processed through the

manifold against SO4 2– concentrations in standards. Compute sample SO4 2– concentration by

comparing sample peak height with standard curve.

6. Precision and Bias

In a single laboratory a sample with an average concentration of about 28 mg SO4 2–/L had a

standard deviation of 0.68 mg/L and a coefficient of variation of 2.4%. In two samples with

added SO4 2–, recoveries were 91% and 100%.

4500-SO4 2– G. Methylthymol Blue Flow Injection Analysis (PROPOSED)

1. General Discussion

a. Principle: At pH 13.0 barium forms a blue complex with methylthymol blue (MTB). This

gives a dark blue base line. The sample is injected into a low, but known, concentration of

sulfate. The sulfate from the sample then reacts with the ethanolic barium-MTB solution and

displaces the MTB from the barium to give barium sulfate and uncomplexed MTB.

Uncomplexed MTB has a grayish color. The pH is raised with NaOH and the color of the gray

uncomplexed MTB is measured at 460 nm. The intensity of gray color is proportional to the

sulfate concentration.

Also see Section 4500-SO4 2–.A and Section 4500-SO4 2–.F, and Section 4130, Flow

Injection Analysis (FIA).

b. Interferences: Remove large or fibrous particulates by filtering sample through glass

wool. Guard against nitrate and nitrite contamination from reagents, water, glassware, and the

sample preservation process.

A cation-exchange column removes multivalent cations such as Ca2+ and Mg2+. A midrange

sulfate standard containing a typical level of hardness as CaCO3 can be run periodically to check

the performance of the column. Any decrease in peak height from that of a sulfate standard

without added CaCO3 indicates the need to regenerate or replace the resin.

Neutralize samples that have pH less than 2. High acid concentrations can displace

multivalent cations from the column.

Orthophosphate forms a precipitate with barium at high pH. If samples are known to be high

in orthophosphate, make a recovery study using added amounts of sulfate, or run a sample blank

containing only the orthophosphate matrix.

Also see Section 4500-SO4 2–.F..

2. Apparatus

Flow injection analysis equipment consisting of:

a. FIA injection valve with sample loop or equivalent.

b. Multichannel proportioning pump.

c. FIA manifold (Figure 4500-SO4 2–:2) with cation-exchange column and flow cell. Relative

flow rates only are shown in Figure 4500-SO4 2–:2. Tubing volumes are given as an example

only; they may be scaled down proportionally. Use manifold tubing of an inert material such as

TFE.

d. Absorbance detector, 460 nm, 10-nm bandpass.

e. Injection valve control and data acquisition system.

3. Reagents

Use reagent water (>10 megohm) for all solutions. To prevent bubble formation, degas

carrier and buffer with helium. Pass He at 140 kPa (20 psi) through a helium degassing tube.

Bubble He through 1 L solution for 1 min. As an alternative to preparing reagents by

weight/weight, use weight/volume.

a. Carrier solution, 0.30 mg SO4 2–/L: To a tared 1-L container, add 0.30 g 1000 mg/L stock

sulfate standard (¶ 3h) and 999.7 g water. Shake or stir to mix. Degas with helium.

b. Barium chloride solution, 6.24mM: To a tared 1-L container, add 1.526 g barium chloride

dihydrate, BaCl2⋅2H2O, and 995 g water. Shake or stir until dissolved. Degas with helium.

c. Hydrochloric acid, HCl, 1.0M: To a tared 1-L container, add 913 g water and 99.6 g conc

HCl (specific gravity 1.20, 37%). CAUTION: Fumes. Shake or stir to mix well. Degas with

helium.

d. Barium—MTB color reagent: NOTE: The purity of the methylthymol blue and the

denaturants in the alcohol are critical. Use the sources specified below, or test the material from

alternative sources for suitability before using.

To a tared 500-mL dry brown plastic bottle, place 0.236 g methylthymol blue,

3,3′-bis[N,N-di(carboxymethyl)amino-methyl]-thymol-sulfonephthalein, pentasodium salt. Add 50 g barium chloride solution (¶ 3b), which may be used to aid in transfer of the dye. Swirl to dissolve. Add 4.0 g of 1.0M HCl (¶ 3c) and mix. The solution should turn orange. Add 71 g

water and 321 g ethanol (ethyl alcohol, specially denatured anhydrous alcohol*#(95)). Stir or

shake to mix well. The pH should be 2.5. Prepare solution the day before use and store,

refrigerated, in a brown plastic bottle. Let warm to room temperature before using, then degas

with helium.

e. Stock sodium hydroxide solution, NaOH, 50% (w/v) solution: To a glass 1-L container

add 500 g water and 500.0 g NaOH. Dilute to 1 L. CAUTION: The solution becomes very hot.

Shake or stir until dissolved. Cool to ambient. Store in a plastic bottle.

f. Working sodium hydroxide solution, NaOH, 0.18M: To a tared plastic 1-L container add

982 g water and 19.8 g stock NaOH solution (¶ 3e). Shake or stir to mix. Degas with helium.

g. Cation exchange column preparation: Prepare approximately 0.5 g ion exchange

resin,†#(96) 50 to 100 mesh, by mixing with sufficient water to make a slurry. Remove one end

fitting from the threaded glass column. Fill column with water and aspirate slurry into column or

let it settle into column by gravity. Take care to avoid trapping air bubbles in column or fittings

at this point and during all subsequent operations. When resin has settled, replace end fitting. To

ensure a good seal, remove any resin particles from the threads of glass, column end, and end

fitting. To store column, join ends of the TFE tubing.

To test column effectiveness, make up two midrange standards, one of only sodium sulfate

and the other with an identical concentration of sodium sulfate but with hardness typical of the

samples. If the column is depleted, the standard with hardness will give a lower response

because the divalent Mg2+ and Ca2+ cations are complexing with the free MTB. If depletion has

occurred, repack column with fresh resin.

h. Stock sulfate standard, 1000 mg SO4 2–/L: Dry approximately 2 g sodium sulfate,

Na2SO4, at 105° overnight. Cool in a desiccator. In a 1-L volumetric flask, add 1.479 g of dried

sodium sulfate to about 800 mL water. Dissolve by swirling, dilute to mark, and mix by

inversion.

i. Standard solutions: Prepare sulfate standards in desired concentration range, using the

stock standard (¶ 3h), and diluting with water.

4. Procedure

Set up a manifold equivalent to that in Figure 4500-SO4 2–:2 and follow method supplied by

manufacturer or laboratory standard operating procedure. Follow quality control protocols

outlined in Section 4020.

5. Calculations

Prepare standard curves by plotting adsorbance of standards processed through the manifold

versus sulfate concentration.

6. Precision and Bias

a. Recovery and relative standard deviation: Table 4500-SO4

2–:I gives results of

single-laboratory studies.

b. MDL: A 180-μL sample loop was used in the method described above. Using a published

MDL method,1 analysts ran 21 replicates of a 5.00-mg SO4 2–/L standard. These gave a mean of

4.80 mg SO4 2–/L, a standard deviation of 0.69 mg SO4 2–/L, and MDL of 1.8 mg SO4 2–/L.

Part 5000 AGGREGATE ORGANIC CONSTITUENTS

5010 INTRODUCTION

5010 A. General Discussion

Analyses for organic matter in water and wastewater can be classified into two general types

of measurements: those that quantify an aggregate amount of organic matter comprising organic

constituents with a common characteristic and those that quantify individual organic compounds.

The latter can be found in Part 6000. The former, described here in Part 5000, have been grouped

into four categories: oxygen-demanding substances, organically bound elements, classes of

compounds, and formation potentials.

Methods for total organic carbon and chemical oxygen demand are used to assess the total

amount of organics present. Gross fractions of the organic matter can be identified analytically,

as in the measurement of BOD, which is an index of the biodegradable organics present, oil and

grease, which represents material extractable from a sample by a nonpolar solvent, or dissolved

organic halide (DOX), which measures organically bound halogens. Trihalomethane formation

potential is an aggregate measure of the total concentration of trihalomethanes formed upon

chlorination of a water sample.

Analyses of organics are made to assess the concentration and general composition of

organic matter in raw water supplies, wastewaters, treated effluents, and receiving waters; and to determine the efficiency of treatment processes.

5010 B. Sample Collection and Preservation

The sampling, field treatment, preservation, and storage of samples taken for organic matter

analysis are covered in detail in the individual introductions to the methods. If possible, analyze

samples immediately because preservatives often interfere with the tests. Otherwise, store at a

low temperature (4°C) immediately after collection to preserve most samples. Use chemical

preservatives only when they are shown not to interfere with the examinations to be made (see

Section 1060). Never use preservatives for samples to be analyzed for BOD. When preservatives

are used, add them to the sample bottle initially so that all portions are preserved as soon as

collected. No single method of preservation is entirely satisfactory; choose the preservative with

due regard to the determinations that are to be made. All methods of preservation may be

inadequate when applied to samples containing significant amounts of suspended matter.

5020 QUALITY ASSURANCE/QUALITY CONTROL

Part 1000 contains important information relevant to analyses included in Part 5000. Give

particular attention to Section 1020B (Quality Control), Section 1060 (Collection and

Preservation of Samples), Section 1080 (Reagent Water), and Section 1090 (Laboratory

Occupational Health and Safety), all of which are critical for many of the Part 5000 methods.

Take special precautions when analyses are performed by independent laboratories. Reliable

use of independent laboratories deserves the same quality assurance procedures observed for

in-house analyses: replicate samples, samples with known additions, and blanks.

Preparation of samples with known additions may not be feasible for certain analyses. In

such cases, consider using a mixture, in varying ratios, of several samples. Use the reported

concentrations in the samples and the proportions in which they were mixed to calculate the

expected concentration in the mixture. Examine laboratory performance using externally

prepared standards and check samples (see Section 1020B).

Reagent water (Section 1080) should give satisfactory results for most of the analyses in Part

5000, but additional purification steps may be needed for certain methods, such as dissolved

organic halogen (DOX) and disinfection by-product formation potential (DBPFP).

5210 BIOCHEMICAL OXYGEN DEMAND (BOD)*#(97)

5210 A. Introduction

- General Discussion

The biochemical oxygen demand (BOD) determination is an empirical test in which

standardized laboratory procedures are used to determine the relative oxygen requirements of

wastewaters, effluents, and polluted waters. The test has its widest application in measuring

waste loadings to treatment plants and in evaluating the BOD-removal efficiency of such

treatment systems. The test measures the molecular oxygen utilized during a specified incubation

period for the biochemical degradation of organic material (carbonaceous demand) and the

oxygen used to oxidize inorganic material such as sulfides and ferrous iron. It also may measure

the amount of oxygen used to oxidize reduced forms of nitrogen (nitrogenous demand) unless

their oxidation is prevented by an inhibitor. The seeding and dilution procedures provide an

estimate of the BOD at pH 6.5 to 7.5.

Measurements of oxygen consumed in a 5-d test period (5-d BOD or BOD5, Section 5210B),

oxygen consumed after 60 to 90 d of incubation (ultimate BOD or UBOD, Section 5210C), and

continuous oxygen uptake (respirometric method, Section 5210D) are described here. Many

other variations of oxygen demand measurements exist, including using shorter and longer

incubation periods and tests to determine rates of oxygen uptake. Alternative seeding, dilution,

and incubation conditions can be chosen to mimic receiving-water conditions, thereby providing

an estimate of the environmental effects of wastewaters and effluents.

The UBOD measures the oxygen required for the total degradation of organic material

(ultimate carbonaceous demand) and/or the oxygen to oxidize reduced nitrogen compounds

(ultimate nitrogenous demand). UBOD values and appropriate kinetic descriptions are needed in

water quality modeling studies such as UBOD: BOD5 ratios for relating stream assimilative

capacity to regulatory requirements; definition of river, estuary, or lake deoxygenation kinetics;

and instream ultimate carbonaceous BOD (UCBOD) values for model calibration.

2. Carbonaceous Versus Nitrogenous BOD

A number of factors, for example, soluble versus particulate organics, settleable and

floatable solids, oxidation of reduced iron and sulfur compounds, or lack of mixing may affect

the accuracy and precision of BOD measurements. Presently, there is no way to include

adjustments or corrections to account for the effect of these factors.

Oxidation of reduced forms of nitrogen, such as ammonia and organic nitrogen, can be

mediated by microorganisms and exert nitrogenous demand. Nitrogenous demand historically

has been considered an interference in the determination of BOD, as clearly evidenced by the

inclusion of ammonia in the dilution water. The interference from nitrogenous demand can now

be prevented by an inhibitory chemical.1 If an inhibiting chemical is not used, the oxygen

demand measured is the sum of carbonaceous and nitrogenous demands.

Measurements that include nitrogenous demand generally are not useful for assessing the

oxygen demand associated with organic material. Nitrogenous demand can be estimated directly

from ammonia nitrogen (Section 4500-NH3); and carbonaceous demand can be estimated by

subtracting the theoretical equivalent of the reduced nitrogen oxidation from uninhibited test

results. However, this method is cumbersome and is subject to considerable error. Chemical inhibition of nitrogenous demand provides a more direct and more reliable measure of

carbonaceous demand.

The extent of oxidation of nitrogenous compounds during the 5-d incubation period depends

on the concentration and type of microorganisms capable of carrying out this oxidation. Such

organisms usually are not present in raw or settled primary sewage in sufficient numbers to

oxidize sufficient quantities of reduced nitrogen forms in the 5-d BOD test. Many biological

treatment plant effluents contain sufficient numbers of nitrifying organisms to cause nitrification

in BOD tests. Because oxidation of nitrogenous compounds can occur in such samples,

inhibition of nitrification as directed in Section 5210B.4e6) is recommended for samples of

secondary effluent, for samples seeded with secondary effluent, and for samples of polluted

waters.

Report results as carbonaceous biochemical oxygen demand (CBOD5) when inhibiting the

nitrogenous oxygen demand. When nitrification is not inhibited, report results as BOD5.

3. Dilution Requirements

The BOD concentration in most wastewaters exceeds the concentration of dissolved oxygen

(DO) available in an air-saturated sample. Therefore, it is necessary to dilute the sample before

incubation to bring the oxygen demand and supply into appropriate balance. Because bacterial

growth requires nutrients such as nitrogen, phosphorus, and trace metals, these are added to the

dilution water, which is buffered to ensure that the pH of the incubated sample remains in a

range suitable for bacterial growth. Complete stabilization of a sample may require a period of

incubation too long for practical purposes; therefore, 5 d has been accepted as the standard

incubation period.

If the dilution water is of poor quality, the BOD of the dilution water will appear as sample

BOD. This effect will be amplified by the dilution factor. A positive bias will result. The

methods included below (Section 5210B and Section 5210C) contain both a dilution-water

check and a dilution-water blank. Seeded dilution waters are checked further for acceptable

quality by measuring their consumption of oxygen from a known organic mixture, usually

glucose and glutamic acid.

The source of dilution water is not restricted and may be distilled, tap, or receiving-stream

water free of biodegradable organics and bioinhibitory substances such as chlorine or heavy

metals. Distilled water may contain ammonia or volatile organics; deionized waters often are

contaminated with soluble organics leached from the resin bed. Use of copper-lined stills or

copper fittings attached to distilled water lines may produce water containing excessive amounts

of copper (see Section 3500-Cu).

5210 B. 5-Day BOD Test

1. General Discussion

a. Principle: The method consists of filling with sample, to overflowing, an airtight bottle of

the specified size and incubating it at the specified temperature for 5 d. Dissolved oxygen is

measured initially and after incubation, and the BOD is computed from the difference between

initial and final DO. Because the initial DO is determined shortly after the dilution is made, all

oxygen uptake occurring after this measurement is included in the BOD measurement.

b. Sampling and storage: Samples for BOD analysis may degrade significantly during

storage between collection and analysis, resulting in low BOD values. Minimize reduction of

BOD by analyzing sample promptly or by cooling it to near-freezing temperature during storage.

However, even at low temperature, keep holding time to a minimum. Warm chilled samples to

20 ± 3°C before analysis.

1) Grab samples—If analysis is begun within 2 h of collection, cold storage is unnecessary.

If analysis is not started within 2 h of sample collection, keep sample at or below 4°C from the

time of collection. Begin analysis within 6 h of collection; when this is not possible because the

sampling site is distant from the laboratory, store at or below 4°C and report length and

temperature of storage with the results. In no case start analysis more than 24 h after grab sample

collection. When samples are to be used for regulatory purposes make every effort to deliver

samples for analysis within 6 h of collection.

2) Composite samples—Keep samples at or below 4°C during compositing. Limit

compositing period to 24 h. Use the same criteria as for storage of grab samples, starting the

measurement of holding time from end of compositing period. State storage time and conditions

as part of the results.

2. Apparatus

a. Incubation bottles: Use glass bottles having 60 mL or greater capacity (300-mL bottles having a ground-glass stopper and a flared mouth are preferred). Clean bottles with a detergent,

rinse thoroughly, and drain before use. As a precaution against drawing air into the dilution

bottle during incubation, use a water seal. Obtain satisfactory water seals by inverting bottles in a

water bath or by adding water to the flared mouth of special BOD bottles. Place a paper or

plastic cup or foil cap over flared mouth of bottle to reduce evaporation of the water seal during

incubation.

b. Air incubator or water bath, thermostatically controlled at 20 ±1°C. Exclude all light to

prevent possibility of photosynthetic production of DO.

3. Reagents

Prepare reagents in advance but discard if there is any sign of precipitation or biological

growth in the stock bottles. Commercial equivalents of these reagents are acceptable and

different stock concentrations may be used if doses are adjusted proportionally.

a. Phosphate buffer solution: Dissolve 8.5 g KH2PO4, 21.75 g K2HPO4, 33.4 g

Na2HPO4⋅7H2O, and 1.7 g NH4Cl in about 500 mL distilled water and dilute to 1 L. The pH

should be 7.2 without further adjustment. Alternatively, dissolve 42.5 g KH2PO4 or 54.3 g

K2HPO4 in about 700 mL distilled water. Adjust pH to 7.2 with 30% NaOH and dilute to 1 L.

b. Magnesium sulfate solution: Dissolve 22.5 g MgSO4⋅7H2O in distilled water and dilute to 1 L.

c. Calcium chloride solution: Dissolve 27.5 g CaCl2 in distilled water and dilute to 1 L.

d. Ferric chloride solution: Dissolve 0.25 g FeCl3⋅6H2O in distilled water and dilute to 1 L.

e. Acid and alkali solutions, 1N, for neutralization of caustic or acidic waste samples.

1) Acid—Slowly and while stirring, add 28 mL conc sulfuric acid to distilled water. Dilute

to 1 L.

2) Alkali—Dissolve 40 g sodium hydroxide in distilled water. Dilute to 1 L.

f. Sodium sulfite solution: Dissolve 1.575 g Na2SO3 in 1000 mL distilled water. This

solution is not stable; prepare daily.

g. Nitrification inhibitor, 2-chloro-6-(trichloromethyl) pyridine.*#(98)

h. Glucose-glutamic acid solution: Dry reagent-grade glucose and reagent-grade glutamic

acid at 103°C for 1 h. Add 150 mg glucose and 150 mg glutamic acid to distilled water and

dilute to 1 L. Prepare fresh immediately before use.

i. Ammonium chloride solution: Dissolve 1.15 g NH4Cl in about 500 mL distilled water,

adjust pH to 7.2 with NaOH solution, and dilute to 1 L. Solution contains 0.3 mg N/mL.

j. Dilution water: Use demineralized, distilled, tap, or natural water for making sample

dilutions.

4. Procedure

a. Preparation of dilution water: Place desired volume of water (¶ 3 j) in a suitable bottle

and add 1 mL each of phosphate buffer, MgSO4, CaCl2, and FeCl3 solutions/L of water. Seed

dilution water, if desired, as described in ¶ 4d. Test dilution water as described in ¶ 4h so that

water of assured quality always is on hand.

Before use bring dilution water temperature to 20 ± 3°C. Saturate with DO by shaking in a

partially filled bottle or by aerating with organic-free filtered air. Alternatively, store in

cotton-plugged bottles long enough for water to become saturated with DO. Protect water quality

by using clean glassware, tubing, and bottles.

b. Dilution water storage: Source water (¶ 3 j) may be stored before use as long as the

prepared dilution water meets quality control criteria in the dilution water blank (¶ 4h). Such

storage may improve the quality of some source waters but may allow biological growth to cause

deterioration in others. Preferably do not store prepared dilution water for more than 24 h after

adding nutrients, minerals, and buffer unless dilution water blanks consistently meet quality

control limits. Discard stored source water if dilution water blank shows more than 0.2 mg/L DO

depletion in 5 d.

c. Glucose-glutamic acid check: Because the BOD test is a bioassay its results can be

influenced greatly by the presence of toxicants or by use of a poor seeding material. Distilled

waters frequently are contaminated with copper; some sewage seeds are relatively inactive. Low

results always are obtained with such seeds and waters. Periodically check dilution water quality,

seed effectiveness, and analytical technique by making BOD measurements on a mixture of 150

mg glucose/L and 150 mg glutamic acid/L as a ‘‘standard’’ check solution. Glucose has an

exceptionally high and variable oxidation rate but when it is used with glutamic acid, the

oxidation rate is stabilized and is similar to that obtained with many municipal wastes.

Alternatively, if a particular wastewater contains an identifiable major constituent that

contributes to the BOD, use this compound in place of the glucose-glutamic acid.

Determine the 5-d 20°C BOD of a 2% dilution of the glucose-glutamic acid standard check

solution using the techniques outlined in ¶s 4d-j. Adjust concentrations of commercial mixtures

to give 3 mg/L glucose and 3 mg/L glutamic acid in each GGA test bottle. Evaluate data as

described in ¶ 6, Precision and Bias.

d. Seeding:

1) Seed source—It is necessary to have present a population of microorganisms capable of

oxidizing the biodegradable organic matter in the sample. Domestic wastewater, unchlorinated

or otherwise-undisinfected effluents from biological waste treatment plants, and surface waters

receiving wastewater discharges contain satisfactory microbial populations. Some samples do

not contain a sufficient microbial population (for example, some untreated industrial wastes,

disinfected wastes, high-temperature wastes, or wastes with extreme pH values). For such wastes

seed the dilution water or sample by adding a population of microorganisms. The preferred seed

is effluent or mixed liquor from a biological treatment system processing the waste. Where such

seed is not available, use supernatant from domestic wastewater after settling at room temperature for at least 1 h but no longer than 36 h. When effluent or mixed liquor from a

biological treatment process is used, inhibition of nitrification is recommended.

Some samples may contain materials not degraded at normal rates by the microorganisms in

settled domestic wastewater. Seed such samples with an adapted microbial population obtained

from the undisinfected effluent or mixed liquor of a biological process treating the waste. In the

absence of such a facility, obtain seed from the receiving water below (preferably 3 to 8 km) the

point of discharge. When such seed sources also are not available, develop an adapted seed in the

laboratory by continuously aerating a sample of settled domestic wastewater and adding small

daily increments of waste. Optionally use a soil suspension or activated sludge, or a commercial

seed preparation to obtain the initial microbial population. Determine the existence of a

satisfactory population by testing the performance of the seed in BOD tests on the sample. BOD

values that increase with time of adaptation to a steady high value indicate successful seed

adaptation.

2) Seed control—Determine BOD of the seeding material as for any other sample. This is

the seed control. From the value of the seed control and a knowledge of the seeding material

dilution (in the dilution water) determine seed DO uptake. Ideally, make dilutions of seed such

that the largest quantity results in at least 50% DO depletion. A plot of DO depletion, in

milligrams per liter, versus milliters of seed for all bottles having a 2-mg/L depletion and a

1.0-mg/L minimum residual DO should present a straight line for which the slope indicates DO

depletion per milliliter of seed. The DO-axis intercept is oxygen depletion caused by the dilution

water and should be less than 0.1 mg/L (¶ 4h). Alternatively, divide DO depletion by volume of

seed in milliliters for each seed control bottle having a 2-mg/L depletion and a 1.0-mg/L residual

DO. Average the results for all bottles meeting minimum depletion and residual DO criteria. The

DO uptake attributable to the seed added to each bottle should be between 0.6 and 1.0 mg/L, but

the amount of seed added should be adjusted from this range to that required to provide

glucose-glutamic acid check results in the range of 198 ± 30.5 mg/L. To determine DO uptake

for a test bottle, subtract DO uptake attributable to the seed from total DO uptake (see ¶ 5).

Techniques for adding seeding material to dilution water are described for two sample

dilution methods (¶ 4 f).

e. Sample pretreatment: Check pH of all samples before testing unless previous experience

indicates that pH is within the acceptable range.

1) Samples containing caustic alkalinity (pH >8.5) or acidity (pH <6.0)—Neutralize samples

to pH 6.5 to 7.5 with a solution of sulfuric acid (H2SO4) or sodium hydroxide (NaOH) of such

strength that the quantity of reagent does not dilute the sample by more than 0.5%. The pH of

dilution water should not be affected by the lowest sample dilution. Always seed samples that

have been pH-adjusted.

2) Samples containing residual chlorine compounds—If possible, avoid samples containing

residual chlorine by sampling ahead of chlorination processes. If the sample has been chlorinated

but no detectable chlorine residual is present, seed the dilution water. If residual chlorine is

present, dechlorinate sample and seed the dilution water (¶ 4 f). Do not test chlorinated/dechlorinated samples without seeding the dilution water. In some samples chlorine

will dissipate within 1 to 2 h of standing in the light. This often occurs during sample transport

and handling. For samples in which chlorine residual does not dissipate in a reasonably short

time, destroy chlorine residual by adding Na2SO3 solution. Determine required volume of

Na2SO3 solution on a 100- to 1000-mL portion of neutralized sample by adding 10 mL of 1 + 1

acetic acid or 1 + 50 H2SO4, 10 mL potassium iodide (KI) solution (10 g/100 mL) per 1000 mL

portion, and titrating with Na2SO3 solution to the starch-iodine end point for residual. Add to

neutralized sample the relative volume of Na2SO3 solution determined by the above test, mix,

and after 10 to 20 min check sample for residual chlorine. (NOTE: Excess Na2SO3 exerts an

oxygen demand and reacts slowly with certain organic chloramine compounds that may be

present in chlorinated samples.)

3) Samples containing other toxic substances—Certain industrial wastes, for example,

plating wastes, contain toxic metals. Such samples often require special study and treatment.

4) Samples supersaturated with DO—Samples containing more than 9 mg DO/L at 20°C

may be encountered in cold waters or in water where photosynthesis occurs. To prevent loss of

oxygen during incubation of such samples, reduce DO to saturation at 20°C by bringing sample

to about 20°C in partially filled bottle while agitating by vigorous shaking or by aerating with

clean, filtered compressed air.

5) Sample temperature adjustment—Bring samples to 20 ± 1°C before making dilutions.

6) Nitrification inhibition—If nitrification inhibition is desired add 3 mg

2-chloro-6-(trichloro methyl) pyridine (TCMP) to each 300-mL bottle before capping or add

sufficient amounts to the dilution water to make a final concentration of 10 mg/L. (NOTE: Pure

TCMP may dissolve slowly and can float on top of the sample. Some commercial formulations

dissolve more readily but are not 100% TCMP; adjust dosage accordingly.) Samples that may

require nitrification inhibition include, but are not limited to, biologically treated effluents,

samples seeded with biologically treated effluents, and river waters. Note the use of nitrogen

inhibition in reporting results.

f. Dilution technique: Make several dilutions of sample that will result in a residual DO of at

least 1 mg/L and a DO uptake of at least 2 mg/L after a 5-d incubation. Five dilutions are

recommended unless experience with a particular sample shows that use of a smaller number of

dilutions produces at least two bottles giving acceptable minimum DO depletion and residual

limits. A more rapid analysis, such as COD, may be correlated approximately with BOD and

serve as a guide in selecting dilutions. In the absence of prior knowledge, use the following

dilutions: 0.0 to 1.0% for strong industrial wastes, 1 to 5% for raw and settled wastewater, 5 to

25% for biologically treated effluent, and 25 to 100% for polluted river waters.

Prepare dilutions either in graduated cylinders or volumetric glassware, and then transfer to

BOD bottles or prepare directly in BOD bottles. Either dilution method can be combined with

any DO measurement technique. The number of bottles to be prepared for each dilution depends on the DO technique and the number of replicates desired.

When using graduated cylinders or volumetric flasks to prepare dilutions, and when seeding

is necessary, add seed either directly to dilution water or to individual cylinders or flasks before

dilution. Seeding of individual cylinders or flasks avoids a declining ratio of seed to sample as

increasing dilutions are made. When dilutions are prepared directly in BOD bottles and when

seeding is necessary, add seed directly to dilution water or directly to the BOD bottles. When a

bottle contains more than 67% of the sample after dilution, nutrients may be limited in the

diluted sample and subsequently reduce biological activity. In such samples, add the nutrient,

mineral, and buffer solutions (¶ 3a through e) directly to individual BOD bottles at a rate of 1

mL/L (0.33 mL/300-mL bottle) or use commercially prepared solutions designed to dose the

appropriate bottle size.

1) Dilutions prepared in graduated cylinders or volumetric flasks—If the azide modification

of the titrimetric iodometric method (Section 4500-O.C) is used, carefully siphon dilution water,

seeded if necessary, into a 1- to 2-L-capacity flask or cylinder. Fill half full without entraining

air. Add desired quantity of carefully mixed sample and dilute to appropriate level with dilution

water. Mix well with a plunger-type mixing rod; avoid entraining air. Siphon mixed dilution into

two BOD bottles. Determine initial DO on one of these bottles. Stopper the second bottle tightly,

water-seal, and incubate for 5 d at 20°C. If the membrane electrode method is used for DO

measurement, siphon dilution mixture into one BOD bottle. Determine initial DO on this bottle

and replace any displaced contents with sample dilution to fill the bottle. Stopper tightly,

water-seal, and incubate for 5 d at 20°C.

2) Dilutions prepared directly in BOD bottles—Using a wide-tip volumetric pipet, add the

desired sample volume to individual BOD bottles of known capacity. Add appropriate amounts

of seed material either to the individual BOD bottles or to the dilution water. Fill bottles with

enough dilution water, seeded if necessary, so that insertion of stopper will displace all air,

leaving no bubbles. For dilutions greater than 1:100 make a primary dilution in a graduated

cylinder before making final dilution in the bottle. When using titrimetric iodometric methods

for DO measurement, prepare two bottles at each dilution. Determine initial DO on one bottle.

Stopper second bottle tightly, water-seal, and incubate for 5 d at 20°C. If the membrane electrode

method is used for DO measurement, prepare only one BOD bottle for each dilution. Determine

initial DO on this bottle and replace any displaced contents with dilution water to fill the bottle.

Stopper tightly, water-seal, and incubate for 5 d at 20°C. Rinse DO electrode between

determinations to prevent cross-contamination of samples.

Use the azide modification of the iodometric method (Section 4500-O.C) or the membrane

electrode method (Section 4500-O.G) to determine initial DO on all sample dilutions, dilution

water blanks, and where appropriate, seed controls.

If the membrane electrode method is used, the azide modification of the iodometric method

(Method 4500-O.C) is recommended for calibrating the DO probe.

g. Determination of initial DO: If the sample contains materials that react rapidly with DO,

determine initial DO immediately after filling BOD bottle with diluted sample. If rapid initial DO uptake is insignificant, the time period between preparing dilution and measuring initial DO

is not critical but should not exceed 30 min.

h. Dilution water blank: Use a dilution water blank as a rough check on quality of unseeded

dilution water and cleanliness of incubation bottles. Together with each batch of samples

incubate a bottle of unseeded dilution water. Determine initial and final DO as in ¶s 4g and j.

The DO uptake should not be more than 0.2 mg/L and preferably not more than 0.1 mg/L

Discard all dilution water having a DO uptake greater than 0.2 mg/L and either eliminate source

of contamination or select an alternate dilution water source..

i. Incubation: Incubate at 20°C ± 1°C BOD bottles containing desired dilutions, seed

controls, dilution water blanks, and glucose-glutamic acid checks. Water-seal bottles as

described in ¶ 4 f.

j. Determination of final DO: After 5 d incubation determine DO in sample dilutions,

blanks, and checks as in ¶ 4g.

5. Calculation

For each test bottle meeting the 2.0-mg/L minimum DO depletion and the 1.0-mg/L residual

DO, calculate BOD5 as follows:

When dilution water is not seeded:

When dilution water is seeded:

where:

D1 = DO of diluted sample immediately after preparation, mg/L,

D2 = DO of diluted sample after 5 d incubation at 20°C, mg/L,

P = decimal volumetric fraction of sample used,

B1 = DO of seed control before incubation, mg/L (¶ 4d),

B2 = DO of seed control after incubation mg/L (¶ 4d), and

f = ratio of seed in diluted sample to seed in seed control = (% seed in diluted

sample)/(% seed in seed control).

If seed material is added directly to sample or to seed control bottles:

f = (volume of seed in diluted sample)/(volume of seed in seed control)

Report results as CBOD5 if nitrification is inhibited.

If more than one sample dilution meets the criteria of a residual DO of at least 1 mg/L and a

DO depletion of at least 2 mg/L and there is no evidence of toxicity at higher sample

concentrations or the existence of an obvious anomaly, average results in the acceptable range.

In these calculations, do not make corrections for DO uptake by the dilution water blank

during incubation. This correction is unnecessary if dilution water meets the blank criteria

stipulated above. If the dilution water does not meet these criteria, proper corrections are

difficult ; do not record results or, as a minimum, mark them as not meeting quality control

criteria.

6. Precision and Bias

There is no measurement for establishing bias of the BOD procedure. The glucose-glutamic

acid check prescribed in ¶ 4c is intended to be a reference point for evaluation of dilution water

quality, seed effectiveness, and analytical technique. Single-laboratory tests using a 300-mg/L

mixed glucose-glutamic acid solution provided the following results:

Number of months: 14

Number of triplicates: 421

Average monthly recovery: 204 mg/L

Average monthly standard deviation: 10.4 mg/L

In a series of interlaboratory studies,1 each involving 2 to 112 laboratories (and as many

analysts and seed sources), 5-d BOD measurements were made on synthetic water samples

containing a 1:1 mixture of glucose and glutamic acid in the total concentration range of 3.3 to

231 mg/L. The regression equations for mean value, Ä , and standard deviation, S, from these

studies were:

Ä = 0.658 (added level, mg/L) + 0.280 mg/L

S = 0.100 (added level, mg/L) + 0.547 mg/L

For the 300-mg/L mixed primary standard, the average 5-d BOD would be 198 mg/L with a

standard deviation of 30.5 mg/L. When nitrification inhibitors are used, GGA test results falling

outside the 198 ± 30.5 control limit quite often indicate use of incorrect amounts of seed. Adjust

amount of seed added to the GGA test to achieve results falling within this range.

a. Control limits: Because of many factors affecting BOD tests in multilaboratory studies

and the resulting extreme variability in test results, one standard deviation, as determined by

interlaboratory tests, is recommended as a control limit for individual laboratories. Alternatively,

for each laboratory, establish its control limits by performing a minimum of 25 glucose-glutamic

acid checks (¶ 4c) over a period of several weeks or months and calculating the mean and

standard deviation. Use the mean ±3 standard deviations as the control limit for future

glucose-glutamic acid checks. Compare calculated control limits to the single-laboratory tests presented above and to interlaboratory results. If control limits are outside the range of 198 ±

30.5, re-evaluate the control limits and investigate source of the problem. If measured BOD for a

glucose-glutamic acid check is outside the accepted control limit range, reject tests made with

that seed and dilution water.

b. Working range and detection limit: The working range is equal to the difference between

the maximum initial DO (7 to 9 mg/L) and minimum DO residual of 1 mg/L multiplied by the

dilution factor. A lower detection limit of 2 mg/L is established by the requirement for a

minimum DO depletion of 2 mg/L.

5210 C. Ultimate BOD Test (PROPOSED)

1. General Discussion

The ultimate BOD test is an extension of the 5-d dilution BOD test as described in 5210B

but with a number of specific test requirements and differences in application. The user should

be familar with the 5210B procedure before conducting tests for UBOD.

a. Principle: The method consists of placing a single sample dilution in full, airtight bottles

and incubating under specified conditions for an extended period depending on wastewater,

effluent, river, or estuary quality.1 Dissolved oxygen (DO) is measured (with probes) initially

and intermittently during the test. From the DO versus time series, UBOD is calculated by an

appropriate statistical technique. For improved accuracy, run tests in triplicate.

Bottle size and incubation time are flexible to accommodate individual sample characteristics

and laboratory limitations. Incubation temperature, however, is 20°C. Most effluents and some

naturally occurring surface waters contain materials with oxygen demands exceeding the DO

available in air-saturated water. Therefore, it is necessary either to dilute the sample or to

monitor DO frequently to ensure that low DO or anaerobic conditions do not occur. When DO

concentrations approach 2 mg/L, the sample should be reaerated.

Because bacterial growth requires nutrients such as nitrogen, phosphorus, and trace metals,

the necessary amounts may be added to the dilution water together with buffer to ensure that pH population. However, if the result is being used to estimate the rate of oxidation of naturally

occurring surface waters, addition of nutrients and seed probably accelerates the decay rate and

produces misleading results. If only UBOD is desired, it may be advantageous to add

supplemental nutrients that accelerate decay and reduce the test duration. When nutrients are

used, they also should be used in the dilution water blank. Because of the wide range of water

and wastewater characteristics and varied applications of UBOD data, no specific nutrient or

buffer formulations are included.

The extent of oxidation of nitrogenous compounds during the prescribed incubation period

depends on the presence of microorganisms capable of carrying out this oxidation. Such

organisms may not be present in wastewaters in sufficient numbers to oxidize significant

quantities of reduced nitrogen. This situation may be reversed in naturally occurring surface

waters. Erratic results may be obtained when a nitrification inhibitor is used;2 therefore, the

specified method precludes use of a nitrogen inhibitor unless prior experimental evidence on the

particular sample suggests that it is acceptable.*#(99) Monitor NO2–-N and NO3–-N to compute

the oxygen equivalency of the nitrification reaction. When these values are subtracted from the

DO vs. time series, the carbonaceous BOD time series can be constructed.3 b. Sampling and

storage: See Section 5210B.1b.

2. Apparatus

a. Incubation bottles: Glass bottles with ground-glass stoppers,†#(100) 2-L (or larger)

capacity. Glass serum bottles of 4- to 10-L capacity are available. Alternatively use

nonground-glass bottles with nonbiodegradable plastic caps as a plug insert. Do not reuse the

plugs because discoloration occurs with continued use. Replace plugs every 7 to 14 d. Do not use

rubber stoppers that may exert an oxygen demand. Clean bottles with a detergent and wash with

dilute HCl (3N) to remove surface films and precipitated inorganic salts; rinse thoroughly with

DI water before use. Cover top of bottles with paper after rinsing to prevent dust from collecting.

To prevent drawing air into the sample bottle during incubation, use a water seal. If the bottle

does not have a flared mouth, construct a water seal by making a watertight dam around the

stopper (or plug) and fill with water from the reservoir as necessary. Cover dam with clean

aluminum foil to retard evaporation. If a 2-L BOD bottle is used, fill reservoir with sample and

cover with a polyethylene cap before incubation.

Place a clean magnetic stirring bar in each bottle to mix contents before making DO

measurement or taking a subsample. Do not remove the magnets until the test is complete.

Alternatively use a series of 300-mL BOD bottles as described in 5210B, if larger bottles are

not available or incubation space is limited.

b. Reservoir bottle: 4-L or larger glass bottle. Close with screw plastic cap or non-rubber

plug.

c. Incubator or water bath, thermostatically controlled at 20 ± 1°C. Exclude all light to

prevent the possibility of photosynthetic production of DO.

d. Oxygen-sensitive membrane electrode: See Section 4500-O.G.2.

3. Procedure

a. River water samples: Preferably fill large BOD bottle (>2 L, or alternatively 6 or more

300-mL BOD bottles) with sample at 20°C. Add no nutrients, seed, or nitrification inhibitor if

in-bottle decay rates will be used to estimate in-stream rates. Do not dilute sample unless it is

known by pretesting or by experience to have a high ultimate BOD (>20 mg/L).

Measure DO in each bottle, stopper, and make an airtight seal. Incubate at 20°C in the dark.

Measure DO in each bottle at intervals of at least 2 to 5 d over a period of 30 to 60 d

(minimum of 6 to 8 readings) or longer under special circumstances. To avoid oxygen depletion

in samples containing NH3-N, measure DO more frequently until nitrification has taken place. If

DO falls to about 2 mg/L, reaerate as directed below. Replace sample lost by the cap and DO

probe displacement by adding 1 to 2 mL sample from the reservoir bottle.

When DO approaches 2 mg/L, reaerate. Pour a small amount of sample into a clean vessel

and reaerate the remainder directly in the bottle by vigorous shaking or bubbling with purified

air (medical grade). Refill bottle from the storage reservoir and measure DO. This concentration

becomes the initial DO for the next measurement. If using 300-mL BOD bottles, pour all of the

sample from the several bottles used into a clean vessel, reaerate, and refill the small bottles.

Analyze for nitrate plus nitrite nitrogen (NO3–-N + NO2–-N) (see Section 4500-NO2– and

Section 4500-NO3–) on Days 0, 5, 10, 15, 20, and 30. Alternatively, determine NO2–-N and

NO3–-N each time DO is determined, thereby producing corresponding BOD and nitrogen

determinations. If the ultimate demand occurs at a time greater than 30 d, make additional

analyses at 30-d intervals. Remove 10 to 20 mL from the bottle for these analyses. Refill bottle

as necessary from the reservoir bottle. Preserve NO2–-N + NO3–-N subsample with H2SO4 to

pH <2 and refrigerate. If the purpose of the UBOD test is to assess the UBOD and not to provide

data for rate calculations, measure nitrate nitrogen concentration only at Day 0 and on the last

day of the test (kinetic rate estimates are not useful when the nitrification reaction is not

followed).

Calculate oxygen consumption during each time interval and make appropriate corrections

for nitrogenous oxygen demand. Correct by using 3.43 × the NH3-N to NO2–-N conversion plus

1.14 × the NO2–-N to NO3–-N conversion to reflect the stoichiometry of the oxidation of NH4+

to NO2– or NO3–.

When using a dilution water blank, subtract DO uptake of the blank from the total DO

consumed. High-quality reagent water without nutrients typically will consume a maximum of 1

mg DO/L in a 30- to 90-d period. If DO uptake of the dilution water is greater than 0.5 mg/L for

a 20-d period, or 1 mg/L for a 90-d period, report the magnitude of the correction and try to

obtain higher-quality dilution water for use with subsequent UBOD tests.

When the weekly DO consumption drops below 1 to 2% of the total accumulative

consumption, calculate the ultimate BOD using a nonlinear regression method.

b. Wastewater treatment plant samples: Use high-quality reagent water (see Section 1080)

for dilution water. Add no nitrification inhibitors if decay rates are desired. If seed and nutrients

are necessary, add the same amounts of each to the dilution water blank. Use minimal sample

dilution. As a rule of thumb, the ultimate BOD of the diluted sample should be in the range of 20

to 30 mg/L. Dilution to this level probably will require two or three sample reaerations during

the incubation period to avoid having dissolved oxygen concentrations fall below 2 mg/L.

Use 2-L or larger BOD bottles (alternatively, multiple 300-mL BOD bottles) for each

dilution. Add desired volume of sample to each bottle and fill with dilution water.

Fill a BOD bottle with dilution water to serve as a dilution water blank. Treat blank the same

as all samples. Follow procedure given in ¶ 3a) and incubate for at least as long as UBOD test.

4. Calculations

An example of results obtained for a wastewater sample, undiluted, without seed and

nutrients, is given in Table 5210:I.

UBOD can be estimated by using a first-order model described as follows:

BODt = UBOD (1 − e–kt)

where:

BODt = oxygen uptake measured at time t, mg/L, and

k = first-order oxygen uptake rate.

The data in Table 5210:I were analyzed with a nonlinear regression technique applied to the

above first-order model.4 However, a first-order kinetic model may not always be the best

choice. Significantly better statistical fits usually are obtained with alternative kinetic models

including sum of two first-order and logistic function models.1,3-8

5. Precision and Bias

The precision of the ultimate BOD test was assessed with a series of replicate tests in a single

laboratory. Interlaboratory studies have not been conducted.

Reference         Replicate No.      UBOD mg/L            Precision Summary*

2                          1                    154                     μ = 151 mg/L

2                    154

3                    145                     CV = 3.5%

5                          1                      10.3

2                       11.1

3                         9.6                   μ = 10.0 mg/L

4                         9.9

5                          9.8                  CV = 5.8%

6                          9.6

6                          1                         12.8                  μ = 12.4 mg/L

2                         12.6

3                         12.6                  CV = 4.4%

4                         11.6

*μ = mean, CV = coefficient of variation.

Bias was assessed by determining the BOD of a known concentration of glucose (150 mg/L)

and glutamic acid (150 mg/L). This solution has a UBOD of 321 mg/L to 308 mg/L, depending

on extent of nitrification. The results of the study conducted in triplicate were:1

Estimated* UBOD                     Theoretical BOD                          Percent Difference

mg/L                                         mg/L

276                               308/321                                     −10/−14

310                               308/321                                       +1/−3

303                               308/321                                        −2/−6

*By statistical model.

5210 D. Respirometric Method (PROPOSED)

1. General Discussion

a. Principle: Respirometric methods provide direct measurement of the oxygen consumed by

microorganisms from an air or oxygen-enriched environment in a closed vessel under conditions

of constant temperature and agitation.

b. Uses: Respirometry measures oxygen uptake more or less continuously over time.

Respirometric methods are useful for assessing: biodegradation of specific chemicals; treatability

of organic industrial wastes; the effect of known amounts of toxic compounds on the

oxygen-uptake reaction of a test wastewater or organic chemical; the concentration at which a

pollutant or a wastewater measurably inhibits biological degradation; the effect of various

treatments such as disinfection, nutrient addition, and pH adjustment on oxidation rates; the

oxygen requirement for essentially complete oxidation of biologically oxidizable matter; the

need for using adapted seed in other biochemical oxygen-uptake measurements, such as the

dilution BOD test; and stability of sludges.

Respirometric data typically will be used comparatively, that is, in a direct comparison

between oxygen uptakes from two test samples or from a test sample and a control. Because of

inherent differences among uses, among seed cultures, among applications of results, and among

instruments, a single procedure for respirometric tests applicable to all cases cannot be defined.

Therefore, only basic recommendations and guidelines for overall test setup and procedure are

given. Follow manufacturer’s instructions for operating details for specific commercial

instruments.

c. Types of respirometers: Four principal types of commercial respirometers are available.

Manometric respirometers relate oxygen uptake to the change in pressure caused by oxygen consumption while maintaining a constant volume. Volumetric respirometers measure oxygen

uptake in incremental changes in gas volume while maintaining a constant pressure at the time of

reading. Electrolytic respirometers monitor the amount of oxygen produced by electrolysis of

water to maintain a constant oxygen pressure within the reaction vessel. Direct-input

respirometers deliver oxygen to the sample from a pure oxygen supply through metering on

demand as detected by minute pressure differences. Most respirometers have been instrumented

to permit data collection and processing by computer. Reaction-vessel contents are mixed by

using a magnetic or mechanical stirring device or by bubbling the gaseous phase within the

reaction vessel through the liquid phase. All respirometers remove carbon dioxide produced

during biological growth by suspending a concentrated adsorbent (granular or solution) within

the closed reaction chamber or by recirculating the gas phase through an external scrubber.

d. Interferences: Evolution of gases other than CO2 may introduce errors in pressure or

volume measurements; this is uncommon in the presence of dissolved oxygen. Incomplete CO2

absorption will introduce errors if appropriate amounts and concentrations of alkaline absorbent

are not used. Temperature fluctuations or inadequate mixing will introduce error. Fluctuations in

barometric pressure can cause errors with some respirometers. Become familiar with the limits

of the instrument to be used.

e. Minimum detectable concentration: Most commercial respirometers can detect oxygen

demand in increments as small as 0.1 mg but test precision depends on the total amount of

oxygen consumed at the time of reading, the precision of pressure or volume measurement, and

the effect of temperature and barometric pressure changes. Upper limits of oxygen uptake rate

are determined by the ability to transfer oxygen into the solution from the gas phase, which

typically is related to mixing intensity. Transfer limits typically range from less than 10 mg

O2/L/h for low-intensity mixing to above 100 mg O2/L/h for high-intensity mixing.

f. Relationship to dilution BOD: Variations in waste composition, substrate concentration,

mixing, and oxygen concentrations from one wastewater source to another generally preclude

use of a general relationship between oxygen uptake by respirometers and the 5-d, 20°C, BOD

(see 5210B, above). Reasonably accurate correlations may be possible for a specific wastewater.

The incubation period for respirometric measurements need not be 5 d because equally valid

correlations can be made between the 5-d dilution BOD and respirometric oxygen uptake at any

time after 2 d.1,2 The point of common dilution and respirometric BOD seems to occur at about 2

to 3 d incubation for municipal wastewaters. Correlations between respirometric measurements

and 5-d BOD for industrial wastes and specific chemicals are less certain. Respirometric

measurements also can provide an indication of the ultimate biochemical oxygen demand

(UBOD) (see Section 5210C). In many cases, it is reasonable to consider that the 28-to 30-d

oxygen uptake is essentially equal to the UBOD.3

More commonly, respirometers are used as a diagnostic tool. The continuous readout of

oxygen consumption in respirometric measurements indicates lag, toxicity, or any abnormalities

in the biodegradation reaction. The change in the normal shape of an oxygen-uptake curve in the first few hours may help to identify the effect of toxic or unusual wastes entering a treatment

plant in time to make operating corrections.

g. Relationship to other test methods and protocols: This method supports most of the

protocols and guidelines established by the European Organization for Economic Co-operation

and Development3 (OECD) that require measurement of oxygen uptake.

h. Sampling and storage:

1) Grab samples—If analysis is begun within 2 h of sample collection, cold storage is

unnecessary. Otherwise, keep sample at or below 4°C from the time of collection. Begin analysis

within 6 h of collection; when this is not possible, store at or below 4°C and report length and

temperature of storage. Never start analysis more than 24 h after grab sample collection.

2) Composite samples—Keep samples at or below 4°C during compositing. Limit

compositing period to 24 h. Use the same criteria as for storage of grab samples, starting the

measurement of holding time from the end of the compositing period. State storage time and

conditions with results.

2. Apparatus

a. Respirometer system: Use commercial apparatus and check manufacturer’s instructions for

specific system requirements, reaction vessel type and volume, and instrument operating

characteristics.

b. Incubator or water bath: Use a constant-temperature room, incubator chamber, or water

bath to control temperature to ±1°C. Exclude all light to prevent oxygen formation by algae in

the sample. Use red, actinic-coated bottles for analysis outside of a darkened incubator.

3. Reagents

Formulations of reagent solutions are given for 1-L volumes, but smaller or larger volumes

may be prepared according to need. Discard any reagent showing signs of biological growth or

chemical precipitation. Stock solutions can be sterilized by autoclaving to provide longer shelf

life.

a. Distilled water: Use only high-quality water distilled from a block tin or all-glass still (see

Section 1080). Deionized water may be used but often contains high bacterial counts. The water

must contain less than 0.01 mg heavy metals/L and be free of chlorine, chloramines, caustic

alkalinity, organic material, or acids. Make all reagents with this water. When other waters are

required for special-purpose testing, state clearly their source and quality characteristics.

b. Phosphate buffer solution, 1.5N: Dissolve 207 g sodium dihydrogen phosphate,

NaH2PO4⋅H2O, in water. Neutralize to pH 7.2 with 6N KOH (¶ 3g below) and dilute to 1 L.

c. Ammonium chloride solution, 0.71N: Dissolve 38.2 g ammonium chloride, NH4Cl, in

water. Neutralize to pH 7.0 with KOH. Dilute to 1.0 L; 1 mL = 10 mg N.

d. Calcium chloride solution, 0.25N: Dissolve 27.7 g CaCl2 in water and dilute to 1 L; 1 mL

= 10 mg Ca.

e. Magnesium sulfate solution, 0.41N: Dissolve 101 g MgSO4⋅7H2O in water and dilute to 1

L; 1 mL = 10 mg Mg.

f. Ferric chloride solution, 0.018N: Dissolve 4.84 g FeCl3⋅6H2O in water and dilute to 1 L; 1

mL = 1.0 mg Fe.

g. Potassium hydroxide solution, 6N: Dissolve 336 g KOH in about 700 mL water and dilute

to 1 L. CAUTION: Add KOH to water slowly and use constant mixing to prevent excessive heat

buildup. Alternately, use commercial solutions containing 30 to 50% KOH by weight.

h. Acid solutions, 1N: Add 28 mL conc H2SO4 or 83 mL conc HCl to about 700 mL water.

Dilute to 1 L.

i. Alkali solution, 1N: Add 40 g NaOH to 700 mL water. Dilute to 1 L.

j. Nitrification inhibitor: Reagent-grade 2-chloro-6-(trichloromethyl) pyridine (TCMP) or

equivalent.3*#(101)

k. Glucose-glutamic acid solution: Dry reagent-grade glucose and reagent-grade glutamic

acid at 103°C for 1 h. Add 15.0 g glucose and 15.0 g glutamic acid to distilled water and dilute

to 1 L. Neutralize to pH 7.0 using 6N potassium hydroxide (¶ 3g). This solution may be stored

for up to 1 week at 4°C.

l. Electrolyte solution (for electrolytic respirometers): Use manufacturer’s recommended

solution.

m. Sodium sulfite solution, 0.025N: Dissolve 1.575 g Na2SO3 in about 800 mL water. Dilute

to 1 L. This solution is not stable; prepare daily or as needed.

n. Trace element solution: Dissolve 40 mg MnSO4⋅4H2O, 57 mg H3BO3, 43 mg

ZnSO4⋅7H2O, 35 mg (NH4)6 Mo7O24, and 100 mg Fe-chelate (FeCl3-EDTA) in about 800 mL

water. Dilute to 1 L. Sterilize at 120°C and 200 kPa (2 atm) pressure for 20 min.

o. Yeast extract solution:3 Add 15 mg laboratory- or pharmaceutical-grade brewer’s yeast

extract to 100 mL water. Make this solution fresh immediately before each test in which it is

used.

p. Nutrient solution:3 Add 2.5 mL phosphate buffer solution (3b), 0.65 mL ammonium

chloride solution (3c), 1.0 mL calcium chloride solution (3d), 0.22 mL magnesium sulfate

solution (3a), 0.1 mL ferric chloride solution (3 f ), 1 mL trace element solution (3n), and 1 mL

yeast extract solution (3o) to about 900 mL water. Dilute to 1 L. This nutrient solution and those

of ¶s n and o above are specifically formulated for use with the OECD method.3 (NOTE: A 10:1

concentrated nutrient solution can be made and diluted accordingly.)

4. Procedure

a. Instrument operation: Follow respirometer manufacturer’s instructions for assembly,

testing, calibration, and operation of the instrument. NOTE: The manufacturer’s stated maximum and minimum limits of measurement are not always the same as the instrument output limits.

Make sure that test conditions are within the limits of measurement.

b. Sample volume: Sample volume or concentration of organic chemicals to be added to test

vessels is a function of expected oxygen uptake characteristics and oxygen transfer capability of

the instrument. Small volumes or low concentrations may be required for high-strength wastes.

Large volumes may be required for low-strength wastes to improve accuracy.

c. Data recording interval: Set instrument to give data readings at suitable intervals.

Intervals of 15 min to 6 h typically are used.

d. Sample preparation:

1) Homogenization—If sample contains large settleable or floatable solids, homogenize it

with a blender and transfer representative test portions while all solids are in suspension. If there

is a concern for changing sample characteristics, skip this step.

2) pH adjustment—Neutralize samples to pH 7.0 with H2SO4 or NaOH of such strength (¶s

3h and ¶ 3i) that reagent quantity does not dilute the sample more than 0.5%.

3) Dechlorination—Avoid analyzing samples containing residual chlorine by collecting the

samples ahead of chlorination processes. If residual chlorine is present, aerate as described in ¶

5) below or let stand in light for 1 to 2 h. If a chlorine residual persists, add Na2SO3 solution.

Determine required volume of Na2SO3 solution by adding 10 mL 1 + 1 acetic acid or 1 + 50

H2SO4 and 10 mL potassium iodide solution (10 g/100 mL) to a portion of the sample. Titrate

with 0.025N Na2SO3 solution to the starch-iodine end point (see Section 4500-Cl.B). Add to the

neutralized sample a proportional volume of Na2SO3 solution determined above, mix, and after

10 to 20 min check for residual chlorine. Re-seed the sample (see ¶ 4h below).

4) Samples containing toxic substances—Certain industrial wastes contain toxic metals or

organic compounds. These often require special study and treatment.3

5) Initial oxygen concentration—If samples contain dissolved oxygen concentrations above

or below the desired concentration, agitate or aerate with clean and filtered compressed air for

about 1 h immediately before testing. Minimum and maximum actual DO concentrations will

vary with test objectives. In some cases, pure oxygen may be added to respirometer vessels to

increase oxygen levels above ambient.

6) Temperature adjustment—Bring samples and dilution water to desired test temperature

(±1°C) before making dilutions or transferring to test vessels.

e. Sample dilution: Use distilled water or water from other appropriate sources free of

organic matter. In some cases, receiving stream water may be used for dilution. Add desired

sample volume to test vessels using a wide-tip volumetric pipet or other suitable volumetric

glassware. Add dilution water to bring sample to about 80% of desired final volume. Add

appropriate amounts of nutrients, minerals, buffer, nitrification inhibitor if desired, and seed

culture as described in ¶s 4 f and h below. Dilute sample to desired final volume. The number of test vessels to prepare for each dilution depends on test objectives and number of replicates

desired.

f. Nutrients, minerals, and buffer: Add sufficient ammonia nitrogen to provide a COD:N:P

ratio of 100:5:1 or a TOC:N:P ratio of 30:5:1. Add 2 mL each of calcium, magnesium, ferric

chloride, and trace mineral solutions to each liter of diluted sample unless sufficient amounts of

these minerals are present in the original sample. Phosphorus requirements will be met by the

phosphate buffer if it is used (1 mL/50 mg/L COD or ultimate BOD of diluted sample usually is

sufficient to maintain pH between 6.8 and 7.2). Be cautious in adding phosphate buffer to

samples containing metal salts because metal phosphates may precipitate and show less toxic or

beneficial effect than when phosphate is not present. For OECD-compatible tests, substitute the

nutrient, mineral, and buffer amounts listed in ¶ 3 p for the above nutrient/ mineral/buffer

quantities.

g. Nitrification inhibition: If nitrification inhibition is desired, add 10 mg

2-chloro-6-(trichloromethyl) pyridine (TCMP)/L sample in the test vessel. Samples that may

nitrify readily include biologically treated effluents, samples seeded with biologically treated

effluents, and river waters.4

h. Seeding: See 5210B.4d1) for seed preparation. Use sufficient amounts of seed culture to

prevent major lags in the oxygen uptake reaction but not so much that the oxygen uptake of the

seed exceeds about 10% of the oxygen uptake of the seeded sample.

Determine the oxygen uptake of the seeding material as for any other sample. This is the

seed control. Typically, the seed volume in the seed control should be 10 times the volume used

in seeded samples.

i. Incubation: Incubate samples at 20°C or other suitable temperature ±1.0°C. Take care that

the stirring device does not increase the temperature of the sample.

5. Calculations

To convert instrument readings to oxygen uptake, refer to manufacturer’s procedures.

Correct oxygen uptake for seed and dilution by the following equation:

C = [A − B(SA/SB)](1000/NA)

where:

C = corrected oxygen uptake of sample, mg/L,

A = measured oxygen uptake in seeded sample, mg,

B = measured oxygen uptake in seed control, mg,

SA = volume of seed in Sample A, mL,

SB = volume of seed in Sample B, mL, and

NA = volume of undiluted sample in Sample A, mL.

6. Quality Control

Periodically use the following procedure to check distilled water quality, instrument quality,

instrument function, and analytical technique by making oxygen uptake measurements using a

mixture of glucose and glutamic acid as a standard check solution.

Adjust water for sample formulation to test temperature and saturate with DO by aerating

with clean, organic-free filtered air. Protect water quality by using clean glassware, tubing, and

bottles.

Prepare a test solution by adding 10 mL glucose-glutamic acid solution (¶ 3k); 6 mL

phosphate buffer (¶ 3b); 2 mL each of ammonium chloride (¶ 3c), magnesium sulfate (¶ 3e),

calcium chloride (¶ 3d), ferric chloride (¶ 3 f), and trace element solution (¶ 3n) to approximately

800 mL water. Add 10 mg nitrification inhibitor (TCMP)/L. Add sufficient seed from a suitable

source as described in ¶ 4h to give a lag time less than 6 h (usually 25 mL supernatant from

settled primary effluent/L test solution is sufficient). Dilute to 1 L. Adjust temperature to 20 ±

1°C.

Prepare a seed blank by diluting 500 mL or more of the seed solution to 800 mL with

distilled water. Add the same amount of buffer, nutrients, and TCMP as in the test solution, and

dilute to 1 L. Adjust temperature to 20 ± 1°C.

Place test solution and seed blank solution in separate reaction vessels of respirometer and

incubate for 5 d at 20°C. Run at least three replicates of each. The seed-corrected oxygen uptake

after 5 d incubation should be 260 ± 30 mg/L. If the value of the check is outside this range,

repeat the test using a fresh seed culture and seek the cause of the problem.

7. Precision and Bias

a. Precision: No standard is available to check the accuracy of respirometric oxygen uptake

measurements. To obtain laboratory precision data, use a glucose-glutamic acid mixture (¶ 6

above) having a known theoretical maximum oxygen uptake value. Tests with this and similar

organic compound mixtures have shown that the standard deviation, expressed as the coefficient

of variation, Cv, is approximately 5% for samples having total oxygen uptakes of 50 to 100 mg/L

and 3% for more concentrated samples.1,2 Individual instruments have different readability

limits that can affect precision. The minimum response or sensitivity of most commercial

respirometers ranges from 0.05 to 1 mg oxygen. Check manufacturer’s specifications for

sensitivity of the instrument at hand.

b. Control limits: To establish laboratory control limits, perform a minimum of 25

glucose-glutamic acid checks over a period of several weeks or months and calculate mean and

standard deviation. If measured oxygen uptake in 5 d at 20°C is outside the 260 ± 30 mg/L

range, re-evaluate procedure to identify source of error. For other samples, use the mean ±3

standard deviations as the control limit.

c. Working range and detection limits: The working range and detection limits are

established by the limits of each commercial instrument. Refer to manufacturer’s specifications.

5220 CHEMICAL OXYGEN DEMAND (COD)*#(102)

5220 A. Introduction

Chemical oxygen demand (COD) is defined as the amount of a specified oxidant that reacts

with the sample under controlled conditions. The quantity of oxidant consumed is expressed in

terms of its oxygen equivalence. Because of its unique chemical properties, the dichromate ion

(Cr2O7 2–) is the specified oxidant in Methods Section 5220B, Section 5220C, and Section

5220D; it is reduced to the chromic ion (Cr3+) in these tests. Both organic and inorganic

components of a sample are subject to oxidation, but in most cases the organic component

predominates and is of the greater interest. If it is desired to measure either organic or inorganic

COD alone, additional steps not described here must be taken to distinguish one from the other. COD is a defined test; the extent of sample oxidation can be affected by digestion time, reagent

strength, and sample COD concentration.

COD often is used as a measurement of pollutants in wastewater and natural waters. Other

related analytical values are biochemical oxygen demand (BOD), total organic carbon (TOC),

and total oxygen demand (TOD). In many cases it is possible to correlate two or more of these

values for a given sample. BOD is a measure of oxygen consumed by microorganisms under

specific conditions; TOC is a measure of organic carbon in a sample; TOD is a measure of the

amount of oxygen consumed by all elements in a sample when complete (total) oxidation is

achieved.

In a COD analysis, hazardous wastes of mercury, hexavalent chromium, sulfuric acid, silver,

and acids are generated. Methods Section 5220C and Section 5220D reduce these waste

problems but may be less accurate and less representative. (See ¶ 2 below.)

1. Selection of Method

The open reflux method (B) is suitable for a wide range of wastes where a large sample size

is preferred. The closed reflux methods (C and D) are more economical in the use of metallic salt

reagents and generate smaller quantities of hazardous waste, but require homogenization of

samples containing suspended solids to obtain reproducible results. Ampules and culture tubes

with premeasured reagents are available commercially. Measurements of sample volumes as

well as reagent volumes and concentrations are critical. Consequently, obtain specifications as to

limits of error for premixed reagents from manufacturer before use.

Determine COD values of >50 mg O2/L by using procedures Section 5220B.4a, Section

5220C.4, or Section 5220D.4. Use procedure Section 5220B.4b to determine, with lesser

accuracy, COD values from 5 to 50 mg O2/L.

2. Interferences and Limitations

Oxidation of most organic compounds is 95 to 100% of the theoretical value. Pyridine and

related compounds resist oxidation and volatile organic compounds will react in proportion to

their contact with the oxidant. Straight-chain aliphatic compounds are oxidized more effectively

in the presence of a silver sulfate catalyst.

The most common interferent is the chloride ion. Chloride reacts with silver ion to

precipitate silver chloride, and thus inhibits the catalytic activity of silver. Bromide, iodide, and

any other reagent that inactivates the silver ion can interfere similarly. Such interferences are

negative in that they tend to restrict the oxidizing action of the dichromate ion itself. However,

under the rigorous digestion procedures for COD analyses, chloride, bromide, or iodide can react

with dichromate to produce the elemental form of the halogen and the chromic ion. Results then

are in error on the high side. The difficulties caused by the presence of the chloride can be

overcome largely, though not completely, by complexing with mercuric sulfate (HgSO4) before

the refluxing procedure. Although 1 g HgSO4 is specified for 50 mL sample, a lesser amount

may be used where sample chloride concentration is known to be less than 2000 mg/L, as long as a 10:1 weight ratio of HgSO4:Cl– is maintained. Do not use the test for samples containing more

than 2000 mg Cl–/L. Techniques designed to measure COD in saline waters are available.1,2

Halide interferences may be removed by precipitation with silver ion and filtration before

digestion. This approach may introduce substantial errors due to the occlusion and carrydown of

COD matter from heterogenous samples.

Ammonia and its derivatives, in the waste or generated from nitrogen-containing organic

matter, are not oxidized. However, elemental chlorine reacts with these compounds. Hence,

corrections for chloride interferences are difficult.

Nitrite (NO2–) exerts a COD of 1.1 mg O2/mg NO2–-N. Because concentrations of NO2– in

waters rarely exceed 1 or 2 mg NO2–-N/L, the interference is considered insignificant and

usually is ignored. To eliminate a significant interference due to NO2–, add 10 mg sulfamic acid

for each mg NO2–-N present in the sample volume used; add the same amount of sulfamic acid

to the reflux vessel containing the distilled water blank.

Reduced inorganic species such as ferrous iron, sulfide, manganous manganese, etc., are

oxidized quantitatively under the test conditions. For samples containing significant levels of

these species, stoichiometric oxidation can be assumed from known initial concentration of the

interfering species and corrections can be made to the COD value obtained.

The silver, hexavalent chromium, and mercury salts used in the COD determinations create

hazardous wastes. The greatest problem is in the use of mercury. If the chloride contribution to

COD is negligible, HgSO4 can be omitted. Smaller sample sizes (see Section 5220C and Section

5220D) reduce the waste. Recovery of the waste material may be feasible if allowed by

regulatory authority.3

3. Sampling and Storage

Preferably collect samples in glass bottles. Test unstable samples without delay. If delay

before analysis is unavoidable, preserve sample by acidification to pH ≤ 2 using conc H2SO4.

Blend (homogenize) all samples containing suspended solids before analysis. If COD is to be

related to BOD, TOC, etc., ensure that all tests receive identical pretreatment. Make preliminary

dilutions for wastes containing a high COD to reduce the error inherent in measuring small

sample volumes.

5220 B. Open Reflux Method

1. General Discussion

a. Principle: Most types of organic matter are oxidized by a boiling mixture of chromic and

sulfuric acids. A sample is refluxed in strongly acid solution with a known excess of potassium

dichromate (K2Cr2O7). After digestion, the remaining unreduced K2Cr2O7 is titrated with

ferrous ammonium sulfate to determine the amount of K2Cr2O7 consumed and the oxidizable

matter is calculated in terms of oxygen equivalent. Keep ratios of reagent weights, volumes, and

strengths constant when sample volumes other than 50 mL are used. The standard 2-h reflux

time may be reduced if it has been shown that a shorter period yields the same results. Some

samples with very low COD or with highly heterogeneous solids content may need to be

analyzed in replicate to yield the most reliable data. Results are further enhanced by reacting a

maximum quantity of dichromate, provided that some residual dichromate remains.

2. Apparatus

a. Reflux apparatus, consisting of 500- or 250-mL erlenmeyer flasks with ground-glass

24/40 neck and 300-mm jacket Liebig, West, or equivalent condenser with 24/40 ground-glass

joint, and a hot plate having sufficient power to produce at least 1.4 W/cm2 of heating surface, or

equivalent.

b. Blender.

c. Pipets, Class A and wide-bore.

3. Reagents

a. Standard potassium dichromate solution, 0.04167M: Dissolve 12.259 g K2Cr2O7,

primary standard grade, previously dried at 150°C for 2 h, in distilled water and dilute to 1000

mL. This reagent undergoes a six-electron reduction reaction; the equivalent concentration is 6 ×

0.04167M or 0.2500N.

b. Sulfuric acid reagent: Add Ag2SO4, reagent or technical grade, crystals or powder, to

conc H2SO4 at the rate of 5.5 g Ag2SO4/kg H2SO4. Let stand 1 to 2 d to dissolve. Mix.

c. Ferroin indicator solution: Dissolve 1.485 g 1,10-phenanthroline monohydrate and 695

mg FeSO4⋅7H2O in distilled water and dilute to 100 mL. This indicator solution may be

purchased already prepared.*#(103)

- Standard ferrous ammonium sulfate (FAS) titrant, approximately 0.25M: Dissolve 98 g

Fe(NH4)2(SO4)2⋅6H2O in distilled water. Add 20 mL conc H2SO4, cool, and dilute to 1000 mL.

Standardize this solution daily against standard K2Cr2O7 solution as follows:

Dilute 25.00 mL standard K2Cr2O7 to about 100 mL. Add 30 mL conc H2SO4 and cool.

Titrate with FAS titrant using 0.10 to 0.15 mL (2 to 3 drops) ferroin indicator.

Molarity of FAS solution

e. Mercuric sulfate, HgSO4, crystals or powder.

f. Sulfamic acid: Required only if the interference of nitrites is to be eliminated (see Section

5220A.2 above).

g. Potassium hydrogen phthalate (KHP) standard, HOOCC6H4COOK: Lightly crush and

then dry KHP to constant weight at 110°C. Dissolve 425 mg in distilled water and dilute to 1000

mL. KHP has a theoretical COD1 of 1.176 mg O2/mg and this solution has a theoretical COD of

500 μg O2/ mL. This solution is stable when refrigerated, but not indefinitely. Be alert to

development of visible biological growth. If practical, prepare and transfer solution under sterile

conditions. Weekly preparation usually is satisfactory.

4. Procedure

a. Treatment of samples with COD of >50 mg O2/L: Blend sample if necessary and pipet

50.00 mL into a 500-mL refluxing flask. For samples with a COD of >900 mg O2/L, use a

smaller portion diluted to 50.00 mL. Add 1 g HgSO4, several glass beads, and very slowly add

5.0 mL sulfuric acid reagent, with mixing to dissolve HgSO4. Cool while mixing to avoid

possible loss of volatile materials. Add 25.00 mL 0.04167M K2Cr2O7 solution and mix. Attach

flask to condenser and turn on cooling water. Add remaining sulfuric acid reagent (70 mL)

through open end of condenser. Continue swirling and mixing while adding sulfuric acid

reagent. CAUTION: Mix reflux mixture thoroughly before applying heat to prevent local heating

of flask bottom and a possible blowout of flask contents.

Cover open end of condenser with a small beaker to prevent foreign material from entering

refluxing mixture and reflux for 2 h. Cool and wash down condenser with distilled water.

Disconnect reflux condenser and dilute mixture to about twice its volume with distilled water.

Cool to room temperature and titrate excess K2Cr2O7 with FAS, using 0.10 to 0.15 mL (2 to 3

drops) ferroin indicator. Although the quantity of ferroin indicator is not critical, use the same volume for all titrations. Take as the end point of the titration the first sharp color change from

blue-green to reddish brown that persists for 1 min or longer. Duplicate determinations should

agree within 5% of their average. Samples with suspended solids or components that are slow to

oxidize may require additional determinations. The blue-green may reappear. In the same

manner, reflux and titrate a blank containing the reagents and a volume of distilled water equal

to that of sample.

b. Alternate procedure for low-COD samples: Follow procedure of ¶ 4a, with two

exceptions: (i) use standard 0.004167M K2Cr2O7, and (ii) titrate with standardized 0.025M FAS.

Exercise extreme care with this procedure because even a trace of organic matter on the

glassware or from the atmosphere may cause gross errors. If a further increase in sensitivity is

required, concentrate a larger volume of sample before digesting under reflux as follows: Add all

reagents to a sample larger than 50 mL and reduce total volume to 150 mL by boiling in the

refluxing flask open to the atmosphere without the condenser attached. Compute amount of

HgSO4 to be added (before concentration) on the basis of a weight ratio of 10:1, HgSO4:Cl–,

using the amount of Cl– present in the original volume of sample. Carry a blank reagent through

the same procedure. This technique has the advantage of concentrating the sample without

significant losses of easily digested volatile materials. Hard-to-digest volatile materials such as

volatile acids are lost, but an improvement is gained over ordinary evaporative concentration

methods. Duplicate determinations are not expected to be as precise as in 5220B.4a.

c. Determination of standard solution: Evaluate the technique and quality of reagents by

conducting the test on a standard potassium hydrogen phthalate solution.

5. Calculation

where:

A = mL FAS used for blank,

B = mL FAS used for sample,

M = molarity of FAS, and

8000 = milliequivalent weight of oxygen × 1000 mL/L.

6. Precision and Bias

A set of synthetic samples containing potassium hydrogen phthalate and NaCl was tested by

74 laboratories. At a COD of 200 mg O2/L in the absence of chloride, the standard deviation was

±13 mg/L (coefficient of variation, 6.5%). At COD of 160 mg O2/L and 100 mg Cl–/L, the

standard deviation was ±14 mg/L (coefficient of variation, 10.8%).

5220 C. Closed Reflux, Titrimetric Method

1. General Discussion

a. Principle: See Section 5220B.1a.

b. Interferences and limitations: See Section 5220A.2. Volatile organic compounds are more

completely oxidized in the closed system because of longer contact with the oxidant. Before

each use inspect culture-tube caps for breaks in the TFE liner. Select culture-tube size according

to block heater capacity and degree of sensitivity desired. Use the 25- × 150-mm tube for

samples with low COD content because a larger volume sample can be treated.

This procedure is applicable to COD values between 40 and 400 mg/L. Obtain higher values

by dilution. Alternatively, use higher concentrations of dichromate digestion solution to

determine greater COD values. COD values of 100 mg/L or less can be obtained by using a more

dilute dichromate digestion solution or a more dilute FAS titrant. Overall accuracy can be

improved by using an FAS titrant which is less than the 0.10M solution specified below. Higher

dichromate concentrations or reduced FAS concentrations probably require titrations to be done

in a separate vessel, rather than in the digestion vessel, because of the volumes of titrant

required.

2. Apparatus

a. Digestion vessels: Preferably use borosilicate culture tubes, 16- × 100-mm, 20- ×

150-mm, or 25- × 150-mm, with TFE-lined screw caps. Alternatively, use borosilicate ampules,

10-mL capacity, 19- to 20-mm diam.

Digestion vessels with premixed reagents and other accessories are available from

commercial suppliers. Contact supplier for specifications.*#(104)

b. Block heater or similar device to operate at 150 ± 2°C, with holes to accommodate

digestion vessels. Use of culture tubes probably requires the caps to be outside the vessel to protect caps from heat. CAUTION: Do not use an oven because of the possibility of leaking

samples generating a corrosive and possibly explosive atmosphere. Also, culture tube caps may

not withstand the 150°C temperature in an oven.

c. Microburet.

d. Ampule sealer: Use only a mechanical sealer to insure strong, consistent seals.

3. Reagents

a. Standard potassium dichromate digestion solution, 0.01667M: Add to about 500 mL

distilled water 4.903 g K2Cr2O7, primary standard grade, previously dried at 150°C for 2 h, 167

mL conc H2SO4, and 33.3 g HgSO4. Dissolve, cool to room temperature, and dilute to 1000 mL.

b. Sulfuric acid reagent: See Section 5220B.3b.

c. Ferroin indicator solution: See Section 5220B.3c. Dilute this reagent by a factor of 5 (1 +

4).

d. Standard ferrous ammonium sulfate titrant (FAS), approximately 0.10M: Dissolve 39.2 g

Fe(NH4)2(SO4)2⋅6H2O in distilled water. Add 20 mL conc H2SO4, cool, and dilute to 1000 mL.

Standardize solution daily against standard K2Cr2O7 digestion solution as follows:

Pipet 5.00 mL digestion solution into a small beaker. Add 10 mL reagent water to substitute

for sample. Cool to room temperature. Add 1 to 2 drops diluted ferroin indicator and titrate with

FAS titrant.

Molarity of FAS solution

e. Sulfamic acid: See Section 5220B.3 f.

f. Potassium hydrogen phthalate standard: See Section 5220B.3g.

4. Procedure

Wash culture tubes and caps with 20% H2SO4 before first use to prevent contamination.

Refer to Table 5220:I for proper sample and reagent volumes. Make volumetric measurements as

accurate as practical; use Class A volumetric ware. The most critical volumes are of the sample

and digestion solution. Use a microburet for titrations. Measure H2SO4 to ±0.1 mL. The use of

hand-held pipettors with non-wetting (polyethylene) pipet tips is practical and adequate. Place

sample in culture tube or ampule and add digestion solution. Carefully run sulfuric acid reagent

down inside of vessel so an acid layer is formed under the sample-digestion solution layer.

Tightly cap tubes or seal ampules, and invert each several times to mix completely. CAUTION:

Wear face shield and protect hands from heat produced when contents of vessels are mixed. Mix

thoroughly before applying heat to prevent local heating of vessel bottom and possible explosive

reaction.

Place tubes or ampules in block digester preheated to 150°C and reflux for 2 h behind a

protective shield. CAUTION: These sealed vessels may be under pressure from gases generated

during digestion. Wear face and hand protection when handling. If sulfuric acid is omitted or

reduced in concentration, very high and dangerous pressures will be generated at 150°C. Cool

to room temperature and place vessels in test tube rack. Some mercuric sulfate may precipitate

out but this will not affect the analysis. Remove culture tube caps and add small TFE-covered

magnetic stirring bar. If ampules are used, transfer contents to a larger container for titrating.

Add 0.05 to 0.10 mL (1 to 2 drops) ferroin indicator and stir rapidly on magnetic stirrer while

titrating with standardized 0.10M FAS. The end point is a sharp color change from blue-green to

reddish brown, although the blue-green may reappear within minutes. In the same manner reflux

and titrate a blank containing the reagents and a volume of distilled water equal to that of the

sample.

5. Calculation

where:

A = mL FAS used for blank,

B = mL FAS used for sample,

M = molarity of FAS, and

8000 = milliequivalent weight of oxygen × 1000 mL/L.

Preferably analyze samples in duplicate because of small sample size. Samples that are

inhomogeneous may require multiple determinations for accurate analysis. Results should agree

within ±5% of their average unless the condition of the sample dictates otherwise.

6. Precision and Bias

Sixty synthetic samples containing potassium hydrogen phthalate and NaCl were tested by

six laboratories. At an average COD of 195 mg O2/L in the absence of chloride, the standard

deviation was ±11 mg O2/L (coefficient of variation, 5.6%). At an average COD of 208 mg O2/L

and 100 mg Cl–/L, the standard deviation was ±10 mg O2/L (coefficient of variation, 4.8%).

5220 D. Closed Reflux, Colorimetric Method

- General Discussion

a. Principle: See Section 5220B.1a. When a sample is digested, the dichromate ion oxidizes

COD material in the sample. This results in the change of chromium from the hexavalent (VI)

state to the trivalent (III) state. Both of these chromium species are colored and absorb in the

visible region of the spectrum. The dichromate ion (Cr2O7 2–) absorbs strongly in the 400-nm

region, where the chromic ion (Cr3+) absorption is much less. The chromic ion absorbs strongly

in the 600-nm region, where the dichromate has nearly zero absorption. In 9M sulfuric acid

solution, the approximate molar extinction coefficients for these chromium species are as

follows: Cr3+ − 50 L/mole cm at 604 nm; Cr2O7 2– − 380 L/mole cm at 444 nm; Cr3+ − 25

L/mole cm at 426 nm. The Cr3+ ion has a minimum in the region of 400 nm. Thus a working

absorption maximum is at 420 nm.

For COD values between 100 and 900 mg/L, increase in Cr3+ in the 600-nm region is

determined. Higher values can be obtained by sample dilution. COD values of 90 mg/L or less

can be determined by following the decrease in Cr2O7 2– at 420 nm. The corresponding

generation of Cr3+ gives a small absorption increase at 420 nm, but this is compensated for in the

calibration procedure.

b. Interferences and limitations: See Section 5220C.1b.

For this procedure to be applicable, all visible light-absorbing interferents must be absent or

be compensated for. This includes insoluble suspended matter as well as colored components. If

either type of interference occurs, the test is not necessarily lost because COD can be determined

titrimetrically as in 5220C.

2. Apparatus

a. See Section 5220C.2. Ensure that reaction vessels are of optical quality. Other types of

absorption cells with varying path lengths may be used. Use the extinction coefficients of the

ions of interest for this approach.

b. Spectrophotometer, for use at 600 nm and/or 420 nm with access opening adapter for

ampule or 16-, 20-, or 25-mm tubes. Verify that the instrument operates in the region of 420 nm

and 600 nm. Values slightly different from these may be found, depending on the spectral

bandpass of the instrument.

3. Reagents

a. Digestion solution, high range: Add to about 500 mL distilled water 10.216 g K2Cr2O7,

primary standard grade, previously dried at 150°C for 2 h, 167 mL conc H2SO4, and 33.3 g

HgSO4. Dissolve, cool to room temperature, and dilute to 1000 mL.

b. Digestion solution, low range: Prepare as in 3a, but use only 1.022 g potassium

dichromate.

c. Sulfuric acid reagent: See Section 5220B.3b.

d. Sulfamic acid: See Section 5220B.3 f.

e. Potassium hydrogen phthalate standard: See Section 5220B.3g.

4. Procedure

a. Treatment of samples: Measure suitable volume of sample and reagents into tube or

ampule as indicated in Table 5220:I. Prepare, digest, and cool samples, blank, and one or more

standards as directed in Section 5220C.4. Note the safety precautions. It is critical that the

volume of each component be known and that the total volume be the same for each reaction

vessel. If volumetric control is difficult, transfer digested sample, dilute to a known volume, and

read. Premixed reagents in digestion tubes are available commercially.

b. Measurement of dichromate reduction: Cool sample to room temperature slowly to avoid

precipitate formation. Once samples are cooled, vent, if necessary, to relieve any pressure

generated during digestion. Mix contents of reaction vessels to combine condensed water and

dislodge insoluble matter. Let suspended matter settle and ensure that optical path is clear.

Measure absorption of each sample blank and standard at selected wavelength (420 nm or 600

nm). At 600 nm, use an undigested blank as reference solution. Analyze a digested blank to

confirm good analytical reagents and to determine the blank COD; subtract blank COD from

sample COD. Alternately, use digested blank as the reference solution once it is established that

the blank has a low COD.

At 420 nm, use reagent water as a reference solution. Measure all samples, blanks, and

standards against this solution. The absorption measurement of an undigested blank containing

dichromate, with reagent water replacing sample, will give initial dichromate absorption. Any

digested sample, blank, or standard that has a COD value will give lower absorbance because of

the decrease in dichromate ion. Analyze a digested blank with reagent water replacing sample to

ensure reagent quality and to determine the reagents’ contribution to the decrease in absorbance

during a given digestion. The difference between absorbances of a given digested sample and the

digested blank is a measure of the sample COD. When standards are run, plot differences of

digested blank absorbance and digested standard absorbance versus COD values for each

standard.

c. Preparation of calibration curve: Prepare at least five standards from potassium hydrogen

phthalate solution with COD equivalents to cover each concentration range. Make up to volume

with reagent water; use same reagent volumes, tube, or ampule size, and digestion procedure as

for samples. Prepare calibration curve for each new lot of tubes or ampules or when standards

prepared in ¶ 4a differ by ≥5% from calibration curve. Curves should be linear. However, some

nonlinearity may occur, depending on instrument used and overall accuracy needed.

5. Calculation

If samples, standards, and blanks are run under same conditions of volume and optical path

length, calculate COD as follows:

Preferably analyze samples in duplicate because of small sample size. Samples that are

inhomogeneous may require multiple determinations for accurate analysis. These should not

differ from their average by more than ±5% for the high-level COD test unless the condition of

the sample dictates otherwise. In the low-level procedure, results below 25 mg/L may tend to be

qualitative rather than quantitative.

6. Precision and Bias

Forty-eight synthetic samples containing potassium hydrogen phthalate and NaCl were

tested by five laboratories. At an average COD of 193 mg O2/L in the absence of chloride, the

standard deviation was ±17 mg O2/L (coefficient of variation 8.7%). At an average COD of 212

mg O2/L and 100 mg Cl–/ L, the standard deviation was ±20 mg O2/L (coefficient of variation,

9.6%). Additional QA/QC data for both high- and low-level procedures may be found

elsewhere.1

5310 TOTAL ORGANIC CARBON (TOC)*#(105)

5310 A. Introduction

1. General Discussion

The organic carbon in water and wastewater is composed of a variety of organic compounds

in various oxidation states. Some of these carbon compounds can be oxidized further by

biological or chemical processes, and the biochemical oxygen demand (BOD), assimilable organic carbon (AOC), and chemical oxygen demand (COD) methods may be used to

characterize these fractions. Total organic carbon (TOC) is a more convenient and direct

expression of total organic content than either BOD, AOC, or COD, but does not provide the

same kind of information. If a repeatable empirical relationship is established between TOC and

BOD, AOC, or COD for a specific source water then TOC can be used to estimate the

accompanying BOD, AOC, or COD. This relationship must be established independently for

each set of matrix conditions, such as various points in a treatment process. Unlike BOD or

COD, TOC is independent of the oxidation state of the organic matter and does not measure

other organically bound elements, such as nitrogen and hydrogen, and inorganics that can

contribute to the oxygen demand measured by BOD and COD. TOC measurement does not

replace BOD, AOC, and COD testing.

Measurement of TOC is of vital importance to the operation of water treatment and waste

treatment plants. Drinking water TOCs range from less than 100 μg/L to more than 25,000 μg/L.

Wastewater may contain very high levels of organic compounds (TOC >100 mg/L). Some of

these applications may include waters with substantial ionic impurities as well as organic matter.

In many applications, the presence of organic contaminants may degrade ion-exchange

capacity, serve as a nutrient source for undesired biological growth, or be otherwise detrimental

to the process for which the water is to be utilized. For drinking waters in particular, organic

compounds may react with disinfectants to produce potentially toxic and carcinogenic

compounds.

To determine the quantity of organically bound carbon, the organic molecules must be

broken down and converted to a single molecular form that can be measured quantitatively. TOC

methods utilize high temperature, catalysts, and oxygen, or lower temperatures (<100°C) with

ultraviolet irradiation, chemical oxidants, or combinations of these oxidants to convert organic

carbon to carbon dioxide (CO2). The CO2 may be purged from the sample, dried, and transferred

with a carrier gas to a nondispersive infrared analyzer or coulometric titrator. Alternatively, it

may be separated from the sample liquid phase by a membrane selective to CO2 into a

high-purity water in which corresponding increase in conductivity is related to the CO2 passing

the membrane.

2. Fractions of Total Carbon

The methods and instruments used in measuring TOC analyze fractions of total carbon (TC)

and measure TOC by two or more determinations. These fractions of total carbon are defined as:

inorganic carbon—the carbonate, bicarbonate, and dissolved CO2; total organic carbon

(TOC)—all carbon atoms covalently bonded in organic molecules; dissolved organic carbon

(DOC)—the fraction of TOC that passes through a 0.45-μm-pore-diam filter; suspended organic

carbon—also referred to as particulate organic carbon, the fraction of TOC retained by a

0.45-μm filter; purgeable organic carbon—also referred to as volatile organic carbon, the

fraction of TOC removed from an aqueous solution by gas stripping under specified conditions;

and nonpurgeable organic carbon—the fraction of TOC not removed by gas stripping.

In most water samples, the inorganic carbon fraction is many times greater than the TOC

fraction. Eliminating or compensating for inorganic carbon interferences requires determinations

of both TC and inorganic carbon to measure TOC. Inorganic carbon interference can be

eliminated by acidifying samples to pH 2 or less to convert inorganic carbon species to CO2.

Subsequent purging of the sample with a purified gas or vacuum degassing removes the CO2 by

volatilization. Sample purging also removes purgeable organic carbon so that the organic carbon

measurement made after eliminating inorganic carbon interferences is actually a nonpurgeable

organic carbon determination: determine purgeable organic carbon to measure TOC. In many

surface and ground waters the purgeable organic carbon contribution to TOC is negligible.

Therefore, in practice, the nonpurgeable organic carbon determination is substituted for TOC.

Alternatively, inorganic carbon interference may be compensated for by separately

measuring total carbon (TC) and inorganic carbon. The difference between TC and inorganic

carbon is TOC.

The purgeable fraction of TOC is a function of the specific conditions and equipment

employed. Sample temperature and salinity, gas-flow rate, type of gas diffuser, purging-vessel

dimensions, volume purged, and purging time affect the division of TOC into purgeable and

nonpurgeable fractions. When separately measuring purgeable organic carbon and nonpurgeable

organic carbon on the same sample, use identical conditions for purging during the purgeable

organic carbon measurement as in purging to prepare the nonpurgeable organic carbon portion

for analysis. Consider the conditions of purging when comparing purgeable organic carbon or

nonpurgeable organic carbon data from different laboratories or different instruments.

3. Selection of Method

The high-temperature combustion method (B) is suitable for samples with higher levels of

TOC that would require dilution for the various persulfate methods (Method C or Method D).

Generally, it also will determine organic carbon from compounds that are chemically refractory

and not determined by Method C or Method D. High-temperature combustion may be desirable

for samples containing high levels of suspended organic carbon, which may not be efficiently

oxidized by persulfate and/or UV methods. Interlaboratory studies have shown biases on the

order of 1 mg/L using older high-temperature instruments. With newer instruments, detection

limits as low as 10 μg/L have been reported. Some high-temperature combustion instruments are

not designed for levels below 1 mg/L. The high-temperature methods accumulate nonvolatile

residues in the analyzer, whereas, in Method C, residuals are drained from the analyzer. Method

C generally provides better sensitivity for lower-level (<1 mg/L) samples. Persulfate and/or UV

oxidation are useful for TOC as low as 10 μg/L. Because the range of sensitivity of the methods

overlaps, other factors may dictate method choice in the range of 1 mg/L to 50 mg/L. A method

may be chosen on the basis of desired precision, ease of use, cost, etc. Method D generally is

equivalent to Method C, but the equipment for Method D is no longer manufactured.

To qualify a particular instrument for use, demonstrate that the single-user precision and bias

given in each method can be reproduced. Also, preferably demonstrate the overall precision by conducting in-house studies with more than one operator.

Evaluate the selected method to ensure that data quality objectives are attained. Evaluate

method detection limit in a matrix as similar as possible to the unknowns as described in Section

1030. Be aware that instrument blanks are handled in a variety of ways in TOC analyzers and

that the true magnitude of the blank may not be readily apparent to the analyst. Some instruments

‘‘zero out’’ much of the blank by adjusting the zero on the detector. Others enter blank values in

units such as mv responses rather than absolute concentrations, whereas other instruments

accumulate the total blank in the system during a blank run. Carefully observe the variability of

low-level measurements and check it any time reagents or instrument operations are changed.

The following methods note that when a water blank is run there is a contribution to the

observed blank value from the level of carbon in the blank water.

The methods show expected single-operator and multiple-laboratory precision. These

equations are based on referenced interlaboratory studies that in some cases were performed on

older equipment. The range of testing is important to observe because the error and bias

generally will be some significant fraction of the low standard. Consult references to determine

type of equipment and conditions of the interlaboratory study. Determine the performance of the

instrument being used by analyzing waters with matrices similar to those of unknowns, using the

procedures outlined in Section 1040B.

5310 B. High-Temperature Combustion Method

1. General Discussion

The high-temperature combustion method has been used for a wide variety of samples, but

its utility is dependent on particle size reduction because it uses small-orifice syringes.

a. Principle: The sample is homogenized and diluted as necessary and a microportion is

injected into a heated reaction chamber packed with an oxidative catalyst such as cobalt oxide,

platinum group metals, or barium chromate. The water is vaporized and the organic carbon is

oxidized to CO2 and H2O. The CO2 from oxidation of organic and inorganic carbon is

transported in the carrier-gas streams and is measured by means of a nondispersive infrared

analyzer, or titrated coulometrically.

Because total carbon is measured, inorganic carbon must be removed by acidification and

sparging or measured separately and TOC obtained by difference.

Measure inorganic carbon by injecting the sample into a reaction chamber where it is

acidified. Under acidic conditions, all inorganic carbon is converted to CO2, which is transferred

to the detector and measured. Under these conditions organic carbon is not oxidized and only

inorganic carbon is measured.

Alternatively, convert inorganic carbonates to CO2 with acid and remove the CO2 by purging

before sample injection. The sample contains only the nonpurgeable organic carbon fraction of

total carbon: a purgeable organic carbon determination also is necessary to measure TOC.

b. Interference: Removal of carbonate and bicarbonate by acidification and purging with

purified gas results in the loss of volatile organic substances. The volatiles also can be lost

during sample blending, particularly if the temperature is allowed to rise. Another important loss

can occur if large carbon-containing particles fail to enter the needle used for injection.

Filtration, although necessary to eliminate particulate organic matter when only DOC is to be

determined, can result in loss or gain of DOC, depending on the physical properties of the carbon-containing compounds and the adsorption or desorption of carbonaceous material on the

filter. Check filters for their contribution to DOC by analyzing a filtered blank. Note that any

contact with organic material may contaminate a sample. Avoid contaminated glassware, plastic

containers, and rubber tubing. Analyze sample treatment, system, and reagent blanks.

Combustion temperatures above 950°C are required to decompose some carbonates.

Systems that use lower temperatures must destroy carbonates by acidification. Elemental carbon

may not be oxidized at lower temperatures but generally it is not present in water samples nor is

it formed during combustion of dilute samples. The advantage of using lower temperatures

(680°C) is that fusion of dissolved salts is minimized, resulting in lower blank values. Gases

evolved from combustion, such as water, halide compounds, and nitrogen oxides, may interfere

with the detection system. Consult manufacturers’ recommendations regarding proper selection

of scrubber materials and check for any matrix interferences.

The major limitation to high-temperature techniques is the magnitude and variability of the

blank. Instrument manufacturers have developed new catalysts and procedures that yield lower

blanks, resulting in lower detection levels.

c. Minimum detectable concentration: 1 mg C/L or less, depending on the instrument used.

This can be achieved with most high-temperature combustion analyzers although instrument

performance varies. The minimum detectable concentration may be reduced by concentrating the

sample, or by increasing the portion taken for analysis.

d. Sampling and storage: If possible, rinse bottles with sample before filling and carry field

blanks through sampling procedure to check for any contamination that may occur. Collect and

store samples in glass bottles protected from sunlight and seal with TFE-backed septa. Before

use, wash bottles with acid, seal with aluminum foil, and bake at 400°C for at least 1 h. Wash

uncleaned TFE septa with detergent, rinse repeatedly with organic-free water, wrap in aluminum

foil, and bake at 100°C for 1 h. Check performance of new or cleaned septa by running

appropriate blanks. Preferably use thick silicone rubber-backed TFE septa with open ring caps to

produce a positive seal. Less rigorous cleaning may be acceptable if the concentration range is

relatively high. Check bottle blanks with each set of sample bottles to determine effectiveness or

necessity of cleaning. Preserve samples that cannot be examined immediately by holding at 4°C

with minimal exposure to light and atmosphere. Acidification with phosphoric or sulfuric acid to

a pH ≤2 at the time of collection is especially desirable for unstable samples, and may be used on

all samples: acid preservation, however, invalidates any inorganic carbon determination on the

samples.

2. Apparatus

a. Total organic carbon analyzer, using combustion techniques.

b. Sampling, injection, and sample preparation accessories, as prescribed by instrument

manufacturer.

c. Sample blender or homogenizer.

d. Magnetic stirrer and TFE-coated stirring bars.

e. Filtering apparatus and 0.45-μm-pore-diam filters. Preferably use HPLC syringe filters

with no detectable TOC blank. Glass fiber or silver membrane filters also can be used. Rinse

filters before use and monitor filter blanks.

3. Reagents

a. Reagent water: Prepare reagents, blanks, and standard solutions from reagent water with a

TOC value less than 2 × the MDL (see Section 1030 and Section 1080).

b. Acid: Phosphoric acid, H3PO4. Alternatively use sulfuric acid, H2SO4.

c. Organic carbon stock solution: Dissolve 2.1254 g anhydrous primary-standard-grade

potassium biphthalate, C8H5KO4, in carbon-free water and dilute to 1000 mL; 1.00 mL = 1.00

mg carbon. Prepare laboratory control standards using any other appropriate

organic-carbon-containing compound of adequate purity, stability, and water solubility.

Preserve by acidifying with H3PO4 or H2SO4 to pH ≤2, and store at 4°C.

d. Inorganic carbon stock solution: Dissolve 4.4122 g anhydrous sodium carbonate,

Na2CO3, in water, add 3.497 g anhydrous sodium bicarbonate, NaHCO3, and dilute to 1000 mL;

1.00 mL = 1.00 mg carbon. Alternatively, use any other inorganic carbonate compound of

adequate purity, stability, and water solubility. Keep tightly stoppered. Do not acidify.

e. Carrier gas: Purified oxygen or air, CO2-free and containing less than 1 ppm hydrocarbon

(as methane).

f. Purging gas: Any gas free of CO2 and hydrocarbons.

4. Procedure

a. Instrument operation: Follow manufacturer’s instructions for analyzer assembly, testing,

calibration, and operation. Adjust to optimum combustion temperature before using instrument;

monitor temperature to insure stability.

b. Sample treatment: If a sample contains gross solids or insoluble matter, homogenize until

satisfactory replication is obtained. Analyze a homogenizing blank consisting of reagent water

carried through the homogenizing treatment.

If inorganic carbon must be removed before analysis, transfer a representative portion (10 to

15 mL) to a 30-mL beaker, add acid to reduce pH to 2 or less, and purge with gas for 10 min.

Inorganic carbon also may be removed by stirring the acidified sample in a beaker while

directing a stream of purified gas into the beaker. Because volatile organic carbon will be lost

during purging of the acidified solution, report organic carbon as total nonpurgeable organic

carbon. Check efficiency of inorganic carbon removal for each sample matrix by splitting a

sample into two portions and adding to one portion an inorganic carbon level similar to that of

the sample. The TOC values should agree; if they do not, adjust sample container, sample

volume, pH, purge gas flow rate, and purge time to obtain complete removal of inorganic carbon.

If the available instrument provides for a separate determination of inorganic carbon (carbonate, bicarbonate, free CO2) and total carbon, omit decarbonation and determine TOC by

difference between TC and inorganic carbon.

If dissolved organic carbon is to be determined, filter sample through 0.45-μm-pore-diam

filter; analyze a filtering blank.

c. Sample injection: Withdraw a portion of prepared sample using a syringe fitted with a

blunt-tipped needle. Select sample volume according to manufacturer’s direction. Stir samples

containing particulates with a magnetic stirrer. Select needle size consistent with sample

particulate size. Other sample injection techniques, such as sample loops, may be used. Inject

samples and standards into analyzer according to manufacturer’s directions and record response.

Repeat injection until consecutive measurements are obtained that are reproducible to within ±

10%.

d. Preparation of standard curve: Prepare standard organic and inorganic carbon series by

diluting stock solutions to cover the expected range in samples within the linear range of the

instrument. Dilute samples higher than the linear range of the instrument in reagent water. Inject

and record peak height or area of these standards and a dilution water blank. Plot carbon

concentration in milligrams per liter against corrected peak height or area on rectangular

coordinate paper. This is unnecessary for instruments provided with a digital readout of

concentration.

With most TOC analyzers, it is not possible to determine separate blanks for reagent water,

reagents, and the entire system. In addition, some TOC analyzers produce a variable and erratic

blank that cannot be corrected reliably. In many laboratories, reagent water is the major

contributor to the blank value. Correcting only the instrument response of standards (which

contain reagent water + reagents + system blank) creates a positive error, while also correcting

samples (which contain only reagents and system blank contributions) for the reagent water

blank creates a negative error. Minimize errors by using reagent water and reagents low in

carbon.

Inject samples and procedural blanks (consisting of reagent water taken through any

pre-analysis steps—values are typically higher than those for reagent water) and determine

sample organic carbon concentrations directly from the readout or measurements by comparing

corrected instrument response to the calibration curve. Instruments with coulometric detectors do

not require calibration curves. Regularly analyze laboratory control samples to confirm

performance of the instrument (see Quality Control, below). These detectors accumulate the

system blank; therefore, monitor system blank regularly.

5. Calculations

Calculate corrected instrument response of standards and samples by subtracting the

reagent-water blank instrument response from that of the standard and sample. Prepare a

standard curve of corrected instrument response vs. TOC concentration. Subtract procedural

blank from each sample instrument response and compare to standard curve to determine carbon

content. Apply appropriate dilution factor when necessary. Subtract inorganic carbon from total carbon when TOC is determined by difference.

NOTE: The reagent water blank may include an instrument contribution not dependent on

reagent-water carbon, and a true response due to reagent-water carbon. When reagent-water

carbon is a significant fraction of reagent-water blank, a negative error no larger than

reagent-water blank is introduced in the sample values. If TOC analyzer design permits isolation

of each of the contributions to the total blank, apply appropriate blank corrections to instrument

response of standards (reagent blank, water blank, system blank) and sample (reagent blank and

system blank).

6. Quality Control

Determine instrument detection limit according to Section 1030.

After every tenth analysis, analyze a blank and a laboratory control sample prepared from a

source of material other than the calibration standards, at a level similar to the analytical

samples. Preferably prepare the laboratory control sample in a matrix similar to that of the

samples. Alternatively, periodically make known additions to samples to ensure recovery from

unknown matrices.

7. Precision

The difficulty of sampling particulate matter on unfiltered samples limits the precision of the

method to approximately 5 to 10%.

Interlaboratory studies of high-temperature combustion methods have been conducted in the

range above 2 mg/L.1 The resulting equation for single-operator precision on matrix water is:

So = 0.027x + 0.29

Overall precision is:

St = 0.044x + 1.49

where:

So = single-operator precision,

St = overall precision, and

x = TOC concentration, mg/L.

8. Reference

1. AMERICAN SOCIETY FOR TESTING AND MATERIALS. 1994. Standard Test Method for

Total and Organic Carbon in Water by High Temperature Oxidation and by

Coulometric Detection. D4129–88. Annual Book of ASTM Standards. American Soc.

Testing & Materials, Philadelphia, Pa.

5310 C. Persulfate-Ultraviolet or Heated-Persulfate Oxidation Method

1. General Discussion

Many instruments utilizing persulfate oxidation of organic carbon are available. They

depend either on heat or ultraviolet irradiation activation of the reagents. These oxidation

methods provide rapid and precise measurement of trace levels of organic carbon in water.

a. Principle: Organic carbon is oxidized to carbon dioxide, CO2, by persulfate in the

presence of heat or ultraviolet light. The CO2 produced may be purged from the sample, dried,

and transferred with a carrier gas to a nondispersive infrared (NDIR) analyzer, or be

coulometrically titrated, or be separated from the liquid stream by a membrane that allows the

specific passage of CO2 to high-purity water where a change in conductivity is measured and

related to the CO2 passing the membrane.

Some instruments utilize an ultraviolet lamp submerged in a continuously gas-purged

reactor that is filled with a constant-feed persulfate solution. The samples are introduced serially

into the reactor by an autosampler or they are injected manually. The CO2 produced is sparged

continuously from the solution and is carried in the gas stream to an infrared analyzer that is

specifically tuned to the absorptive wavelength of CO2. The instrument’s microprocessor

calculates the area of the peaks produced by the analyzer, compares them to the peak area of the

calibration standard stored in its memory, and prints out a calculated organic carbon value in milligrams per liter.

Other UV-persulfate instruments use continuous-flow injection of the sample into the

instrument. Removal of inorganic carbon by vacuum degassing is provided optionally. The

sample is acidified and persulfate added. Sample flow is split; one channel passes to a delay coil

while the other passes through the UV reactor. The CO2 from each stream is separated from the

sample stream by membranes selectively permeable to CO2 that allow the CO2 to pass into

high-purity water where change in conductivity is measured. CO2 from the non-UV-irradiated

stream represents inorganic carbon. CO2 from the irradiated stream represents TC. The

instrument automatically converts the detector signals to unit of concentration (mg/L or μg/L).

The TOC is calculated as the difference between the TC and inorganic carbon channels.

Heated-persulfate instruments utilize a digestion vessel heated to 95 to 100°C. Samples are

added by direct injection, loop injection, line injection, or autosampler. After inorganic carbon

is removed by acidification and sparging, a measured amount of persulfate solution is added to

the sample. After an oxidation period, the resulting CO2 is sparged from the solution and carried

to an infrared analyzer specifically tuned to the absorptive wavelength of CO2. The instrument’s

microprocessor converts the detector signal to organic carbon concentrations in mg/L based on

stored calibration data.

b. Interferences: See Section 5310B.1. Insufficient acidification will result in incomplete

release of CO2.

The intensity of the ultraviolet light reaching the sample matrix may be reduced by highly

turbid samples or with aging of the ultraviolet source, resulting in sluggish or incomplete

oxidation. Large organic particles or very large or complex organic molecules such as tannins,

lignins, and humic acid may be oxidized slowly because persulfate oxidation is rate-limited.

However, oxidation of many large biological molecules such as proteins and monoclonal

antibodies proceeds rapidly. Because the efficiency of conversion of organic carbon to CO2 may

be affected by many factors, check efficiency of oxidation with selected model compounds

representative of the compounds of interest in a matrix representative of the sample.

Some instruments give low results for certain difficult-to-oxidize compounds under certain

conditions. The following compounds are difficult to oxidize, are sufficiently soluble in water,

and can be mixed and measured accurately at trace levels: urea, nicotinic acid, pyridine,

n-butanol, acetic acid, leucine, acetonitrile, octoxynol-9, tartaric acid, 1,10-phenanthroline, 1-

glutonic acid, 2-propanol, and sodium dodecylbenzenesulfonate. Use these compounds as matrix

additions to evaluate oxidation efficiency.

Persulfate oxidation of organic molecules is slowed in samples containing significant

concentrations of chloride by the preferential oxidation of chloride; at concentrations above

0.05% chloride, oxidation of organic matter may be inhibited. To remove this interference add

mercuric nitrate*#(106) to the persulfate solution in UV-persulfate instruments, or extend

reaction time and/or increase amount of persulfate solution in heated-persulfate instruments.

With any organic carbon measurement, contamination during sample handling and treatment

is a likely source of interference. This is especially true of trace analysis. Take extreme care in

sampling, handling, and analysis of samples below 1 mg TOC/L.

c. Minimum detectable concentration: Concentration of 0.01 mg TOC/L can be measured by

some instruments if scrupulous attention is given to minimizing sample contamination and

method background. See Section 1030 for procedures to evaluate the MDL for a specific

instrument. Use the high-temperature combustion method (B) for high concentrations of TOC or

dilute the sample, ensuring that the dilution process does not contaminate the sample.

d. Sampling and storage: See Section 5310B.1d.

2. Apparatus

a. Total organic carbon analyzer utilizing persulfate oxidation principle.

b. Sampling and injection accessories, as specified by the instrument manufacturer.

3. Reagents

a. Reagents listed in Section 5310B.3.

b. Persulfate solution: Different instrument manufacturers recommend different forms and

concentrations of peroxydisulfate. Typical preparations are as follows:

1) Sodium peroxydisulfate, 10%: Dissolve 100 g reagent in water; bring volume to 1 L.

2) Ammonium peroxydisulfate, 15%: Dissolve 150 g reagent in water; bring volume to 1 L.

3) Potassium peroxydisulfate, 2%: Dissolve 20 g reagent in water; bring volume to 1 L.

Check blank values from reagents and, if values are high, purify reagent or use a

higher-purity source.

4. Procedure

a. Instrument operation: Follow manufacturer’s instructions for assembly, testing,

calibration, and operation.

b. Sample preparation: If a sample contains gross particulates or insoluble matter,

homogenize until a representative portion can be withdrawn through the syringe needle,

autosampler tubing, or sample inlet system of continuous on-line monitor.

If dissolved organic carbon is to be determined, filter sample and a reagent water blank

through 0.45-μm filter. HPLC syringe filters have been found to pass water without

contamination. Glass fiber or silver membrane filters also can be used. Check filter blanks

regularly.

To determine nonpurgeable organic carbon, transfer 15 to 30 mL sample to a flask or test

tube and acidify to a pH of 2. Purge according to manufacturer’s recommendations. In some

instruments this is performed internally. Check efficiency of inorganic carbon removal for each

sample matrix by splitting a sample into two portions; to one of the portions, add inorganic

carbon to a level similar to that of the sample. The TOC values should agree. If the values do not agree, adjust conditions such as sample container, sample volume, pH, purge-gas flow rate, and

purge time to obtain complete removal of inorganic carbon.

c. Sample injection: See Section 5310B.4c.

d. Standard curve preparation: Prepare an organic carbon standard series over the range of

organic carbon concentrations in the samples. Run standards and blanks and record analyzer’s

response. Determine instrument response for each standard and blank. Unless carbon dioxide is

trapped and desorbed, producing consistent peak heights, determinations based on peak height

may be inadequate because of differences in the rate of oxidation of standards and samples.

Correct instrument response of standards by subtracting reagent water blank and plot organic

carbon concentration in milligrams per liter against corrected instrument response. For

instruments providing a digital computation of concentration, this is not necessary. Be sure that

the instrument’s algorithm includes blank correction and linearity of response. Analyze

standards having concentrations above and below those determined in the samples, preferably

prepared in a similar matrix, to confirm proper instrument operation.

5. Calculation

See Section 5310B.5, or use instrument manufacturer’s procedure.

6. Quality Control

See Section 5310B.6.

7. Precision and Bias

Interlaboratory studies of persulfate and/or UV with NDIR detection methods have been

conducted in the range of 0.1 mg/L to 4 000 mg/L of carbon.1 The resulting equation for organic

carbon, single-operator precision is:

So = 0.04x + 0.1

Overall precision is expressed as:

St = 0.08x + 0.1

where:

So = single-operator precision,

St = overall precision, and

x = TOC concentration, mg/L.

An interlaboratory study was conducted for the membrane conductivity method,†#(107)

covering samples with 1 to 25 mg/L organic carbon concentrations. The resulting equation for

single-operator precision is:

So = 0.012x − 0.022

Overall precision is expressed as:

St = 0.027x + 0.09

where terms are defined as above.

8. Reference

1. AMERICAN SOCIETY FOR TESTING AND MATERIALS. 1994. Standard Test Method for

Total Carbon in Water by Ultraviolet, or Persulfate Oxidation, or Both, and Infrared

Detection. D4839-88. Annual Book of ASTM Standards. American Soc. Testing &

Materials, Philadelphia, Pa.

5310 D. Wet-Oxidation Method

1. General Discussion

The wet-oxidation method is suitable for the analyses of water, water-suspended sediment

mixtures, seawaters, brines, and wastewaters containing at least 0.1 mg nonpurgeable organic

carbon/L. The method is not suitable for the determination of volatile organic constituents.

a. Principle: The sample is acidified, purged to remove inorganic carbon, and oxidized with

persulfate in an autoclave at temperatures from 116 to 130°C. The resultant carbon dioxide

(CO2) is measured by nondispersive infrared spectrometry.

b. Interferences: See Section 5310B.1 and Section 5310C.1.

c. Minimum detectable concentrations: High concentrations of reducing agents may

interfere. Concentration of 0.10 mg TOC/L can be measured if scrupulous attention is given to

minimizing sample contamination and method background. Use the high-temperature

combustion method (B) for high concentrations of TOC.

d. Sampling and storage: See Section 5310B.1d.

2. Apparatus

a. Ampules, precombusted, 10-mL, glass.

b. Ampule purging and sealing unit.

c. Autoclave.

d. Carbon analyzer.

e. Homogenizer.

3. Reagents

In addition to the reagents specified in Section 5310B.3a, Section 5310B.3c, Section

5310B.3e, and Section 5310B.3 f, the following reagents are required:

a. Phosphoric acid solution, H3PO4, 1.2N: Add 83 mL H3PO4 (85%) to water and dilute to 1

L with water. Store in a tightly stoppered glass bottle.

b. Potassium persulfate, reagent-grade, granular. Avoid using finely divided forms.

4. Procedure

Follow manufacturer’s instructions for instrument assembly, testing calibration, and operation. Add 0.5 mL 1.2N H3PO4, solution to precombusted ampules.

To analyze for dissolved organic carbon, follow the filtration procedure in Method B.

Homogenize sample to produce a uniform suspension. Rinse homogenizer with reagent water

after each use. Pipet water sample (10.0 mL maximum) into an ampule. Adjust smaller volumes

to 10 mL with reagent water. Prepare one reagent blank (10 mL reagent water plus acid and

oxidant) for every 15 to 20 water samples. Prepare standards covering the range of 0.1 to 40 mg

C/L by diluting the carbon standard solution. Immediately place filled ampules on purging and

seating unit and purge them at rate of 60 mL/min for 6 min with purified oxygen. Add 0.2 g

potassium persulfate using a dipper calibrated to deliver 0.2 g to the ampule. Seal samples

according to the manufacturer’s instructions. Place sealed samples, blanks, and a set of standards

in ampule racks in an autoclave and digest 4 h at temperature between 116 and 130°C.

Set sensitivity range of carbon analyzer by adjusting the zero and span controls in accordance

with the manufacturer’s instructions. Break combusted ampules in the cutter assembly of the

carbon analyzer, sweep CO2 into the infrared cell with nitrogen gas, and record area of each CO2

peak. CAUTION: Because combusted ampules are under positive pressure, handle with care to

prevent explosion.

5. Calculations

Prepare an analytical standard curve by plotting peak area of each standard versus

concentration (mg/L) of organic carbon standards. The relationship between peak area and

carbon concentration is curvilinear. Define operating curves each day samples are analyzed.

Report nonpurgeable organic carbon concentration as follows: 0.1 mg/L to 0.9 mg/L, one

significant figure; 1.0 mg/L and above, two significant figures.

6. Quality Control

See Section 5310B.6.

7. Precision and Bias

Multiple determinations of four different concentrations of aqueous potassium acid phthalate

samples at 2.00, 5.00, 10.0, and 40.0 mg C/L resulted in mean values of 2.2, 5.3, 9.9, and 38

mg/L and standard deviations of 0.13, 0.15, 0.11, and 1.4, respectively.

Precision also may be expressed in terms of percent relative standard deviation as follows:

Number of Replicates       Mean                   Relative Standard Deviation

mg/L                                        %

9                                  2.2                                  5.9

10                                 5.3                                  2.8

10                                 9.9                                  1.1

10                               38.0                                  3.7

8. Bibliography

WILLIAMS, P.M. 1969. The determination of dissolved organic carbon in seawater: A comparison

of two methods. Limnol. Oceanogr. 14:297.

OCEANOGRAPHY INTERNATIONAL CORP. 1970. The total carbon system operating manual.

College Station, Tex.

MacKinnon, M.D. 1981. The measurement of organic carbon in seawater. In E.K. Duursma &

R. Dawson, eds., Marine Organic Chemistry: Evolution, Composition, Interactions and

Chemistry of Organic Matter in Seawater. Elsevier Scientific Publishing Co., New York,

N.Y.

2320 ALKALINITY*#(29)

2320 A. Introduction

1. Discussion

Alkalinity of a water is its acid-neutralizing capacity. It is the sum of all the titratable bases.

The measured value may vary significantly with the end-point pH used. Alkalinity is a measure

of an aggregate property of water and can be interpreted in terms of specific substances only

when the chemical composition of the sample is known.

Alkalinity is significant in many uses and treatments of natural waters and wastewaters.

Because the alkalinity of many surface waters is primarily a function of carbonate, bicarbonate,

and hydroxide content, it is taken as an indication of the concentration of these constitutents. The

measured values also may include contributions from borates, phosphates, silicates, or other

bases if these are present. Alkalinity in excess of alkaline earth metal concentrations is

significant in determining the suitability of a water for irrigation. Alkalinity measurements are

used in the interpretation and control of water and wastewater treatment processes. Raw

domestic wastewater has an alkalinity less than, or only slightly greater than, that of the water

supply. Properly operating anaerobic digesters typically have supernatant alkalinities in the

range of 2000 to 4000 mg calcium carbonate (CaCO3)/L.1 2. Reference

1. POHLAND, F.G. & D.E. BLOODGOOD. 1963. Laboratory studies on mesophilic and

thermophilic anaerobic sludge digestion. J. Water Pollut. Control Fed. 35:11.

2320 B. Titration Method

1. General Discussion

a. Principle: Hydroxyl ions present in a sample as a result of dissociation or hydrolysis of

solutes react with additions of standard acid. Alkalinity thus depends on the end-point pH used.

For methods of determining inflection points from titration curves and the rationale for titrating

to fixed pH end points, see Section 2310B.1a.

For samples of low alkalinity (less than 20 mg CaCO3/L) use an extrapolation technique

based on the near proportionality of concentration of hydrogen ions to excess of titrant beyond

the equivalence point. The amount of standard acid required to reduce pH exactly 0.30 pH unit is

measured carefully. Because this change in pH corresponds to an exact doubling of the hydrogen

ion concentration, a simple extrapolation can be made to the equivalence point.1,2

b. End points: When alkalinity is due entirely to carbonate or bicarbonate content, the pH at

the equivalence point of the titration is determined by the concentration of carbon dioxide (CO2)

at that stage. CO2 concentration depends, in turn, on the total carbonate species originally

present and any losses that may have occurred during titration. The pH values in Table 2320:I

are suggested as the equivalence points for the corresponding alkalinity concentrations as

milligrams CaCO3 per liter. ‘‘Phenolphthalein alkalinity’’ is the term traditionally used for the

quantity measured by titration to pH 8.3 irrespective of the colored indicator, if any, used in the

determination. Phenolphthalein or metacresol purple may be used for alkalinity titration to pH

8.3. Bromcresol green or a mixed bromcresol green-methyl red indicator may be used for pH 4.5.

c. Interferences: Soaps, oily matter, suspended solids, or precipitates may coat the glass

electrode and cause a sluggish response. Allow additional time between titrant additions to let

electrode come to equilibrium or clean the electrodes occasionally. Do not filter, dilute,

concentrate, or alter sample.

d. Selection of procedure: Determine sample alkalinity from volume of standard acid

required to titrate a portion to a designated pH taken from ¶ 1b. Titrate at room temperature with

a properly calibrated pH meter or electrically operated titrator, or use color indicators. If using

color indicators, prepare and titrate an indicator blank.

Report alkalinity less than 20 mg CaCO3/L only if it has been determined by the

low-alkalinity method of ¶ 4d.

Construct a titration curve for standardization of reagents.

Color indicators may be used for routine and control titrations in the absence of interfering

color and turbidity and for preliminary titrations to select sample size and strength of titrant (see

below).

e. Sample size: See Section 2310B.1e for selection of size sample to be titrated and normality

of titrant, substituting 0.02N or 0.1N sulfuric (H2SO4) or hydrochloric (HCl) acid for the

standard alkali of that method. For the low-alkalinity method, titrate a 200-mL sample with

0.02N H2SO4 from a 10-mL buret.

f. Sampling and storage: See Section 2310B.1 f.

2. Apparatus

See Section 2310B.2.

3. Reagents

a. Sodium carbonate solution, approximately 0.05N: Dry 3 to 5 g primary standard Na2CO3

at 250°C for 4 h and cool in a desiccator. Weigh 2.5 ± 0.2 g (to the nearest mg), transfer to a 1-L

volumetric flask, fill flask to the mark with distilled water, and dissolve and mix reagent. Do not

keep longer than 1 week.

b. Standard sulfuric acid or hydrochloric acid, 0.1N: Prepare acid solution of approximate

normality as indicated under Preparation of Desk Reagents. Standardize against 40.00 mL 0.05N

Na2CO3 solution, with about 60 mL water, in a beaker by titrating potentiometrically to pH of

about 5. Lift out electrodes, rinse into the same beaker, and boil gently for 3 to 5 min under a

watch glass cover. Cool to room temperature, rinse cover glass into beaker, and finish titrating to

the pH inflection point. Calculate normality:

where:

A = g Na2CO3 weighed into 1-L flask,

B = mL Na2CO3 solution taken for titration, and

C = mL acid used.

Use measured normality in calculations or adjust to 0.1000N; 1 mL 0.1000N solution = 5.00 mg

CaCO3.

c. Standard sulfuric acid or hydrochloric acid, 0.02N: Dilute 200.00 mL 0.1000N standard

acid to 1000 mL with distilled or deionized water. Standardize by potentiometric titration of 15.00 mL 0.05N Na2CO3 according to the procedure of ¶ 3b; 1 mL = 1.00 mg CaCO3.

d. Bromcresol green indicator solution, pH 4.5 indicator: Dissolve 100 mg bromcresol green,

sodium salt, in 100 mL distilled water.

e. Mixed bromcresol green-methyl red indicator solution:3 Use either the aqueous or the

alcoholic solution:

1) Dissolve 100 mg bromcresol green sodium salt and 20 mg methyl red sodium salt in 100

mL distilled water.

2) Dissolve 100 mg bromcresol green and 20 mg methyl red in 100 mL 95% ethyl alcohol or

isopropyl alcohol.

f. Metacresol purple indicator solution, pH 8.3 indicator: Dissolve 100 mg metacresol purple

in 100 mL water.

g. Phenolphthalein solution, alcoholic, pH 8.3 indicator.

h. Sodium thiosulfate, 0.1N: See Section 2310B.3i.

4. Procedure

a. Color change: See Section 2310B.4b.

b. Potentiometric titration curve: Follow the procedure for determining acidity (Section

2310B.4c), substituting the appropriate normality of standard acid solution for standard NaOH,

and continue titration to pH 4.5 or lower. Do not filter, dilute, concentrate, or alter the sample.

c. Potentiometric titration to preselected pH: Determine the appropriate end-point pH

according to ¶ 1b. Prepare sample and titration assembly (Section 2310B.4c). Titrate to the

end-point pH without recording intermediate pH values and without undue delay. As the end

point is approached make smaller additions of acid and be sure that pH equilibrium is reached

before adding more titrant.

d. Potentiometric titration of low alkalinity: For alkalinities less than 20 mg/L titrate 100 to

200 mL according to the procedure of ¶ 4c, above, using a 10-mL microburet and 0.02N standard

acid solution. Stop the titration at a pH in the range 4.3 to 4.7 and record volume and exact pH.

Carefully add additional titrant to reduce the pH exactly 0.30 pH unit and again record volume.

5. Calculations

a. Potentiometric titration to end-point pH:

where:

A = mL standard acid used and N = normality of standard acid

Or

where:

t = titer of standard acid, mg CaCO3/mL.

Report pH of end point used as follows: ‘‘The alkalinity to pH _______ = _______ mg

CaCO3/L’’ and indicate clearly if this pH corresponds to an inflection point of the titration

curve.

b. Potentiometric titration of low alkalinity:

Total alkalinity, mg CaCO3/L

where:

B = mL titrant to first recorded pH,

C = total mL titrant to reach pH 0.3 unit lower, and

N = normality of acid.

c. Calculation of alkalinity relationships: The results obtained from the phenolphthalein and

total alkalinity determinations offer a means for stoichiometric classification of the three

principal forms of alkalinity present in many waters. The classification ascribes the entire

alkalinity to bicarbonate, carbonate, and hydroxide, and assumes the absence of other (weak)

inorganic or organic acids, such as silicic, phosphoric, and boric acids. It further presupposes the

incompatibility of hydroxide and bicarbonate alkalinities. Because the calculations are made on a

stoichiometric basis, ion concentrations in the strictest sense are not represented in the results,

which may differ significantly from actual concentrations especially at pH > 10. According to

this scheme:

1) Carbonate (CO3

2–) alkalinity is present when phenolphthalein alkalinity is not zero but is

less than total alkalinity.

2) Hydroxide (OH–) alkalinity is present if phenolphthalein alkalinity is more than half the

total alkalinity.

3) Bicarbonate (HCO3

–) alkalinity is present if phenolphthalein alkalinity is less than half the total alkalinity. These relationships may be calculated by the following scheme, where P is

phenolphthalein alkalinity and T is total alkalinity (¶ 1b):

Select the smaller value of P or (T −P). Then, carbonate alkalinity equals twice the smaller

value. When the smaller value is P, the balance (T −2P) is bicarbonate. When the smaller value

is (T−P), the balance (2P−T ) is hydroxide. All results are expressed as CaCO3. The

mathematical conversion of the results is shown in Table 2320:II. (A modification of Table

2320:II that is more accurate when P = 1/2T has been proposed.4)

Alkalinity relationships also may be computed nomographically (see Carbon Dioxide,

Section 4500-CO2). Accurately measure pH, calculate OH– concentration as milligrams CaCO3

per liter, and calculate concentrations of CO3

2– and HCO3

– as milligrams CaCO3 per liter from

the OH– concentration, and the phenolphthalein and total alkalinities by the following equations:

CO3

2– = 2P − 2[OH–]

HCO3

– = T − 2P + [OH–]

Similarly, if difficulty is experienced with the phenolphthalein end point, or if a check on the

phenolphthalein titration is desired, calculate phenolphthalein alkalinity as CaCO3 from the

results of the nomographic determinations of carbonate and hydroxide ion concentrations:

P = 1/2 [CO3

2–] + [OH–]

6. Precision and Bias

No general statement can be made about precision because of the great variation in sample

characteristics. The precision of the titration is likely to be much greater than the uncertainties

involved in sampling and sample handling before the analysis.

In the range of 10 to 500 mg/L, when the alkalinity is due entirely to carbonates or

bicarbonates, a standard deviation of 1 mg CaCO3/L can be achieved. Forty analysts in 17

laboratories analyzed synthetic samples containing increments of bicarbonate equivalent to 120

mg CaCO3/L. The titration procedure of ¶ 4b was used, with an end point pH of 4.5. The

standard deviation was 5 mg/L and the average bias (lower than the true value) was 9 mg/L.5

Sodium carbonate solutions equivalent to 80 and 65 mg CaCO3/L were analyzed by 12

laboratories according to the procedure of ¶ 4c.6 The standard deviations were 8 and 5 mg/L,

respectively, with negligible bias.6 Four laboratories analyzed six samples having total

alkalinities of about 1000 mg CaCO3/L and containing various ratios of carbonate/bicarbonate

by the procedures of both ¶ 4a and ¶ 4c. The pooled standard deviation was 40 mg/L, with

negligible difference between the procedures.

2510 CONDUCTIVITY*#(35)

2510 A. Introduction

Conductivity, k, is a measure of the ability of an aqueous solution to carry an electric

current. This ability depends on the presence of ions; on their total concentration, mobility, and

valence; and on the temperature of measurement. Solutions of most inorganic compounds are

relatively good conductors. Conversely, molecules of organic compounds that do not dissociate

in aqueous solution conduct a current very poorly, if at all.

1. Definitions and Units of Expression

Conductance, G, is defined as the reciprocal of resistance, R:

where the unit of R is ohm and G is ohm−1 (sometimes written mho). Conductance of a solution

is measured between two spatially fixed and chemically inert electrodes. To avoid polarization at

the electrode surfaces the conductance measurement is made with an alternating current signal.1

The conductance of a solution, G, is directly proportional to the electrode surface area, A, cm2,

and inversely proportional to the distance between the electrodes, L, cm. The constant of

proportionality, k, such that:

is called ‘‘conductivity’’ (preferred to ‘‘specific conductance’’). It is a characteristic property of

the solution between the electrodes. The units of k are 1/ohm-cm or mho per centimeter.

Conductivity is customarily reported in micromhos per centimeter (μmho/cm).

In the International System of Units (SI) the reciprocal of the ohm is the siemens (S) and

conductivity is reported as millisiemens per meter (mS/m); 1 mS/m = 10 μmhos/cm and 1 μS/

cm = 1 μmho/cm. To report results in SI units of mS/m divide μmhos/cm by 10.

To compare conductivities, values of k are reported relative to electrodes with A = 1 cm2 and

L = 1 cm. Absolute conductances, Gs, of standard potassium chloride solutions between

electrodes of precise geometry have been measured; the corresponding standard conductivities,

ks, are shown in Table 2510:I.

The equivalent conductivity, Λ, of a solution is the conductivity per unit of concentration. As

the concentration is decreased toward zero, Λ approaches a constant, designated as Λ°. With k in

units of micromhos per centimeter it is necessary to convert concentration to units of equivalents

per cubic centimeter; therefore:

Λ = 0.001k/concentration

where the units of Λ, k, and concentration are mho-cm2/equivalent, μmho/cm, and equivalent/L,

respectively. Equivalent conductivity, Λ, values for several concentrations of KCl are listed in

Table 2510:I. In practice, solutions of KCl more dilute than 0.001M will not maintain stable

conductivities because of absorption of atmospheric CO2. Protect these dilute solutions from the

atmosphere.

2. Measurement

a. Instrumental measurements: In the laboratory, conductance, Gs, (or resistance) of a

standard KCl solution is measured and from the corresponding conductivity, ks, (Table 2510:I) a

cell constant, C, cm−1, is calculated:

Most conductivity meters do not display the actual solution conductance, G, or resistance, R;

rather, they generally have a dial that permits the user to adjust the internal cell constant to

match the conductivity, ks, of a standard. Once the cell constant has been determined, or set, the

conductivity of an unknown solution,

ku = CGu

will be displayed by the meter.

Distilled water produced in a laboratory generally has a conductivity in the range 0.5 to 3

μmhos/cm. The conductivity increases shortly after exposure to both air and the water container.

The conductivity of potable waters in the United States ranges generally from 50 to 1500

μmhos/cm. The conductivity of domestic wastewaters may be near that of the local water supply,

although some industrial wastes have conductivities above 10 000 μmhos/cm. Conductivity

instruments are used in pipelines, channels, flowing streams, and lakes and can be incorporated

in multiple-parameter monitoring stations using recorders.

Most problems in obtaining good data with conductivity monitoring equipment are related to

electrode fouling and to inadequate sample circulation. Conductivities greater than 10 000 to 50

000 μmho/cm or less than about 10 μmho/cm may be difficult to measure with usual

measurement electronics and cell capacitance. Consult the instrument manufacturer’s manual or

published references.1,5,6

Laboratory conductivity measurements are used to:

• Establish degree of mineralization to assess the effect of the total concentration of ions on

chemical equilibria, physiological effect on plants or animals, corrosion rates, etc.

• Assess degree of mineralization of distilled and deionized water.

• Evaluate variations in dissolved mineral concentration of raw water or wastewater. Minor

seasonal variations found in reservoir waters contrast sharply with the daily fluctuations in

some polluted river waters. Wastewater containing significant trade wastes also may show a

considerable daily variation.

• Estimate sample size to be used for common chemical determinations and to check results of a

chemical analysis.

• Determine amount of ionic reagent needed in certain precipitation and neutralization reactions,

the end point being denoted by a change in slope of the curve resulting from plotting

conductivity against buret readings.

• Estimate total dissolved solids (mg/L) in a sample by multiplying conductivity (in micromhos

per centimeter) by an empirical factor. This factor may vary from 0.55 to 0.9, depending on the

soluble components of the water and on the temperature of measurement. Relatively high

factors may be required for saline or boiler waters, whereas lower factors may apply where

considerable hydroxide or free acid is present. Even though sample evaporation results in the

change of bicarbonate to carbonate the empirical factor is derived for a comparatively constant

water supply by dividing dissolved solids by conductivity.

• Approximate the milliequivalents per liter of either cations or anions in some waters by

multiplying conductivity in units of micromhos per centimeter by 0.01.

b. Calculation of conductivity: For naturally occurring waters that contain mostly Ca2+,

Mg2+, Na+, K+, HCO3

−, SO4

2−, and Cl− and with TDS less than about 2500 mg/L, the

following procedure can be used to calculate conductivity from measured ionic concentrations.7

The abbreviated water analysis in Table 2510:II illustrates the calculation procedure.

At infinite dilution the contribution to conductivity by different kinds of ions is additive. In

general, the relative contribution of each cation and anion is calculated by multiplying

equivalent conductances, λ°+ and λ°−, mho-cm2/equivalent, by concentration in equivalents per

liter and correcting units. Table 2510:III contains a short list of equivalent conductances for ions

commonly found in natural waters.8 Trace concentrations of ions generally make negligible

contribution to the overall conductivity. A temperature coefficient of 0.02/deg is applicable to

all ions, except H+ (0.0139/deg) and OH− (0.018/deg).

At finite concentrations, as opposed to infinite dilution, conductivity per equivalent decreases

with increasing concentration (see Table 2510:I). For solutions composed of one anion type and

one cation type, e.g., KCl as in Table 2510:I, the decrease in conductivity per equivalent with

concentration can be calculated, ±0.1%, using an ionic-strength-based theory of Onsager.9

When mixed salts are present, as is nearly always the case with natural and wastewaters, the

theory is quite complicated.10 The following semiempirical procedure can be used to calculate

conductivity for naturally occurring waters:

First, calculate infinite dilution conductivity (Table 2510:II, Column 4):

k° = Σ|zi|(λ°+i)(mMi) + Σ|zi|(λ°−i)(mMi)

where:

|zi| = absolute value of the charge of the i-th ion,

mMi = millimolar concentration of the i-th ion, and

λ°+i, λ°−i = equivalent conductance of the i-th ion.

If mM is used to express concentration, the product, (λ°+) (mMi) or (λ°−)(mMi), corrects the

units from liters to cm3. In this case k° is 578.2 μmho/cm (Table 2510:II, Column 4).

Next, calculate ionic strength, IS in molar units:

IS = Σzi

2(mMi)/2000

The ionic strength is 15.33/2000 = 0.00767 M (Table 2510:II, Column 5).

Calculate the monovalent ion activity coefficient, y, using the Davies equation for IS ≤ 0.5

M and for temperatures from 20 to 30°C.9,11

y = 10−0.5[IS1/2/(1 + IS1/2) − 0.3IS]

In the present example IS = 0.00767 M and y = 0.91.

Finally, obtain the calculated value of conductivity, kcalc, from: kcalc = k°y2

In the example being considered, kcalc = 578.2 × 0.912 = 478.8 μmho/cm versus the reported

value as measured by the USGS of 477 μmho/cm.

For 39 analyses of naturally occurring waters,7,12 conductivities calculated in this manner

agreed with the measured values to within 2%.

2510 B. Laboratory Method

1. General Discussion

See Section 2510A.

2. Apparatus

a. Self-contained conductivity instruments: Use an instrument capable of measuring

conductivity with an error not exceeding 1% or 1 μmho/cm, whichever is greater.

b. Thermometer, capable of being read to the nearest 0.1°C and covering the range 23 to

27°C. Many conductivity meters are equipped to read an automatic temperature sensor.

c. Conductivity cell:

1) Platinum-electrode type—Conductivity cells containing platinized electrodes are available

in either pipet or immersion form. Cell choice depends on expected range of conductivity.

Experimentally check instrument by comparing instrumental results with true conductivities of

the KCl solutions listed in Table 2510:I. Clean new cells, not already coated and ready for use,

with chromic-sulfuric acid cleaning mixture [see Section 2580B.3a2)] and platinize the

electrodes before use. Subsequently, clean and replatinize them whenever the readings become

erratic, when a sharp end point cannot be obtained, or when inspection shows that any platinum

black has flaked off. To platinize, prepare a solution of 1 g chloroplatinic acid, H2PtCl6⋅6H2O,

and 12 mg lead acetate in 100 mL distilled water. A more concentrated solution reduces the time

required to platinize electrodes and may be used when time is a factor, e.g., when the cell

constant is 1.0/cm or more. Immerse electrodes in this solution and connect both to the negative

terminal of a 1.5-V dry cell battery. Connect positive side of battery to a piece of platinum wire

and dip wire into the solution. Use a current such that only a small quantity of gas is evolved.

Continue electrolysis until both cell electrodes are coated with platinum black. Save platinizing

solution for subsequent use. Rinse electrodes thoroughly and when not in use keep immersed in

distilled water.

2) Nonplatinum-electrode type—Use conductivity cells containing electrodes constructed

from durable common metals (stainless steel among others) for continuous monitoring and field

studies. Calibrate such cells by comparing sample conductivity with results obtained with a

laboratory instrument. Use properly designed and mated cell and instrument to minimize errors

in cell constant. Very long meter leads can affect performance of a conductivity meter. Under

such circumstances, consult the manufacturer’s manual for appropriate correction factors if

necessary.

3. Reagents

a. Conductivity water: Any of several methods can be used to prepare reagent-grade water.

The methods discussed in Section 1080 are recommended. The conductivity should be small

compared to the value being measured.

b. Standard potassium chloride solution, KCl, 0.0100M: Dissolve 745.6 mg anhydrous KCl

in conductivity water and dilute to 1000 mL in a class A volumetric flask at 25°C and store in a

CO2-free atmosphere. This is the standard reference solution, which at 25°C has a conductivity

of 1412 μmhos/cm. It is satisfactory for most samples when the cell has a constant between 1 and 2 cm−1. For other cell constants, use stronger or weaker KCl solutions listed in Table 2510:I.

Care must be taken when using KCl solutions less than 0.001M, which can be unstable because

of the influence of carbon dioxide on pure water. For low conductivity standards, Standard

Reference Material 3190, with a certified conductivity of 25.0 μS/cm ± 0.3 μS/cm, may be

obtained from NIST. Store in a glass-stoppered borosilicate glass bottle.

4. Procedure

a. Determination of cell constant: Rinse conductivity cell with at least three portions of

0.01M KCl solution. Adjust temperature of a fourth portion to 25.0 ± 0.1°C. If a conductivity

meter displays resistance, R, ohms, measure resistance of this portion and note temperature.

Compute cell constant, C:

C, cm–1 = (0.001412)(RKCl)[1 + 0.0191(t − 25)]

where:

RKCl = measured resistance, ohms, and

t = observed temperature, °C.

Conductivity meters often indicate conductivity directly. Commercial probes commonly

contain a temperature sensor. With such instruments, rinse probe three times with 0.0100M KCl,

as above. Adjust temperature compensation dial to 0.0191 C−1. With probe in standard KCl

solution, adjust meter to read 1412 μmho/cm. This procedure automatically adjusts cell constant

internal to the meter.

b. Conductivity measurement: Thoroughly rinse cell with one or more portions of sample.

Adjust temperature of a final portion to about 25°C. Measure sample resistance or conductivity

and note temperature to ±0.1°C.

5. Calculation

The temperature coefficient of most waters is only approximately the same as that of

standard KCl solution; the more the temperature of measurement deviates from 25.0°C, the

greater the uncertainty in applying the temperature correction. Report temperature-compensated

conductivities as ‘‘μmho/cm 25.0°C.’’

- When sample resistance is measured, conductivity at 25°C is:

- where:

- k = conductivity, μmhos/cm,

C = cell constant, cm−1,

Rm = measured resistance of sample, ohms, and

t = temperature of measurement.

b. When sample conductivity is measured without internal temperature compensation

- conductivity at 25°C is:

where:

km = measured conductivity in units of μmho/cm at t°C, and other units are defined as

above.

For instruments with automatic temperature compensation and readout directly in μmho/cm

or similar units, the readout automatically is corrected to 25.0°C. Report displayed conductivity

in designated units.

6. Precision and Bias

The precision of commercial conductivity meters is commonly between 0.1 and 1.0%.

Reproducibility of 1 to 2% is expected after an instrument has been calibrated with such data as

- is shown in Table 2510:I.

2520 SALINITY*#(36)

2520 A. Introduction

1. General Discussion

Salinity is an important unitless property of industrial and natural waters. It was originally

conceived as a measure of the mass of dissolved salts in a given mass of solution. The

experimental determination of the salt content by drying and weighing presents some

difficulties due to the loss of some components. The only reliable way to determine the true or

absolute salinity of a natural water is to make a complete chemical analysis. However, this

method is time-consuming and cannot yield the precision necessary for accurate work. Thus, to

determine salinity, one normally uses indirect methods involving the measurement of a physical

property such as conductivity, density, sound speed, or refractive index. From an empirical

relationship of salinity and the physical property detemined for a standard solution it is possible to calculate salinity. The resultant salinity is no more accurate than the empirical relationship.

The precision of the measurement of a physical property will determine the precision in salinity.

Following are the precisions of various physical measurements and the resultant salinity

- presently attainable with commercial instruments:

Property             Precision of Measurement               Precision of Salinity

Conductivity         ±0.0002                                   ±0.0002

Density                 ±3 × 10–6 g/cm3                           ±0.004

Sound speed        ±0.02 m/s                                 ±0.01

Although conductivity has the greatest precision, it responds only to ionic solutes. Density,

although less precise, responds to all dissolved solutes.

2. Selection of Method

In the past, the salinity of seawater was determined by hydrometric and argentometric

methods, both of which were included in previous editions of Standard Methods. In recent years

the conductivity (2520B) and density (2520C) methods have been used because of their high

sensitivity and precision. These two methods are recommended for precise field and laboratory

work.

3. Quality Assurance

Calibrate salinometer or densimeter against standards of KCl or standard seawater. Expected

precision is better than ±0.01 salinity units with careful analysis and use of bracketing standards.

2520 B. Electrical Conductivity Method

1. Determination

See Conductivity, Section 2510. Because of its high sensitivity and ease of measurement,

the conductivity method is most commonly used to determine salinity.1 For seawater

measurements use the Practical Salinity Scale 1978.2-5 This scale was developed relative to a

KCl solution. A seawater with a conductivity, C, at 15°C equal to that of a KCl solution

containing a mass of 32.4356 g in a mass of 1 kg of solution is defined as having a practical

salinity of 35. This value was determined as an average of three independent laboratory studies.

The salinity dependence of the conductivity ratio, Rt, as a function of temperature (t °C,

International Practical Temperature Scale 1968) of a given sample to a standard S= 35 seawater

is used to determine the salinity

S = a0 + a1Rt

1/2 + a2Rt +a3Rt

3/2 + a4Rt

2 + a5Rt

5/2 + S

where S is given by

and:

a0 = 0.0080    b0 = 0.0005

a1 = −0.1692   b1 = −0.0056

a2 = 25.3851   b2 = −0.0066

a3 = 14.0941   b3 = −0.0375

a4 = −7.0261   b4 = 0.0636

a5 = 2.7081    b5 = −0.0144

valid from S = 2 to 42, where:

To measure the conductivity, use a conductivity bridge calibrated with standard

seawater*#(37) with a known conductivity relative to KCl, following manufacturer’s

instructions and the procedures noted in Section 2510. If the measurements are to be made in

estuarine waters, make secondary calibrations of weight-diluted seawater of known conductivity

to ensure that the bridge is measuring true conductivities.

The Practical Salinity Scale recently has been extended to low salinities6 to give an equation

that is valid from 0 to 40 salinity. The equation is:

2520 C. Density Method

1. Determination

With a precise vibrating flow densimeter, it is possible to make rapid measurements of the

density of natural waters. The measurements are made bypassing the sample through a vibrating

tube encased in a constant-temperature jacket. The solution density (ρ) is proportional to the

square of the period of the vibration (τ).

ρ = A +Bτ2

where A and B are terms determined by calibration, B being determined by calibration with a

densimeter with standard seawater. The difference between the density of the sample and that of

pure water is given by:

ρ − ρ0 = B(τ2 − τ0

2)

where τ and τ0 are, respectively, the periods of the sample and water. The system is calibrated

with two solutions of known density. Follow manufacturer’s recommendations for calibration.

These two solutions can be nitrogen gas and water or standard seawater and water. The salinity

of the sample can be determined from the 1 atm international equation of state for seawater. This

equation relates (ρ −ρ0) to the practical salinity (S) as a function of temperature.1

ρ (kg/m3) = ρ0+ AS +BS3/2 + CS2

where:

A = 8.244 93 × 10–1 − 4.0899 × 10–3t

+ 7.6438 × 10–5t2 − 8.2467 ×10–7t3 + 5.3875 ×10–9t4,

B = −5.724 66 × 10–3 + 1.0227 × 10–4t − 1.6546 × 10–6t2,

C = 4.8314 × 10–4,

and the density of water is given by:

ρ0 = 999.842 594 + 6.793 952 × 10–2t − 9.095 290× 10–3 t2

+ 1.001 685 × 10–4t3 − 1.120 083 ×10–6t4 + 6.536 332

× 10–9t5

Perform simple iteration by adjusting S until it gives the measured ρ − ρ0 at a given temperature.

If the measurements are made at 25°C, the salinity can be determined from the following

equation:

S = 1.3343 (ρ − ρ0) + 2.155 306 ×10–4 (ρ − ρ0)2 −1.171 16 × 10–5 (ρ −ρ0)3

which has a τ = 0.0012 in S. Approximate salinities also can be determined from densities or

specific gravities obtained with a hydrometer at a given temperature (Section 210B, 16th

edition).

2520 D. Algorithm of Practical Salinity

Because all practical salinity measurements are carried out in reference to the conductivity

of standard seawater (corrected to S = 35), it is the quantity Rt that will be available for salinity

calculations. Rt normally is obtained directly by laboratory salinometers, but in situ

measurements usually produce the quantity R, the ratio of the in situ conductivity to the standard

conductivity at S = 35, t = 15°C, p = 0 (where p is the pressure above one standard atmosphere

and the temperature is on the 1968 International Temperature Scale). R is factored into three

parts, i.e.,

R =RprtRt

where:

Rp = ratio of in situ conductivity to conductivity of the same sample

at the same temperature, but at p = 0 and rt = ratio of

conductivity of reference seawater, having a practical salinity

of 35, at temperature t, to its conductivity at t = 15°C. From

Rp and rt calculate Rt using the in situ results, i.e.,

Rp and rt can be expressed as functions of the numerical values of the in situ parameters, R, t,

and p, when t is expressed in °C and p in bars (105 Pa), as follows:

where:

e1 = 2.070 ×10–4, d1 = 3.426 ×10–2,

e2 = −6.370 ×10–8, d2 = 4.464 ×10–4,

e3 = 3.989 ×10–12, d3 = 4.215 ×10–1,

and d4 = −3.107 ×10–3,

and

rt =c0 + c1t +c2t2 + c3t3 +c4t4

where:

c0 = 0.676 609 7,

c1 = 2.005 64 × 10–2,

c2 = 1.104 259 ×10–4,

c3 = −6.9698 × 10–7,and

c4 = 1.0031 × 10–9.