- A point P has a spherical coordinate of r = 6378788.489 m, spherical latitude of 9.039544, and longitude of 38.761301. Calculate the rectangular coordinate.

- A point A on sphere has the following rectangular coordinate X = 4961166.873 m, Y = 3898630.315 m, and Z = 936187.758 m. Calculate the spherical coordinates.

- Convert the following rectangular coordinate in Adindan Datum to Geodetic coordinate: X = 4962536.72 m, 3898588.309 m, and Z = 930033.327 m.

A) Using Closed Formula

B) Using Iteration (Use MATLAB)

- The Grand Ethiopian Renaissance Dam is located at  = 35.092716, geodetic latitude of  = 11.213525, h = 996 m in WGS 84 datum. Calculate the rectangular coordinate of the point.

- A point P in front of 5 Kilo Campus Library has geodetic coordinate of  = 38.763150,  = 9.040810, h = 2477 m. Compute a small change in geodetic and rectangular coordinate for this point. Assume the change in rectangular coordinate is 1 mm and for geodetic coordinate 1 sec (for angles, lat and long) and 1 mm (for ellipsoidal height). (Use MATLAB)

- Use a point on Question No 5 and determine the reduced and geocentric latitude.

- For the following rectangular coordinate, compute the topocentric coordinate of Point A (N, E, U) with geodetic coordinate  = 57.395483300,  = 11.925395149, h = 43.372. (Use MATLAB)

Xp=3370641.970

Yp=711866.128

Zp=5349796.160

X=3098889.388

Y=1011032.696

Z=5463980.133

- A geodetic triangle formed by point A, B and C is as shown in figure below. Compute the spherical excess.

| Point | Lat | Long | Remark |
| --- | --- | --- | --- |
| A | 9.821519° | 49.779333° | WGS 84 |
| B | 1.776369° | 35.642067° | WGS 84 |
| C | 15.345004° | 34.341945° | WGS 84 |

- A point A along a geodetic line has a coordinate of  = 3930’ N,  = 1510’ E, and A = 3536’. A Point B on the same geodetic line has  = 4535’ N.  Compute the geodetic azimuth at point B. Compute also the minimum geodetic azimuth.

- Point A has spherical coordinate of   = 9.821519°,  = 49.779333° with geodetic azimuth 80  Compute the spherical coordinate of  point B if the distance between A and B is 1867 km.

- Point A and C has spherical coordinate of   = 9.821519°,  = 49.779333° and  = 15.345004° = 34.341945° respectively. Compute geodetic azimuth from point A to C, from C to A and distance between A and C.