Problem 3 – Map Projection

- Find the X, Y and k on the sphere for a point with  = 9 N and  = 40 E in Mercator projection. The central meridian o = 39 E and the Radius of the sphere, R = 6371 km.

- Find ,  on the sphere for the point with Mercator projection coordinates X = 4268.600 km and Y = 12600.600 km. The central meridian o = 39 E and the radius of the sphere, R = 6371 km.

- Find the X, Y and k on the Clarke 1880 ellipsoid for a point with  = 9 N and  = 40 E. The central meridian o = 39 E W. Use Mercator projection.

- Find the X, Y and k on the GRS 80 ellipsoid for a point with  = 9 N and  = 40 E. The central meridian o = 39 E. Use Mercator projection.

- Find ,  on Clarke 1880 ellipsoid for the point with Mercator projection coordinates X = 4140150 m and Y = 11700000. The central meridian o = 39 E.

- Find the X, Y and k on the Clarke 1880 ellipsoid for a point with  = 9 N and  = 40 E in Transverse Mercator projection. The central meridian o = 39 E.

- Find ,  on GRS 80 ellipsoid for the point with Transverse Mercator projection coordinates X = 6110700.158 m and Y = 98762.683 m. The central meridian o = 39 E.

- Solve Question No 6 for UTM projection.

Exercises

- For the following data

Day: March 1, 1981 Mid night (UT1 = 0)

Equation of the Equinox = -0.8 sec

A point P has coordinates: Height angle (h) = 8025’34’’, Declination () = 511’14’’, Azimuth (A) = 938’11’’

- Compute Julian Date and Modified Julian Date.

- GAST for Time = 22 hr 50 min 42 sec

- Astronomical longitude and latitude for point P

- A point P in Addis has geodetic coordinates of  = 9,  = 40, h = 2150 m in Clarke 1880 ellipsoid. If the geodetic parameter of Clarke 1880 ellipsoid changed by xo = 1m, yo = 1m, zo = 1m, a= 1m, f= 0.006m, calculate the change in geodetic coordinates  , , h.

- The following data is collected for precise leveling

| Station | BS | FS | Gravity | Remark |
| --- | --- | --- | --- | --- |
| 1 | 0.928 | 1.080 | 9.80132316 | Height = 2100 m |
| 2 | 1.212 | 0.765 | 9.8013227 |  |
| 3 | 1.668 | 0.775 | 9.8013216 |  |
| 4 | 2.969 | 0.311 | 9.80132019 |  |
| 5 | 2.938 | 0.367 | 9.80131975 |  |
| 6 | 3.437 | 2.658 | 9.80131931 |  |
| 7 | 2.658 | 0.208 | 9.80131861 |  |
| 8 | 2.500 | 0.728 | 9.80131800 |  |
| 9 | 2.729 | 0.339 | 9.80131714 |  |
| 10 | 2.075 | 0.349 | 9.80131628 |  |
| 11 | 3.825 | 0.375 | 9.80131579 |  |
| 12 | 1.760 | 0.645 | 9.80131539 |  |
| 13 | 3.165 | 0.490 | 9.80131487 |  |
| 14 | 2.055 | 0.950 | 9.80131434 |  |
| 15 | 3.330 | 0.455 | 9.80131291 |  |
| 16 | 1.830 | 1.418 | 9.80131116 |  |

Calculate the elevation of Station 16 using geopotential number and check the height with conventional method.

- Let the X, Y, and Z coordinates of point P with respect to the instantaneous pole are:

| X | 4827804 |
| --- | --- |
| Y | 4051009 |
| Z | 991498.6 |

If the pole has rotation angle of 5’’ and 8’’ with respect to CIO,

- Calculate the X, Y and Z coordinates with respect to CIO

- Calculate the change in geodetic coordinates if the point has geodetic latitude and longitude of 9 and 40 respectively.

Project

- Determine the appropriate two-dimensional coordinate transformation for the following data at a 0.01 level of significance when transforming the XY coordinates into the EN coordinate system.

- Using a weighted three-dimensional coordinate transformation and the follow set of data:

(a) Determine the transformation parameters and their standard deviations.

(b) Compute the XYZ coordinates and their standard deviations for points 7 to 10.