Water and Wastewater Treatment

Problems on Filtration, Disinfection, Softening and Adsorption

- A rapid filter plant has units with a design capacity of 8000 m3/d at a rate of 120 m/d. What should be the capacity of the elevated storage tank if it is to hold sufficient amount of water for two consecutive 5-min washes at a rate of 0.8 m/min?

- A groundwater supply has the following analysis:

Ca2+ = 3.7 meq/L		HCO3- = 4.0 meq/L

Mg2+ = 1.0 meq/L		SO42- = 1.2 meq/L

Na+ = 1.0 meq/L		Cl- = 1.0 meq/L

K+ = 0.5 meq/L		CO2 = 0.40 meq/L

Calculate the chemical dose needed and the residue produced for excess lime softening. (Assume excess lime = 1.25 meq/L; lower limits for removal of calcium and magnesium, respectively are 0.6 meq/L and 0.2 meq/L)

- Results of a chlorine demand test on a raw water are as follows:

| Sample No. | Chlorine dosage (mg/L) | Residual chlorine after 10-min contact (mg/L) |
| --- | --- | --- |
| 1 2 3 4 5 6 7 8 | 0.20 0.40 0.60 0.80 1.00 1.20 1.40 1.60 | 0.19 0.36 0.50 0.48 0.20 0.40 0.60 0.80 |

Sketch the chlorine demand curve. What is the chlorine dose required to attain a combined residual of 0.5 mg/L and a free residual of 0.5 mg/L? What is the combined residual present when a free residual of 0.5 mg/L exists?

- Water with a flow rate of 1000 l/min is to be treated with powdered activated carbon (PAC) to reduce residual organics from an initial concentration of 5 mg/L to equilibrium concentration of 1 mg/L. The Freundlich adsorption isotherm was found to be a good model to describe the process with a best fit line equation of lnq = 5 + 0.5ln[C].

  - Determine the required mass of PAC

  - Calculate the annual cost of PAC if its unit price is $0.5 per kg