For cotm   3rd year     2nd semester regular and extension students.

Construction equipment and plant management course supportive examples and answers

Prepared by Inst. Meseret  Hu.(Msc)

Equipment production estimation example

- Estimate the production in loose cubic yards per hour for a medium-weight clamshell excavating loose earth. Heaped bucket capacity is1cuyd (0.75m 3 ).The soil is common earth with a bucket fill factor of 0.95. Estimated cycle time is 40 s. Job efficiency is estimated at 50 min/h.

Solution

Production (LCY/h )  =3600 * 1*0.95*50/60=71 LCY/h

40

Production (LCm/h)  =3600 * 0.75*0.95*50/60=71 LCm/h

40

- 8 km of gravel road requires reshaping and leveling. You estimate that 5 passes of a motor grader will be required. Based on operator skill, machine characteristics, and job conditions, you estimate 1st and 2nd passes at 2.06, 3rd 4th passes at 4.0 km/hr, and 5th pass at 4.82 km/hr. If job efficiency is 0.80, how many grader hours will be required for this job?

Solution

T1,2=    2*8         =9.71hr

2.06*0.8

T3,4=    2*8       =5 hr

4.0*0.8

T5=    1*8       =2.1 hr

4.82*0.8

∑=16.81hr

- Estimate the production of smooth steel wheel roller which is compacting a road under the following conditions: average speed of the roller is 7.6 km/hr, compacted lift 25 cm, effective roller width is 2.2m, number of pass required is 8 and job efficiency is 50 min/hr.

Solution

Production =   10*W*S*L*E

P

= 10*2.2*7.6*25*0.833

8

=435.4 cm3/hr

Equipment cost calculation example

Note

COST OF OWNING AND OPERATING CONSTRUCTION EQUIPMENT

- Factors affecting the cost include:

- Cost of equipment delivered to the owner.

- The severity of the conditions under which it is used.

- The care with which it is maintained and repaired.

- The number of hours used.

- The demand for used equipment (salvage value = SV)

2. Estimate Cots: includes

- - Depreciation Costs.

- – Maintenance & Repair.

- – Investment cost.

- – Fuel consumption.

- – Lubricating Oil.

a- Depreciation Costs:

Depreciation is the loss in value of equipment resulting from use or age (useful life). Assume a unit of equipment will decrease in value from its original total cost at a uniform rate. There are 3 three methods for influential the cost of depreciation.

- Straight Line Method.

- Declining – Balance Method.

- Sum of the Years Digit Method.

b– Maintenance & Repair.

 Vary with the type , the service and the care.

 Expressed as a % of annual cost of depreciation

 Experience records serve as a guide in estimating these costs.

c– Investment cost.

Includes interest on the money invested, taxes of all types , insurance and storage as a % of ~ 15-25%.

d– Fuel consumption.

Two types of engines used in construction equipments: Gasoline and Diesel engines

Diesel engine consumption     =0.04 gal / hp. hr.

Lubricant oil

q=     hp*f*0.006lb/hp.hr     +    c

7.4 lb/gal                            t

    - q = quantity assumed, gal / hr.

    - hp. = rated horse power for engine.

    - c. = capacity of crankcase, gal

    - f. =operating factor

    - t. = # no. of hours between changes.

    - The above formula based on :

    - An operating factor of 60 % Quantity of oil consumed per rated horsepower hour, between changes, will be 0.006 lb.

- Calculate the ownership cost per hour for an excavator powered by a 250-hp engine based on the following data:

- Purchase price (P) = 420,000 Br

- Salvage value (F) = 250,000 Br

- Operation factor = 50%

- Useful life (N)   = 6 years

- Working hours per year = 2000

- Maintenance and repair costs = 110% of annual depreciation

- Diesel fuel price   = 3.8/gallon

- Fuel consumption   = 0.04 gallon/hp/hr

- Lube oil cost   = 10% of fuel

- Interest rate (i)   = 10%

Solution

Depreciation (assume straight-line) = (420000 – 250000) / 6 = 28333.33 Br /year

Investment annual cost is calculated as follows:

Annual investment=4200000.11.161.16-1 -420000/6 -2500000.11.16-1-250000/6

Annual investment = (420000 × 0.2296 – 70000) – (250000 × 0.1296 – 41666.67)    =

26432 – (- 9264.82) = 35696.82 Br /year

Maintenance and repair cost = 1.1 × 28333.33 = Br 31166.67/year Then, the total yearly

Costs = 28333.33 + 35696.82 + 31166.67 = 95196.81 Br/year

Accordingly, the hourly cost = 95196.81 / 2000 = Br 47.6/hr

Fuel consumption = 250 × 0.04 × 0.5 = 5 gallon/hr Fuel cost = 5 × 3.8 = Br 19/hr

Lubricate oil cost = 19 × 0.1 = Br 1.9/hr

- Calculate the hourly rate of equipment based on the following data:

- Purchase price (P)  = LE460,000

- Salvage value (F)  = LE40,000

- Useful life (N)   = 10 years

- Working hours per year = 2000 years

- Annual maintenance costs = 10% of purchase price

- Annual operating costs = LE47,000

- Interest rate (i)   = 15%

Solution

Depreciation (assume straight-line) = (460000 – 40000) / 10 = LE42000/year

Investment annual cost is calculated as follows:

Annual investment=4600000.151.15101.1510-1 -460000/10 -400000.151.1510-1-40000/10

Annual investment = 47684 Br/year

Maintenance and repair cost = 0.1 × 460000 = Br 46000/year

Operating costs =47000 Br/year

Then, the total annual costs = 42000 + 47684 + 46000 + 47000 = 182684 Br/year

Accordingly, the hourly cost = 182684/ 2000 = 91.34 Br/hr

- Determine the probable cost per hour for owning and operating a 25 cu-yd  heaped capacity bottom dumb wagon with six rubber tires, the following information will apply:

    - Engine 250 hp, Diesel.

    - Crankcase capacity 14 gallon

    - Time between oil changes 80 hr

    - Operating factor 60 %

    - Useful life 5 years – 2000 hr/yr – with no salvage value

    - Life of tires 5,000 hr

    - Repair of tires 15% of tire depreciation

    - Cost delivered including freight and taxes = $ 92,623

    - Cost of tires =$ 12,113

    - M & R = 50% OF Depreciation

    - Investment rate = 15%

Solution

1- fuel  consumed per hr = 250 * 0.6 * 0.04 = 6.0 gal

2- Lubricating oil consumed per hr:

q=     250*0.6 *0.006lb/hp.hr     +    14

7.4 lb/gal                                   80

=0.3gal/hr

3. Cost of owner

Cost delivered including freight and taxes = $ 92,623

Less cost of tires                                        = ($ 12,113)

Net cost less tires                                        = $ 80,510

Average annual investment cost (AAI) = P(n+1)/2n = 92,623 (5+1)/2*5 = $ 55,744

No salvage value case

4-Annual cost:

Depreciation = (80,510 – SV) / 5                                                                                     = $ 16,102

Maintenance and Repair = 50% * 15,102                                                                      = $ 8,051

Investment = 15% * (AAI).                                                                          = $ 8,362                        Total annual fixed costs                                                                                                = $ 32,515

5-Hourly cost:

Fixed cost = 32,515 / 2000 hr                                                                               = $ 16.26        Tire depreciation =12,113 / 5,000                                           = $ 2.42                                                                  Tire repairs = 0.15 * 2.42                                                                                                  = $ 0.36

Fuel = $4 * 6 gal                                                                                                  = $ 24.00 Lubricating oil = 0.3 * $15                                                     = $ 4.50                                                             Total cost per hour... Excluding labor                                                                            = $ 47.54

Say the labor takes $10/ hr

Then the total cost = $57.54

You must add profit margin By about 10 to 15 %